0660102020575

1

2#

9[.............................................................]

FOREWORD

TM

GEM  DOS    is  a  single-user operating system designed for the Motorola..

TM

MC68000   or  compatible  68000  microprocessor.    GEM  DOS  requires  the

following minimum system configuration:

*   68000 microprocessor

*   Bit-mapped  video  controller  card  and  a  display  with  320  by 200

resolution

*   128K bytes of Random Access Memory (RAM)

*   160K bytes of Read-Only Memory (ROM)

*   An optical or track mouse or other pointing device

The following configuration represents a system that can exercise the  full

capabilities of GEM DOS:

*   68010, 68020, or 68070 advanced microprocessor

*   High-resolution,  color  bit-mapped  display and controller (640 by 400

pixel resolution, barrel-shifter hardware for very  high  speed  screen

refresh)

*   256K bytes of RAM

*   192K bytes of ROM

*   Mouse

*   Disk drives and controllers

*   Optical drives and controllers

*   Printer and communication ports

This  guide describes the programming interface to GEM DOS and is part of a

documentation set that also includes these manuals:

*   The GEM DOS User's Guide

*   The C Language Programming Guide for  GEM  DOS  (supplement  to  the  C

Language Programming Guide for CP/M-68K..)

*   The Mince User's Guide, prepared by Mark of the Unicorn, Inc.

*   The  Kermit  User's  Guide  prepared  by Columbia University Center for

Computing Activities

*   The GEM DOS Supplement to the Kermit User's Guide

This guide assumes you are an  experienced  programmer  familiar  with  the

Motorola  68000  assembly  language and the C programming language.  If you

are not  familiar  with  GEM  DOS,  refer  to  the  manuals  listed  above,

especially  the  GEM  DOS  User's  Guide.  If you are not familiar with the

Motorola 68000 assembly language, refer to the following Motorola manuals:

*   16-Bit Microprocessor User's Manual, third edition MC68000UM(AD3)

*   MC68000 Resident Structured Assembler Reference Manual M68KMASM(D4)

The following is a summary of the contents of this guide:

*   Section  1  discusses   the   GEM   DOS   system   architecture,   file

specification, and terminology.

*   Section  2  describes  the  programming  loading  procedure,  transient

program environment, and how to exit a transient program.

*   Section 3 describes the GEM DOS comand file (transient program) format.

*   Section 4 lists the system function call categories and  describes  the

system functions and their entry parameters and return codes.

*   Section 5 describes the GEM DOS assembler.

*   Section 6 describes the GEM DOS linkers.

*   Section 7 describes the GEM DOS archive utility.

*   Section 8 describes the DUMP, SIZE68, and SENDC68 utilities.

TM

*   Section 9 describes the SID-68K   debugger.

*   Appendix A lists the BIOS functions.

*   Appendix B shows the format of the GEM DOS base page.

*   Appendix C contains a decimal-ASCII-hexadecimal conversion table.

*   Appendix D lists the error messages returned by the GEM DOS programming

utilities.

*   Appendix E describes the Motorola S-Record format.

INTRODUCTION TO GEM DOS

1.1  SYSTEM ARCHITECTURE

Table 1-1 describes the three modules that comprise GEM DOS.

Table 1-1.  GEM DOS System Modules

Module

Console Command Processor

(CCP)

Basic Disk Operating System

(BDOS)

Basic I/O System

(BIOS)

1.2  TRANSIENT PROGRAMS

The Transient Program Area (TPA) consists  of  all  the  contiguous  memory

space  not  occupied  by  the  operating system when GEM DOS is resident in

memory.  GEM DOS loads executable command files from  disk  into  the  TPA.

Command  files  are  also  called  transient  programs because they are not

memory resident nor configured within the  operating  system.    Section  3

describes the format of a GEM DOS command file.

Section 2, "The CCP and Transient Programs," describes how

Section  2, "The CCP and Transient Programs," describes how GEM DOS formats

the TPA for programs it loads into memory for execution.

1.3  PROGRAMMING TOOLS

Table 1-2 lists the assembly language programming tools provided  with  GEM

DOS.  These tools are described in Sections 5 through 9 of this guide.

Table 1-2.  GEM DOS Programming Tools

Program   Purpose

AR68      Creates and modifies object module libraries

AS68      Processes   assembly  language  source  modules  to  create

relocatable object module library files

DUMP      Displays files in hexadecimal and ASCII notation

TM

LINK68    A linker that combines assembled or compiled object modules

with other modules from a runtime library

LO68      Another linker

NM68      Prints the symbol table of an object or command file

RELOC     Converts  relocatable  object  modules  from  one format to

another

SENDC68   Converts a command file to the Motorola S-record format

TM

SID-68K   Tests and debugs programs

SIZE68    Displays the size of each program  segment  within  one  or

more command files

XREF      Creates  a  table  of the symbols defined and referenced by

object files

Use AS68 to assemble the file and either LINK68 or LO68 to link it.    AR68

creates  library  modules  that can be linked with LINK68 or LO68.  Both of

the linkers use and create the same file formats.  However, if you are  not

linking with runtime modules, LO68 provides better performance.

NM68 is described in Section 3.

GEM  DOS  also supports a C language compiler for the 68000 microprocessor.

See the C Language Programming Guide for GEM DOS.

1.4  GEM DOS FILE SPECIFICATION

The GEM DOS file specification consists of four fields:    a  one-character

drive  designation  delimited  by  a  colon (d:), a directory path in which

directory names are delimited by backslashes (\), a one- to eight-character

filename,  and  an optional one- to three-character filetype separated from

the filename by a period (.).  Directories and files have the  same  naming

conventions.  The format of a file specification is shown below.

Format:

d:\path\filename.typ

Example:

A:\LIBRARY\OBJS\FLOATER.OBJ

The  drive designation, path, and filetype fields are optional.  The colon,

backslash, and period delimiters are required only when you  include  these

fields in the file specification.

Drive   designation   values   range   from  A  through  P  when  the  BIOS

implementation supports 16 drives, the maximum  number  that  GEM  DOS  can

support.  See Section 4.5, "File Management Functions."

The   characters  in  the  filename  and  filetype  fields  cannot  contain

delimiters (the colon, backslash,  and  period).    The  CCP  automatically

converts  a  command line and its file specifications (if any) to uppercase

internally before parsing them.

Table 1-3 lists some additional characters you should avoid using  in  your

file specifications.  These characters are reserved because several GEM DOS

built-in commands and utilities have special uses for them.

Table 1-3.  Delimiter Characters

Character

[]

()

<>

=

*

&

,

!

|

?

/

\

$

.

:

;

+

-

1.5  WILDCARDS

GEM DOS supports two wildcards, the question mark (?) and the  asterisk(*).

Several  utilities  and  BDOS functions allow you to specify wildcards in a

file specification to perform the operation or  function  on  one  or  more

files.

The ? wildcard matches any one character in the character position occupied

by  this  wildcard.    For  example,  the  file  specification  WH?TMAN.OBJ

indicates  that  the  third  letter of the filename can be any alphanumeric

character if the remainder of the file specification matches.  Thus, the  ?

wildcard matches exactly one character position.

The  * wildcard matches one or more characters in the field or remainder of

a field that this wildcard occupies.  GEM DOS internally pads the field  or

remaining portion of the field occupied by the * wildcard with ?  wildcards

before searching for a match.  For  example,  GEM  DOS  converts  the  file

specification  G*.BAT  to G???????.BAT before searching for a matching file

specification.  Thus, any file that starts with the  letter  G  and  has  a

filetype of BAT matches this file specification.

For  details  on  wildcard support by specific BDOS functions, refer to the

description of the function in Section 4 of this  guide.    For  additional

details  on  these wildcards and support by GEM DOS utilities, refer to the

GEM DOS User's Guide and Sections 5 through 9 of this guide.

1.6  GEM DOS TERMINOLOGY

Table 1-4 list the terminology used throughout this guide to  describe  GEM

DOS values and program components.

Table 1-4.  GEM DOS Terminology

Term      Meaning

Nibble    4-bit value

Byte      8-bit value

Word      16-bit value

Longword  Signed 32-bit value

Address   32-bit value that specifies a storage location

Offset    A  fixed  displacement  that references a storage location,

other data source, or destination

Text Segment The   section  of  a  program  that  contains  the  program

instructions

Data Segment The section of a program that contains initialized data

bss       The  block  storage  segment is a section of a program that

contains uninitialized data

Table 1-5 describes conventions used in this guide.

Table 1-5.  GEM DOS Programmer's Guide Conventions

Convention

Meaning

[]        Square   brackets   in  a  command  line  enclose  optional

parameters.

nH        The capital  letter  H  follows  numeric  values  that  are

represented in hexadecimal notation.

(n)       BDOS function numbers are enclosed in parentheses when they

appear in text.

Ctrl-X    The mnemonic Ctrl-X indicates that you are to hold down the

key labeled Ctrl while you press the key represented by the

variable X.

End of Section 1

THE CCP AND TRANSIENT PROGRAMS

This section discusses the Console Command Processor  (CCP),  built-in  and

transient commands, loading and exiting transient programs, and the GEM DOS

program execution model.

2.1  CCP BUILT-IN AND EXTERNAL COMMANDS

After an initial cold start, GEM DOS displays  a  sign-on  message  at  the

console.   The drive containing the system disk is logged in automatically.

GEM DOS displays its "{a}" prompt  to  indicate  the  currently  logged  in

drive,  and  that  the  operating system is ready to receive a command line

from the console.

In response to the prompt, a user types the filename of a batch or  command

file  with  its  command  tail, if required.  GEM DOS supports two types of

commands, built-in and transient.  Built-in commands are part of  the  CCP.

Transient  commands  are  loaded  in  the  TPA  and do not reside in memory

allocated to GEM DOS.  The GEM DOS User's Guide describes the built-in  and

transient commands that GEM DOS supports.

A  transient  command is a machine-readable program file that GEM DOS loads

from disk into memory for execution.  Section 3  describes  the  format  of

transient command files.

When the user enters a command line, the CCP parses it and tries to execute

the file specified.  The CCP asssumes a file is a command file when it  has

the  filetype  BAT.    When  the  user specifies only the filename, the CCP

searches the current directory  and  drive  for  a  file  with  a  matching

filename and a filetype of BAT.  If the CCP does not locate a batch file to

match the filename specified, it searches for a command file whose filetype

is PRG.

2.2  LOADING A PROGRAM

Either  the  CCP or a transient program can load a program into memory with

the BDOS Load and Execute a Process Function (4B) described in Section 4.7.

After  the  program is loaded, the TPA contains the program segments (text,

data, and bss), a user stack, and a base page.  A base page exists for each

program  loaded  into  memory.   The base page is a 256-byte data structure

that  defines  a  program's  operating  environment.    The  base  page  is

initialized  by  the  Load and Execute a Process (4B) function described in

Section 4.7.  GEM DOS allocates 256 bytes (minimum) for the program's  user

stack.

2.2.1  The TPA and Base Page Initialization

The format of the Transient Program Area (TPA) that GEM DOS establishes for

an application program is shown in Figure 2-1.

__________ <-------- High TPA

|          |<---- Initial Stack Pointer

|  Appli-  |

|  cation  |

|   User   |

|   Area   |

|          |

------------

|    BSS   |

------------

|   Data   |

------------

|   Text   |

------------

| Base Page|

------------<-------- Low TPA

Figure 2-1.  GEM DOS TPA Format

The initial stack pointer is directed at the top end of the TPA.  The  base

page  contains  pointers  as  described  in Section 2.3.  To insure maximum

compatibility with future releases of  GEM  DOS,  it  is  recommended  that

memory  not  required  by your program be freed for use by other processes.

The top of the stack should be moved down to an acceptable  value  adjacent

to  or  within  the bss, and an M_SHRINK (4A) function should be performed.

If additional memory is needed later, perform an  M_FREE  (49)  or  M_ALLOC

(48) function to determine the amount of available memory (specify nbytes =

-1 for the M_ALLOC call); follow this by another call to M_ALLOC to have  a

portion  of  that  memory  allocated.  These operations reformat the TPA as

shown in Figure 2-2.

---------- <---------- High TPA

| Available|

| to other |

| Processes|

------------<----------- Stack Pointer

|  Appli-  |

|  cation  |

|  User    |

|  Area    |

____________

|   BSS    |

------------

|   DATA   |

------------

|   TEXT   |

------------

| BASE PAGE|

------------<----------- Low TPA

Figure 2-2.  Adjusted TPA

2.3  BASE PAGE FORMAT

Table 2-1 lists the byte offset, length, value, and briefly  describes  the

fields  in  the  base page that Load and Execute a Process (4B) initializes

for transient programs.

Table 2-1.  Base Page Format

Base Page

Offset    Length  Value        Description

00        4     p_lowtpa     Base address of the TPA

04        4     p_hitpa      High address of the TPA + 1

08        4     p_tbase      Base address of text (code)

0C        4     p_tlen       Length (in bytes) of text (code)

10        4     p_dbase      Base address of initialized data

14        4     p_dlen       Length (in bytes) of data

18        4     p_bbase      Base address of BSS uninitialized

data

1C        4     p_blen       Length of BSS uninitialized data

2C        4     p_env        Pointer to the environment string

x80       x80    p_cmdlin     Command line image

Note:  All other base page fields are reserved.

2.4  EXITING TRANSIENT PROGRAMS

GEM DOS supports two ways of exiting  a  transient  program  and  returning

control to the CCP:

*   Interactively, when the user types Ctrl-C at the console

*   A programmed return to the CCP through:

--  Program Terminate (00)

--  Terminate and Stay Resident (31)

--  Terminate Process (4C)

Typing  Ctrl-C  from  the  console  returns  control to the CCP only if the

program uses any of the following functions:

*   Read Character from Standard Input Device (01)

*   Write Character to Standard Output Device (02)

*   Read Character from Standard Input, No Echo (08)

*   Write String to Standard Output Device (09)

*   Read Edited String from Standard Input Device (0A)

*   Check Status of Standard Output Device (10)

Ctrl-C terminates execution of the main program and any additional programs

loaded beyond the CCP level.

End of Section 2

GEM DOS COMMAND FILE FORMAT

The  GEM  DOS  command  file  format  is  the  output  of the RELMOD format

conversion utility.  RELMOD accepts a CP/M-68K.. format relocatable  output

file generated by LINK68 or LO68, the GEM DOS linkers, and converts it to a

GEM DOS relocatable file.   RELMOD,  LINK68,  and  LO68  are  described  in

Section 6.

Command  files  consist  of  a  file  header  followed by the text and data

segments.    Optionally,  a  command  file  contains  a  symbol  table  and

relocation  information.    These  components  are  shown in Figure 3-1 and

described in the following sections.

______________________

|         File         |

|        Header        |

|______________________|

|       Text and       |

|     Data Segments    |

|______________________|

|     Symbol Table     |

|     (if present)     |

|______________________|

|    Relocation Data   |

|     (if present)     |

|______________________|

Figure 3-1.  GEM DOS Command File Layout

3.1  THE HEADER AND PROGRAM SEGMENTS

The  figure  3-2  shows  the  format  of  the  header   for   non-segmented

(contiguous) executable files.

+--------+--------+

00H |      601AH      |

+--------+--------+--------+--------+

02H |  Number of bytes in text segment  |

+--------+--------+--------+--------+

06H |  Number of bytes in data segment  |

+--------+--------+--------+--------+

0AH |      Number of bytes in bss       |

+--------+--------+--------+--------+

0EH |  Number of bytes in symbol table  |

+--------+--------+--------+--------+

12H |        Reserved, must be 0        |

+--------+--------+--------+--------+

16H |        Reserved, must be 0        |

+--------+--------+--------+--------+

1AH |     Reserved    |

+--------+--------+

Figure 3-2.  GEM DOS Command File Header

The  14-word,  601AH-type  header  denotes a relocatable file that contains

contiguous text, data, and block storage segments.  Note that GEM DOS  does

not support programs with non-contiguous segments.

The fields at offsets 12H, 16H, and 1AH are reserved and must be zero.  The

text segment image follows the field at offset 1AH.

The text segment contains  the  executable  instruction  code.    The  data

segment  contains  the  initialized  data  variables.    The  text and data

segments must be relocatable.

The inclusion of the symbol table and relocation information in the command

file is controlled by the GEM DOS linkers, described in Section 6.

3.2  SYMBOL TABLE

The  symbol  table  defines the symbols referenced by a command file.  Each

entry in the symbol table is associated with an index  that  indicates  its

entry  number.    Entries are numbered sequentially starting with zero.  As

shown in Figure 3-3, each symbol table entry is  composed  of  seven  words

that describe the symbol's name, type, and value.

+--------+--------+

0 |                 |

+      Symbol     +

|                 |

+                 +

|       Name      |

+                 +

6 |                 |

+--------+--------+

8 |       Type      |

+--------+--------+

A |                 |

+      Value      +

C |                 |

+--------+--------+

Figure 3-3.  Symbol Table Entry

Table 3-1 describes the fields of a symbol table entry.

Table 3-1.  Symbol Table Entry Fields

Field     Definition

Name      Symbol   name,   null-padded   right  if  less  than  eight

characters.

Type      Symbol type as indicated by the following values:

0100H:  bss-based relocatable

0200H:  text-based relocatable

0400H:  data-based relocatable

0800H:  external reference

1000H:  equated register

2000H:  global

4000H:  equated

8000H:  defined

Value     Symbol value where the value can be  an  address,  register

number, value of an expression, and so forth. Note that the

linkers interpret the value when  the  symbol  is  external

(0800H) as the size of a common region.

3.3  PRINTING THE SYMBOL TABLE

You  can  print the symbol table of an object or command file with the NM68

utility.  Use the following command line to invoke NM68:

{a}NM filename

NM68 operates on object or command files.  NM68 does not sort the  symbols;

rather, it prints them in the order in which they appear in the file.  Each

symbol name is printed, followed by its  value  and  one  or  more  of  the

following type-descriptors:

*   equ (equated)

*   global

*   equreg (equated register)

*   external

*   data

*   text

*   bss

*   abs (absolute)

You  can  use  the I/O redirection facility to store NM68 output in a file.

To redirect NM68 output to a file, specify a greater-than-sign  (>)  and  a

file  specification  following  the filename for which NM68 is to print the

symbol table.

3.4  RELOCATION INFORMATION

The relocation information indicates the offset  between  longwords  to  be

relocated.   The offset from the beginning of the text segment to the first

longword that requires relocation is stored as  a  longword  following  the

symbol  table.  Following that, bytes are used to store succeeding offsets.

If the offset between two longwords to be relocated  is  greater  than  254

bytes,  the relocation information has a byte of 1 for each multiple of 254

bytes, plus a byte whose value indicates any  remainder.    A  zero  offset

value  indicates the end of relocation information.  Note that GEM DOS does

not support 16-bit relocatable objects.

See Figure 3-3.  Table 3-2 lists the  relocation  offset  byte  values  and

their meanings.

.                          .

.                          .

.__________________________.

|       Symbol Table       |

|__________________________|

|      Offset of 1st       |

|   Longword to Relocate   |

|     (Longword Value)     |

|__________________________|

|    Succeeding Offsets    |

|       (Byte Values)      |

|__________________________|

|          End of          |

|  Relocation Information  |

|        (Zero Byte)       |

|__________________________|

Figure 3-4.  Location and Format of Relocation Information

Table 3-2.  Relocation Offset Byte Table

Byte

Value

0

1

2 through 254

255

*

Plus 254 times the number of "1" bytes that preceded this byte.

All relocation information is relative to the beginning load address (start

of text).

End of Section 3

SYSTEM FUNCTION CALLS

This section describes the GEM  DOS  system  function  calls,  their  entry

parameters, return values, and error codes.

4.1  INTRODUCTION

The  GEM  DOS  system  calls  are  divided  into  the following categories:

Character Device I/O, File  Management,  Drive  and  Directory  Management,

Program Control, Memory Management, and Time.  Table 4-1 lists each GEM DOS

system call according to these categories.  Also listed in  Table  4-1  are

references  to  the  sections  of  this  chapter that describe each call in

detail.

Table 4-1.  GEM DOS System Call Categories

Number

Character Device I/O, Section 4.4

01

02

03

04

05

06

07

08

09

0A

0B

10

11

12

13

File Management, Section 4.5

1A

2F

3C

3D

3E

3F

Table 4-1.  (continued)

Number

40

41

42

43

44

45

46

4E

4F

56

57

Drive and Directory Management, Section 4.6

0E

19

36

39

3A

3B

47

Program Control, Section 4.7

00

25

30

31

35

4B

4C

Memory Management, Section 4.8

48

49

4A

Table 4-1.  (continued)

Number

Time, Section 4.9

2A

2B

2C

2D

4.2  FUNCTION CALL PARAMETERS

The GEM DOS parameter-passing convention supports function call  references

from C and other high-level languages.

A function call reference from a high-level language constructs a parameter

block that  contains  the  number  of  the  function  call,  any  necessary

parameters,  and  other relevant data (such as a drive code or file handle)

and performs a TRAP #1 instruction.  GEM DOS then processes  the  reference

and executes the appropriate actions.

The  general  format of a GEM DOS function call parameter block is shown in

Figure 4-1.

Low Memory

+--------------------------------+ (Parameter

|         Function Number        |  Block Address)

+--------------------------------+

|          1st Parameter         |

+--------------------------------+

|          2nd Parameter         |

+--------------------------------+

|          3rd Parameter         |

+--------------------------------+

.                                .

.                                .

.                                .

+--------------------------------+ High memory

Figure 4-1.  Function Call Parameter Block Format

The positions of the parameters and  the  function  number  in  Figure  4-1

conform  to  the  order  in  which  they  are  placed on the stack by the C

language.  Specifically, the C code:

CALL (functionnumber, P1, P2, P3, ... Pn)

loads the Pn parameter into the  parameter  block  first,  followed  by  P3

through  P1,  until  the function number is the last entry in the parameter

block.

4.2.1  GEM DOS File and Device Handles

F_CREATE  (3C),  F_OPEN  (3D),  and  F_DUP  (45)  return  a  16-bit  binary

identifier  value  in register D0.L.  This identifier is called a "handle."

The handle is used to later identify files or devices that have been opened

or  created  with  the system call that originally returned it.  Although a

handle is 16 bits long, your program should test the entire long word value

returned  in  D0.L  to determine if the value represents an error code or a

valid handle.

Table 4-2 lists the handles that identify devices.

Table 4-2.  GEM DOS Device Handles

Handle

Value        Device

4          Standard list device

Output can be redirected.

3          Standard AUX device

I/O can be redirected.

2          Standard error device (stdErr)

I/O can be redirected.

1          Standard output device (stdOut)

Output can redirected at

the command line.

0          Standard input device (stdIn)

Input can be redirected at

the command line.

Your program can use these handles without previously opening them.    Your

program can redirect I/O for these devices with F_FORCE (46).

Your  program  can  open  the following devices by specifying a device name

with the F_OPEN (3D) function:

*   The console device, CON:

*   The auxiliary device, AUX:

*   The list device, PRN:

*   The bit bucket, NUL:

Note that I/O for the devices opened by name cannot be redirected.  See the

descriptions of F_OPEN (3D) and F_FORCE (46) in Section 4.5.

4.2.2  Return Values

GEM  DOS function calls return all byte, word, and long data in register D0

by default.  Those function calls that return more  information  place  the

address of an information return buffer in D0 or fill in a caller-specified

buffer.  The individual function call descriptions explain how data returns

to the caller.

4.2.3  Error Return Codes

The  GEM  DOS  function  calls place a negative number in D0 to indicate an

error return.  See the  individual  function  call  descriptions  for  more

complete information.

Table  4-3  lists the error codes returned by the GEM DOS function calls in

register D0.L.  Also listed in Table 4-3 are the  numbers  of  the  PC  DOS

function call error codes that correspond to the GEM DOS error codes.

Table 4-3.  BDOS Error Codes

GEM DOS    PC DOS

Return    Equiv.  Code         Error Report

-32L       1     EINVFN       Invalid function

-33L       2     EFILNF       File not found

-34L       3     EPTHNF       Path not found

-35L       4     ENHNDL       No handles left (too many open f

-36L       5     EACCDN       Access denied

-37L       6     EIHNDL       Invalid handle

-39L       8     ENSMEM       Insufficient memory

-40L       9     EIMBA        Invalid memory block address

-46L       15    EDRIVE       Invalid drive specified

-49L       18    ENMFIL       No more files

-64L             ERANGE       Range error

-65L             EINTRN       Internal error

-66L             EPLFMT       Invalid program load format

-67L             EGSBF        Setblock failure (due to growth

restrictions)

4.3  GEM DOS SYSTEM CALL SUMMARY

Table  4-4 lists the hexadecimal number and mnemonic of each GEM DOS system

function call.  The table includes the parameters a process  must  pass  to

the system call and the values the system returns to the calling process.

Table 4-4.  GEM DOS System Call Summary

Hex    Mnemonic             Input Parameters

0     P_TERM0              none

1     C_CONIN              none

2     C_CONOUT             Character

3     C_AUXIN              none

4     C_AUXOUT             Character

5     C_PRNOUT             Character

6     C_RAWIO              FF/Character

7     C_RAWCIN             none

8     C_NECIN              none

9     C_CONWS              *Character String

A     C_CONRS              *Input Buffer

B     C_CONIS              none

E     D_SETDRV             Drive Code

10    C_CONOS              none

11    C_PRNOS              none

12    C_AUXIS              none

13    C_AUXOS              none

19    D_GETDRV             none

1A    F_SETDTA             DTA (Long Value)

Table 4-4.  (continued)

Hex    Mnemonic             Input Parameters

25    S_SETVEC             Vector#

Address

2A    T_GETDATE            none

Bits 0-4 = day (1-31)

Bits 5-8 = month (1-12)

Bits 9-15 = year (0-119)

2B    T_SETDATE            Date Word:

Bits 0-4 = (1-31)

Bits 5-8 = (1-12)

Bits 9-15 = (0-119)

2C    T_GETTIME            none

Bits 0-4 = 2-sec increments

Bits 5-10 = minutes

Bits 10-15 = hours

2D    T_SETTIME            Time Word (binary):

Bits 0-4 = 2-sec increments

Bits 5-10 = minutes

Bits 10-15 = hours

2F    F_GETDTA             none

30    S_VERSION            none

31    P_TERMRES            Memory Size

Exit Code

35    S_GETVEC             Vector#

36    D_FREE               *Return Buffer

Drive Code           # of Clusters

Bytes/Sectors

Sects/Clusters

D0.L = Error Code

39    D_CREATE             *Path String

3A    D_DELETE             *Path String

Table 4-4.  (continued)

Hex    Mnemonic             Input Parameters

3B    D_SETPATH            *Path String

3C    F_CREATE             *Path String

Attribute

3D    F_OPEN               *Path String

Access Mode

3E    F_CLOSE              Handle

3F    F_READ               Handle

# of Bytes           Error Code

*Buffer

40    F_WRITE              Handle

# of Bytes           Error Code

*Buffer

41    F_DELETE             *Path String

42    F_SEEK               Offset

Handle               Error Code

Mode Value

43    F_ATTRIB             *Path String

Mode Value           Error Code

Attribute

44    F_IOCTL              Subfunction #

Handle or Drive Code Bit Map of Device Information

Byte Count or Bit Map

*Buffer              # of Bytes Read/Written

I/O Status

Media Type

45    F_DUP                Handle

Error Code

46    F_FORCE              Standard Handle

Duplicate Handle

Table 4-4 (continued)

Hex    Mnemonic             Input Parameters

47    D_GETPATH            *Path Buffer

Drive Code

48    M_ALLOC              # of Bytes or -1L

(for # of available bytes)

0 on Failure

49    M_FREE               Memory Block Addr

4A    M_SHRINK             Memory Block Addr

# of bytes to keep

4B    P_EXEC               Load Value

*Path String         Base Page Addr

*Command Tail        Error Code

*Environment Strings

4C    P_TERM               Status Code

4E    F_SFIRST             *Path String

Attribute

4F    F_SNEXT              none

56    F_RENAME             *Path String

*Path String (New Name)

57    F_DATIME             *Time/Date Buffer

Buffer

Handle

Get/Set Flag

Conventions used in Table 4-4:

*         =   Pointer to

#         =   Number

Addr      =   Address

DTA       =   Disk Transfer Address

Handle    =   File or Device Handle

Path String   =

directory path, and filename

4.4  CHARACTER DEVICE I/O FUNCTIONS

Table  4-5  lists  the  number,  mnemonic,  and  name of the GEM DOS system

function described in this section.

Table 4-5.  Character Device I/O Functions

Hex     Mnemonic

01      C_CONIN

02      C_CONOUT

03      C_AUXIN

04      C_AUXOUT

05      C_PRNOUT

06      C_RAWIO

07      C_RAWCIN

08      C_NECIN

09      C_CONWS

0A      C_CONRS

0B      C_CONIS

10      C_CONOS

11      C_PRNOS

12      C_AUXIS

13      C_AUXOS

Function 01:  Read Character from Standard Input Device

C Interface:

LONG

c_conin ()

{

}

Parameter Block:

+--------+--------+

0 | function code 01|

+--------+--------+

Return Values:

Register D0.L:

(in least significant byte)

Description     C_CONIN reads a character from the  standard  input  device

and echoes it to the standard output device.

If  a  GEM...  VDI compatible console is the standard input

device, C_CONIN places the console scan  code  in  the  low

byte of the high word returned in D0.  Otherwise, this byte

is set to 00.  See Figure 4-2.

If  C_CONIN  reads  a  Ctrl-C  character,  it  flushes  the

type-ahead buffer and executes a warm boot operation.

Hb Hw    Lb Hw    Hb Lw    Lb Lw

+--------+--------+--------+--------+

|   00   |  code  |   00   |  char  |

|        | or  00 |        |        |

+--------+--------+--------+--------+

Figure 4-2.  D0 Contents Returned from C_CONIN

Function 02:  Write Character to Standard Ouput Device

C Interface:

VOID

c_conout (c)

WORD     c;

{

}

Parameter Block:

+--------+--------+

0 | function code 02|

+--------+--------+

2 |        c        |

+--------+--------+

Parameters:

c         Character to be displayed in lower 8 bits,

upper 8 bits should be set to zero

Description    C_CONOUT  displays  the  character contained in the lower 8

bits of the c parameter on the standard output device.   To

ensure  compatibility  with future extensions to the 16-bit

character set, the upper 8 bits of c should be zero.

C_CONOUT translates tab characters into  columns  of  eight

characters.    This function also checks for Ctrl-S (scroll

stop), Ctrl-Q (strat scroll), and Ctrl-P  (printer  switch)

and  processes  Ctrl-C, which aborts the operation and warm

boots the system.

Function 03:  Read Character from Standard Auxiliary Device

C Interface:

LONG

c_auxin ()

{

}

Parameter Block:

+--------+--------+

0 | function code 03|

+--------+--------+

Return Values:

Register D0.L:

Description    C_AUXIN receives a character from the  auxiliary  port  and

returns it in D0.L.

Function 04:  Write Character to Standard Auxiliary Device

C Interface:

VOID

c_auxout (c)

WORD c;

{

}

Parameter Block:

+--------+--------+

0 | function code 04|

+--------+--------+

2 |        c        |

+--------+--------+

Parameters:

c      Character to be sent to the auxiliary port in

lower 8 bits, upper 8 bits should be set to zero

Description     C_AUXOUT  sends the character contained in the lower 8 bits

of the c parameter  to  the  auxiliary  port.    To  ensure

compatibility   with   future   extensions  to  the  16-bit

character set, the upper 8 bits of c should be zero.

Function 05:  Write Character to Standard Print Device

C Interface:

VOID

c_prnout (c)

WORD c;

{

}

Parameter Block:

+--------+--------+

0 | function code 05|

+--------+--------+

2 |        c        |

+--------+--------+

Parameters:

c         Character to be sent to the standard printer

device in lower 8 bits, upper 8 bits shouldbe

set to zero

Description     C_PRNOUT sends the character contained in the lower 8  bits

of  the  c  parameter  to  the standard printer device.  To

ensure compatibility with future extensions to  the  16-bit

character set, the upper 8 bits of c should be zero.

Function 06:  Raw I/O to Standard Console

C Interface:

LONG

c_rawio (parm)

WORD    parm;

{

}

Parameter Block:

+--------+--------+

0 | function code 06|

+--------+--------+

2 |      parm       |

+--------+--------+

Parameters:

parm      Specifies the operation to be performed:

(FF   To read a character from StdIn

char Character to be sent to StdOut)

Return Values:

Register D0.L:

(param = FF)

0 Character not available

(parm = FF)

Description    If  parm is FF, C_RAWIO reads a character from the standard

input device and returns it it D0.W.  If a character is not

available from the standard input device D0.L returns zero.

If  parm  is  not  FF,  C_RAWIO assumes the word contains a

character to be sent to the standard output device.

Function 07:  Raw Input from Standard Input Device

C Interface:

LONG

c_rawcin ()

{

}

Parameter Block:

+--------+--------+

0 | function code 07|

+--------+--------+

Return Values:

Register D0.L:

Description    C_RAWCIN reads a character from the standard  input  device

and  returns  it  in  D0.L  without  echoing it to standard

output device.

C_RAWCIN does not perform a check for control characters.

Function 08:     Read Character from Standard Input, No Echo

C Interface:

LONG

c_necin ()

{

}

Parameter Block:

+--------+--------+

0 | function code 08|

+--------+--------+

Return Values:

Register D0.L:

Description    C_NECIN reads a character from the  standard  input  device

and  returns  it  in  D0.L  without  echoing it to standard

output device.

Control characters are interpreted  by  this  function  and

have  their  proper  effect; see "Line Editing Commands" in

Section 1 of the GEM DOS User's Guide.

Function 09:    Write String to Standard Output Device

C Interface:

VOID

c_conws (p)

BYTE     *p;

{

}

Parameter Block:

+--------+--------+

0 | function code 09|

+--------+--------+

2 |                 |

+        p        +

4 |                 |

+--------+--------+

Parameters:

p       Points to a null-terminated string of characters

to be written to the standard output device

Description    C_CONWS  transmits  the  null-terminated  character  string

pointed to by p to the standard output device, character by

character.  This  function  also  processes  Ctrl-C,  which

aborts the operation and warm boots the system.

Function 0A:  Read Edited String from Standard Input Device

C Interface:

VOID

c_conrs ()

BYTE     *p;

{

}

Parameter Block:

+--------+--------+

0 | function code 0A|

+--------+--------+

2 |                 |

+        p        +

4 |                 |

+--------+--------+

Parameters:

p         Points to an input buffer

Description    C_CONRS  reads  characters  from the caller's default input

device and writes them to the input buffer  pointed  to  by

the  p  parameter.  The format of the input buffer is shown

in Figure 4-3.

0        1        2                          MAX - 1

|--------+--------+--------+--------+ - - - +--------|

|  MAX   | NCHARS | CHARACTERS . . .|       |   0DH  |

|--------+--------+--------+--------+ - - - +--------|

Figure 4-3.  C_CONRS Occurrence

Byte  0  of  the  input  buffer  specifies  the  number  of

characters the buffer can hold.  C_CONRS sets byte 1 to the

number of characters received.  This value does not include

the terminator character, a carriage return (0DH).  C_CONRS

writes the characters it  reads  from  the  standard  input

device  into  the buffer beginning at the third byte.  When

the system call reads a carriage return, it  stops  writing

to the buffer.

When  the  input buffer fills to one less than the value of

byte 0, C_CONRS ignores additional  input  and  sounds  the

bell until it reads a carriage return.

C_CONRS  performs  line-editing functions on the characters

as they are  read  from  the  console  (Backspace,  Ctrl-E,

Ctrl-Q,  Ctrl-R,  Ctrl-S,  Ctrl-U,  Ctrl-X)  and performs a

warmboot on Ctrl-C input.

Function 0B:    Check Status of Standard Input Device

C Interface:

LONG

c_conis (int)

{

}

Parameter Block:

+--------+--------+

0 | function code 0B|

+--------+--------+

Return Values:

Register D0.L:

0 If no character available

Description    C_CONIS checks the status of the  caller's  standard  input

device to determine if a character is ready to be received.

Function 10:  Check Status of Standard Output Device

C Interface:

LONG

c_conos ()

{

}

Parameter Block:

+--------+--------+

0 | function code 10|

+--------+--------+

Return Values:

Register D0.L:

0 If console unavailable

Description    C_CONOS checks the status of the caller's console device to

determine if it is ready to receive a character.

This function  also  processes  Ctrl-C,  which  aborts  the

operation and warm boots the system.

Function 11:  Check Status of Standard Print Device

C Interface:

LONG

c_prnos ()

{

}

Parameter Block:

+--------+--------+

0 | function code 11|

+--------+--------+

Return Values:

Register D0.L:

0 If printer unavailable

Description    C_PRNOS   checks  the  status  of  the  printer  device  to

determine if it is ready to receive a character.

Function 12:  Check Status of Standard Auxiliary Device Input

C Interface:

LONG

c_auxis ()

{

}

Parameter Block:

+--------+--------+

0 | function code 12|

+--------+--------+

Return Values:

Register D0.L:

0 If no character available

Description     C_AUXIS checks the input status of  the  caller's  standard

auxiliary device to determine if a character is ready to be

received.

Function 13:  Check Status of Standard Auxiliary Device Output

C Interface:

LONG

c_auxos ()

{

}

Parameter Block:

+--------+--------+

0 | function code 13|

+--------+--------+

Return Values:

Register D0.L:

0 If it is unavailable

Description    C_AUXOS checks the status of the caller's auxiliary  device

to determine if it is ready to receive a character.

4.5  FILE ACCESS FUNCTIONS

Table  4-6 lists the number, mnemonic, and name of the GEM DOS system calls

described in this section.

Table 4-6.  File Access Function Calls

Hex     Mnemonic

1A      F_SETDTA

2F      F_GETDTA

3C      F_CREATE

3D      F_OPEN

3E      F_CLOSE

3F      F_READ

40      F_WRITE

41      F_DELETE

42      F_SEEK

43      F_ATTRIB

45      F_DUP

46      F_FORCE

4E      F_SFIRST

4F      F_SNEXT

56      F_RENAME

57      F_DATIME

Function 1A:  Set Disk Transfer Address

C Interface:

VOID

f_setdta (b)

BYTE *b;

{

}

Parameter Block:

+--------+--------+

0 | function code 1A|

+--------+--------+

2 |                 |

+       addr      +

4 |                 |

+--------+--------+

Parameters:

addr      Address of DTA buffer

Description    The  44-byte  buffer  whose  location  is  specified   with

F_SETDTA  may  begin  on  an even or odd address.  F_SFIRST

(4E) and F_SNEXT (4F) use the DTA  to  return  file  search

information.

If  you do not set the DTA, GEM DOS uses the default DTA at

offset 80 in the Base Page.    F_GETDTA  (2F)  returns  the

current location of the DTA buffer.

Function 2F:  Get Disk Transfer Address

C Interface:

LONG

f_getdta ()

{

}

Parameter Block:

+--------+--------+

0 | function code 2F|

+--------+--------+

Return Values:

Register D0.L:

Description    F_GETDTA  returns  the current address of the DTA buffer in

D0.L.  You can set the DTA by calling F_SETDTA (1A).   Note

that  only  F_SFIRST  (4E)  and F_SNEXT (4F) use the DTA to

return file search information.

Function 3C:  Create a File

C Interface:

LONG

f_create (name,attr)

BYTE        *name;

WORD        attr;

{

}

Parameter Block:

+--------+--------+

0 | Function code 3C|

+--------+--------+

2 |                 |

+       name      +

4 |                 |

+--------+--------+

6 |       attr      |

+--------+--------+

Parameters:

name      Points to a null-terminated string that

specifiesthe drive, path, and name of the

file to be created

attr      Specifies the file's attributes:

01   Read-only

02   Hidden

04   System, Hidden

08   Entry is a volume label

Return Values:

Register D0.L:

-34L (Path not found)

-35L (No handles left)

-36L (Access Denied)

Description    F_CREATE  returns  error  code  -36L  (Access  Denied)   to

indicate  that  a  file with the name specified in the name

string already exists or that the directory is full.

Function 3D:    Open a File

C Interface:

LONG

f_open (pname,mode)

BYTE    *pname;

WORD mode;

{

}

Parameter Block:

+--------+---------+

0 | function code  3D|

+--------+---------+

2 |                  |

+      pname       +

4 |                  |

+--------+---------+

6 |       mode       |

+--------+---------+

Parameters:

pname     Points to a null-terminated string that

specifiesthe drive, path, and name of the

file or device to open

mode      File access mode:

0    Read access only

1    Write access only

2    Read/Write access

Return Values:

Register D0.L:

-33L (File not found)

-35L (No handles left)

-36L (Access Denied)

Description    F_OPEN positions the file pointer at the beginning  of  the

file.    You  can  change  the  pointer's location with the

F_SEEK (42) function call.

Use the handle returned by F_OPEN to refer to the file  for

subsequent  read,  write,  seek, and close operations.  You

can use F_ATTRIB (43)  to  get  the  file's  attribute  and

F_DATIME to get or set its date and time stamps.

Function 3E:    Close a File

C Interface:

LONG

f_close (handle)

WORD      handle;

{

}

Parameter Block:

+--------+--------+

0 | Function code 3E|

+--------+--------+

2 |      handle     |

+--------+--------+

Parameters:

handle    File handle of file to be closed

Return Values:

Register D0.L:

-37L (Invalid handle)

Description    The  handle  you specify in the parameter block on entry to

F_CLOSE should be  one  that  was  previously  returned  by

F_OPEN (3D).

All  disk  buffers associated with the file are flushed and

the file's directory entry is updated on successful  return

from this call.

Function 3F:    Read from a File

C Interface:

LONG

f_read (handle, cnt, pbuffer)

WORD       handle;

LONG       cnt;

BYTE       *pbuffer;

{

}

Parameter Block:

+--------+--------+

0 | Function code 3F|

+--------+--------+

2 |      handle     |

+--------+--------+

4 |                 |

+       cnt       +

6 |                 |

+--------+--------+

8 |                 |

+     pbuffer     +

A |                 |

+--------+--------+

Parameters:

handle    File handle (returned from F_OPEN) of file to

be read

cnt       Number of bytes to be read

pbuffer   Points to the buffer into which the file's

contents are to be read

Return Values:

Register D0.L:

-37L (Invalid handle)

Function 40:    Write to a File

C Interface:

LONG

f_write (handle, cnt, pbuffer)

WORD       handle;

LONG       cnt;

BYTE       *pbuffer;

{

}

Parameter Block:

+--------+--------+

0 | function code 40|

+--------+--------+

2 |      handle     |

+--------+--------+

4 |                 |

+       cnt       +

6 |                 |

+--------+--------+

8 |                 |

+     pbuffer     +

A |                 |

+--------+--------+

Parameters:

handle    File handle (returned from F_OPEN)

cnt       Number of bytes to be written

pbuffer   Points to the buffer that contains

the bytes to be written

Return Values:

Register D0.L:

-36L (Access Denied)

-37L (Invalid handle)

Function41:    Delete a File

C Interface:

LONG

f_delete (name)

BYTE        *name;

{

}

Parameter Block:

+--------+--------+

0 | function code 41|

+--------+--------+

2 |                 |

+       name      +

4 |                 |

+--------+--------+

Parameters:

name      Points to a null-terminated string that

specifies the drive, path, and name of the

directory entry to be deleted

Return Values:

Register D0.L:

-33L (File not found)

-36L (Access Denied)

Function 42:  Seek File Pointer

C Interface:

LONG

f_seek (softs, handle, smode)

LONG     softs;

WORD     handle;

WORD     smode;

{

}

Parameter Block:

+--------+--------+

0 | function code 42|

+--------+--------+

2 |                 |

+      softs      +

4 |                 |

+--------+--------+

6 |      handle     |

+--------+--------+

8 |      smode      |

+--------+--------+

Parameters:

softs     A signed value, N, used to indicate byte

offset negative values are useful in modes 1

and 2 below +N moves pointer toward end of

file, -N toward beginning of file

handle    File handle (returned from F_OPEN)

smode     Indicates how the file pointer is to be moved:

0  Move pointer N bytes from the beginning of

the file

1  Move pointer N bytes from its current

location

2  Move pointer N bytes from the end of the

file

Return Values:

Register D0.L:

from beginning of file)

-37L (Invalid handle)

-32L (Invalid function)

Function 43:    Get/Set File Attributes

C Interface:

LONG

f_attrib (p, wrt, mod)

BYTE     *p;

WORD     wrt,mod;

{

}

Parameter Block:

+--------+--------+

0 | function code 43|

+--------+--------+

2 |                 |

+        p        +

4 |                 |

+--------+--------+

6 |       wrt       |

+--------+--------+

8 |       mod       |

+--------+--------+

Parameters:

p         Points to a null-terminated string that

containsthedrive, path, and file name

wrt       Specifies the operation to be performed:

0    Return the file's attributes in D0.L

1    Set the file's attributes according to

mod

mod       Indicates file attributes:

01H  Read-only

02H  Hidden from directory search

04H  System, hidden from directory search

08H  Entry contains the volume label

10H  Entry is a subdirectory

20H  Archive (file written to and closed)

Return Values:

Register D0.L:

-33L (File not found)

-34L (Path not found)

Function 44:    I/O Control

C Interface:

LONG

f_ioctl (subfun, hndrv, cntval, bufptr)

WORD subfun, hndrv, cntval;

LONG bufptr;

{

}

Parameter Block:

+--------+--------+

0 | function code 44|

+--------+--------+

2 |      subfun     |

+--------+--------+

4 |      handrv     |

+--------+--------+

6 |      cntval     |

+--------+--------+

8 |                 |

+      bufptr     +

A |                 |

+--------+--------+

Parameters:

subfun    F_IOCTL subfunction code value as follows:

-1   Substitute interrupt vector

0    Get device information

1    Reserved

2    Read from device control channel

3    Write to device control channel

4    Read from drive control channel

5    Write to drive control channel

6    Get input status

7    Get output status

8    Get media type

hndrv     Handle or drive code, subfunction dependent.

SeeTable 4-7, below.

cntval    Byte count or bit map of values, subfunction

dependent.

bufptr    Pointer to buffer or, for subfunction -1, the

address of device's interrupt vector.

Return Values:

Device's current interrupt vector for subfunction -1

Bit map of device information for subfuction 0

Number of bytes transferred for subfunctions 2-5

Disk file or character device input status for

subfunction 6

Disk file or character device output status for

subfunction 7

Media type for subfunction 8

Description    On  entry  to  F_IOCTL,  offset  02  of the parameter block

contains a  subfunction  code  value  that  determines  the

operation  to  be  performed.   Other entry parameters vary

according to the subfunction you call.  Table 4-7 lists the

F_IOCTL  subfunction calling restrictions.  Descriptions of

the individual subfunctions follow the table.

Table 4-7.  F_IOCTL Calling Restrictions

Character              Disk

Subfunction  Device     Standard      File    Drive           Byte

Number     Handle  Handle  Handle  Number          Count/Buffer

-1         X

0         X       X       X

2         X       X                        X

3         X       X                        X

4                                 X        X

5                                 X        X

6         X       X       X

7         X       X       X

8                                 X

Subfunction -1: Specify a character device handle at offset 04, 0 at offset

06,  and  the  address of the device's interrupt handler at

offset 08 of the F_IOCTL parameter block.   Subfunction  -1

returns  the  current interrupt vector for the device whose

handle you specify.  Note  that  only  a  character  device

handle can be specified for this subfunction.

Subfunction 0:  Specify a disk file or character device handle at offset 04

of the F_IOCTL parameter block.  Subfunction 0 returns  the

bit  map  of  device  information  shown  below or an error

number (-35L, -37L) in register D0.L.

If bit 7 = 0:

Bits   Meaning

0-5    Drive number

6     Reserved

7     0 = Disk file

8-D    Reserved

E     1 = Device can process

control strings

F     Reserved

If bit 7 = 1:

Bits   Meaning

0     1 = CON: input (stdin)

1     1 = CON: output (stdout)

2     1 = NUL:

3     1 = CLK:

4-6    Reserved

7     1 = Character device

8-D    Reserved

E     1 = Device can process

control strings

F     Reserved

Subfunction 2:  Specify the handle of a character device at offset  4,  the

number  of bytes to be read at offset 6, and the address of

the  buffer  into  which  this  subfunction  is   to   read

characters from the device's control channel at offset 8 of

the F_IOCTL parameter block.

Subfunction 2 returns the number of bytes read or an  error

number in register D0.L.

Subfunction 3   Specify  the  handle  of a character device at offset 4, of

the number of bytes to be written to the  device's  control

channel  at  offset  6,  and the address of the buffer from

which the bytes are to written at offset 8 of  the  F_IOCTL

parameter block.

Subfunction  3  returns  the number of bytes written to the

device control channel or an error number in register D0.L.

Subfunction 4   Specify the number of the drive at offset 4, the number  of

bytes to be read from the drive's control channel at offset

6, and the address of a buffer at offset 8 of  the  F_IOCTL

parameter block.

The drive number can be 0 for  the  default  drive,  1  for

drive  A, and so on up to the number of drives supported by

your particular system.

Subfunction 4 returns the number of  bytes  read  from  the

drive's control channel or an error number in register D0.L

Subfunction 5   Specify  the number of the drive at offset 4, the number of

bytes to written to the drive's control channel  at  offset

6, and the buffer from which the bytes are to be written at

offset 8 of the F_IOCTL parameter block.

Subfunction 5 returns the number of bytes  written  to  the

drive's  control  channel  or  an  error number in register

D0.L.

Subfunction 6   Specify the handle of a disk file  or  device  whose  input

status  you  want  returned  at  offset  4  of  the F_IOCTL

parameter block.  If the handle you specify is a disk file,

subfunction 6 returns zero in D0.L when the file pointer is

at EOF, or a non-zero  value.    If  the  file  pointer  is

repositioned after reaching EOF, this subfunction returns a

non-zero value again.

If the handle you specify is for a  device,  subfunction  6

returns a non-zero value in D0.L to indicate a ready state,

or zero to indicate that the device is not ready.

Subfunction 7   Specify the handle of a disk file or  device  whose  output

status  you  want  returned  at  offset  4  of  the F_IOCTL

parameter block.  If the handle you specify is for  a  disk

file,  subfunction  7  returns  zero  in D0.L when the file

pointer is at EOF, or a non-zero value.  If the pointer  is

repositioned  after  reaching EOF, this subfunction returns

non-zero again.

If the handle you specify is for a  device,  subfunction  7

returns  a  non-zero  value to indicate a ready state, or a

zero value to indicate that the device  is  not  ready  for

output.

Subfunction 8   Specify  the  number  of  the  drive  whose  media type you

returned at offset 4 of the F_IOCTL parameter  block.    If

the  drive  number  you  specify is valid, this subfunction

returns zero to indicate that the drive contains  removable

media  or  a  non-zero  value  to  indicate  that the drive

contains fixed media.

The drive number you specify  may  be  0  for  the  default

drive,  1 for drive A, and so on up to the number of drives

supported by your particualr system.

Function 45:  Duplicate a File Handle

C Interface:

LONG

f_dup (stdhnd)

WORD stdhnd;

{

}

Parameter Block:

+--------+--------+

0 | function code 45|

+--------+--------+

2 |     stdhnd      |

+--------+--------+

Parameters:

stdhnd    Standard file handle

Return Values:

Register D0.L:

-35L (No handles left)

-37L (Invalid handle)

Description    F_DUP accepts a standard file handle as input and returns a

non-standard  handle that refers to the same file or device

at the same position.

If you use F_SEEK  (42)  to  change  the  location  of  the

pointer  for either handle, the pointer of its duplicate is

also moved.

See Table 4-2 for the GEM DOS standard handle values.

Function 46:  Force File Handle

C Interface:

LONG

f_force (stdhnd, nsthnd)

WORD stdhnd, nsthnd;

{

}

Parameter Block:

+--------+--------+

0 | function code 46|

+--------+--------+

2 |     stdhnd      |

+--------+--------+

4 |     nsthnd      |

+--------+--------+

Parameters:

stdhnd    Standard file handle

nsthnd    Non-standard file handle

Return Values:

Register D0.L:

-37L (Invalid handle)

Description    F_FORCE forces the  standard  device  handle  specified  at

offset  02 of the parameter block to point to the same file

or device as the non-standard handle  specified  at  offset

04.

If  the  file  whose  handle you specify in stdhnd is open,

F_FORCE closes before returning.  If you use F_SEEK (42) to

change  the  location of the pointer for either handle, the

pointer of its duplicate is also moved.

See Table 4-2 for the GEM DOS standard handle values.

Function 4E:    Search for First Occurrence of Filespec

C Interface:

LONG

f_sfirst (pspec, attr)

BYTE      *pspec;

WORD      attr;

{

}

Parameter Block:

+--------+--------+

0 | function code 4E|

+--------+--------+

2 |                 |

+      pspec      +

4 |                 |

+--------+--------+

6 |      attr       |

+--------+--------+

Parameters:

pspec     Points to a null-terminated string that

specifies the drive, path and name of the file

to search for; only the file name portion of

pspec may contain * or ? wildcards

attr      Specifies the search attributes:

00H  File is normal file entry

01H  File is read-only

02H  File is hidden from directory search

04H  File is system file

08H  Entry is a volume label

10H  Entry is a subdirectory

20H  File's archive bit is set

Return Values:

Register D0.L:

-33L (File not found)

-49L (No more files)

Description    F_SFIRST  searches  for  the  first  directory  entry  that

matches pspec according to the value of attr.

F_SFIRST uses the value of attr in the following manner:

*   If  attr  is  00H,  the  search is restricted to normal

directory entries.  F_SFIRST does  not  return  entries

for  volume  labels,  subdirectories,  hidden files, or

system files.

*   If attr is set to the hidden (02H) or system (04H) file

values,  they  are  included with the normal entries in

the search set.  You can use F_SFIRST to search for all

directory  entries except volume labels by setting attr

for hidden, system, and directory.

*   If attr  is  set  to  the  volume  label  value  (08H),

F_SFIRST restricts its search to volume labels.

If  F_SFIRST finds a directory entry that matches pspec and

attr, it formats the DTA as shown in Table 4-8.    F_GETDTA

(2F) returns the current location of the DTA buffer.

Table 4-8.  DTA Contents from F_SFIRST Call

DTA Offset    Contents

0 - 20      Reserved for OS use

21 File's attributes

22 - 23     File's time stamp

24 - 25     File's date stamp

26 - 29     Longword of file's size

30 - 43     File's name and extension

Function 4F:    Search for Next Occurrence of Filespec

C Interface:

LONG

f_snext()

{

}

Parameter Block:

+--------+--------+

0 | function code 4F|

+--------+--------+

Return Values:

Register D0.L:

-49L (No more files)

Description     F_SNEXT   uses  the  parameters  specified  in  a  previous

F_SFIRST (4E) or F_SNEXT call to locate the  next  matching

directory  entry.    Offsets 0 to 20 of the DTA must remain

untouched from the previous SFIRST or SNEXT call.

If F_NEXT locates a matching entry, it updates the contents

of  the  DTA buffer initialized by the previous F_SFIRST or

F_NEXT call as shown in Table 4-8, above.

Function 56:    Rename a File

C Interface:

LONG

f_rename (res, p1, p2)

WORD   res;

BYTE   *p1;

BYTE   *p2;

{

}

Parameter Block:

+--------+--------+

0 | function code 56|

+--------+--------+

2 |    Must be 0    |

+--------+--------+

4 |                 |

+        p1       +

6 |                 |

+--------+--------+

8 |                 |

+        p2       +

A |                 |

+--------+--------+

Parameters:

p1      Points to a null-terminated string that contains

the drive, path, and name of the file to be

renamed

p2      Points to a null-terminated string that contains

the path and new name for the file specified in

p1

Return Values:

Register D0.L:

-34L (Path not found)

-36L (Access denied)

Description    The word at offset 2 in the parameter block is reserved for

system  use  and  must be zero.  The file name specified in

the p2 string must not exist.  If you specify  a  drive  in

the  p2  string, it must be the same drive specified in the

string pointed to by p1.

You can specify a different directory path in the p2 string

so   that   the   operation   moves  the  file  to  another

subdirectory on the same drive.

F_RENAME returns -36L in D0 if the read/only  attribute  of

the file to be renamed is set.

Function 57:    Get/Set File Date and Time Stamps

C Interface:

VOID

f_datime (buf, h, set)

BYTE     *buff;

WORD     h;

WORD     set;

{

}

Parameter Block:

+--------+--------+

0 | function code 57|

+--------+--------+

2 |                 |

+       buff      +

4 |                 |

+--------+--------+

6 |        h        |

+--------+--------+

8 |       set       |

+--------+--------+

Parameters:

buff      Points to a buffer that contains the time in

the first word and the date in the second word

h         Contains the file handle returned from F_OPEN

set       A flag that specifies the operation to be

performed:

0    Set date and time

1    Return date and time to buff

Description    The  format  of  the  date and time contained in the buffer

pointed to by buff is as follows:

Time (word 0):

Bits 00 - 04

Bits 05 - 10

Bits 11 - 15

Date (word 1):

Bits 00 - 04

Bits 05 - 08

Bits 09 - 15

4.6  DRIVE AND DIRECTORY MANAGEMENT FUNCTIONS

Table 4-9 lists the number, mnemonic, and name  of  the  GEMDOS  drive  and

directory management functions.

Table 4-9.  Drive and Directory Management Functions

Hex     Mnemonic

0E      D_SETDRV

19      D_GETDRV

36      D_FREE

39      D_CREATE

3A      D_DELETE

3B      D_SETPATH

47      D_GETPATH

Function 0E:    Set Default Drive

C Interface:

LONG

d_setdrv (newdrv)

WORD newdrv;

{

}

Parameter Block:

+--------+--------+

0 | function code 0E|

+--------+--------+

2 |      newdrv     |

+--------+--------+

Parameters:

newdrv    Specifies the number for the current drive:

0    Drive A

1    Drive B

3    Drive C

.

.

.

15   Drive P

Return Values:

Register D0.L:

Bit  0 = A

Bit  1 = B

Bit  2 = C

.        .

.        .

.        .

Bit 15 = P

Description    D_SETDRV  designates the drive whose number is specified at

offset 04 in the parameter block as the current  drive  and

returns a bit map of the system drives in D0.L.

Function 19:  Get Default Drive

C Interface:

LONG

d_getdrv ()

{

}

Parameter Block:

+--------+--------+

0 | function code 19|

+--------+--------+

Return Values:

Register D0.L:

Description    D_GETDRV  returns  the number of the current drive in D0.L,

where: 0 = A, 1 = B, 2 = C, up to 15 = P

Function 36:  Get Drive Free Space

C Interface:

LONG

d_free (pbuffer,dr)

BYTE *pbuffer

WORD dr;

{

}

Parameter Block:

+--------+--------+

0 | function code 36|

+--------+--------+

2 |                 |

+     pbuffer     +

4 |                 |

+--------+--------+

6 |       dr        |

+--------+--------+

Parameters:

pbuffer   Points to the buffer into which D_FREE returns

drive information

dr        Specifies the drive code where:

0 = Current Drive, 1 = Drive A, 2=Drive B, etc.

Return Values:

Register D0.L:

-46L (Invalid drive specified)

Description    The  format  of  the  buffer  pointed  to  by  pbuffer   on

successful return from D_FREE is shown in Figure 4-4.

+--------+--------+--------+--------+

0 |   Number of Free Bytes on Drive   |

+--------+--------+--------+--------+

4 |   Number of Available Clusters    |

+--------+--------+--------+--------+

8 |          Bytes per Sector         |

+--------+--------+--------+--------+

C |  Number of Sectors per Cluster    |

+--------+--------+--------+--------+

Figure 4-4.  D_FREE Information Return Buffer

Function 39:   Create a Subdirectory

C Interface:

LONG

d_create (path),

BYTE   *path;

{

}

Parameter Block:

+--------+--------+

0 | function code 39|

+--------+--------+

2 |                 |

+       path      +

4 |                 |

+--------+--------+

Parameters:

path      Points to a null-terminated string that

containsthedrive and pathname of the

subdirectory to be created

Return Values:

Register D0.L:

-34L (Path not found)

-36L (Access denied)

Description    D_CREATE  does not change the directory path if any element

of the pathname contained  in  the  path  string  does  not

exist.  If D0.L contains 0 on return from D_CREATE, the new

subdirectory is created at the end of the pathname.

Function 3A:    Delete a Subdirectory

C Interface:

LONG

d_delete (path)

BYTE    *path;

{

}

Parameter Block:

+--------+--------+

0 | function code 3A|

+--------+--------+

2 |                 |

+       path      +

4 |                 |

+--------+--------+

Parameters:

path      Points to a null-terminated string that

containsthedrive and pathname of the

subdirectory to be deleted

Return Values:

Register D0.L:

-34L (Path not found)

-36L (Access denied)

-65L (Internal error)

Description    If the subdirectory to be deleted is  not  empty,  D_DELETE

returns  -36L  (Access  denied).   D_DELETE also returns an

error code if you attempt to delete  the  current  or  root

directories.

Function 3B:    Set Current Directory

C Interface:

LONG

d_setpath (path)

BYTE       *path;

{

}

Parameter Block:

+--------+--------+

0 | function code 3B|

+--------+--------+

2 |                 |

+       path      +

4 |                 |

+--------+--------+

Parameters:

path      Points to a null-terminated string that

containsthedrive and pathname of the

directory to be made current

Return Values:

Register D0.L:

-34L (Path not found)

Description    If  D0.L  contains  0 on return from D_SETPATH, the current

directory is set to correspond with the path string.

Function 47:    Get Current Directory

C Interface:

LONG

d_getpath (pathbuf, drive)

BYTE    *pathbuf;

WORD    drive;

{

}

Parameter Block:

+--------+--------+

0 | function code 47|

+--------+--------+

2 |                 |

+     pathbuf     +

4 |                 |

+--------+--------+

6 |      drive      |

+--------+--------+

Parameters:

pathbuf   Points to a 64-byte buffer into which

D_GETPATH writesthe complete pathname of the

current directory

drive     Contains the drive code where:

0=current drive, 1=drive A, 2 = drive B,etc

Return Values:

Register D0.L:

-46L (Invalid drive)

Description    D_GETPATH writes the pathname (beginning from the root)  of

the  current  directory  for  the  drive  specified  in the

parameter block into the null-terminated buffer pointed  to

by pathbuf.  D_GETPATH does not include the drive letter in

the  pathname  nor  does  it  begin  the  pathname  with  a

backslash (\).

4.7  PROGRAM CONTROL FUNCTIONS

Table 4-10 lists the number, mnemonic, and name of the GEM DOS system calls

described in this section.

Table 4-10.  System and Program Control Function Calls

Hex     Mnemonic

00      P_TERM0

25      S_SETVEC

30      S_VERSION

31      P_TERMRES

35      S_GETVEC

4B      P_EXEC

4C      P_TERM

Function 00:    Program Terminate

C Interface:

VOID

p_term0 ()

{

}

Parameter Block:

+--------+--------+

0 | function code 00|

+--------+--------+

Description    P_TERM0 terminates the calling process and returns  to  the

parent process with a return code of 0.

Function 25:    Set Exception Vector

C Interface:

LONG

s_setvec (vecnum, address)

WORD vecnum;

LONG address;

{

}

Parameter Block:

+--------+--------+

0 | function code 25|

+--------+--------+

2 |     vecnum      |

+--------+--------+

4 |                 |

+     address     +

6 |                 |

+--------+--------+

Parameters:

vecnum    Vector number

address   New address for vector number

Return Values:

Register D0.L:

Description    S_SETVEC  associates  a  vector  number  with the specified

address.

Function 30:    Get Version Number

C Interface:

WORD

s_version ()

{

}

Parameter Block:

+--------+--------+

0 | function code 30|

+--------+--------+

Return Values:

Register D0.W:

Description    The lower byte of the word  returned  in  D0  contains  the

major  version  number;  the  upper byte contains the minor

version number.  0001 = version 1.00.

Function 31:    Terminate and Stay Resident

C Interface:

VOID

p_termres (n_bytes,rc)

LONG n_bytes

WORD rc;

{

}

Parameter Block:

+--------+--------+

0 | function code 31|

+--------+--------+

2 |                 |

+     n_bytes     +

4 |                 |

+--------+--------+

6 |        rc       |

+--------+--------+

Parameters:

n_bytes   Memory size in bytes

rc        Exit code

Description    P_TERMRES allows  the  calling  process  to  terminate  the

currently  running  process  and leave the amount of memory

specified in n_bytes (offset 02) resident.

The rc field at offset 06 of the P_TERMRES parameter  block

contains  an  exit  code  that  is  returned  to the parent

process in register D0.

Note that your use of P_TERMRES  might  make  your  program

difficult  to  port  to  future  versions of this operating

system.

Function 35:    Get Exception Vector

C Interface:

LONG

s_setvec (vecnum)

WORD vecnum;

{

}

Parameter Block:

+--------+--------+

0 | function code 35|

+--------+--------+

2 |     vecnum      |

+--------+--------+

Parameters:

vecnum    Vector number

Return Values:

Register D0.L:

Description    S_GETVEC returns the address associated with the  specified

vector number.

Function 4B:    Load or Execute a Process

C Interface:

LONG

p_exec (load, pcspec, pcmdln, penvstr)

WORD      load;

BYTE      *pcspec;

BYTE      *pcdln;

BYTE      *penvstr;

{

}

Parameter Block:

+--------+--------+

0 | function code 4B|

+--------+--------+

2 |      load       |

+--------+--------+

4 |                 |

+     pcspec      +

6 |                 |

+--------+--------+

8 |                 |

+      pcdln      +

A |                 |

+--------+--------+

C |                 |

+     penvstr     +

E |                 |

+--------+--------+

Parameters:

load      Indicates the type of load function to be

performed:

0    Load and execute program

3    Load overlay, do not execute

pcspec    Points to a null-terminated string that

specifiesthe drive, path, and name of the

file to be loaded

pcdln     Points to a command tail that includes

redirectioninformation

penvstr   Points to the ASCII strings that describe the

configuration of the process's environment

Return Values:

Register D0.L:

on load/execute (load = 0)

Base page address

on load overlay (load=3)

-33L (File not found)

-39L (Insufficient memory)

-66L (Invalid program load format)

Description    The  address  contained  in  the  pcdln  field is placed at

offset 80H in the base page of the program being loaded.

The process environment strings pointed to by  the  penvstr

field  at  offset  0C  in  the  P_EXEC  parameter block are

formatted as a series of null-terminated  strings.    These

environment  strings  are  terminated  by a null word.  The

address contained in the penvstr field is copied  into  the

p_env field at offset 2C of the base page.

The  child  process exit code returned by P_TERMRES in D0.L

can be specified at offset 02 in the parameter  block  used

to invoke Terminate Process (4CH).

See Section for a description of the base page format.

Function 4C:    Terminate Process

C Interface:

VOID

p_term (code)

WORD code;

{

}

Parameter Block:

+--------+--------+

0 | function code 4C|

+--------+--------+

2 |       code      |

+--------+--------+

Parameters:

code      Exit code returned to parent process from

Load or Execute a Process (4B).

Return Values:

Register D0.L:

Error code on failure

Description    P_TERM terminates the current process and transfers control

to  the  calling  process.    All  files  opened   by   the

terminating process are closed.

4.8  MEMORY MANAGEMENT FUNCTIONS

This section describes the following GEM DOS function calls:

*   Allocate Memory (M_ALLOC, 48)

*   Free Allocated Memory (M_FREE, 49)

*   Shrink Size of Allocated Memory (M_SHRINK, 4A)

M_FREE  (49)  and  M_SHRINK  calls  require  the  address of a memory block

previously returned by M_ALLOC (48) as  a  parameter.    M_ALLOC  allocates

memory  in  units  of  bytes  and  can also be used to return the amount of

currently available memory.

The GEM DOS command line interpreter allocates all of available  memory  to

most of the programs that are loaded under its control.

Function 48:    Allocate Memory

C Interface:

LONG

m_alloc (nbytes)

LONG    nbytes;

{

}

Parameter Block:

+--------+--------+

0 | function code 48|

+--------+--------+

2 |                 |

+      nbytes     +

4 |                 |

+--------+--------+

Parameters:

nbytes    The number of bytes to allocate or

-1L (FFFFFFFF) to return the maximum

amount of available memory

Return Values:

Register D0.L:

block's starting address

Number of available bytes

(nbytes = FFFFFFFF)

0 on failure

Description    If   the  nbytes  field  of  the  M_ALLOC  parameter  block

specifies the number of bytes of memory  to  allocate,  the

function  call returns a pointer to the starting address of

a memory block in D0.L.  If the allocation operation fails,

D0.L returns zero.

If  you  set  nbytes to -1L (FFFFFFFF), M_ALLOC returns the

maximum number of available bytes.

Function 49:    Free Allocated Memory

C Interface:

LONG

m_free (maddr)

LONG    maddr;

{

}

Parameter Block:

+--------+--------+

0 | function code 49|

+--------+--------+

2 |                 |

+      maddr      +

4 |                 |

+--------+--------+

Parameters:

maddr     Starting address of memory block to be freed

Return Values:

Register D0.L:

-40L (Invalid memory block address)

Description    M_FREE returns  a  memory  block  previously  allocated  by

M_ALLOC  (48)  to  the  system.   If the maddr field of the

parameter block  does  not  specify  an  address  that  was

returned by a previous M_ALLOC call, M_FREE returns -40L in

D0.L to indicate that you have used an invalid memory block

address.

Function 4A:    Shrink Size of Allocated Memory

C Interface:

LONG

m_shrink (res, mp, size)

res;

BYTE *mp;

LONG size;

{

}

Parameter Block:

+--------+--------+

0 | function code 4A|

+--------+--------+

2 |    Must be 0    |

+--------+--------+

4 |                 |

+       mp        +

6 |                 |

+--------+--------+

8 |                 |

+       size      +

A |                 |

+--------+--------+

Parameters:

mp      Beginning address of memory block to be returned

size    Length, in bytes, of memory to be returned

Return Values:

-40L (Invalid memory block address)

-67L (Growth restriction failure)

Description    The word at offset 2 in the parameter block is reserved for

system use and must be zero.   mp  must  specify  a  memory

block address that was previously returned by M_ALLOC (48).

If the value you pass to M_SHRINK in the size field of  the

parameter  block does not specify a reduction in the memory

block size, M_SHRINK returns -67L in D0.L

Use M_SHRINK to return memory that your  program  does  not

require  to  the  system for allocation to other processes.

Before calling M_SHRINK, move your  program's  stack  space

down  into  an  area  of  memory  that  you have previously

reserved and know  will  accomodate  your  program's  stack

size.

If  your  program  later  requires  additional memory, call

M_ALLOC to determine the amount of memory available (nbytes

= -1).  Then call M_ALLOC with nbytes set to the portion of

available memory your requires.

4.9  TIME FUNCTIONS

This section describes the Get Date (2A), Set Date (2B), Get Time (2C), and

Set Time (2D) functions.

Function 2A:    Get Date

C Interface:

WORD

t_getdate ()

{

}

Parameter Block:

+--------+--------+

0 | function code 2A|

+--------+--------+

Return Values:

Register D0.W:

Bits 00 - 04 Day, 1 - 31

Bits 05 - 08 Month, 1 - 12

Bits 09 - 15 Year (since 1980), 0 - 119

Function 2B:    Set Date

C Interface:

LONG

t_setdate (date)

WORD date;

{

}

Parameter Block:

+--------+--------+

0 | function code 2B|

+--------+--------+

2 |       date      |

+--------+--------+

Parameters:

date      Contains the date to be set in the format:

Bits 00 - 04 Day, 1 - 31

Bits 05 - 08 Month, 1 - 12

Bits 09 - 15 Year (since 1980), 0 - 119

Return Values:

Register D0.L:

-1L (Error) if date is not valid

Function 2C:    Get Time

C Interface:

WORD

t_gettime ()

{

}

Parameter Block:

+--------+--------+

0 | function code 2C|

+--------+--------+

Return Values:

Register D0.W:

Bits 00 - 04 Binary number of two-second increments

Bits 05 - 10 Binary number of minutes

Bits 11 - 15 Binary number of hours

Function 2D:    Set Time

C Interface:

LONG

t_settime (time)

WORD time;

{

}

Parameter Block:

+--------+--------+

0 | function code 2D|

+--------+--------+

2 |       time      |

+--------+--------+

Parameters:

time  Contains the time to be set in the format:

Bits 00 - 04 Binary number of two-second increments

Bits 05 - 10 Binary number of minutes

Bits 11 - 15 Binary number of hours

Return Values:

Register D0.L:

-1L (Error) if time is not valid

End of Section 4

AS68 ASSEMBLER

5.1  ASSEMBLER OPERATION

The  GEM DOS Assembler, AS68, assembles an assembly language source program

forexecutiononthe68000microprozessor.Itproducesarelocatable

object file and, optionally, a listing.  You can find a summary of the AS68

instruction set at the end of this section.  Exceptions  and  additions  to

the standard Motorola instruction set appear in sections 5.6 and 5.7.

5.2  INITIALIZING AS68

If  the  file AS68SYM.DAT is not on your disk, you must create this file to

initialize AS68 before you can use AS68 to assemble files.   To  initialize

AS68, specify the AS68 command, the -I option, and the filename AS68INIT as

shown below.

{a}AS68 -I AS68INIT

AS68 creates the output file AS68SYMB.DAT,  which  AS68  requires  when  it

assembles  programs.  After you create this file, you need not specify this

command line again unless you reconfigure your system to have different TPA

boundaries.

5.3  INVOKING THE ASSEMBLER (AS68)

Invoke AS68 by entering a command with the following form:

AS68 [-F pathname] [-P] [-S pathname] [-U] [-L] [-N] [-I]

[-O object filename]

source pathname [>listing pathname]

Table 5-1 lists and describes the AS68 command line options.

Table 5-1.  Assembler Options

Option         Meaning

-F pathname    Specifies  the  directory  in  which the temporary files are

created.  If this option is not specified, AS68 creates  the

temporary files in the current default directory.

-I             Initializes the assembler.  See Section 5.2 for details.

-P             If  specificed,  AS68  produces  and prints a listing on the

standard output device, which, by default, is  the  console.

Redirect  the  listing,  including error messages, to a file

with the >listing  filename  parameter.    Note  that  error

messages  are  produced  whether  or  not  the  -P option is

specified.  No listing  is  produced,  however,  unless  you

specify the -P option.

-S pathname    Indicates   the   directory   that  contains  the  assembler

initialization file, AS68SYMB.DAT.   This  file  is  created

when  you initialize AS68.  AS68 reads the file AS68SYMB.DAT

before it assembles a source file.  If you  do  not  specify

this option, AS68 assumes the initialization file is located

in the current default directory.

-U             Causes all undefined symbols in the assembly to  be  treated

as global references.

-L             Ensures  all  address  constants are generated as longwords.

Use the -L option for programs that require  more  than  64K

for  execution  or  if the TPA is not contained in the first

64K bytes of memory.  If -L is not specified, the program is

assembled  to  run  in the first 64K bytes of memory.  If an

address in the assembly does not fit  within  one  word,  an

error occurs.

-N             Disables  optimization  of  branches  on forward references.

Normally, AS68 uses  the  2-byte  form  of  the  conditional

branch  and  the  4-byte  BSR  instruction wherever possible

(instead of the 6-byte JSR  instruction)  to  speed  program

execution and reduce instruction size.

-T             Enables AS68 to accept the 68010 microprocessor opcodes.

source filename

Specifies  the  file  to  assemble;  you  must  supply  this

parameter.

>listing filename

Sends a program listing to the standard output  device.  Use

the  greater-than symbol, >, to direct the listing to a disk

file.  The listing includes assembler error messages.   Note

that  if you do not specify -P with a listing filename, only

the error messages are redirected to the listing file.

5.4  ASSEMBLY LANGUAGE DIRECTIVES

Table 5-2 lists the AS68 directives.

Table 5-2.  AS68 Directives

Directive       Meaning

.comm label, expression

The  comm (common) directive specifies a label and the size

of a common area that  programs  assembled  separately  can

share.  The LO68 linker links all areas with the same label

to the same address.  The largest common area  of  a  group

with  the  same  label  determines  the  final  size of the

program common area.

.data           The data directive instructs AS68 to change  the  assembler

base segment to the data segment.

.bss            The bss (block storage segment) directive instructs AS68 to

changetheassemblerbasesegmenttotheblockstorage

segment.   You cannot assemble instructions and data in the

bss.  However, you can define symbols and  reserve  storage

in the bss with the ds directive.

.dc operand[,operand,...]

The dc (define constant)  directive  defines  one  or  more

constants  in  memory.    The  operands  can  be symbols or

expressions assigned numeric values by  AS68,  or  explicit

numeric  constants in decimal or hexadecimal, or strings of

ASCII characters.  You must separate operands with  commas.

You  must  enclose  string  constants  in  single quotation

marks.  Each ASCII character is assigned  a  full  byte  of

memory.  The eighth bit is always 0.

You  can  specify the length of each constant with a single

letter parameter (byte = b, word = w, longword = l).    You

must separate the letter from the dc with a period as shown

in the following explanations.

.dc.b           The constants are byte constants.   If  you

specify  an odd number of bytes, AS68 fills

the odd byte on the right with zeros unless

the   next   statement   is   another  dc.b

directive.  When the next  statement  is  a

dc.b directive, the dc.b uses the odd byte.

Byte constants are not relocatable.

Table 5-2.  (continued)

Directive       Meaning

.dc.w           The constants are word constants.   If  you

specify  an odd number of bytes, AS68 fills

the last word on the right  with  zeros  to

force  an even byte count.  The only way to

specify an odd number of bytes is  with  an

ASCII  constant.    Word  constants  can be

relocated.

.dc.l           The constants are longword constants.    If

less  than  a  multiple  of  four  bytes is

entered, AS68 fills the  last  longword  on

the right with zeros to force a multiple of

four bytes.    Longword  constants  can  be

relocated.

.ds operand     The   define   storage   directive   (ds)  reserves  memory

locations.  The contents of the memory that it reserves  is

not  initialized.    The  operand  specifies  the number of

bytes, words, or longwords that  this  directive  reserves.

The  notation for these size specifications is shown below.

.ds.b           reserves memory locations in bytes

.ds.w           reserves memory locations in words

.ds.l           reserves memory locations in longwords

.end            The end directive informs AS68 that  no  more  source  code

follows  this  directive.    Code,  comments,  or  multiple

carriage returns cannot follow this directive.

.endc           The endc directive denotes the end  of  the  code  that  is

conditionally  assembled.  It is used with other directives

that conditionally assemble code.

.equ (or =) expression

The  equate  directive  (equ or =) assigns the value of the

expression in the operand field to the symbol in the  label

field  that  precedes  the directive.  The syntaxes for the

equate directive are

label .equ expression

label = expression

Table 5-2.  (continued)

Directive       Meaning

The label and operand fields are required.  The label  must

be  unique;  it  cannot  be  defined  anywhere  else in the

program.  The expression cannot include an undefined symbol

or  one  that is defined following the expression.  Forward

references to symbols are not allowed for this directive.

.even           The even directive increments the location counter to force

an  even  boundary.    For  example,  if specified when the

location  counter  is  odd,   the   location   counter   is

incremented  by  one  so  that the next instruction or data

field begins on an even boundary in memory.

.globl label[,label...]

.xdef label[,label...]

.xref label[,label...]

These directives make the label(s) external.  If the labels

are  defined  in the current assembly, this statement makes

them available to other routines during a load by LO68.  If

the  labels  are  not defined in the current assembly, they

become undefined external references, which LO68  links  to

external  values with the same label in other routines.  If

you  specify  the  -U  option,  the  assembler  makes   all

undefined labels external.

.ifeq expression

.ifne expression

.ifle expression

.iflt expression

.ifge expression

.ifgt expression

These  directives  test  an  expression  against zero for a

specified condition.  If the expression is true,  the  code

following is assembled; if false, the code is ignored until

an  end  conditional  directive  (endc)  is  found.     The

directives and the conditions they test are:

.ifeq           equal to zero

.ifne           not equal to zero

.ifle           less than or equal to zero

.iflt           less than zero

.ifge           greater or equal to zero

.ifgt           greater than zero

Table 5-2.  (continued)

Directive       Meaning

.ifc 'string1', 'string2'

.ifnc 'string1', 'string2'

The  conditional string directive compares two strings. The

'c' condition is true if the strings are exactly the  same.

The 'nc' condition is true if they do not match.

.offset expression

The offset directive creates a  dummy  storage  section  by

defining  a  table  of  offsets  with  the  define  storage

directive (ds).  The storage definitions are not passed  to

the  linker.    The  offset  table  begins  at  the address

specified in the expression.  Symbols defined in the offset

table  are  internally  maintained.    No  instructions  or

code-generating directives, except  the  equate  (equ)  and

register  mask  (reg) directives, can be used in the table.

The offset directive is terminated by one of the  following

directives:

bss

data

end

section

text

.org expression The  absolute  origin  directive  (org)  sets  the location

counter  to  the  value  of  the  expression.    Subsequent

statements  are assigned absolute memory locations with the

new value of the location counter.  The  expression  cannot

contain any forward, undefined, or external references.

.page           The page directive causes a page break which forces text to

print on the top of the next page.  It does not require  an

operand or a label and it does not generate machine code.

The  page directive allows you to set the page length for a

listing of code.  If you use this directive and  print  the

source code by specifying the -P option in the AS68 command

line, pages break at predefined rather than random  places.

The  page  directive does not appear on the printed program

listing.

Table 5-2.  (continued)

Directive       Meaning

.reg reglist    The register mask directive builds a register mask that can

be  used  by a movem instruction.  (See Table 1-1.)  One or

more registers can be listed  in  ascending  order  in  the

format:

R?[-R[/R?[-R?...]...]]

Replace   the  R  in  the  above  format  with  a  register

reference.  Any of the following mnemonics are valid:

A0-A7

D0-D7

R0-R15

The following example illustrates a sample register list.

A2-A4/A7/D1/D3-D5

You can also use commas to separate registers as follows:

A1,A2,D5,D7

.section #      The section directive defines a base segment.  The sections

can  be numbered from 0 to 15 inclusive.  Section 14 always

maps to data.  Section  15  is  bss.    All  other  section

numbers denote text sections.

.text           The  text  directive instructs AS68 to change the assembler

base segment to the text  segment.    Each  assembly  of  a

program begins with the first word in the text segment.

5.5  SAMPLE COMMANDS INVOKING AS68

{a}as68 -u -l test.s

This  command assembles the source file TEST.S and produces the object file

TEST.O.  Error messages appear on the screen.  Any  undefined  symbols  are

treated as global.

{a}as68 -p  smpl.s > smpl.l

This  command assembles the source file SMPL.S and produces the object file

SMPL.O.  The program must run in the first  64K  of  memory;  that  is,  no

address  can  be  larger  than 16 bits.  Error messages and the listing are

directed to the file SMPL.L.

5.6  ASSEMBLY LANGUAGE DIFFERENCES

The syntax differences between the AS68 assembly  language  and  Motorola's

assembly language are described in the following list.

*   In  AS68,  all assembler directives are optionally preceded by a period

(.).  For example,

.equ or equ

.ds or ds

*   AS68 does not support, but accepts and ignores the  following  Motorola

directives:

comline

mask2

idnt

opt

*   The  Motorola  .set  directive  is  implemented as the equate directive

(equ).

*   AS68  accepts  upper-  and  lowercase  characters.    You  can  specify

instructions  and  directives  in  either  case.    However, labels and

variables are case-sensitive.  For example, the label START  and  Start

are not equivalent.

*   For AS68, all labels must terminate with a colon (:).  For example,

A:

FOO:

However,  if  a  label begins in column 1, it need not terminate with a

colon.

*   For AS68, ASCII string constants can be enclosed in  either  single  or

double quotes.  For example,

'ABCD'

"ac14"

*   For AS68, registers can be referenced with the following mnemonics:

r0-r15

R0-R15

d0-d7

D0-D7

a0-a7

A0-A7

Upper-  and  lowercase  references are equivalent.  Registers R0-R7 are

the same as D0-D7 and R8-R15 are the same as A0-A7.

*   Use caution when manipulating the location counter forward in AS68.  An

expression  can  move  the  counter  forward only.  The unused space is

filled with zeros in the text or data segments.

*   For AS68, comment lines can begin  with  an  asterisk  followed  by  an

equals  sign  (*  =),  but only if one or more spaces exist between the

asterisk and the equals sign as follows:

* = This command loads R1 with zeros.

* = Branch to subroutine XYZ.

Be sure to include a space after the asterisk, as the location  counter

is manipulated with a statement of the form:

*=expr

*   For AS68, the syntax for short form branches is bxx.b rather than bxx.s

*   The  Motorola assembler supports a programming model in which a program

consists of a maximum of 16  separately  relocatable  sections  and  an

optional  absolute  section.  The AS68 distributed with GEMDOS does not

support this model.  Instead, AS68 supports a model in which a  program

contains three segments, text, data, and bss as described in Section 3,

"Command File Format."

5.7  ASSEMBLY LANGUAGE EXTENSIONS

The following enhancements have been added to AS68  to  make  the  assembly

language more efficient:

*   When  the  instructions  add,  sub,  and  cmp  are used with an address

register in the source or destination, they generate  adda,  suba,  and

cmpa.   When the clr instruction is used with an address register (Ax),

it generates sub Ax, Ax.

*   add, and, cmp, eor, or, sub are allowed with immediate  first  operands

and  generate addi, andi, cmpi, eori, ori, and subi instructions if the

second operand is not register-direct.

*   All  branch  instructions  generate  short  relative   branches   where

possible, including forward references.

*   Any  shift  instruction  with  no shift count specified assumes a shift

count of one.  For example, asl rl is equivalent to asl #1,rl.

*   A jsr instruction is changed to a bsr instruction if the resulting  bsr

instruction is shorter than the jsr instruction.

*   The   .text   directive   causes  the  assembler  to  begin  assembling

instructions in the text segment.    The  .data  directive  causes  the

assembler to begin assembling initialized data in the data segment.

*   The .bss directive instructs the assembler to begin defining storage in

the bss.  No instructions or constants can be placed in the bss because

the  bss  is  for uninitialized data only.  However, the .ds directives

can be used to define storage locations, and the location  counter  (*)

can be incremented.

*   The .globl directive in the form:

.globl label[,label] ...

makes  the  labels  external.    If  they  are  otherwise  defined  (by

assignment or appearance as a label),  they  act  within  the  assembly

exactly  as  if  the  .globl  directive  were not given.  However, when

linking this program with other programs, these symbols  are  available

to  other  programs.   Conversely, if the given symbols are not defined

within the current assembly, the linker can combine the output of  this

assembly with that of others which define the symbols.

*   The  common  directive  (comm)  defines  a  common region, which can be

accessed by programs that are assembled separately.  The syntax for the

common directive is

.comm label, expression

The  expression  specifies  the number of bytes allocated in the common

region.  If several programs  specify  the  same  label  for  a  common

region,  the  size  of  the  region  is  determined by the value of the

largest expression.

The common directive assumes the label is an undefined external  symbol

in  the current assembly.  However, the linker, LO68, is special-cased,

so all external symbols, which are not  otherwise  defined,  and  which

have  a  non-zero value, are defined to be in the bss, and enough space

is left after the symbol to hold expression bytes.  All  symbols  which

become  defined  in  this  way  are  located  before all the explicitly

defined bss locations.

*   The .even directive causes the location counter (*), if  positioned  at

an  odd  address,  to  be advanced by one byte so the next statement is

assembled at an even address.

*   The instructions move, add, and sub, specified with an immediate  first

operand  and  a  data  (D)  register as the destination, generate Quick

instructions, where possible.

5.8  ERROR MESSAGES

Appendix D lists the error messages generated by AS68.

5.9  INSTRUCTION SET SUMMARY

This section contains two tables that describe  the  assembler  instruction

set  distributed  with  GEMDOS.   Table 5-3 summarizes the assembler (AS68)

instruction set.  Table 5-4 lists variations on the instruction set  listed

in  Table  5-3.   For details on specific instructions, refer to Motorola's

16-Bit Microprocessor User's Manual, third edition, MC68000UM(AD3).

Table 5-3.  Instruction Set Summary

Instruction     Description

abcd            Add Decimal with Extend

add             Add

and             Logical AND

asl             Arithmetic Shift Left

asr             Arithmetic Shift Right

bcc             Branch Conditionally

bchg            Bit Test and Change

bclr            Bit Test and Clear

bra             Branch Always

bset            Branch Test and Set

bsr             Branch to Subroutine

btst            Bit Test

chk             Check Register Against Bounds

clr             Clear Operand

cmp             Compare

dbcc            Test Condition, Decrement, and Branch

divs            Signed Divide

divu            Unsigned Divide

eor             Exclusive OR

exg             Exchange Registers

ext             Sign Extend

illegal         Illegal Instruction

jmp             Jump

jsr             Jump to Subroutine

lea             Load Effective Address

link            Link Stack

lsl             Logical Shift Left

lsr             Logical Shift Right

move            Move

movem           Move Multiple Registers

movep           Move Peripheral Data

muls            Signed Multiply

mulu            Unsigned Multiply

Table 5-3. (continued)

Instruction     Description

nbcd            Negate Decimal with Extend

neg             Negate

nop             No Operation

no              One's Complement

or              Logical OR

pea             Push Effective Address

reset           Reset External Devices

rol             Rotate Left without Extend

ror             Rotate Right without Extend

roxl            Rotate Left with Extend

roxr            Rotate Right with Extend

rte             Return From Exception

rtr             Return and Restore

rts             Return from Subroutine

sbcd            Subtract Decimal with Extend

scc             Set Conditional

stop            Stop

sub             Subtract

swap            Swap Data Register Halves

tas             Test and Set Operand

trap            Trap

trapv           Trap on Overflow

tst             Test

unlk            Unlink

Table 5-4.  Variations of Instruction Types

Instruction         Variation

add       add       Add

adda      Add Address

addq      Add Quick

addi      Add Immediate

addx      Add with Extend

Table 5-4. (continued)

Instruction         Variation

and       and       Logical AND

andi      AND Immediate

andi to ccr

andi to sr

cmp       cmp       Compare

cmpa      Compare Address

cmpm      Compare Memory

cmpi      Compare Immediate

eor       eor       Exclusive OR

eori      Exclusive OR Immediate

eori to ccr

eori to sr

move      move      Move

movea     Move Address

moveq     Move Quick

move to ccr

move to sr

move from sr

move to usp

neg       neg       Negate

negx      Negate with Extend

or        or        Logical OR

ori       OR Immediate

ori to ccr

ori to sr OR Immediate to Status Register

sub       sub       Subtract

suba      Subtract Address

subi      Subtract Immediate

subq      Subtract Quick

subx      Subtract with Extend

End of Section 5

GEM DOS LINKERS

Two linkers are provided with GEM DOS:   LINK68  and  LO68.    Both  create

relocatable command files in the format required by the operating system.

Note  that  both  LINK68  and  LO68  produce  files in CP/M-68K relocatable

format.  Use the RELMOD  format  conversion  utility  to  translate  linker

output  files  from  CP/M-68K  format  to  the  GEM DOS relocatable format.

RELMOD is described in Section 6.3.

6.1  LINK68

LINK68 is a linkage editor that combines object files into a command  file.

LINK68  accepts object files produced by other language processors, such as

AS68 or the C compiler.  LINK68  produces  an  executable  file  in  either

contiguous  or  noncontiguous  command  file  format. Noncontiguous command

files are not supported by GEM DOS.  See Section 3 for a description of the

contiguous file format.

LINK68  resolves  all  references  to external symbols and concatenates the

object files in the order specified.  The  entry  point  of  the  resulting

command file is the first instruction in the first object file.

Note:  You must use LINK68 to create an executable command file even when a

single object file contains no unresolved references.

If you use the AS68 common directive to specify a  common  area  shared  by

separate  modules,  LINK68  resolves all common areas with the same name to

the same address in the bss segment.   If  more  than  one  file  specifies

static  storage  with  the  same  name,  LINK68  uses  the largest size for

allocation.

6.1.1  Invoking LINK68

To invoke LINK68, enter a command of the following form:

LINK68 [file =] object-file-1 [,object-file-2,...object-file-n]

If you invoke LINK68  without  any  command  tail,  the  linker  lists  the

options, and returns to the operating system.

The  first  file  specification is the name of the command file you want to

create, and object-file-1 through object-file-n are  the  object  files  to

link. For example, the following command creates the output file MATH:

{a}link68 math = sin,cos,tan

If  you omit the filename to the left of the equal sign, LINK68 creates the

output file using the first filename in the command line, and  assigns  the

default filetype 68K.  For example, the following command creates SIN.68K:

{a}link68 sin,cos,tan

6.1.2  LINK68 Command-Line Options

When  you  invoke LINK68, you can specify command-line options that control

the link operation.  There are two kinds  of  options:  global  and  local.

Global  options  apply  to  the entire link operation.  Local options apply

only to the individual files being linked. You enclose  global  options  in

square brackets immediately preceding the output filename (if specified) in

the command line.  You enclose local options in square brackets immediately

following the filename to which they apply.

You  can use spaces between filenames to improve readability in the command

line, and you can put more than one option in square brackets by separating

them  with  commas.  LINK68 also allows you to abbreviate an option name to

its shortest unambiguous form.  Table 6-1  lists  the  LINK68  options  and

explains their use.  Abbreviated forms are in parentheses.

Note:  GEM  DOS  requires that the linked file have the magic number 601AH.

LINK68 produces the output file  with  this  magic  number  only  when  you

request  a  relocatable  file with contiguous text, data, and bss segments.

Consequently, to create a command file to run under GEM DOS, do not  select

the  ABSOLUTE,  DATABASE[n],  BSSBASE[n],  or  TEXTBASE[n] options when you

invoke LINK68.

Table 6-1.  LINK68 Command-line Options

Option          Function

ABSOLUTE (AB)   Tells  LINK68  to  generate  an  absolute  file   with   no

relocation  bits.    The  default is a relocatable program.

Note that command files must be relocatable  to  run  under

GEM DOS.

ALLMODS (AL)    Tells  LINK68  to  load all modules from a library, even if

they are not referenced.  The default is  to  include  only

those modules that are actually referenced.

BSSBASE[n] (B[n])

Sets the base address for the  uninitialized  data  segment

(bss)  in  discontiguous  programs.  The n is a hexadecimal

value.  The default value is the first even word after  the

data segment.

Table 6-1. (continued)

Option          Function

COMMAND (C)     Tells  LINK68  that  the  following named file contains the

rest of the command line.  LINK68 ignores the rest  of  the

main  command  line.  Nested command files are not allowed.

The format of this option is:

COMMAND [filename]

where filename is the  file  containing  the  rest  of  the

command line.

DATABASE[n] (D[n])

Specifies  the  base  address  of  the  data   segment   in

discontiguous programs.  The n is a hexadecimal value.  The

default is the first even word after the text segment.

IGNORE (IG)     Tells LINK68 to ignore 16-bit address overflow.

INCLUDE (IN)    Tells LINK68 to load an unreferenced module from a library.

The   format  for  this  option  is  the  following,  where

module-name is the module you want to load:

filename [INCLUDE [module-name]]

LOCALS (L)      Tells LINK68 to put local symbols in the symbol table.  The

default  is no local symbols.  LOCALS only applies from the

point in the command line that it appears.

The NOLOCALS option turns this option off.  Use LOCALS  and

NOLOCALS  in combination to put local symbols from specific

files in the symbol table.   LINK68  always  ignores  local

symbols starting with L.

NOLOCALS (NO)   See LOCALS.

SYMBOLS (S)     Tells  LINK68  to  put the symbol table in the output file.

The default is no symbol table in the output file.

TEMPFILES[d:]  (TEM[d:])

Tells  LINK68  to  use  drive  d  for temporary files.  The

default is the currently  logged-in  drive.    If  you  use

TEMPFILES,  it  must precede any input files on the command

line.

Table 6-1. (continued)

Option          Function

TEXTBASE[n] (TEX[n])

Specifies  the base address for the text segment.  The n is

a hexadecimal value.  The default is 0H.

UNDEFINED (U)   Tells LINK68 to ignore the presence of undefined symbols in

the  input files.  LINK68 lists the undefined symbols, then

continues processing.  The default action is  to  list  any

undefined symbols, then stop processing.

The  following  are  examples  of  LINK68  command lines.  Addresses are in

hexadecimal.  The first command line example links the  files  FOOMAIN  and

FOOLIB  into  a command file named FOOBAZ.  It also tells LINK68 to include

the symbol table in FOOBAZ, and place the temporary files on drive B.

{a}link68 [sym, tem[b:]] foobaz = foomain, foolib

The next example tells LINK68 to  read  the  command  line  from  the  file

LINKIT.INP.  Note that closing brackets are not needed.

{a}link68 [com[linkit.inp

The file LINKIT.INP might contain the following commands:

{a}link68 [ab, tex[500], d[2a00], b[3000]] screen =

scrnsl [l], iolib[al]

This  command creates the file SCREEN from the files SCRNS1 and IOLIB.  The

command tells LINK68 to create SCREEN as an absolute  file  with  the  text

segment  starting  at  500H,  the  data  segment starting at 2A00H, and the

initialized data segment starting at  3000H.    It  also  tells  LINK68  to

include local symbols from SCRNS1 and all the modules in IOLIB.

6.1.3  REDIRECTING DIAGNOSTIC OUTPUT

Normally,  LINK68 sends all diagnostic output to the console.  However, you

can redirect this output by using the > character in the command line.  For

example, the following command creates MYFILE.68K on drive A, using drive B

for the temporary files, and sends  the  diagnostics  output  to  the  file

LNKMSGS.TXT on drive D:

{a}link68 [tem[b:] myfile.68k = moda, modb >d:lnkmsgs.txt

6.2  LO68

The  LO68  linker  combines  AS68  assembled (object) programs to produce a

relocatable command file.  All  external  references  are  resolved.    The

linker  must  be  used  to  create  executable programs, even when a single

object program contains no unresolved references.   The  argument  routines

are  concatenated in the order specified.  The entry point of the output is

the first instruction of the first routine.

Note: GEM DOS requires that the linked file have the  magic  number  601AH.

LO68  produces the output file with this magic number only when you request

a  relocatable  file  with  contiguous  text,  data,  and   bss   segments.

Consequently,  to create a command file to run under GEM DOS, do not select

the -T, -Z, -D, -B, and -S options and select the -R option when you invoke

LO68.

Appendix D lists the error messages that LO68 displays.

6.2.1  Invoking LO68

Invoke  LO68  by  entering  a  command  of  the  following  form. Table 6-2

describes the options.

LO68 [-F pathname] [-R] [-S] [-I] [-Uname]

[-O filename] [-X] [-Zaddress]

[-Daddress] [-Baddress] object filename [object filename]

[>message filename]

Table 6-2.  LO68 Options

Option          Meaning

-F pathname     Specifies the path name to the directory in which temporary

files are created.

-R              Preserves  the  relocation bits so the resulting executable

program is relocatable.

-S              Strips output (if specified); the output is stripped of the

symbol table and relocation bits.

-I              Does  not  generate 16-bit address overflow messages.  When

you assemble a program without  the  AS68  -L  option,  the

linker  might  generate  address  overflow  messages if the

program contains addresses longer than 16 bits.

-Uname          Forces linking of a library module that resolves  the  name

parameter, even if the name is not referred to by any other

module being linked.  Normally, library  modules  are  only

linked  when they are needed to resolve references in other

modules.  You can use this option to create a program  from

a  library if the module resolving the name parameter calls

other modules in the library.

-O filename     Gives the object file produced by LO68  the  filename  that

you  specify  following  the  -O option.  The -O option and

filename are separated by one or more spaces.   If  you  do

not specify a filename, the object file has the name C.OUT.

-X              Includes all local symbols in the symbol table except those

that begin with the letter L. If not specified,  LO68  puts

only  global  symbols  in  the  symbol  table.  This option

allows you to discard compiler internally generated  labels

that  begin with the letter L while retaining symbols local

to routines.

-Taddress*

-Zaddress*

The -T and -Z options  are  equivalent.    The  hexadecimal

address  given  is defined by LO68 as the beginning address

for the text segment.  This address defaults to zero, or it

can  be  specified as any even hexadecimal number between 0

and FFFFFFFF.  This option is useful for  putting  programs

in  ROM.    Hexadecimal  characters  can be in uppercase or

lowercase.

Table 6-2. (continued)

Option          Meaning

-Daddress*      The hexadecimal address given is defined  by  LO68  as  the

beginning  address  for  the  data  segment.   This address

defaults to the  next  byte  after  the  end  of  the  text

segment,  or  it  can  be specified as any even hexadecimal

number between 0 and FFFFFFFF.  This option  is  especially

useful  for  for  putting  programs  in  ROM.   Hexadecimal

address characters can be in uppercase or lowercase.

-Baddress*      The hexadecimal address given is defined  by  LO68  as  the

beginning  address  for  the bss.  This address defaults to

the next byte after the end of the data segment, or it  can

be  specified  as any even hexadecimal number between 0 and

FFFFFFFF.

object filename [object filename]

The  name  of  one  or  more  object  files produced by the

assembler AS68.  These  are  the  object  files  that  LO68

links.

>message filename

If  specified,  error  messages  produced   by   LO68   are

redirected  to  the file that you specify immediately after

the greater-than sign  (>).    If  you  do  not  specify  a

filename,  error  messages  are  written  to  the  standard

default output device,  which  typically  is  your  console

terminal.

6.2.2  Sample Commands Invoking LO68

{a}lo68 -s -o test.68k test.o

This  command links assembled file TEST.O into file TEST.68K and strips out

the symbol table and relocation bits.

{a}lo68 -t4000 -d8000 -bc000 a.o b.o c.o

This command links assembled files A.O, B.O, and C.O to the default  output

file  C.OUT.    The text segment starts at location 4000H; the data segment

starts at location 8000H; and the bss starts at location C000H.

{a}lo68 -i -o test.68k test.o test1.o > error

This command links assembled files TEST.O and  TEST1.O  to  file  TEST.68K.

Any 16-bit address overflow errors are ignored; error messages are directed

to the file ERROR.

6.3  RELMOD FORMAT CONVERSION UTILITY

LINK68 and LO68 produce files in CP/M-68K  relocatable  format.    Use  the

RELMOD  command to translate linker output file from CP/M-68K format to GEM

DOS relocatable format.

The RELMOD command line is in the following form:

RELMOD inputfile outputfile

where inputfile is the CP/M-68K format file  produced  by  the  linker  and

outputfile is the name of the file translated to GEM DOS format.

RELMOD can also be used to translate GEM DOS format files back to CP/M-68K.

This is done by simply specifying the GEM DOS file as the  input  file  and

the CP/M-68K file as the output file.

End of Section 6

ARCHIVE UTILITY

7.1  INTRODUCTION

The  Archive  utility,  AR68, creates a library or replaces, adds, deletes,

lists, or extracts object modules in an existing library.  AR68 can be used

on  the  C  Run-time Library distributed with GEM DOS and documented in the

GEM DOS Supplement to the C Language Programmer's Guide for CP/M-68K.

7.2  AR68 SYNTAX

To invoke AR68, specify the  components  of  the  following  command  line.

Optional components are enclosed in square brackets ([]).

AR68 DRTWX[AV][F pathname][OPMOD]ARCHIVE

OBMOD1[OBMOD2...][>filespec]

You  can  specify  multiple  object  modules in a command line provided the

command line does not exceed 127 bytes.  The  delimiter  character  between

components consists of one or more spaces.

Table 7-1 lists and describes the components of the AR68 command line.

Table 7-1.  AR68 Command Line Components

Component Meaning

AR68      Invokes  the Archive utility.  However, if you specify only

the AR68 command, AR68 returns the following  command  line

syntax and system prompt.

{a}ar68

usage:  AR68 DRTWX[AV][F pathname] [OPMOD] ARCHIVE

OBMOD1 [OBMOD2...][>filespec]

{a}

DRTWX     Indicates  you must specify one of these letters as an AR68

command.  Each of these one-letter commands and its options

are described in Section 7.4.

AV        Indicates  you  can specify one or both of these one-letter

options.  These options are described with the commands  in

Section 7.4.

F pathname

Specifies the path to the directory in which the  temporary

file  created  by  AR68  resides.    If  no  path  name  is

specified, the current default directory  is  used.    AR68

creates  a temporary file called AR68.TMP that AR68 uses as

a scratch pad area.

OPMOD     Indicates an object module  within  the  library  that  you

specify.    The  OPMOD  parameter indicates the position in

which additional object modules reside when you incorporate

modules in the library and specify the A option.

ARCHIVE   File specification of the library.

OBMOD1 [OBMOD2 ...]

Indicates one or more object modules in a library that AR68

deletes, adds, replaces, or extracts.

>filespec Redirects the output to the file specification you specify,

rather than sending  the  output  to  the  standard  output

device, which is usually the console device (CONSOLE).  You

can redirect the  output  for  any  of  the  AR68  commands

described in Section 7.4.

7.3  AR68 OPERATION

AR68  sequentially  parses  the  command  line  once.    AR68 searches for,

inserts, replaces, or deletes object modules in the library in the sequence

in  which you specify them in the command line.  Section 7.4 describes each

of the commands AR68 supports.

When AR68 processes a command, it creates a temporary file called AR68.TMP,

which  it  uses  as  a  scratch  pad.  After the operation is complete AR68

erases AR68.TMP.  However, AR68.TMP  is  not  always  erased  if  an  error

occurs.    If this occurs, erase AR68.TMP with the ERA command and refer to

Appendix D for error messages output by AR68.

7.4  AR68 COMMANDS AND OPTIONS

This  section  describes  AR68  commands  and  their  options.     Examples

illustrate  the effect and interaction between each command and the options

it supports.

7.4.1  The D Command

The D command deletes from the library one or more object modules specified

in the command.  The D command supports the following option:

V          Lists  the  modules  in  the  library  and  indicates which

modules are retained and deleted by the D command.   The  V

option  precedes  modules  retained in the library with the

lowercase letter c and modules  deleted  from  the  library

with the lowercase letter d as follows:

{a}ar68 dv myrah.arc orc.o

c red.o

c blue.o

d orc.o

c white.o

{a}

The  D  command  deletes  the module ORC.O from the library

MYRAH.ARC.  In addition  to  listing  the  modules  in  the

library,  the V option indicates which modules are retained

and deleted.

7.4.2  The R Command

The R command creates a library when the one specified in the command  line

does  not exist, or replaces or adds object modules to an existing library.

You must specify one or more object modules.

You can replace more than one object module in the  library  by  specifying

the  module  names in the command line.  However, when the library contains

two or more modules with the same name, AR68 replaces only the first module

it finds that matches the one specified in the command line.  AR68 replaces

modules already in the library only if you specify their names prior to the

names  of  new  modules  to  be  added to the library.  For example, if you

specify the name of a module you want replaced after the name of  a  module

you  are  adding  to  the library, AR68 adds both modules to the end of the

library.

By default, the R command adds new modules to the end of the library.   The

R command adds an object module to a library if:

*   The object module does not already exist in the library.

*   You specify the A option in the command line.

*   The  name  of  the  module  follows  the name of a module that does not

already exist in the library.

The A option indicates where AR68 adds modules to the library.  You specify

the relative position by including the OPMOD parameter with the A option.

The  R  command  also supports the V option, which lists the modules in the

library and indicates the result of the operation performed on the library.

Both the A and V options are described below.

A           The A option adds one or more object modules following the

module specified in the command line:

{a}ar68 rav sdav.o myrah.arc work.o mail.o

c much.o

c sdav.o

a work.o

a mail.o

c less.o

The RAV command adds the object modules WORK.O  and  MAIL.O

after  the  module  SDAV.O in the library MYRAH.ARC.  The V

option, described below,  lists  all  the  modules  in  the

library.   New modules are preceded by the lowercase letter

a and existing modules are preceded by the lowercase letter

c.

V          The  V  option  lists the object modules that the R command

replaces or adds.

{a}ar68 rv jnnk.man nail.o wrench.o

c saw.o

c ham.o

r nail.o

c screw.o

a wrench.o

{a}

The R command replaces the object module  NAIL.O  and  adds

the  module WRENCH.O to the library JNNK.MAN.  The V option

lists object modules in the  library  and  indicates  which

modules  are replaced or added.  Each object module that is

replaced is preceded with the lowercase letter r  and  each

one that is added is preceded with the lowercase letter a.

7.4.3  The T Command

The  T  command  requests  that AR68 print a table of contents or a list of

specified modules in the library.  The T command prints a table of contents

of  all modules in the library only when you do not specify names of object

modules in the command line.  It supports the following option.

V          The V option displays the size of each file in the table of

contents as shown in the following example.

{a}ar68 tv wine.bad

rw-rw-rw- 0/0    6818 rose.o

rw-rw-rw- 0/0    2348 white.o

rw-rw-rw- 0/0     396 red.o

{a}

The  T  command  prints  a table of contents in the library

WINE.BAD.  In  addition  to  listing  the  modules  in  the

library,  the  V  option  requests the size of each module.

The character string rw-rw-rw- 0/0 that precedes the module

size  is  meaningless  for GEM DOS. However, if the file is

transferred to  a  UNIX...  system,  the  character  string

denotes  the  file  protection  and  file  owner.  The size

specified by the decimal number that  precedes  the  object

module name indicates the number of bytes in the module.

7.4.4  The W Command

The  W  command  writes  a  copy  of an object module in the library to the

>filespec parameter specified in the command line.  This command allows you

to  extract  a copy of a module from a library and rename the copy when you

write it to another disk, as shown below.  For this command, the  >filespec

parameter is required.

{a}ar68 w go.arc now.o > b:\root\newd\file.o

The  W  command  writes  a copy of the object module NOW.O from the library

GO.ARC to the file FILE.O in the NEWD subdirectory on drive B.

7.4.5  The X Command

The X command extracts a copy of one or more object modules from a  library

and writes them to the default disk.  If no object modules are specified in

the command line, the X command extracts a  copy  of  each  module  in  the

library. The X command supports the following option.

V          The  V  option  lists  only  those  modules  the  X command

extracts from the library.    It  precedes  each  extracted

module with the lowercase letter x as follows:

{a}ar68 xv jnnk.man saw.o ham.o screw.o

x saw.o

x ham.o

x screw.o

7.5  AR68 ERRORS

When  AR68  incurs  an  error  during  an  operation,  the operation is not

completed.  The original library is not modified  if  the  operation  would

have  modified  the  library.  Thus, no modules in the library are deleted,

replaced, added, or extracted.  Refer to  Appendix  D  for  error  messages

output by AR68.

When  you  specify  the >filespec parameter in the command line to redirect

the output and one or more errors occur, the error messages are sent to the

output  file.    Thus,  you  cannot detect the errors without displaying or

printing the file to which the output was sent.  If  the  contents  of  the

output  file  is  an object file (see the W command), you must use the DUMP

utility described in Section 8 to read any error messages.

End of Section 7

MORE PROGRAMMING UTILITIES

8.1  INTRODUCTION

This section describes the DUMP,  SIZE68,  SENDC68,  and  XREF  programming

utilities.    DUMP  displays the contents of files in hexadecimal and ASCII

notation.  SIZE68 displays the total size of a memory  image  command  file

and  the  size  of  each  of  its  program segments.  XREF produces a cross

reference symbol table for GEM DOS object files.  SENDC68 creates a file of

Motorola  S-records  from  a  command  file.    (Refer  to Appendix E for a

detailed description of the S-record format.)

8.2  DUMP UTILITY

The DUMP utility (DUMP) displays the contents of a GEM  DOS  file  in  both

hexadecimal  and  ASCII  notation.  You can use DUMP to display any GEM DOS

file regardless of the format of its contents (binary data, ASCII text,  an

executable file).

8.2.1  Invoking DUMP

Invoke  DUMP  by  entering a command with the following input components in

the following format:

DUMP [ -sxxxx ] filename1 [ >filename2 ]

Table 8-1 lists the DUMP command line components and their meanings.

Table 8-1.  DUMP Command Line Components

Component Meaning

-sxxxx    xxxx is an optional offset (in hexadecimal) into the  file.

If  specified, DUMP starts dumping the contents of the file

from the byte-offset xxxx and continues until  it  displays

the  contents  of the entire file.  By default, DUMP starts

dumping the contents of the file from the beginning of  the

file until it dumps the contents of the entire file.

filename1 Name of the file you want to dump.

>filename2

The greater than sign (>) followed by a filename or logical

device  optionally  redirects  the output of DUMP.  You can

specify any valid GEM DOS  specification,  or  one  of  the

logical device names, CON: (console) or LST: (list device).

If you do not specify this optional parameter,  DUMP  sends

its output to the console.

8.2.2  DUMP Output

DUMP  sends  its  output  to  the  console  (or  to  a  file  or device, if

specified), 8 words per line, in the following format:

rrrr oo (ffffff):  hhhh hhhh hhhh hhhh hhhh hhhh hhhh hhhh

*aaaaaaaaaaaaaaaa*

The components of a DUMP command line are as follows:

Component       Meaning

rrrr            Record number (GEM  DOS  records  are  128  bytes)  of  the

current line of the display.

oo              Offset  (in  hex  bytes)  from the beginning of the GEM DOS

record.

ffffff          Offset (in hex bytes) from the beginning of the file.

hhhh            Contents of the file displayed in hexadecimal.

aaaaaaaa        Contents of the file displayed as ASCII characters.  If any

character is not representable in ASCII, it is displayed as

a period (.).

8.2.3  DUMP Examples

In the following example, DUMP is invoked to  display  the  contents  of  a

command file that contains data in both binary and ASCII form.

{a}dump dump.68k

0000 00 (000000):  601a 0000 1b34 0000 011d 0000 0e5e 0000

*`....4.......^..*

0000 10 (000010):  0000 0000 0000 0000 0900 ffff 6034 4320

*............`4C *

0000 20 (000020):  5275 6e74 696d 6520 436f 7079 7269 6768 *Run-

time Copyright*

0000 30 (000030):  7420 3139 3832 2062 7920 4469 6769 7461 *t

1982 by Digita*

0000 40 (000040):  6c20 5265 7365 6172 6368 2056 3031 2c30 *l

Research V01.0*

0000 50 (000050):  3320 206f 0004 2268 0018 2649 d3e8 001c *3

o.."h..&ISh..*

. . . . (and so on) . . .

8.3  SIZE68 UTILITY

The SIZE68 utility (SIZE68) indicates if the program segments within one or

more command files are contiguous or non-contiguous, displays the  size  of

each program segment and the symbol table, and reports if the command files

are relocatable or non-relocatable.    SIZE68  displays  both  decimal  and

hexadecimal  values  for  the  sizes of the program segments and the symbol

table.  GEM DOS command files usually have a filetype of .PRG or .REL.  The

total  size of a command file's segments returned by SIZE68 and the size of

a command file returned by the DIR command are not equal.   The  file  size

returned  by  SIZE68  includes  the size of the text, data, and bss program

segments and the size of the symbol table but does not include the size  of

the  header  and and relocation bits.  For more details on the DIR command,

refer to the GEM DOS User's Guide.

8.3.1  Invoking SIZE68

Invoke SIZE68 by entering a command line with the following format:

SIZE68 filename [filename2 filename3, ... ] [ >outfile ]

The SIZE68 command line components have the following meaning:

Component       Meaning

filename        File specification  of  a  file  whose  size  you  want  to

determine.

filename2

filename3 ....

One  or  more additional file specifications of files whose

sizes you want to determine. SIZE68  can  process  multiple

files, provided the command line does not exceed 128 bytes.

Note that SIZE68 also accepts wildcard file specifications.

>outfile        Specifies the file specification to which SIZE68 sends  its

output.      If   you   do   not  specify  an  output  file

specification, SIZE68 sends the output to the console.  For

the  output file specification, you can specify a valid GEM

DOS filename, or one of  the  logical  device  names,  CON:

(console) or LST: (list device).

8.3.2  SIZE68 Examples and Output

This  section  contains  two  examples  that  show SIZE68 command lines and

output.

1.  The following SIZE68 command line example returns information about the

program segments in one command file.

{a}SIZE68 SIZE68.PRG

SIZE68.PRG:

Contiguous

.text length            =   9312        2460

.data length            =   1178         49A

.bss length             =   9140        23B4

Symbol table length     =      0           0

Start of .text          =      0           0

File is relocatable.

SIZE68.PRG  contains  a  9312-byte  (decimal) text segment, a 1179-byte

(decimal) data segment, and a 9140-byte (decimal) bss; the segments are

contiguous  and  SIZE68  is relocatable.  Hexadecimal notations for the

decimal values are displayed in the last column of SIZE68 output.

2.  The following SIZE68 command line uses a wildcard file specfication  to

return information on an object file, a command file, and a text file.

{a}SIZE68 FI*.*

FIND.O:

Contiguous

.text length            =   1072         430

.data length            =    188          BC

.bss length             =      0           0

Symbol table length     =   1708         6AC

Start of .text          =      0           0

File is relocatable.

FIND.PRG:

Contiguous

.text length            =   9888        26A0

.data length            =   1060         424

.bss length             =   9396        24B4

Symbol table length     =      0           0

Start of .text          =      0           0

File is relocatable.

FILE.MSG:

Not a program file.

Notice  that  when  you  specify  a  file  that  is  not a command file

(FILE.MSG, an ASCII file, for example), SIZE68 displays:

Not a program file.