Newsgroups: comp.sys.atari.st.tech

Subject: STe Programming info (long)

Date: 13 Feb 91 10:52:16 GMT

Organization: Computing Laboratory, U of Newcastle upon Tyne, UK NE17RU

I notice that alot of people have been clammering for information on the

STe recently. Some time ago a Mathew Lodge of ST world (UK mag) wrote

the following. This information has appeared here before in 3 parts and

has been published by various people in various magazines.

It should be noted that you should not hold this information to be

gosble and if you wan't to be sure of correct information get the

Developers kit. So if there are any errors don't blame Atari or Mathew

lodge but yourself.

Now there is no excuse for not writing STe specific SW so how about some

nice Demos.

Dave Halliday

(D.C.Halliday@newcastle.ac.uk)

---------------- Mathew Lodge's Original Posts Follow -------------

OK, here is the documentation for STE DMA sound output. More documentation

when I've typed it in. Screen blanker postings when I've got a copy of uuencode

... please be patient

----------------------- cut here ----------------------------------

Information from "Atari STE Developer Information Addendum"

STE DMA Sound registers

Register    Access      Description

FF8900      R/W      00 - sound disabled (reset state)

01 - sound enabled, disable at end of frame

11 - sound enabled, repeat forever

FF8902      R/W      Frame base address (high)

FF8904      R/W      Frame base address (middle)

FF8906      R/W      Frame base address (low)

FF8908      RO       Frame address counter (high)

FF890A      RO       Frame address counter (middle)

FF890C      RO       Frame address counter (low)

FF890E      R/W      Frame end address (high)

FF8910      R/W      Frame end address (middle)

FF8912      R/W      Frame end address (low)

FF8920      R/W      Sound mode control:

xxxx xxxx m000 00rr

Where for m:

0 Stereo mode

1 Monophonic mode

Where for rr:

00  6258 Hz sample rate (reset state)

01 12517 Hz sample rate

10 25033 Hz sample rate

11 50066 Hz sample rate

FF8922      R/W      MICROWIRE data register

FF8924      R/W      MICROWIRE mask register

Volume/controller commands (device address is always 10)

--------------------------------------------------------

011 DDD DDD Set master volume

000 000 -80 dB

010 100 -40 dB

101 XXX   0 dB

101 xDD DDD Set left channel volume

00 000 -40 dB

01 010 -20 dB

10 1xx   0 dB

100 xDD DDD Set right channel volume

00 000 -40 dB

01 010 -20 dB

10 1xx   0 dB

010 xxD DDD Set treble

0 000 -12 dB

0 110   0 dB

1 100 +12 dB

001 xxD DDD Set bass

0 000 -12 dB

0 110   0 dB

1 100 +12 dB

000 xxx xDD Set mix

00 -12 dB

01 Mix GI sound output (ST sound chip)

10 Do not mix GI sound output

11 Reserved

----------------------------------------------------------

Sampled sound data is stored in memory as a series of bytes, which

represent a speaker displacement from -128 to +127. Zero represents the

neutral or middle speaker position. Playback is programmable at one of

four rates : 50kHz, 25kHz, 12.5kHz or 6.25kHz.

During the horizontal blanking phase, samples are fetched from memory by

the DMA sound chip, and fed into a Digital to Analogue Converter (DAC).

The output of the DAC is then filtered by a four-pole low pass filter to

a frequency equal to around 40% of the sample frequency. The signal then

passes through a two pole 16kHz low-pass filter, and fed into the

National Semiconductor Volume/Tone controller (LMC1992). The final

output is available from the RCA jacks on the back of the STE, which can

be fed into an amplifier and hence to speakers, headphones etc.

Both stereo and mono sample replay is provided, but both stereo channels

are mixed along with the ST's sound chip output for monitor speaker

output. Sound chip output can also be sent to the stereo output jacks as

well.

In stereo playback mode, the same data is regarded as words, with the

high byte of the word being the sample for the left channel, and the low

byte the right channel sample. In mono mode, each byte is output to both

left and right stereo channels, but data is still fetched one word at a

time. This means that mono sample data must always be an even number of

bytes.

Samples are grouped together in frames. Each frame can be played once,

or repeated automatically forever (until stopped). Two registers are

loaded with the frame start and end address - the end address is

actually the first byte beyond the end of the sample. Thus a 512 byte

sample with a frame start address of 101024 would have a frame end

address of 101536. Table One gives the location and description of each

DMA sound register.

Actually playing a sample is really quite straightforward. Simply

assemble the data in memory, load the start and end addresses, set

stereo or mono mode and the playback frequency. Finally, write a one to

the sound control register, and the sample will play once.

Producing continuous sound and linking frames together are the next

steps, and hardware support is provided for these processes. The DMA

sound chip produces a 'DMA sound active' signal which is connected to

the external input of MFP Timer A. This signal is a one when samples are

being played, and zero otherwise. At the end of a repeated frame, this

line goes from one to zero, and then back to one again. Thus setting

Timer A into event countdown mode allows you to generate an interrupt

when a frame has been played a set number of times.

Frame repetition is seamless - there is no time delay between the end of

a sample, and the start of it's replay, because the frame start and end

registers are double buffered. Writing to these registers actually

places the data into a holding area, and the contents of the holding

area actually go into the true registers when the chip is idle (at the

end of the frame, if one is currently being played).

Thus, if you wanted to play two consecutive frames, you would write the

start and end addresses, and set the control register to three. The

first frame will start playing, and you can immediately write the start

and end addresses of the next frame, without waiting for the first frame

to finish. There will still be an interrupt from Timer A at the end of

the first frame, and you could use that to load the address of a further

frame, and so on.

One further thing to note is that the 'DMA sound active' signal is also

exclusive-ORed with the 'monochrome monitor detect' signal, and fed into

the GPIP I7 input of the MFP. This was provided to enable interrupt

driven sound without using the last free timer of the MFP. It is a

little more difficult to use, since you will get a different signal edge

depending on whether a mono or colour monitor is attached, as well as an

interrupt at the end of every frame.

Monochrome monitors ground the 'mono detect' line, resulting in a zero

when the bit is read from the MFP. Colour monitors don't ground the line

(it is left floating), and the bit reads one. When DMA sound is active,

this situation is inverted (because of the XOR with the 'DMA sound

active line'). TOS actually looks at this bit during vertical blank

time, to see if the monitor has been changed, but TOS on any machine

with the DMA sound chip has been appropriately modified to avoid

problems.

Finally, the 'DMA sound active' line goes from active to idle (one to

zero) after the last sample has been fetched. There is a four-word FIFO

(First In, First Out) buffer inside the chip, so it will be eight sample

times (in stereo mode) before the sound actually finishes. If you do not

reload the frame registers in this time, then the join between samples

will not be seamless.

The volume and tone controller of the STE is connected via a MICROWIRE

bus interface. The idea behind this is that further devices can be added

to the bus in the future. The MICROWIRE bus is a simple three wire

serial connection, with a protocol to allow multiple devices to be

controlled individually.

In the general case, the data stream consists of N address bits,

followed by zero or more don't care bits, and then M bits of data. The

actual hardware interface in the STE consists of two 16 bit read/write

registers, one for the data to be shifted out, and a mask indicating

which bits are valid.

A one in any bit of the mask indicates that the corresponding bit in the

data register is valid. Data transmission starts as soon as the data

register has been written to, so the mask register must be loaded first.

Sending takes approximately sixteen micro-seconds, and if the data

register is read during this time, a 'snap-shot' of the data being

shifted out will be obtained. This means that if you wait for either

register to return to its original state, you can be sure that sending

has been completed.

The volume/tone controller is addressed by a two bit address field of

%10 (binary) and a nine bit data field. Table One details the commands

that can be sent to the device, and the addresses of the MICROWIRE

registers in the STEs memory map. Actually sending these commands is

easier than it looks. Simply set the mask register to $07FF, and place

the data in the lower nine bits with %10 in the upper two bits.

For example, setting the mask to $07FF and the data register to $04C4

will set the master volume to $14. That's all there is to it!

Regards,

Mathew Lodge

***********************************************************************

* c/o Dept. Computer Science  * "Baldrick, fetch me a turkey _so    *

* University of York    * big_, you'd have thought its mother *

* Heslington         * had been rodgered by an Omnibus"    *

* York, UK        *                 *

* YO1 5DD         * JANET : SOCS18@uk.ac.york.vaxa      *

***********************************************************************

Video Harware Modifications

===========================

Addr     Access Size    Use

======   ====== ====    =================================================

FF8204    R/W    6      Video address counter (high)

FF8206    R/W    8      Video address counter (middle)

FF8208    R/W    8      Video address counter (low)

The change here is that these registers are now

read and write, allowing the programmer to

update the video refresh address to any word

boundary _at any time_.

FF820C    R/W    8      Video base address (low) (VBASELO) This register

didn't exist on the ST, but on the STE it allows

the positioning of the screen base address on

any _WORD_ boundary

FF820E    ??     8      Line offset (LINEWID) - the number of extra words

added to the address counter at the end of each

line, _MINUS ONE DATA FETCH_. This allows for

virtual screens that are wider than the actual

screen display. Clearing this register means the

STE acts like an ordinary ST.

FF8240 to FF825E        Colour palette

There are now four bits for each of the red,

green and blue components. To give backward

compatibility with the ST, the least significant

bit is above the most significant bit. Thus the

register layout is as follows:

xxxx 0321 0321 0321;  x=don't care

RED  GRN  BLUE

FF8264    ??    4       Horizontal bit-wise scroll (HSCROLL). Allows the

start of each line to be delayed by 0-15 bits,

thus giving instant horizontal scrolling.

Horizontal fine scrolling isn't quite as trivial as it looks. The pixel offset

is loaded into HSCROLL, and the documentation then says the following about

the LINEWID register :

"If you are actively scrolling (HSCROLL<>0), this register should

contain the additional width of the display line _minus one data fetch_

(in low resolution one data fetch would be four words, one word for

monchrome etc.)"

The reason for the extra data fetch becomes clearer if you think about

what is actually happening. If you fine scroll by n bits, then n bits

are effectively missed off the left hand edge of the screen. But to get

a complete line of pixels, n bits must be added to the right hand side

of the screen. This constitues one extra data fetch for the display

processor beyond the usual requirement.

For example, if you had three low resolution pictures side by side in

memory, and you wanted to scroll across them, you would set LINEWID to

160 when HSCROLL=0 (no extra data fetch). When HSCROLL<>0, LINEWID would

be set to 156 (four less due to the extra data fetch done automatically

by the display processor).

Vertical scrolling is trivial - simply set the video address base to the

required address at horizontal blanking time.

That's all folks - for now - more info on the new controller ports soon.

Regards,

Mathew Lodge

***********************************************************************

* c/o Dept. Computer Science  * "Baldrick, fetch me a turkey _so    *

* University of York    * big_, you'd have thought its mother *

* Heslington         * had been rodgered by an Omnibus"    *

* York, UK        *                 *

* YO1 5DD         * JANET : SOCS18@uk.ac.york.vaxa      *

***********************************************************************

STE CONTROLLER PORTS

Address Usage                Description

=================================================

FF9200  xxxx xxxx xxxx 3210  Fire buttons

FF9202  UDLR UDLR UDLR UDLR  Joystick directions

Joy3 Joy2 Joy1 Joy0

FF9210  xxxx xxxx NNNN NNNN  Paddle 0 X

FF9212  xxxx xxxx NNNN NNNN  Paddle 0 Y

FF9214  xxxx xxxx NNNN NNNN  Paddle 1 X

FF9216  xxxx xxxx NNNN NNNN  Paddle 1 Y

FF9220  xxxx xxNN NNNN NNNN  Light pen/gun X

FF9222  xxxx xxNN NNNN NNNN  Light pen/gun Y

Four new joystick ports were added, directly interfaced to the 68000

bus, rather than using another IKBD controller. The table above gives the

locations of the hardware registers for the joysticks, paddles and light

gun/pen inputs. Note that the joystick registers are bidirectional, and

can therefore be written as well as read. If the registers are  written,

then the outputs are driven until the registers are read again.

Two paddle controllers can be connected with eight bit resolution on

each axis, and the current position of each of the four paddles are

reported in the registers shown in the table. No information is given in

the data sheets as to the voltage range that the A/D converters function

in, but I would guess that it is 0-5V. The triggers on the paddles are

bits zero and one of $FF9202 (Joystick 0 left and right.)

The light gun/pen position is reported in $FF9220 and $FF9222. The

x-axis accuracy is four pixels in low resolution, eight pixels in medium

res., and sixteen pixels in high resolution. The y-axis position is

accurate to within one pixel in all modes. The x position only ranges

from 0 to 319, and therefore must be left shifted for other screen

modes. Again, no electrical information is provided on how to

implement a light gun/pen.

Regards,

Mathew Lodge

***********************************************************************

* c/o Dept. Computer Science  * "Baldrick, fetch me a turkey _so    *

* University of York    * big_, you'd have thought its mother *

* Heslington         * had been rodgered by an Omnibus"    *

* York, UK        *                 *

* YO1 5DD         * JANET : SOCS18@uk.ac.york.vaxa      *

***********************************************************************