02200000000801

1

2

9[...............................................................]0

MC 68000 ASSEMBLY LANGUAGE COURSE PART VIII by Mark van den Boer

Program Control Instructions

This  class of instructions enables a programmer to create  loops

and  IF-THEN-ELSE  like  decisions.  That's  why  it's  the  most

important group of instructions and every programmer should  have

a  thorough knowledge of this group.  This class of  instructions

are specifically meant to affect the program counter.

Instructions:  Bcc  (cc stands for Condition Code)

Syntax:        Bcc  <address>

Data sizes:    Byte  or word.  This implicates that  the  branch-

instructions  can branch in an area of  32K.  When

using a branch with a byte offset you can put a .S

suffix behind the instruction e.g.  BEQ.S  .  When

using a branch with a word offset you can put a .W

suffix behind the instruction e.g.  BEQ.W  .  Most

assemblers  will  determine if the short  or  word

form is needed. Also most assemblers will optimize

word-branches to byte-branches whenever possible.

Condition codes affected:

None

Function: Test  a  combination of the NZVC-flags in  the  status-

register and conditionally perform a branch to  another

address. If the testing of the condition codes is true,

then  the  branch  will be  taken,  in  the  other  the

instruction  immediately following the Bcc  instruction

will  be executed. A total of 15 possible variations of

this instruction are listed below.

BCC: where  CC stands for Carry Clear.  The  branch  is

taken if the C-bit is 0. This instruction is often

used   in  combination  with  shift   and   rotate

instructions.

BCS: where CS stands for Carry Set. The branch is taken

if  the  C-bit  is  1.  This  instruction  is  the

counterpart of the BCC-instruction.

BEQ: where EQ stand for EQual.  The branch is taken  if

the  Z-bit is 1.  This instruction is  often  used

after a TST-instruction or CMP-instruction.

BNE: where NE stands for Not Equal. The branch is taken

if  the  Z-bit  is  0.  This  instruction  is  the

counterpart of the BNE-instruction.

BPL: where PL stands for PLus.  The branch is taken  if

the  N-bit is 0.  This instruction is  often  used

after a TST-instruction or CMP-instruction.

BMI: where MI stands for MInus.  The branch is taken if

the   N-bit  is  1.   This  instruction   is   the

counterpart of the BPL-instruction.

BVC: where VC stands for oVerflow Clear.  The branch is

taken if the V-bit is 0. This instruction is often

used after an Integer Arithmetic instruction  like

ADD, SUB, MUL etc.

BVS: where  VS stands for oVerflow Set.  The branch  is

taken if the V-bit is 1.  This instruction is  the

counterpart of the BVC-instruction.

BRA: where   RA   stands  for   bRanch   Always.   This

instruction is often used at the end of a loop  to

go back to the beginning of the loop.

Branches often used after an arithmetic operation on

two's complement numbers.

BGE: where GE stands for Greater or Equal.  This branch

is  taken  if the N and V-bits  contain  the  same

value.

BGT: where GT stands for Greater Than.  This branch  is

taken in the following cases:

- N is 1, V is 1, Z is 0

- N is V is Z is 0

BLE: where LE stands for Lower or Equal. This branch is

taken in the following cases:

- Z is 1

- N and V-bits contain different values

BLT: where  LT  stands for Less Than.  This  branch  is

taken  if  the  N  and  V-bits  contain  different

values.

Brances often used after an arithmetic operation on

unsigned numbers.

BHI: where HI stands for HIgher.  This branch is  taken

if the N and V-bits contain the same value.

BLS: where LS stands for Lower or Same.  This branch is

taken  if  the  C  and  Z-bits  contain  different

values.

Example:

This  shows  a piece of a C-program and  an  equivalent

piece  of  a PASCAL-program which are  translated  into

assembler. (variabele is signed)

C:

if (variable == 1 || variable > 4) variable = 5;

else var *= 3;

PASCAL:

if (variable == 1) or (variable > 4)

then variable := 5

else variable := variable * 3

* Most assemblers will optimize the branch-instructions

* to the short forms

CMP.W   #1,variable

BEQ     L10000

CMP.W   #4,variable

BLE     L2

L10000:

MOVE.W  #5,variable

BRA     L3

L2:

MOVE.W  variable,R0

MULS    #3,R0

MOVE.W  R0,variable

L3:

Instructions:  DBcc  (cc stands for Condition Code)

Syntax:        DBcc  Dn,<address>

Data sizes:    byte  or word.  This implicates that  the  branch-

instructions can branch in an area of 32K.  Dn  is

considered to contain a word.

Condition codes affected:

None

Function:

The  group of Decrement and Branch (DBcc)  instructions

provide  an efficient way of creating loops.  They  are

nearly  always placed at the end of a loop.  First  the

condition   is  tested,   then  the   dataregister   is

decremented.  The  branch  is taken  in  the  following

cases:

- Dn is -1;

- The condition cc in DBcc is satisfied.

There  are 16 possible variations of this  instruction.

They  all are nearly the same as the  Bcc-instructions,

with two exceptions. These are:

DBF or DBRA:

This  loop can only be terminated by  count  since

the other condition can never be satisfied.

DBT: Only performs a decrement on the dataregister  and

never branches.  To me this seems a pretty useless

instruction,  which is only there to make the DBcc

series logically complete.

Example:

This  piece of code is an efficient  implementation  of

the strcpy-function of the C-language.  A0 contains the

address  of  the  source string  and  A1  contains  the

address  of the destination string.  In C the end of  a

string is marked by a byte containing 0.

MOVE.W  #$ffff,D0

LOOP:  MOVE.B  (A0)+,(A1)+

DBEQ    D0,LOOP

This  piece of code can easily be transformed into  the

strncpy-function  by  loading D0 with  the  appropriate

value.

Instructions:  Scc  (cc stands for Condition Code)

Syntax:        Scc  <address>

Data sizes:    byte.

Condition codes affected:

None

Function: Sets a byte to $ff if the condition codes satisfie.  If

the  condition is not satisfied the byte is set  to  0.

This  group of 16 instructions is rarely  used.  Nearly

all forms are the same as the DBcc group except for the

following two instructions:

SF:  the same as a CLR.B instruction

ST:  the same as a MOVE.B #$ff, <address>

Example:

Be inventive, invent one yourself!

Instruction:   BSR, JSR

Syntax:        BSR  <address>

JSR  <address>

Data sizes:    none

Condition codes affected:

none

Addressing modes allowed (only for JSR):

Destination:

(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

Function: The  BSR  (Branch  to  SubRoutine)  and  JSR  (Jump  to

SubRoutine)   instructions   are   used   for   calling

subroutines.  BSR  can branch in a range  of  32K.  JSR

should  be  used when a jump out of the  32K  range  is

needed.   Some   assemblers  optimize  JSR   into   BSR

instructions  whenever  possible,  since  BSR  is  more

efficient   than   JSR.   When  executing   a   BSR/JSR

instruction,  the  68000 first pushes the PC  (program-

counter) on the stack and then load the PC with the new

address. See below for the RTS (ReTurn from Subroutine)

instruction.

Instruction:   RTS

Syntax:        RTS

Data sizes:    none

Condition codes affected:

none

Function: Counterpart  of BSR/JSR instructions.  Reloads  the  PC

with  the value on top of the stack.  This  value  will

nearly  always have been put on top of the stack  by  a

BSR/JSR instruction.

Example:  * the strcpy function discussed before

STRCPY:

MOVE.W  #$FFFF,D0

LOOP:  MOVE.W  (A0)+,(A1)+

DBEQ    D0, LOOP

RTS

* some other code

BEGIN:

MOVE.L  #SOURCE,A0

MOVE.L  #DEST,A1

JSR     STRCPY

RTS

* the strings are put in a data area

.DATA

* 80 bytes for every string

SOURCE .DS.B   80

DEST   .DS.B   80

* .DS.B means Define Storage Byte

* so 80 bytes are define as storage for each string

Instruction:   JMP

Syntax:        JMP  <ea>

Data sizes:    none

Condition codes affected:

none

Addressing modes allowed:

Destination:

(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

Function: Transfer program control to another address.  The PC is

loaded  with the specified address.  In fact this is  a

variant  of  the MOVE instruction.  In  this  case  the

destination register is inherently defined,  namely the

PC-register.  Therefore we could translate JMP <ea>  to

MOVE.L <ea>,PC .

Instruction:   RTR

Syntax:        RTR

Data sizes:    none

Condition codes affected:

none

Function: ReTurn    and   Restore.    Counterpart   of    BSR/JSR

instructions.  Reloads the PC with the value on top  of

the stack.  This value will nearly always have been put

on top of the stack by a BSR/JSR instruction.  The only

difference  with the RTS is that with this  instruction

also  the CCR is reloaded.  This instruction is  rarely

used  but  comes  in  handy when  one  doesn't  want  a

subroutine to influence the condition codes. Before the

JSR instruction you should use the instruction:

MOVE.B CCR,-(A7)

which pushes the CCR on the stack

Next time:  The last part of the instruction set.  These are  the

instructions  which can only be executed when supervisor-mode  is

active.

Originally published in ST NEWS Volume 2 Issue 8.