02200000000801

1

2

9[...............................................................]0

ASSEMBLY LANGUAGE COURS PART IX by Mark van den Boer

System Control instructions

In this part of the course the last group of instructions will be

explained.  This group of instructions deals with the supervisor-

mode  and are therefore sometimes referred to as  system  control

instructions.  To  remind  you:  the  S-bit  in  the  SR  (status

register)  of  the  68000  determines whether  the  68000  is  in

supervisor-mode or not.  Many of these instructions deal with  so

called  exceptions.  Exception is another word for interrupt  and

these are used to force program control immediately to a specific

routint exception handler routine.  Exceptions are used to detect

situations  that  are  urgent and need to  be  handled  directly.

Therefore  every  exception has a vector  assigned  to  it.  This

vector is a pointer to a routine which performs some action which

should  be  taken when such an exception  occurs.  The  exception

vectors are located in the first 256 longwords of memory.

Instruction:   CHK

Syntax:        CHK <ea>,Dn

Data sizes:    word

Condition codes affected:

X    not affected

N    Set if Dn is less than zero,  cleared if <ea>

less than Dn, in all other cases undefined

Z

V

C    always undefined

Addressing modes allowed:

Source:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

#

Destination:

Dn

Function: Compares the contents of the effective address  operand

with  the data-register.  If the data register is  less

than zero (the data register is always considered to be

a  signed word) or greater than the contents  of  <ea>,

then   an  exception  occurs.   The  pointer  to   this

exception-routine  is  located  at  address  $18.  This

instruction  is  used to check if a  data  register  is

within  a  range.   It  is  often  used  by  high-level

languages   such  as  PASCAL  to  perform   array-bound

checking.

Examples:

Instruction              Before              After

CHK #50,D0               D0=45               D0=45

No exception occured, if D0 had been 51

or greater then an exception would have

occured.

Instruction:   MOVE USP       (privileged instruction)

Syntax:        MOVE USP,An    or   MOVE An,USP

Data sizes:    long

Condition codes affected:

X

N

Z

V

C    not affected

Addressing modes allowed:

See syntax

Function: As  you all should know,  the 68000 has in fact two  A7

registers.  One A7 register is used when in supervisor-

mode,  the  other  when in usermode (this  is:  not  in

supervisor-mode.   It  is  sometimes  desirable  for  a

program  which is executing in supervisor mode to  know

the value of the usermode A7-register. This instruction

provides  a  way to obtain and change the value  of  A7

usermode-register.

Example:

Instruction              Before              After

MOVE USP,A6              A7user=$12345678    A7user=$12345678

A6    =$00000000    A6    =$12345678

A7sup =$87654321    A7sup =$87654321

Instruction:   RESET

Syntax:        RESET          (privileged instruction)

Data sizes:    none

Condition codes affected:

X

N

Z

V

C    not affected

Function: Reset all external devices. A device can be a chip like

the 6850.

Instruction:   RTE            (privileged instruction)

Syntax:        RTE

Data sizes:    none

Condition codes affected:

none

Function: Every exception is terminated by this  instruction.  It

can be compared to RTS. The only difference is that RTE

will  restore  the  SR  in  addition.   Note  that   an

exception-routine   has  the  responsibility  to   save

registers if this is important.

Instruction:   STOP           (privileged instruction)

Syntax:        STOP #

Data sizes:    word

Condition codes affected:

All set as a direct result of the operand

Addressing modes allowed:

Source:

#

Function: Stop execution of a program until an exception  occurs.

The  operand  stored  in the SR.  Note  that  with  the

operand  a minimum interrupt level can  be  determined.

With  this  instruction it is possible to  wait  for  a

videochip interrupt to occur.

Example:

STOP #%0010011000011111

Wait  for  an exception with a priority of 6  or  7  to

occur and set the XNZVC-bits.

Instruction:   TRAP

Syntax:        TRAP #

Data sizes:    # must be >=0 and <=15

Condition codes affected:

X

N

Z

V

C    not affected

Function: This  instruction generates an exception.  The  operand

indicates  an  exception number.  The vectors  for  the

exceptions  are located at addresses $80 to  $BC.  This

instruction  is  mainly  used to  allow  programs  that

execute in user-mode to call supervisor-mode  routines.

This  way  a  user can be given a  number  of  specific

functions. In the ST trap vectors 2, 14 and 15 are used

for GEM,  BIOS and XBIOS functions.  In the case of the

ST  the TRAP-instructions is preceded  by  instructions

that  put  function numbers and  parameters  for  these

functions  on  the stack.  This way it is  possible  to

assign groups of functions to one trap-vector.

Note  that  when  calling  a  TRAP  in  user-mode   the

stackpointers  change (supervisor-mode  and  user-mode,

remember??). Thus, the MOVE USP instruction can be used

to retrieve parameters that had been put on the stack.

Instruction:   TRAPV

Syntax:        TRAPV

Data sizes:    none

Condition codes affected:

none

Function: When  the  V-bit  is  set  an  exception  occurs.   The

exception vector is located at address $1C. When the V-

bit is clear nothing happens.  This instruction can  be

used by high-level languages to inform the user that an

overflow error has occured.