Degas decompression

Save valuable disk space by storing

screens in compressed Degas Elite format

Each month cover disk submissions arrive by the sackload and poor old postman

Pat's back is under a tremendous strain. The overwhelming majority of submitted

programs are superb and if it were possible we would love to use them all.

However, the storage capacity of ST disks is limited to around 800k which

means that only quite small programs can be squeezed into the limited space

available. Unfortunately, the remaining submissions, excellent though they

often are, must be rejected simply because we haven't enough space for them. We

wish we had more, but we're stuck with the ST's 800k disks.

(It's interesting to note that a few years ago 800k disks would have seemed

increadably large, but over the years programs have steadily grown in size and

complexity and a 16k word processor or spreadsheet - such as View and Viewsheet

on the BBC Micro - seems unthinkable now!)

One of the reasons why programs take up such a lot of disk space is that

they require one or more graphic screens. These are most commonly stored as

either Neochrome or Degas screens which occupy over 32k of disk space. A

typical game may require a loading screen which is displayed while the program

is booting up and a title/menu/main screen which shows the credits, options and

prompts you to press fire to start.

Since each screen is 32k long, requiring just two of these means that 64k

of disk space is consumed, even before you start writing a line of program

code! What is needed is a way of reducing the amount of space the graphics

occupy.

We can't easily do anything about Neochrome pictures, but every Degas Elite

owner will know that their art package can save screens in a highly compressed

form. In fact, this is the default Save mode.

You can tell a compressed Degas picture from its name, to be more specific,

its file extension - the name ends in PC1, PC2 or PC3. (P for picture, C for

compressed and the three numbers indicate the screen resolution: 1 is low res

colour, 2 is medium res colour and 3 is high res monochrome.)

A degas compressed screen is typically around 12k to 16k long and that is

less than half the space required for a standard screen. So if compressed

screens could be substituted for standard ones in our programs then we would

half the disk space required by the graphics.

How are Degas compressed screens stored and how can we load and decompress

them so that they can be used in our own programs? The first 34 bytes of the

picture file makes up a header that contains two flags in the first word and

the colour palette in the next 16 words.

The first byte holds the compression flag in bit seven and the second byte

holds the screen mode (zero, one or two). The result of this is that the first

word of a compressed low res picture is $8000, medium res is $8001 and

monochrome is $8002.

From byte 34 onwards is the compressed data that we'll have to decompress

before the picture can be displayed on the screen. A fairly simple, but

effective, variation on run length encoding is used, so decoding is just as

straightforward.

Read the first byte of data and if it is 128 then ignore it and go on to

the next byte. (I'm not sure why this value is required, maybe it's for padding

out an odd length file to make it even.)

If the byte - call it N - is less than 128 then copy the next N+1 bytes

straight into the screen memory as it is raw picture data. If the byte is

greater than 128 then it is a signed single byte number and the following byte

must be copied -N+1 times (subtract the byte from 256 to get the signed

number).

Here is a quick summary of the values to expect:

Byte  0... 127 = copy next n+1 bytes

Byte -1...-127 = copy next byte -n+1 times

Byte      -128 = ignore

Two complications muck up a very simple system, but they aren't too

difficult to program around. The picture data is compressed one scan line

(horizontal row of pixels) at a time and also one bit plane at a time. Let's

see what this means.

Dividing the screen into scan lines isn't a problem as we can decompress

one line at a time and copy it to the screen. Since monochrome screens only

have one bit plane we can ignore this further complication if we limit

ourselves to decompressing mono only Degas pictures.

It's time now to look at some program code and to keep the programming

simple I'll use HiSoft BASIC. Here is the main part of the code used in

SHOW_PC3.BAS in the LISTINGS folder on this month's cover disk:

OPEN "I",#1,file$                      :' open the file for input

colour$ = INPUT$(34,#1)                :' get colour palette - ignore it!

scrstart& = FNphysbase&                :' get screen start address

FOR scan% = 0 TO 399                   :' 400 scan lines (rows of pixels)

addr& = scrstart& + 80*scan%         :' get address of current scan line

nextline& = addr& + 80               :' get address of next scan line

DO

b% = ASC(INPUT$(1,#1))             :' read a byte from picture file

IF b%<128 THEN

FOR i% = 0 TO b%                 :' copy next n+1 bytes literally

POKEB addr&,ASC(INPUT$(1,#1))

addr& = addr& + 1

NEXT

END IF

IF b%>128 THEN

byte% = ASC(INPUT$(1,#1))

FOR i% = 1 TO 256-b%+1           :' copy byte% -b%+1 times

POKEB addr&,byte%

addr& = addr& + 1

NEXT

END IF

LOOP UNTIL addr&>=nextline&          :' finished scan line?

NEXT                                   :' next scan line

CLOSE #1                               :' close the file

A FOR...NEXT loop sorts out the scan line problem and the DO...LOOP

decompresses each line. The scan line start address is calculated by

multiplying the number of scan lines by 80 (because there are 80 bytes per

line) and adding on the screen start address.

Two short FOR...NEXT loops poke the decompressed data into the screen

memory. Which one is chosen depends on whether the flag byte is greater or less

than 128.

This BASIC program quite easily translates into 68000 machine code (it's

called SHOW_PC3.S in the listings folder). Here is the main loop which decodes

the 400 scan lines:

MOVE #399,D7            400 scan lines

MOVEA.L A5,A4           A5 = current scan line address

loop1   ADDA.L #80,A4           A4 = next scan line address

loop2   f_read buffer,1,in      read 1 byte

MOVE.B buffer(A6),D0    D0 = byte

CMP.B #128,D0           < 128 ?

BPL skip1

MOVE D0,D6              loop counter

ANDI #$FF,D6

loop3   f_read buffer,1,in      read one byte into buffer

MOVE.B buffer(A6),(A5)+ store byte in screen

DBRA D6,loop3

BRA next

skip1   CMP.B #129,D0           > 128 ?

BMI next

MOVE D0,D6              loop counter

NOT D6

ANDI #$FF,D6

ADDQ.L #1,D6

f_read buffer,1,in      read one byte into buffer

loop4   MOVE.B buffer(A6),(A5)+ store byte in screen

DBRA D6,loop4

next    MOVE.L A5,D0            done whole scan line?

CMP.L A4,D0

BMI loop2               back to loop2 if not done

MOVEA.L A4,A5           next scan line address

DBRA D7,loop1           done all scan lines?

Now let's take a look at how low res colour pictures can be decompressed.

There are four bit planes and each plane is compressed separately. Think of the

screen as being made up of (8,000) groups of four words. The first word in each

group belongs to the first bit plane, the second word is the second bit plane,

the third word is the third and the fourth is the fourth.

What you must do is decompress each scan line as before, but do it four

times - the first time decompressing the first word in each group, then the

second time decompressing the second word, the third time its the turn of the

third word and finally comes the fourth word.

The addition of a simple FOR...NEXT loop to our mono decompressor copes

with the extra bit planes. Each word (two bytes) is poked into the screen

memory then we skip forward over the other bit plane words in the group:

scrstart& = FNphysbase&

FOR scan%=0 TO 199

FOR plane%=0 TO 6 STEP 2

addr&=scrstart& + 160*scan% + plane%

nextline&=addr& + 160

DO

b% = ASC(INPUT$(1,#1))

IF b%<128 THEN

FOR i% = 0 TO b%

POKEB addr&,ASC(INPUT$(1,#1))

IF addr& AND 1 THEN addr&=addr&+7 ELSE addr&=addr&+1

NEXT

END IF

IF b%>128 THEN

byte% = ASC(INPUT$(1,#1))

FOR i% = 1 TO 256-b%+1

POKEB addr&,byte%

IF addr& AND 1 THEN addr&=addr&+7 ELSE addr&=addr&+1

NEXT

END IF

LOOP UNTIL addr&>=nextline&

NEXT

NEXT

CLOSE #1

Note that in low res colour each scan line occupies 160 bytes of memory and

that there are 200 lines (horizontal rows of pixels) on the screen.

What I haven't yet mentioned, is the colour palette. As this is quite

simple I have left it until last. All you need to do is read in words 1 to 16

of the header and set the 16 colours of palette according to the contents of

these words. Here's a HiSoft BASIC routine to set the palette:

OPEN "I",#1,file$

dummy$ = INPUT$(2,#1)

FOR i% = 0 TO 15

a% = 256*ASC(INPUT$(1,#1)) + ASC(INPUT$(1,#1))

dummy% = FNsetcolor%(i%,a%)

NEXT

All the programming examples are in fairly straightforward BASIC code

(apart from a little 68000) and you should be able to convert them into GFA or

Fast BASIC, or STOS.

If the decompression routine seems slow it is because of the slow ST disk

drives and it's not a result of inefficient code. To speed up the program load

the compressed picture into a buffer and decompress the data in memory.

HARDWARE: ALL STs, MONO AND COLOUR