072000202050000144000004004010000

11

2Page #1

F0110030001

9[...................................................................]1010

OVERVIEW

========

DC-PORT is a serial expansion cartridge for your Atari ST computer.  With

DC-PORT you get two more RS232 serial ports.  DC-PORT does not support the

CTS/RTS and RING DETECT lines.  If you need these, use the main ST serial

port.

INSTALLATION

============

1) Turn off your computer.

NOTE:

Always turn the POWER switch OFF when inserting or removing DC-PORT.

This will protect the electronic components and prolong the life of

your ATARI ST computer.

2) Plug the edge connector of DC-PORT into the cartridge port.

3) Plug the cable, with female DB25 connector, coming from DC-PORT into

the ST's main serial port.  The label on DC-PORT identifies this cable

as: To ST

4) Plug the 3 DC-PORT cables into DC-PORT using the end with RJ12 (phone)

jack connector.

NOTE:

DO NOT attempt to plug your telephone line directly into DC-PORT.

Doing so WILL damage DC-PORT, and void your warranty.  DC-PORT is

designed to be used with an RS232 compatible device.  The RJ12

connectors are *NOT* designed for direct connection to the telephone line.

5) Connect the male DB25 connectors to any RS232 compatible device, such as

a modem, scanner, printer, another computer, etc.

SPECIFICATIONS

==============

DC-PORT supports the following RS232 lines:

(numbers indicate the RJ12 pin number)

TRANSMIT                    (1)

RECEIVE                     (2)

DATA TERMINAL READY [DTR]   (3)

NOT USED                    (4)

CARRIER DETECT (CD)         (5)

GROUND                      (6)

USAGE

=====

To use DC-PORT, you must either be using a DC-PORT compatible program, or

you must have the DC-PORT device handler installed.

The program DCPORT.PRG adds devices to the ST's BIOS corresponding to the

two extra ports.  Access to the ports is provided through BIOS compatible

commands.

To INSTALL the DC-PORT handler, copy DCPORT.PRG into the AUTO folder of your

boot disk.

When DCPORT.PRG runs, it will display a bootup message indicating installation

of the handler, as well as a handler version number.  Should an upgrade of

hardware require a change in device access, you would need only replace the

DC-PORT handler program.

PROGRAMMING

===========

Programming to communicate using DC-PORT is provided through BIOS compatible

calls.  This section details use of these calls for DC-PORT and the regular

serial port.

----

BIOS

----

These subroutines are accessed using the TRAP #13 call.

All examples are calling sequences from 'C'

Numbers in parenthesis indicate the function number

All the examples will use the following declarations:

#define AUX  0x0002     /* main ST serial port, DC-PORT 0 */

#define DCP0 0x8000     /* DC-PORT 0, main ST serial port */

#define DCP1 0x8100     /* DC-PORT 1 */

#define DCP2 0x8200     /* DC-PORT 2 */

#define DCP3 0x8300     /* DC-PORT 3 */

#define DCP4 0x8400     /* DC-PORT 4 */

char input;             /* character input */

long status;            /* status - where applicable */

int  port;              /* DC-PORT select */

port = DCP1;

FUNCTIONS

---------

int Bconstat(dev)   (1)

int dev;

Return character-device input status, D0.L will be 0x0000 if no

character is available, or 0xffff if (at least one) character is

available from the device.

Example:

if (Bconstat(port))

input = Bconin(port);

NOTE:

For ease of use, you can OR the value of the DC-PORT select with

the AUX: device number.  We will use this syntax throughout the

document.

Example (from above):

if (Bconstat(port|AUX))

input = Bconin(port|AUX);

-------------------

int Bconin(dev)     (2)

int dev;

dev is the device number as defined above.

Does not return until a character has been input (busy-wait).  It returns

the character value in D0.L, with the high word zero.

Example:

input = Bconin(port|AUX);

-------------------

void Bconout(dev,c) (3)

int dev, c;

dev is the device number as defined above.

Output character 'c' to the device.  Does not return until the character

has been written.

Example:

Bconout(port|AUX, 'A');

-------------------

long Bcostat(dev)   (8)

int dev;

dev is the device number as defined above.

Returns character output status:

-1  Device is ready to send (no waiting on next device-output call).

0  Device is not ready to send.

Example:

status = Bcostat(port|AUX);

-----

XBIOS

-----

These subroutines are accessed using the TRAP #14 call.

All examples are calling sequences from 'C'

Numbers in parenthesis indicate the function number.

Examples are as above.

FUNCTIONS

---------

long Iorec(devno)   (14)

int devno;

Returns a pointer to a serial device's input buffer record.  dev is one of:

devno   Device

-----   ------

0x0000  RS232   (ST main serial port, DC-PORT 0)

0x8000  DCP0    (DC-PORT 0, ST main serial port)

0x8100  DCP1    (DC-PORT 1)

0x8200  DCP2    (DC-PORT 2)

0x8300  DCP3    (DC-PORT 3)

0x8400  DCP4    (DC-PORT 4)

The structure of the record is:

struct iorec {

long    ibuf;   /* pointer to buffer */

int ibufsiz;    /* size of the buffer */

int ibufhd;     /* head index */

int ibuftl;     /* tail index */

int ibuflow;    /* low-water mark */

int ibufhi;     /* hi-water mark */

};

For RS232, an output buffer record immediately follows the input buffer

record.  The format of the output buffer record is identical.

'ibuf' points to the device's buffer.  'ibufsiz' is the buffer's size.

'ibufhi' is the buffer's high-water mark.  'ibuflow' is the buffer's

low-water mark.

If flow control is enabled and the number of characters in the buffer

reaches the high-water mark, the ST requests (according to the flow control

protocol) the sender to stop sending characters.  When the number of

characters in the buffer drops below the low-water mark, the ST tells the

sender to resume transmission.

The flow control operation is similar for the RS232 output record.

-------------------

long Rsconf(port|speed, flowctl, ucr, rsr, tsr, scr)

int speed, flowctl, ucr, rsr, tsr, scr;

This function initializes the rs232 port.

SPEED sets the baud rate for the RS232 port as follows:

SPEED          BAUD RATE

-----          ---------

0              19200

1               9600

2               4800

3               3600 *

4               2400

5               2000

6               1800

7               1200

8                600

9                300

10                200 *

11                150

12                134

13                110

14                 75

15                 50 *

FLOW sets the flow control as follows:

FLOW           TYPE OF FLOW CONTROL

----           --------------------

0            No flow control (default value)

1            XON/XOFF

2            RTS/CTS *

3            Both XON/XOFF and RTS/CTS *

UCR, RSR, TSR, and SCR set the corrsponding 68901 registers.  A -1 for

any one of these parameters will not set the register:

Bit in UCR     Meaning

----------     -------

0         Not used

1         Parity 1=even parity, 0=odd parity

2         Parity enable; 1=enabled

3,4         Start/Stop bits:

Bit 4    Bit 3     Start bits   Stop bits

-----    -----     ----------   ---------

0        0       Do not use

0        1           1            1

1        0           1            1.5

1        1           1            2

5,6         Work length:

Bit 6    Bit 5     Word length

-----    -----     -----------

0        0          8 bits

0        1          7 bits

1        0          6 bits

1        1          5 bits

7         Must be set to 1

RSR, TSR, and SCR should be set to -1.

NOTE: Selections indicated with a * are only supported in DCPORT0.

-------------------

offgibit(port|mask)  (29)

int mask;

This function sets the DTR line on one of the serial ports.

MASK has a value of $EF:

Offgibit(port|$EF)

-------------------

ongibit(port|mask)   (30)

int mask;

This function drops the DTR line on one of the serial ports.

MASK has a value of $10:

Ongibit(port|$10)

------

GEMDOS

------

These subroutines are accessed using the TRAP #1 call.

All examples are calling sequences from 'C'

Numbers in parenthesis indicate the function number.

Examples are as above.

FUNCTIONS

---------

Cauxin(dev)     (3)     NOTE: The standard GEMDOS call has no parameters.

int dev;

dev is as defined above.

Return a character from the specified serial port (dev).

Example:

input = Cauxin(port);

-------------------

Cauxout(chr)    (4)

int chr;        NOTE: The standard GEMDOS call specifies a char

Write a character to the serial port defined in the upper byte of the

int 'chr.'

Example:

Cauxout(port|'A');  /* with port = 0x0000 it has no effect */

-------------------

long Cauxis()    (18)

Returns -1 [nonzero] if char is available from the main ST serial port;

0 otherwise.

-------------------

long Cauxos()    (19)

Returns -1 [nonzero] if char is ready to receive a character, 0 if it is

unavailable (for the main ST serial port).

FOR FURTHER ASSISTANCE

======================

If you require further assistance from us, give our support BBS a call.  We

will be more than happy to help you with any difficulties which you find.

WARRANTY

========

DC-PORT is guaranteed to be free from material defects for one year from the

date of purchase.

DC-PORT cables are guaranteed to be from material defects for 30 days from

the date of purchase.

Double Click Software will either replace or repair the defective unit, at

the discretion of Double Click Software.

Under no conditions will Double Click Software be responsible for damaged

caused due to neglect.  If the seal on DC-PORT is broken, the warranty is

voided immediately.

Double Click Software is not be responsible for any damage to your equipment,

reputation, profit-making ability or mental or physical condition caused by

the use or misuse of our hardware and/or software.

ADDITIONAL INFORMATION

======================

Mailing address:

Double Click Software

P.O. Box 741206

Houston, Texas 77236-1206

Support BBS:

(713)944-0108

------------------------------------------------------------------------------

This manual and its contents are copyright (c) 1989 Double Click Software.

This manual may be distributed freely so long as it is not edited in any form.

------------------------------------------------------------------------------