README.DOC

==========

Please note that a unit number (UNIT 0 or UNIT 1) and a

disk  type  will be requested for each function in FHDX

3.1.  The default unit is UNIT 0.

The correct sequence of the test procedure is :

CONTROLLER TEST --> DAISY-CHAIN TEST --> FORMAT

--> INPUT DEFECT LIST --> BURN_IN TEST --> SHIP

as displayed on the menu bar.  But, each  function  can

also be run independently in no specific order.

WARNING!!

ALL functions, EXCEPT Daisy-Chain Test  and  Ship,  are

destructive.   Please be sure to back up everything (if

possible) that you may want before you proceed.

Interleave factor:

------------------

(1)  MFM - 1 to 1 with 17 sectors per track.

(2)  RLL - 1 to 1 with 26 sectors per track.

April 13, 1989

- 2 -

Functional description of each program:

---------------------------------------

(1)  CONTROLLER TEST --

This program tests if the controller  interacting  with

the  drive  is  functioning  correctly.  It will try to

format the drive as a 2-head,  20-cylinder  device  (to

make  it  efficient).   The  format parameters for this

test are specified in the WINCAP file  under  the  'ct'

entries.

(2)  DAISY-CHAIN TEST --

This program tests if the daisy-chain port  works.   It

will  do  a non- destructive markbad on unit 1 (for 128

sectors, to be efficient).  The program will first save

the  data the 128 sectors on unit 1, WRITE a worst case

test pattern to that 128 sectors, READ data  back  from

that  128  sectors,  VERIFY  that the data read back is

exactly the same as what was written onto it, and WRITE

the  original  data  back onto those 128 sectors.   The

parameters for this test are specified  in  the  WINCAP

file under the 'dc' entry.

(3)  FORMAT --

This program formats the entire surface  of  the  drive

and  creates  a  default partition when format is done.

The default partition schemes for the different  drives

are  specified  in the WINCAP file.  There is no FORMAT

track command available.  This FORMAT program will  NOT

delete  existing defective sector information logged on

the drive earlier  (those  entered  manually  and  also

those found during the burn-in test).  The program will

save the defective sector information from  the  begin-

ning  of  the  drive, format it, partition it, and then

write the defective sector information  back  onto  the

drive.  The FORMAT program and the PARTITION program in

HDX 2.x will also save the  existing  defective  sector

information logged on the drive by FHDX.

April 13, 1989

- 3 -

(4)  INPUT DEFECT LIST --

This program logs at the beginning of  the  drive,  the

defects entered manually from the defect list generated

by the vendors.  These defects  inputted  will  NOT  be

deleted by the FORMAT program or the BURN-IN program in

FHDX, and will NOT be deleted by the FORMAT program  or

the PARTITION program in HDX 2.x.

(5)  BURN-IN TEST --

This program  tests  all  sectors  of  the  drive,  one

cylinder at a time, in the following order -

0, (N/4), 1, (N/4)+1, ... (N/4)-1, (N/2),

(N/2), (3N/4), (N/2)+1, (3N/4)+1, ... (3N/4)-1, N.

where N is the number of cylinders on the drive.   This

order is much faster than the 0, N, 1, (N-1), 2, (N-2),

... order.

The program was designed to test the cylinders  in  the

order  specified above, so as to do seek tests combined

with  write/read/verify  tests.   When   testing   each

cylinder, the program will SEEK to that cylinder, WRITE

a worst case test pattern (the  pattern  is  0x6cdb6cdb

for  MFM drives and 0xc623d5ab for RLL drives) to every

sector on that  cylinder,  READ  data  back  from  that

cylinder, and VERIFY that the data read back is exactly

the same as what was written to it.  A sector  will  be

logged on the drive as bad if there is a write error, a

read error or a data error on that sector.  The program

also does a 4-bit rotation on the test pattern for each

cycle of the burn-in test (e.g. For  RLL  drives,  test

pattern for cycle 1 is 0x6333, test pattern for cycle 2

is 0x3633, etc...).

All bad sectors found on a  drive  during  the  burn-in

test will be logged at the beginning of the drive.  The

logged defective sector information will  be  preserved

by the FORMAT program and the INPUT DEFECT LIST program

in FHDX, and also by the FORMAT program and the  PARTI-

TION program in HDX 2.0.

April 13, 1989

- 4 -

(6)  SHIP --

This program parks the heads  of  a  drive.   You  MUST

switch  off  the power of the drive once its heads have

been parked.

The ONLY way to erase the defect  list  logged  on  the

drive  is  to choose not to keep the existing list when

the INPUT DEFECT LIST program prompts you.

April 13, 1989