-------------------------------------------------------------------

File name:	TCPSPEED.DOC		Revision date:	1999.11.14

Created by:	Ronald Andersson	Creation date:	1999.11.14

-------------------------------------------------------------------

All of this file is released as PD info. Thus it may (and should)

be spread to STinG users everywhere, to help them optimize STinG.

-------------------------------------------------------------------

Released by:	Ronald Andersson	mailto:dlanor@ettnet.se

STinG home:	http://www.ettnet.se/~dlanor/

AKA:	http://dlanor.atari.org/

AKA:	http://sting.atari.org/

-------------------------------------------------------------------

Speed sensitive STinG configuration settings.

---------------------------------------------

The TCP module affects speed of most STinG servers and clients,

so its proper configuration is vital to efficient STinG usage.

All settings described below interact with this module, and can

cause profound changes in how packets are exchanged between the

servers and clients, whether in local traffic with STinG being

used at each end, or over Internet through some ISP.

-------------------------------------------------------------------

First and foremost, STinG timeslice period, set in DEFAULT.CFG as

the variable THREADING and in the 'Delay:' dragbar of STING.CPX .

Its value should be kept low,  10-15 for fast machines like TT or

accelerated Falcon, but in the range 20-50 for slow machines. Too

high values will cause slow traffic handling,  while too low ones

on slow machines will cause the  whole  system to get 'sluggish'.

But on fast machines the minimum value (10 ms) is the best choice.

-------------------------------------------------------------------

Second,  DEF_RTT  has less influence in old modules but,  with

introduction of modern TCP 'slow-start' and 'congestion avoidance'

methods, it has become of vital importance to use a DEF_RTT value

that allows the new algorithms to work properly.  Nowdays this is

also a required property of all TCP implementations, as specified

in several RFC documents.  The minimum setting they allow for use

on the Internet is 1500 ms,  so DEF_RTT should always be at least

1500.  This is set in DEFAULT.CFG,  and should be 'mirrored' also

in the STNGPROT.CPX .  If another value has been saved in the CPX

you must also change that to 1500 (or higher) and save settings.

NB: In all release archives of STinG DEFAULT.CFG has contained a

value of DEF_RTT = 50, which is no longer functional. That is

a value useful only on a LAN based on fast machines with high

bit rates on all ports, so delays are kept to a minimum.  For

access to Internet this value is forbidden.  The minimum that

is allowed there is 1500, and the settings in future releases

will be modified in this regard.

-------------------------------------------------------------------

Third,  MTU and MSS should be chosen as the highest ones that the

ISP allows,  as this increases speed for bulk traffic.  Many ISPs

always tell users to use  576 and 536,  because that old standard

is accepted by all  RFC-compliant  software,  even though most of

these ISPs can work with the new 'semi-standard' of 1500 and 1460

just as well.  Crossexamine your ISP on this,  and use the higher

values if supported by the ISP. MTU settings exist in DEFAULT.CFG,

DIAL.SCR,  and in the STING.PRT saved by STNGPORT.CPX.  Best LAN

operation will result if all ports use the same MTU.  MSS setting

exists in  DEFAULT.CFG and this is mirrored in  STNGPROT.CPX. The

latter setting only needs to be saved if you have saved a changed

setting that way before  (if CPX no longer mirrors DEFAULT.CFG).

-------------------------------------------------------------------

Fourth, RCV_WND, the TCP reception window size, limits the amount

of data received before ISP halts to wait for ACK packets. Higher

values allow faster bulk transfer, but ONLY if other settings are

already optimized.  Otherwise the effects will be negative,  thus

triggering unnecessary congestion.  In old TCP modules this would

always happen, so they never worked well with large windows. With

the new TCP modules I myself use RCV_WND = 32000, but I do advice

against going above 32767,  as sloppy programmers may have caused

a sign error for higher values in some implementations.  Some RFC

documents also  warn  against that possibility.  RCV_WND settings

exist in DEFAULT.CFG and that's mirrored in STNGPROT.CPX. For the

latter change the same rules apply as for  DEF_RTT and MSS above:

If the change of  DEFAULT.CFG is 'mirrored' in the CPX on reboot,

then you don't have to save it separately in the CPX.

-------------------------------------------------------------------

End of:		TCPSPEED.DOC

-------------------------------------------------------------------