DigiPoint Documentation                        16/05/94 v3.60 - Page 1

DigiPoint - The English Documentation (Version 3.60, 16/05/94)

DigiPoint  (DP)  is an all-in-one "singing and  dancing"  amateur

radio  software for the Atari ST/TT/Falcon  computers.  The  user

interface is consequently GEM-driven, the communication interface

is  BIOS-driven,  the result is a high grade of compatibility  to

all existing and maybe futur operating systems.

DP has three basic parts:

- the packet radio host terminal

- the bulletin board system

- the satellite calculations

Even when using GEM, one cannot promise that such a complexity is

operational  for  you,   the  user,  at  the  first  sight.  This

documentation tries to focus most of the features and commands of

the DigiPoint System.  Sometimes,  you may find the  explanations

somewhat  cryptic,  don't  worry about that:  DP is  in  its  way

unique. It unites basic packet radio user terminal functions with

a  highly  complex  mail  system.  You can run  DP  as  a  simple

terminal,  but you can even install a state-of-the-art public bbs

system  with  it.  If  you are doing  the  second,  you  have  to

understand  all  the crap  concerning  mail  forwarding,  message

filtering  etc.,  but  as one should have  some  experience  when

acting  as a sysop I'm hopeful that those chapters are clear  for

you.

Please  excuse  the  errors  in  this  documentation.  My  native

language is german, not english.

I  would like to thank those amateurs that helped to develop  DP.

Without them, their technical, mental and sometimes psychological

help,  their testings and sometimes their creativity to  confront

DP  with totally useless command combinations it would  not  have

been possible to create this stable and multifunctional software.

Beyond many others, I would like to mention:

F6FBB Jean Paul (Toulouse),  DL7QG Gerd (Berlin), DG0FT Rene

(Strausberg), DL4YBG Mark (Berlin), DL7AND Michael (Berlin),

DL1XAO Odo (Hamburg),  DL7APN Chris (Berlin),  DL1HAZ Walter

(Hamburg),  PE1NIB  Chris (Wijchen),  F6GIA Jean  Marie  (Le

Tholy),  LX9EG  Norby (Luxembourg),  DL1GJI  Jimy  (Mengen),

DF2AU Georg (Brunswick).

And, for her love, Nikoline.

Thank you all!

DigiPoint Documentation                        16/05/94 v3.60 - Page 2

Content

Page

1.   Copyright                                              4

2.   System Requirements                                    5

3.   Supported TNC For Packet Radio                         6

4.   Installation                                           8

4.1.      The Keywords in PRCONF.DP                         9

4.2.      The QRGLIST.DP Configuration                      14

4.3.      The PATHLIST.DP Configuration                     14

4.4.      Different Language Files                          14

4.5.      Starting DP For The First Time                    14

5.   General                                                16

5.1.      The Windows                                       16

5.2.      The Dialog GENERAL                                16

5.3.      The Dialog FUNCTION KEYS                          18

5.4.      The Batch Files                                   18

5.5.      The Online Help System                            19

5.6.      Memory Management                                 19

5.7.      The GEM Message Interface                         20

5.8.      The Command Window                                21

5.9.      The Debug Window                                  21

6.   Packet Radio Operations                                22

6.1.      The Basic Packet Radio Commands                   22

6.2.      The 'special' Keys                                23

6.3.      Additional Commands                               24

6.4.      The Internal Remote Commands                      25

6.5.      The External Remote Commands                      31

6.6.      The Language Files                                32

6.7.      The Text Convert Routine                          33

6.8.      The File GREETING.DP                              33

6.9.      The Menu PARAMETERS -> PACKET                     35

6.10.     The Packet Terminal Pop-Up Menu                   37

6.11.     The Packet Terminal Read Generator For Mailboxes  38

6.12.     The Automatic Password Generation                 39

6.13.     The File QRT.DP                                   40

6.14.     The File WELCOME.DP                               40

6.15.     The File NODE.DP                                  40

6.16.     The Status Display                                40

6.17.     The Connect Status                                41

7.   The Satellite Calculations                             42

7.1.      The Available Satellite Calculations Commands     42

7.2.      How To Get Keplerian Elements                     48

7.3.      The ROTARY Interface                              49

8.   The Mailbox                                            50

8.1.      Board Oriented System vs.

MSG Number Oriented System                        51

DigiPoint Documentation                        16/05/94 v3.60 - Page 3

8.2.      A Typical Session In TheBox / DP - Syntax         51

8.3.      The Internal Commands Of The BBS (COMMANDS.BOX)   53

8.4.      Command Description                               56

8.5.      The Configuration Files                           70

8.5.1.         RUBRIKEN.BOX                                 70

8.5.2.         TRANSFER.BOX                                 71

8.5.3.         BADNAMES.BOX                                 71

8.5.4.         TCPIP.BOX                                    71

8.5.5.         BEACON.BOX                                   72

8.5.6.         AUTOBOX.DIR                                  72

8.5.7.         F6FBB.BOX                                    73

8.5.8.         The .SF Files                                73

8.6.      The Language Files                                77

8.7.      The Autorouting Feature                           78

8.8.      The Bulletin ID System                            78

8.9.      The Read Generator (II)                           79

8.10.     The Dialog BOX 1                                  79

8.11.     The Dialog BOX 2                                  82

8.12.     Message Filtering                                 83

8.13.     External Commands                                 84

8.14.     The Folder NEWMAIL                                86

8.15.     Timeouts                                          86

8.16.     Multiple Commands                                 86

8.17.     Output Abort                                      86

8.18.     Erase / MyBBS Forwarding                          86

8.19.     The File CONFIG.BOX                               87

8.20.     The Auto Import Feature                           87

8.21.     The S&F - Resume Mode                             87

9.   The Modem Port                                         88

9.1.      The File MODEM.BOX                                88

9.2.      Starting Modem Operation                          89

APPENDIX

A.   The German Keytable                                    90

B.   Station Identifiers (SID)                              90

C.   External BBS Commands: The DP User Record Structure    90

D.   The Index Files (.IDX)                                 91

E.   The Checklist Format (BOXLOG.DP)                       92

F.   The File HPATH.BOX                                     92

G.   The Extended Forward Protocol                          93

H.   The Static Huffman Compression Table                   94

DigiPoint Documentation                        16/05/94 v3.60 - Page 4

1. Copyright

DigiPoint is copyrighted by Joachim Schurig, Berlin, 1991-1994.

It is not public domain, but you are free to copy the archives if

you don't take money for that  service!  Therefore,  professional

PD-services are excluded of this general licence. Furthermore, DP

is  only  free for radio amateurs and the non-commercial  use  in

amateur radio networks.

DP  reached a high grade of reliability.  Nonetheless I  have  to

disclaim that I cannot guarantee for safe and proper work of  DP.

Also,  all  responsability for occuring damages in  hardware  and

sofware remains at yours.

I know well enough that the following has no effect, but anyway:

Think  about to support the project with a little  donation.  The

invested work, time and other means are not to be discussed here,

but  always  keep  in mind that its up to  YOU  to  motivate  the

ongoing of the development.  The fact that DP is distributed free

of charges doesn't mean that it isn't worth some.  But as I  know

how  difficult it is to pay software if one has no or  only  poor

income (I'm a student) I decided to distribute it this  way.  So,

everyone may decide if and how much the program is worth for  him

and if he can express that in cash.  This is no moral debate  and

shouldn't become one.

The authors adress:           Donations:

Joachim Schurig               Postgiroamt Hamburg

DL8HBS                        BLZ: 200 100 20

Bergmannstr. 67               Konto: 55 66 94 - 202

D-10961 Berlin                Germany

Germany

BBS: DL8HBS @ DB0GR.#BLN.DEU.EU

2. System Requirements

For  proper  operation of DP,  you should use a  harddisk  and  a

minimum of 1 Megabyte RAM. The proposed amount of RAM is at least

2  Megabtyes.  The  more,  the faster and  safer  DP  works.  The

DP_MINI.PRG  is  a DP-Version without BBS  and  SAT-calculations,

this will work without harddisk and with only 1 Megabyte RAM.

DP  assumes that you are booting a GDOS (grafical DOS  expansion,

available by ATARI and many others) and that you speeded up  your

screen output with either NVDI,  TurboST,  QuickST or others.  If

you don't, DP will work, but slower than acceptable (at least for

me).  This  is  the price to be paid for the  output  independent

grafic routines of DP,  but what you get is worth a lot:  You can

DigiPoint Documentation                        16/05/94 v3.60 - Page 5

use  whatever grafics mode you want,  high  resolution  monitors,

external grafic cards,  screenblasters and multitasking  environ-

ments.

DP  works well with the following TOS  versions:  TOS  1.04,  TOS

2.06,  the TT-TOS,  the Falcon-TOS,  MultiTOS,  Mag!x,  MultiGEM,

KAOS-TOS.  It is not to be expected that problems will occur with

future TOS releases.

DP works extremly slow with TOS 1.2, this TOS is highly buggy and

you  should  replace  it the fastest you can.  TOS  1.0  was  not

tested,  but  the reliability should be about the same as of  TOS

1.2.

Please  note that some of the screen enhancers have  problems  to

display  the (vectorized) world map drawn by DP in the  SAT-mode.

This is NOT a problem of DP.  Also,  I was told about problems of

NVDI 2.5 to display inverted text,  but this seems to be  limited

to that version number.  One user told about problems on the  TT:

when  displaying a POP-UP - Menu in a special  constellation,  DP

crashes.  I  can't  follow that,  the routine works well  on  all

grafic cards, I have never seen that error.

When using other keyboards than the german, especially the french

one,  some of the menu shortcuts are difficult to generate as for

example  the  german  'M' is represented by  a  french  ','.  The

english  keyboard seems all right at this point.  Note that  this

only concerns some function key definitions,  anyway you can  use

the  mouse to solve that problem.  In the appendix you  find  the

german keyboard table, so you can compare it with yours.

Note  also  that  most  TOS versions  have  bugs,  some  of  them

undetected.   The  TOS interface  of  DP  follows  the  available

documentation  by  Atari.  The  Falcon TOS seems to  mix  up  the

different serial ports,  a reasonable number of patch programs is

available by different sources.

All  yet known TOS versions have problems to  administrate  large

harddisks  with  many folders.  You are recommended  to  use  the

FOLDR100.PRG, especially when the message '*** no system ram, TOS

halted'  appears!  This is no message of DP,  it's a mess of  the

TOS. If you don't take this serious, don't wonder about problems.

Also,  you should install the HSMODEM1.PRG in your AUTO - folder.

It  is distributed with DP,  the author is  Harun  Scheutzow.  It

fixes some problems concerning keyboard and serial port.  Read at

least the QUICK.TXT concerning HSMODEM1.

The best yet known TOS versions are: KAOS TOS and Mag!x 2.0, both

are commercial products.

DP works fine in multitasking environments.  I cannot propose the

use of MultiTOS,  it works much too slow.  I am using Mag!x  2.0,

this  multitasking  system works even faster than  TOS  2.06  and

seems to be much more stable than MultiTOS.

DigiPoint Documentation                        16/05/94 v3.60 - Page 6

IMPORTANT:  Whenever you have problems with  DP,  crashes,  wrong

files in the BBS or others,  reboot WITHOUT all AUTO\ -  programs

and  WITHOUT  all accessories.  Many many of these  programs  are

buggy. Wonderful examples are some TT-Desk versions for the ST.

DP  works absolutely reliable in hundreds of  installations.  So,

FIRST CHECK YOUR SYSTEM CONFIGURATION.

To destroy some of the myths about DP:  For read/write operations

on  the  file  system,  only GEMDOS functions  are  used.  DP  is

absolutely unable to damage FAT's, bootsectors etc. I write that,

because  many  users of DP seem to be frightened about  the  many

harddisk accesses of DP.  It is in the nature of a message system

that  it  has to read and write files  on  disk.  But,  as  those

operations are numerous, DP is very effective in detecting either

driver bugs or harddisk sector errors. Don't blame DP for that.

And, please, if a function does not work the way you expected it,

think  about  that  you might have entered  a  wrong  command  or

configurated  something  wrong.  DP is not a  one-day  work,  the

creation took nearly three years. You are not the first using the

program (and hopefully you won't be the last).

3. Supported TNC For Packet Radio

-    DP  supports the WA8DED - Hostmode.  You can use every  TNC2

with a WA8DED - EPROM inside, of course also the TheFirmware

of  NORD><LINK  which is a highly  improved  clone.  DP  was

developed with TheFirmware.  You should use at least version

2.6 of TheFirmware (don't mix that up with v2.6 of WA8DED!).

That  and the following versions have a  hostmode  expansion

that  speeds up the communication between computer and  TNC.

The  number of channels is not limited by DP (well,  it  is,

some 45 or so),  so you can use 6,  8,  10, 27, 32 - channel

versions of TheFirmware. As long as there is enough space on

the  distribution disk,  you find the newest version of  the

TheFirmware on the disk,  compiled for 10 channels. You have

to  burn an EPROM of the type 27256 (CMOS) and insert it  in

your TNC2.

-    DP supports the TNC3 - TurboHostmode, which is a derivate of

the WA8DED-Hostmode for multiport-TNC.

-    DP  supports TFST.  TFST is a small program that has  to  be

started  before DP,  it offers a communication interface  to

all  KISS-Mode TNC such as KAM,  PK232.  TFST is hamware  by

OE1HHC, please read his instructions.

-    DP supports the PK232 Hostmode.  But beware!  That implemen-

tation  is not error free.  I would propose you to  run  the

DigiPoint Documentation                        16/05/94 v3.60 - Page 7

PK232  either  with TFST in KISS-Mode or to use the  TFPK  -

EPROM  by DL4YBG.  TFPK is an implementation of  TheFirmware

for the PK232. With that EPROM, the PK232 acts as a TNC2.

DP  supports  up to 4 TNC (even on a  single  serial  port),  the

maximal number of channels is limited to 45.  So don't use 4  32-

channel TNCs.

DigiPoint Documentation                        16/05/94 v3.60 - Page 8

4. Installation

At this moment,  the DP - archives should be completely  unpacked

at your harddisk. A folder structure as

DP\

BOX\

SAT\

PACKET\

HELP\

should be visible.  The main folder DP\ may also be named DP_ST\,

you can rename it the way you like it.

Even though DP is a GEM program,  the configuration requires your

help.  Assuming  that you want to install DP with TNC's (it  also

works without,  the packet radio terminal is switched off in that

case),  the  most important and maybe most difficult work  is  to

config the file DP\PACKET\PRCONF.DP .

You should do that before starting DP.

It  is  a simple text file and can be edited  with  every  ASCII-

editor.  It  contains  keywords  (such as UTC=  )  and  comments,

comments are starting with a '#'.

Please revise NOW the example delivered with DP.

DigiPoint Documentation                        16/05/94 v3.60 - Page 9

4.1. The Keywords In PRCONF.DP

CONNECT=  If  your  local digipeater uses  uncommon  link  status

messages, you can define the used message for a connect

setup.  Default is:  'connected to'. Upper/lowercase is

not checked. Usually you can uncomment this parameter.

RECONNECT=Here,  you define a message for the reconnect state  of

your local digipeater. Default is 'reconnected to'.

FAILURE=  Here,  you define a message for the link failure  state

of your local digipeater.  Default is a set of messages

such  as:  'loop detected',  'link  failure',  'invalid

call', and so on.

CONT=     Defines if a user that is not mentioned in  GREETING.DP

(see below) may connect the DP-Terminal.  1 = Yes,  0 =

No.

CONB=     Defines if a user that is not mentioned in  GREETING.DP

(see below) may connect the DP-BBS. 1 = Yes, 0 = No.

CONN=     Defines if a user that is not mentioned in  GREETING.DP

(see below) may connect the DP-Node. 1 = Yes, 0 = No.

CONFTEXT= The text following this keyword is sent to a user  that

is excluded by one of the above selections (CONT, CONB,

CONN). Example: CONFTEXT=sri, port reserved.

RMP=      RMP  allows you to set the remote access  directory  of

DP. Per default, this is DP\REMOTE\.

Example:

RMP=C:\UPLOAD\PACKET\REMOTE\

RUN=      RUN  allows you to set the directory where DP  searches

for the external RUN-programs.

Default is DP\PACKET\RUN\

SVP=      SVP presets the SAVE-Path of DP.

Default is DP\SAVE\

This  is only a preset,  whenever you open a  SAVE-File

you can determine a new path.

BOXSSID=  SSID  of your callsign that activates  immediately  the

bbs  when connected.  Note that of course you  have  to

define at least one TNC-channel with the desired  SSID.

0 = call without SSID.

Example: BOXSSID=7

NODESSID= SSID of your callsign that activates immediately the DP

-node when connected.  Note that of course you have  to

define at least one TNC-channel with the desired  SSID.

0 = call without SSID.

Example: NODESSID=9

DigiPoint Documentation                       16/05/94 v3.60 - Page 10

HOT=      Interval for the 'hold'-timer. After the specified time

the  string  defined  with HOS=  (see  below)  will  be

transmitted  if  no other data was transmitted  in  the

intervall. This is useful to prevent timeouts.

Example: HOT=30

(30 minutes).

HOS=      Text  that  will be sent when the  hold  timer  expires

(default: a single blank). The text is converted in the

text-convert-routine.

Example: HOS=SH/DX 5\r

UTC=      Difference between system clock and UTC in seconds.

Example: UTC=-3600

POP=      Seconds for POP-UP-Menus.

Example: POP=30

CTE=      Send WELCOME.DP to every connecting station? 0 = No , 1

= Yes

Example: CTE=1

REM=      Remote commands active? 0 = No , 1 = Yes

Example: REM=1

MYCALLS0= Defines  a  list  of MyCalls to be  used  for  TNC  #0.

Default  for  every channel is your  personal  callsign

without SSID

Example: MYCALLS0=0:NOCALL,1:NOCALL,4:NOCALL-7

MYCALLS1= The same for TNC #1

IS0=      Defines a beacon text for TNC #0

Example: IS0=DP Mail System -7 = BOX , -9 = NODE

IT0=      Defines a beacon timer for TNC #0

Example: IT0=30

(30 minutes, 0 = OFF)

IS1=

IT1=

The same for TNC #1

PAC=      PACLEN (default). When using NET/ROM - Nodes, never set

it > 234

The  paclen  is  the maximum length  of  a  transmitted

packet. The maximum value is 256.

Example: PAC=234

PLS=      PACLEN (per TNC), max. 234 for VHF, 80 for SW!

Example: PLS=0:180,1:80

EBL=      Amount of lines for write buffer (default) - max.  9999

lines.

The  write  buffer  is the part  of  the  split  screen

DigiPoint Documentation                       16/05/94 v3.60 - Page 11

terminal where you pretype your text.

Example: EBL=20

EBS=      Amount of lines for write buffer (per channel)

Example: EBS=1:100,2:50

REC=      Amount  of  lines  for  backscroll  of  received   text

(default) - max. 30000 lines.

Example: REC=50

RVS=      Amount of lines for backscroll per channel

Example: RVS=0:200,1:200,10:150

NOTE:  The  write buffer and backscroll settings are  responsible

for the memory DP needs. The lower those values are, the less RAM

is  required.  The allocation of those buffers  is  dynamic,  the

backscroll  buffer  of channel 10 will never be reserved  if  you

never start a connect on that channel.

The  following six keywords are the most important in the  config

file. They determine where DP searches what kind of TNC. If these

settings don't fit the reality,  DP may crash,  be worried,  stop

running,  eat your lunch or do anything other that has nothing to

do with the usual job of the program.

The keywords are:

CFG=TNCS:

CFG=PORTx:

CFG=TYPEx:

CFG=CHANSx:

CFG=BAUDx:

CFG=IDENTx:

The 'x' is a TNC - number.  If you have only one TNC, all 'x' are

'0',  if  you  have  two,  you have to define two  sets  of  this

keywords, one with '0' and another with '1'.

CFG=TNCS:      Defines the amount of TNC's.

Example: CFG=TNCS:1

CFG=PORTx:     Defines  the BIOS-port where DP will find  a  TNC.

Usual ports are:

1 (RS232),  3 (Midi),  6..? (extended serial ports

of the TT, Falcon etc.)

Example: CFG=PORT0:1

CFG=TYPEx:     Valid keywords are:  TNC2 ,  PK232 ,  TFST .  This

defines the communication protocole DP will use.

TNC2  = WA8DED - hostmode

PK232 = PK232  - hostmode

TFST  = KISS (only valid if TFST.PRG is executed!)

Example: CFG=TYPE0:TNC2

DigiPoint Documentation                       16/05/94 v3.60 - Page 12

CFG=CHANSx:    This defines the amount of channels of your TNC.

Example: CFG=CHANS0:10

CFG=BAUDx:     This  defines  the  baudrate to  be  used  between

computer and TNC.  This has nothing to do with the

radio baud rate!  Always use the fastest  baudrate

possible.

Example: CFG=BAUD0:19200

CFG=IDENTx:    This defines the prompt to be used on a TNC. After

execution of several packet terminal commands,  DP

displays a prompt.

The  string is converted in the  text-convert-rou-

tine.

Example: CFG=IDENT0:\Q

(What  results  in the output of the  actual  fre-

quency).

A definition for two TNC could look like this:

CFG=TNCS:2

CFG=PORT0:01

CFG=TYPE0:PK232

CFG=CHANS0:10

CFG=BAUD0:9600

CFG=IDENT0:\Q

CFG=PORT1:11

CFG=TYPE1:TNC2

CFG=CHANS1:10

CFG=BAUD1:9600

CFG=IDENT1:\Q

The  CFG=PORTx:  - command has a special feature:  DP is able  to

switch multiple TNC on serial port 1.  The used handshake signals

are  DTR and RTS,  an external multiplexer has to select the  TxD

and RxD - lines of the corresponding TNC.  With those two signals

you can select 4 TNC on port 1. CFG=PORT0:01 and CFG=PORT1:11 are

defining TNC #0 connected to sub-port 0 of serial port 1,  TNC #1

connected to sub-port 1 of serial port 1.

When  using the TFST interface,  set PORT to 5.  This is a  dummy

value (BIOS port 5 normally is the screen).

If at this point the communication between DP and your TNC(s)  is

established, you might want to send initial TNC-commands. This is

provided by the following commands:

INI=      All  that follows INI= is sent in command mode  to  all

defined  TNC on system startup.  You might  switch  the

DigiPoint Documentation                       16/05/94 v3.60 - Page 13

monitor on, set a ctext etc.

Example: INI=M UISC

INIx=     Same  as  INI=,  but  the  text is  only  sent  to  one

specified TNC.

Example: INI0=T 30

DEI=      This  is the same as INI=,  except that those  commands

are sent when leaving DP.

DEIx=     see above

Please note that a subset of the above keywords was introduced by

SP of DL1MEN. It was his special wish to be mentioned here, so he

is.

4.2. The QRGLIST.DP Configuration

By  different reasons,  it is important that DP always knows  the

correct  radio  frequency of its TNC(s).  This is realized  by  a

call-scan procedure.  Some calls could be regarded as beacons, as

they never make qsy, e.g. your local digipeater.

When  DP monitors a frame sent by such a 'beacon',  the  internal

qrg setting is updated.  The file DP\PACKET\QRGLIST.DP contains a

set of callsigns and corresponding frequencies, but of course, as

the  most of you is not living in Berlin,  it has to  be  altered

according  to  your  local  situation.  The  file  contains  some

informations about the definition format.

4.3. The PATHLIST.DP Configuration

The informations found in this file are used for the auto-connect

feature of DP.  Please read the instructions at the header of the

file. For a first start of DP you can leave this file unchanged.

4.4. Different Language Files

DP is completely multilingual. The GEM surface of DP exists up to

now in four lingual versions:

DP.RSC      = german

DPENG.RSC   = english

DPFRANC.RSC = french

DPNED.RSC   = dutch

DigiPoint Documentation                       16/05/94 v3.60 - Page 14

Please select a language by renaming the file to DP.RSC.  You can

delete the other .RSC - files on your copy of DP.

4.5. Starting DP For The First Time

Now, after all necessary modifications of PRCONF.DP, start DP.PRG

(or DP_MINI.PRG).

An install dialog will appear.  Please enter your callsign,  your

WW - QTH-Locator and two passwords (at least 8 chars length). The

first  password is for the bones,  I excluded  the  corresponding

mechanism,  but  the  second one will be used to  allow  you  (or

someone other) to login as a sysop via the radio.  You can  alter

this password later.

The  last you have to do is to define a default editor.  You  can

use ASCII-editors,  such as TEMPUS,  7UP,  QED. If you don't have

such  a  program,  define WORDPLUS.  Some  funny  people  defined

DP.PRG.  This might not be the best idea. Click on the 'search' -

button of the dialog to define the path.

If all entrys of that dialog are valid: Welcome to DP.

DigiPoint Documentation                       16/05/94 v3.60 - Page 15

5. General

5.1. The Windows

DP has two different ways to handle and display windows.  You can

use  normal GEM - windows,  with slide bars,  closers and so  on.

Alternately  you can use 'smart' GEM - windows.  The second  type

has  no  border  elements and requires much  less  screen  space.

Anyway,  also the second type is a true GEM window.  To switch to

this  type of display,  select the Menu PARAMETERS -  OTHERS  and

click off 'GEM-Windows'.

You  might  ask yourself how to handle the  'smart'  windows.  In

fact,  all usual objects,  as sizer,  close box,  fuller are exi-

sting, but invisible. Try it...

To  scroll such a window with the mouse,  two different  concepts

are existing:  packet terminal windows are scrollable by clicking

in  the first or last line of the receive window,  the  ViewIt  -

window  (used to display/view a file) reacts like GUCK.TTP -  the

good  old viewer by Marcel Waldvogel.  Left/right  mousebutton  =

up/down, both buttons = close.

Note that these scroll options are also available with the  'full

GEM  windows',  the  scrolling  is  much  faster  than  that  one

generated by the scroll bars.

Note also that not every window has a scrollable content.

When using the normal GEM windows,  you will remark problems when

moving  or  sizing such a  window.  Randomly,  a  POP-UP-Menu  is

generated by DP,  this normally only should happen when you click

in the MIDDLE of a window.  It is a well known bug of the GEM, it

sends an erroneous event to the event handler of DP.  The onliest

way to fix this error is to use the ARROWFIX or WINX -programs or

KAOS-TOS / MultiTOS / Mag!x.

5.2. The Dialog GENERAL

Clicking on the menu entry PARAMETERS -> GENERAL, a dialog opens.

The section MISCELANGELOUS has the items

TX active           if  this  switch is off,  every  TNC  is

deactivated. This is done by sending the

TNC commands X 0 and Y 0.  When  reacti-

vating,  Y is set to the number of allo-

wed channels before switching to 0.

clock visible       if  selected,  the  UTC clock of  DP  is

DigiPoint Documentation                       16/05/94 v3.60 - Page 16

displayed in the upper right part of the

menu  bar.  This may cause  problems  in

multitasking  environments  if   several

programs  display  clocks.  So  you  can

switch it off.

sounds              if  not selected,  all sound  output  is

inhibited.  Concerns  dis/connect  bell,

CTR-G and others.

tabulators          Hm. I believe that is to be treatened as

a  dummy  switch hi.  DP  has  sometimes

problems with tabs,  sometimes not. That

aint worse.

log                 if   not  selected,   no  log  file   is

written.  Refer to your legislation. The

log  file is to be found in  the  folder

DP\VERWALTG\.

password protect.   This  doesn't work no more,  I  excluded

the corresponding mechanism.  So  forget

it.

The section COMPATIBILITY contains the elements

GEM windows         If  selected,  border elements are  used

for the windows.

quick dialogs       if selected, a quick screen copy routine

is  active when displaying a  dialog  to

restore  the  screen  after  ending  the

dialog.  If  not,  the screen has to  be

redrawn,  what takes more time. The fun-

ction needs some 10 kByte of RAM and  is

switched  off automatically if there  is

not enough.

watchdog            activates  the  built-in  watchdog.  The

watchdog  creates  a  RESET  if  DP   is

stopped for more than 5 minutes.  If you

configure  your  boot sequence  the  way

that DP is autostarted, this is a useful

feature  for  stand alone  bbs  systems.

Note: an external watchdog always is the

better  way,  but this one  works  quite

reliable for a simple software solution.

If DP crashes and returns to the  dektop

the watchdog is still running. Please do

not restart DP,  because whatever you do

next,   the  watchdog  expires  after  5

minutes.  If  you are present,  cause  a

RESET, if not, the watchdog will do so.

DigiPoint Documentation                       16/05/94 v3.60 - Page 17

5.3.The Dialog FUNCTION KEYS

In this dialog,  you can set up your function keys.  Simply enter

the text to be typed when depressing ALT-F1 - ALT-F10. Some chars

have a special function:

#    if  found at the beginning of a line will be  converted

to <ESC>.

%    generates an end of line (as <RETURN>)

&    generates the ALT-C

^    if found at the beginning of a line indicates that  the

following string has to be interpreted as filename. The

file  is  opened  and sent line by  line  as  a  BATCH.

Example: ^D:\DP\TEST.BAT

CTRL-D    inserts the current date when pressing the function key

CTRL-T    inserts the current time when pressing the function key

The  function  key string is sent to the  uppermost  window  when

pressing the key.

5.4. The Batch Files

As was written in section 5.3.  you can define batch files to  be

executed.  The  batch files contain simple text to be typed  line

per line. There is only one control sequence:

If a line starting with '^' is found,  the following word is

interpreted as filename.  The file is opened and sent as  an

included batchfile. There is no depth limit. Example:

^C:\NEW\BATCH2.DP

If the line starts with '^CONVERT',  this is not interpreted

as a batch name but toggles the string convert routine to be

active on the batch output. '^CONVERT ON' and '^CONVERT OFF'

are  the legal commands.  The convert routine  is  described

later on in the packet radio section.

Per default, CONVERT is OFF.

5.5. The Online Help System

The  online  help is an open system,  you can easily  create  new

DigiPoint Documentation                       16/05/94 v3.60 - Page 18

helpfiles. The distributed ones are mostly in german.

The  functionability is as follows:  when clicking on  that  menu

item,  a file select box requests you the name of the helpfile to

be  openend.   Then,  definition  strings  are  searched  in  the

helpfile.  They  are  alphabetically sorted and displayed  in  an

'index  page'.  Now  you can click with the mouse on one  of  the

words,  the adjacent help text is displayed.  Also in this  text,

you  can click on any word.  If this word is one of  those  shown

before in the 'index page', the text is displayed and so on. This

is good for hints as: 'see also -> ....'

To go back,  click the close box of the window until you are back

in the 'index page'.

How to create a help file?

A help file contains indexes and text.  It is a full ASCII  file.

An  index is marked by framing it with '@@' at both sides of  the

word.  Note  that you cannot use multiple words as  indexes.  All

text following the index until the next index definition (or  the

end of the file) is the help text related to that index.

An example:

@@TNC@@

A tnc is something that hangs around in your radio shack and

causes a big noise floor on the shortwave.

@@DP@@

DP seems to be the program you're using at the moment.

The  files may be packed.  Use the DP_PACK.PRG.  The packed  help

files  MUST have the extension .HLX ,  the unpacked .HLP  .  Take

that  serious,  DP  will  crash if you  tinker  around  with  the

extensions.

5.6. Memory Management

DP uses a fully dynamic memory management.  No buffer is  blocked

when  not in use.  This makes DP a perfect multitasking  program.

Many others, as WordPlus (that I am using to type this text while

DP runs in the background),  reserve nearly the whole system RAM.

That's rubbish.

This  technique implies also a disadvantage:  If another  program

reserves  the whole RAM while DP is running,  it may happen  that

some buffers cannot be opened.  Normally,  this only affects  the

display (the backscroll buffers).  But when the bbs is sorting  a

new mail and nearly no RAM is found,  it needs much more time  as

when  there is enough RAM...  and some functions do not  work  at

DigiPoint Documentation                       16/05/94 v3.60 - Page 19

all.

Also,  the size of the backscroll buffers may be too big for your

system RAM,  but DP will not create a warning on startup  because

it doesn't knows at that moment.  By the way,  thats not serious,

the buffer will be smaller than desired and a lot of warnings may

appear in the debug window, but thats all.

Pressing ALT-2 will free ALL buffers.  This may help if you  need

free RAM.

5.7. The GEM Message Interface

DP  offers  programmers  an interface to  communicate  using  GEM

messages.  Don't  expect  too much,  it was  a  quick  hack.  GEM

messages  may  be sent by parallel applications  in  multitasking

environments or by desk accessories.

How does it work?

Your  application  has to send a standard message buffer  to  DP,

filled  with some parameters.  The message buffer is an array  of

integer,

msg = array[0..7] of integer

msg[0]    must be filled with 9166 (this triggers DP)

msg[1]    must  be  filled with the APPL_HANDLE of  DP  (use  the

APPL_FIND AES routine to get the HANDLE)

msg[2]    must  be filled with the command number (it  determines

which action DP will perform)

msg[3]    must be filled with YOUR APPL_HANDLE

msg[4]

and

msg[5]    are opcode dependent

msg[6]

and

msg[7]    should be cleared

The  first thing happening when DP receives such a message is  an

acknowledge,  DP sends you the message back,  using the value  of

9167 in msg[0] and exchanging the values of msg[1] and [3].  Now,

you know that communication works.

Supported commands (selected in msg[2]):

0    echoes command range in msg[4] and [5].  With DP  v3.60

you find 0 in msg[4] and 4 in msg[5],  what means  that

the commands 0 to 4 are accessible.

1    indicates  DP that you want to import a mail  into  the

bbs.  msg[4]  and [5] contain the adress of  a  string,

DigiPoint Documentation                       16/05/94 v3.60 - Page 20

this string is the filename (with path) of the mail  to

be sorted.  The string is of C-type (ending with  ASCII

0).  msg[4]  contains the LSB,  msg[5] the MSB  of  the

adress. Refer to section 8.14. for the proper format of

the file.  One hint:  if the filename is SENDING (+  an

unsignificant extension),  the mail will be put in  the

forward. If the name is another, it's only treatened as

a local import.

2    msg[4]  and  [5] contain the adress of a string  to  be

displayed in DP's debug window. The string is of C-type

(ending with ASCII 0).  msg[4] contains the LSB, msg[5]

the MSB of the adress.

3    msg[4]  is  the  kbshift()-state and  msg[5]  the  full

scancode of a key that should be 'typed' by DP. The key

is  sent  to the uppermost window of  DP.  I  have  not

tested  this,  maybe  it  only  works  if  the  sending

application is not the possessor of the front window.

4    forces DP to free all buffers (as if you pressed ALT-2)

5.8. The Command Window

The command window is a rudiment of older versions.  The  onliest

thing  it serves today is to enter a new remote password for  the

packet  radio terminal,  to request the amount of free RAM or  to

monitor  the MODEM port (take care,  for error free  communiction

with high baud rates you should leave the port unmonitored).

5.9. The Debug Window

The debug window is a display-only window.  If you have problems,

open it.  In some error conditions DP displays messages there. It

is not a real help for you,  but while programming new versions I

use that window extensively.  But,  to minimize program length, I

delete  most  of  the  error  messages if  all  is  ready  to  be

distributed.

DigiPoint Documentation                       16/05/94 v3.60 - Page 21

6. Packet Radio Operations

6.1. The Basic Packet Radio Commands

Open  a packet radio window.  This is done by selecting the  menu

option GENERAL -> NEW WINDOW. You can switch to your desired TNC-

channel using the function keys F1 - F10. ALT-M selects channel 0

(the monitor).  The actual channel is indicated in the upper left

corner  of  the  window.  You can also  switch  the  channels  by

clicking  in  the inverted line that separates the  pretype  area

from  the receive screen.  If you have more than 10 channels  per

TNC,  you can successively select the desired channel by multiple

clicks or function key presses.

Now  type  some signs.  They appear in the first section  of  the

window. They are sent after pressing RETURN. But - the channel is

not yet connected.  Instead of an error message,  DP builds up  a

local  loopback.  You  are  connected to your  own  node  (except

channel 0 is selected,  then,  all typed data is sent in  unproto

mode).

Now you want to connect someone.

Type ESC.        (this initializes a TNC-command)

Type C           (this is the TNC-Command (connect))

Type a callsign  (try the one of your local digipeater)

Type RETURN

After  some  seconds  a connection  should  be  established.  The

loopback mode of DP is automatically switched off.  All that  you

type will be sent to the station you connected above.  To end the

connection,  press  ALT-D  (this generates  internally  the  TNC-

command  ESC-D,  of course you could type ESC D RETURN to do  the

same).

Please  refer to your TNC documentation to find out  which  other

commands are possible.  Normaly, three commands are sufficient to

work with DP:

ESC C (start connect)

ESC D (end connect)

ESC I (set callsign of this channel)

DigiPoint Documentation                       16/05/94 v3.60 - Page 22

6.2. The 'special' Keys

Within  a packet radio window,  the following function  keys  are

active:

ALT-C          Starts  a connect using the autorouter.  The  file

DP\PACKET\PATHLIST.DP has to be valid!

Example: ALT-C DB0XYZ <RETURN>

ALT-D          Disconnects the QSO.

ALT-F          Sends  a textfile (to be choosen in the  appearing

file select box).

ALT-J          Switches the channel in the converse mode,  or  if

it was in converse,  switches back to normal  con-

versation mode.

ALT-M          Switches  to  the monitor channel.  You  can  also

click  on the very left position of  the  inverted

line in the window.

ALT-N          Ends a QSO-monitor on this channel.  You can  also

click on the corresponding position of the  inver-

ted line in the window.

ALT-O          Toggles sysop-mode for this channel.

ALT-P          Toggles printer on/off for this channel.

ALT-S          Toggles save on/off for this channel.

ALT-W          Starts save on this channel, all yet received data

(from the start of the backscrollbuffer) is  saved

first. Switch off by pressing ALT-S.

SHIFT-SHIFT-ALT-1   locks/unlocks the keyboard and menues.

ALT-2          Clears  all buffers of all packet  radio  channels

(when you ran out of memory...).

CTRL-HOME      clears buffers of this window.

SHIFT-CTRL-D   Inserts the current date.

SHIFT-CTRL-T   Inserts the current time.

F1-F10         Switch channels.

The scroll control:

===================

numerical +         scrolls page down.

DigiPoint Documentation                       16/05/94 v3.60 - Page 23

numerical -         scrolls page up.

SHIFT-numerical +   scrolls line down.

SHIFT-numerical +   scrolls line up.

HOME                scrolls to start of buffer.

SHIFT-HOME          scrolls to end of buffer.

6.3. Additional Commands

<ESC> QRG      Sets the actual QRG of this TNC.

Example: <ESC> QRG 430900 <RETURN>

<ESC> SB       Send Binary.  Sends a file in binary mode.  If  no

filename  was added to the command,  a  fileselect

box appears.

Example: <ESC> SB <RETURN>

<ESC> SB C:\TEST.BIN <RETURN>

<ESC> PAC      Sets the maximum packet length to be used on  this

TNC.

Example: <ESC> PAC 200 <RETURN>

<ESC> LP       Load Pathlist. After editing the PATHLIST.DP while

DP  is running,  you can reload the  altered  file

with this command.

Example: <ESC> LP <RETURN>

<ESC> LV       Changes  the  current  userlevel  of  a  connected

station. Example: <ESC> LV 6 <RETURN>

<ESC> HOLD     Toggles  the hold function for  this  channel.  If

defined in PRCONF.DP , a string is sent every time

the hold - timer expires.  The timer is reset when

typing something else on this channel.  This  fun-

ction is used to prevent timeouts,  especially  in

DX-clusters.

<ESC> //COMMAND Instead  of COMMAND  you can  type  every  remote

command  of DP (see below).  The command  is  then

executed  as if the station you are  connected  to

has requested it.

<ESC> ///TEXT  Instead of TEXT you can type any signs.  The  text

is  then treated by DP as if the station  you  are

connected to has typed it.

DigiPoint Documentation                       16/05/94 v3.60 - Page 24

6.4. The Internal Remote Commands

DP  has  a  set  of functions to be  activated  by  the  external

connected  station.  To  distinguish those commands  from  normal

conversationary  text,  they have to be sent as first words of  a

line, starting with two '/', for example

//HELP

When  a  channel  is in the node mode,  the leading  //  are  not

necessary.  This is,  by the way,  the onliest difference between

'normal'  mode  and  node mode.  In  node  mode,  every  line  is

treatened as a command.

The built-in (or internal) remote commands of DP are:

//All <text>   = send <text> to all connected channels

//Box          = starting mailbox

//BBS          = //BOX

//CAT          = show file directory

//COMP         = (de)select static huffman data compression

//Connect      = cross-connect (//C Call Port)

//CONVers      = convers-mode

//CStatus      = connect status

//DBase        = data base module ( e.g. //DBASE NEW YORK )

//DIR          = //CAT

//Disconnect   = quick disconnect

//Echo text    = reply 'text'

//Help         = help

//Info         = show system information

//Kill         = erase a file (sysop only, //KILL C:\DP\DP.CFG )

//Language     = show/set language

//Level        = request own userlevel

//MHeard       = show heard stations

//News         = latest news

//Param        = show tnc-parameters

//PREfix       = prefix-list : e.g. //PREF 3B8

//Quit         = disconnect

//Read <File>  = read a file in text mode

//RESet        = creates a system reset (sysop only!)

//Ring         = call SYSOP

//RPRG <File>  = read a file in binary mode

//RTt          = echo runtime test (round trip time)

//Sat          = activate satellite calculations

//SYStem       = password routine for sysop login

//Time         = time

//User         = //CStatus

//Version      = show version number etc.

//Write <File> = open a file for write in text mode

//WPRG <File>  = open a file for write in binary mode

//<n> <text>   = send <text> to channel <n>

DigiPoint Documentation                       16/05/94 v3.60 - Page 25

Most  of the remote command set should  be  understandable.  Some

specific commands are explained below:

//COMP

//COMP  ON enables the huffman frame encoder  (and  decoder,

too).  Every  transmitted frame is  compressed.  The  static

algorithm provides a data reduction of about 30% with simple

ASCII text,  but binary datas will be longer than before. In

that case, the compression is switched off for that frame.

//COMP has four parameters: ON OFF 1 0

For  operators use,  only ON and OFF are useful.  //COMP  ON

will echo a //COMP 1 to the requesting  station,  permitting

that  both sides switch on the mode  simultaneously.  //COMP

OFF echoes a //COMP 0 string.

You can enter a line '//COMP ON' in your CTEXT,  so whenever

a  station is connecting you,  that also supports  the  com-

pressed format,  the feature will be activated.  With others

this will generate an error message and nothing happens.

Is this encrypted transmission?

No,  of course not. The compression algorithm is for example

also used in PACTOR,  and as DP exercises, it is monitorable

by  any other station that has included a  huffman  decoder.

The encoding table is found in Appendix H.

The //comp - method was first used by DK4NB in SP 9.00,  the

table is his setup.

//CAT , //READ , //RPRG , //WRITE , //WPRG , //KILL

These commands are directly manipulating the file system  on

your  harddisk.  A normal remote user has a  limited  access

only to the REMOTE - folder,  usually DP\REMOTE\.  Note that

also sub-folders of REMOTE are accepted.

After priviledging the remote user as a sysop,  he has  full

access to every partition of your harddisk.

The //KILL - command is a sysop-only - command.

//READ and //WRITE are useful for plain ASCII files.

//RPRG  and  //WPRG  is  the command  set  for  binary  file

transmission.  When  these two commands are called with  two

filenames  in the option field (//WPRG TEST.BIN  EPROM.BIN),

the following will be done:  DP creates a file TEST.BIN  and

generates a //RPRG EPROM.BIN,  what results in the transmis-

sion of EPROM.BIN of the remote user,  saved as TEST.BIN  on

DigiPoint Documentation                       16/05/94 v3.60 - Page 26

the local drive.

The  binary  transmission is checked  by  a  CRC,  erroneous

transmissions are deleted. The transmission protocol is well

known by THP,  SP,  GP and other packet programs, unfortuna-

tely it has nothing to do with the YAPP transfer protocol.

If  someone is interested in the full specifications of  the

format,  write me a letter.  One hint: the CRC is calculated

(unfortunately, but not my fault) not the best possible way,

and  it  is incompatible to other CRC  used  within  amateur

radio (PACSAT broadcasts,  frame check sequences of (A)X.25,

PACTOR  CRC,  SMACK KISS Mode).  This is a historical  fact.

But:  you can (for own programs) omit the CRC in the  binary

transmission header, the protocol works without...

DP has a special feature: the auto - compression of a binary

transfer.  If  the  remote station was identified as  a  DP-

station  (this  happens automatically at the  start  of  the

connect),  the binary file is first compressed with the -lha

1- - algorithm,  then sent and automatically decompressed on

the  other  side.  Therefore it is VERY useful  to  use  the

binary mode also for simple text files. The compression rate

is  very  high  with text files.  Archiver  files  are  sent

uncompressed  (.LZH,  .ZIP,  .ARC,  .ZOO),  the  compression

result  won't be worth the work.  The binary  routines  work

well with TheBox - BBS - systems in binary up/download (this

is  a  german standard,  DP uses the same for  its  built-in

BBS). BGET/BPUT with version 5.15 of f6fbb works, too.

Note that starting with v3.60.00,  DP supports a resume mode

in  binary transmissions.  If connected to another DP or  SP

user the programs will communicate automatically and in case

of  a retransmit after a link failure restart the  transmis-

sion at the point it was interrupted (crash recovery).

The protocol expansion was made by DK4NB for SP v9.00.

Write  me  a letter if you want the full  specifications  of

this mode.

//CONNECT

DP offers a powerful cross-connect mode.

A  user may connect any station 'via' DP (don't mix that  up

with  AX.25 - 'via' - digipeating).  Three  different  cases

have to be distinguished:

1) //CONNECT DL9XYZ

This  is the most simple case.  DP will generate  a  connect

request on the same TNC the user came in.  If the connection

is established,  data transfer between DL9XYZ and the remote

user is completely transparent.

DigiPoint Documentation                       16/05/94 v3.60 - Page 27

2) //CONNECT DL9XYZ 1

DP  is  forced  to  establish the  connect  on  TNC  #1  (if

available).  This  allows  you to use DP as  a  crossconnect

digi.

3) //CONNECT DB0ABC

This  looks like case 1),  but note:  if a connect  path  to

DB0ABC is found in the PATHLIST.DP - definitions,  a succes-

sive connect is generated,  such as:  C DB0AAA,  C DB0AAB, C

DB0ABC

The user will be informed about every fragment of path while

connecting,  in  the result it is a  transparent  connection

between  the  remote user and a maybe far  away  digipeater,

connected by DP in several logical steps.  DP selects  auto-

matically  the  outgoing TNC,  if none was selected  by  the

user.

IMPORTANT:  All connect features of DP require defined digi-

peater  link  status  messages.  The built  in  set  is  the

standard  english  one.   The  most  important  message  is:

'Connected  to' (in whatever,  upper-  or  lowercase).  Some

french  digipeaters  do not match that  standard,  they  use

french texts.  Fortunately, our world offers many languages,

but  please,  understand  that a machine interface  is  less

flexible  than  a  human...  Link status  messages  are  the

essential  of a packet radio connect,  try to convince  your

local sysop of the problems he causes with his changes. Meme

que je suis infecte par la francophilie.

Starting  with v 3.60,  you can define additional texts  for

the link status messages. See above: The File PRCONF.DP. So,

even french is no more a problem hi.

//CONVERS

DP  has  a  built-in converse  mode.  Several  stations  may

connect DP and enter //CONVERS,  what results in a conferen-

ce. You can start the convers on the console of DP in a free

channel,  so you can participate.  When pressing ALT-J,  the

syop  can  force a channel to convers mode and  vice  verse.

Multiple  DP-nodes  can be connected to  one  'big'  convers

node. Connect a far away DP-node, type //CONV and press ALT-

J on the same channel,  that's all.  Every user on both  (or

more) DP-nodes is in touch with every other convers - user.

//DBASE

DigiPoint Documentation                       16/05/94 v3.60 - Page 28

This  starts a request to the built-in database of  DP.  The

database  contains  informations about nearly  every  german

village  and a large amount of cities worldwide.  Please  do

not  try  to add informations in the database  file.  It  is

indexed  by CITIES.IDX,  and every change leads to  inconsi-

stence.

Special informations for programers:

Revise  CITIES.LOC.  The  structure of the  file  should  be

logical.

The  format of CITIES.IDX is an ARRAY['A'..'Z','A'..'Z']  of

LongInteger,  where  the  first field refers  to  the  first

letter of an entry, the second field to the second letter of

an  entry  in CITIES.LOC.  The LongInteger -  value  is  the

offset from the start of CITIES.LOC to the actual  position.

All special signs (such as ,) are treatened as 'Z'.  Every

offset value in CITIES.IDX points to the first apperance  of

the  two-letter-combination in CITIES.LOC.  So,  if  a  user

types '//DBASE Berlin',

DP  looks  in the index table,  searches for the  offset  of

['B','E'],  reads  CITIES.LOC starting with the  offset  and

compares  all  following  entries  with  the  full   string,

'Berlin'. The search is aborted with the first appearance of

'BF'.

The geographical coordinates are in decimal format.

The database files are in DP\VERWALTG\

(VERWALTUNG is the german word for ADMINISTRATION).

//PREFIX

This starts a request to the prefix - database of DP.

//PREFIX 3B8 gives you all informations about Mauritius. The

prefix  database  was  created by DJ3CW  Werner  who  kindly

allowed the use by DP.

The  prefix  database  structure  is  more  easy  than   the

CITIES.IDX.  First of all it is completely line oriented, so

you  can  correct wrong entries in PREFIX.INF  with  a  text

editor,  or add new ones.  The PREFIX.SUB contains  informa-

tions about prefix equations,  such as ZZ = PY.  It is a set

of two-line records,  where the second line will replace the

first one while searching.  Both files have to be alphabeti-

cally sorted from Z .. A and 9 .. 0

The database files are in DP\VERWALTG\

//SYSTEM

DigiPoint Documentation                       16/05/94 v3.60 - Page 29

The  //SYSTEM - command allows a remote user to log in as  a

sysop.  The proceeding works similary to TheNet -  standard:

DP responses the request with 5 numbers,  those numbers  are

corresponding  to 5 signs of the password string defined  in

the  install dialog of DP.  The remote user has to send  the

five correct signs (without spaces) as answer.  If they were

correct,  he  is accepted as sysop.  The user may  send  the

//SYSTEM  command as often as he wants,  with false  answers

and  only one right answer between,  to increase the  safety

against monitoring hackers.

//RESET

A  sysop  can force a reset of  the  computer.  The  //RESET

command has three different options:

1) //RESET

Writes an info about the following reset to every  connected

station, disconnects all users, saves the current parameters

and resets the system

2) //RESET sri, system maintenance works for the next hour

Will write the text 'sri, ...' and proceeds as 1)

3) //RESET +

Immediate reset.  No message, no disconnect, no save, just a

reset.

6.5. The External Remote Commands

You  can add an unlimited number of external remote  commands  to

extend the internal ones. All you have to do is to write a little

(and,  please,  fast  working) program and place it in  the  RUN-

Directory (normally DP\PACKET\RUN\).  The program may not have an

extension  even  though it is a program hi.  When  DP  detects  a

remote request that doesn't fits on the internal remote commands,

it  revises the RUN-Directory and tries to start a  program  with

the  name of the remote command.  This program may create a  file

OUT.TXT,  the content should be pure ASCII text.  If this file is

found after execution, DP will send it and delete it.

When  starting  an external remote program,  the  commandline  is

filled with some parameters:

first all parameters the user added to the remote command, then a

sysop-flag (0 if no sysop level,  1 if sysop), then the userlevel

and as last parameter the callsign of the user.

DigiPoint Documentation                       16/05/94 v3.60 - Page 30

When, for example, DL9ZZZ types

//REMTEST check

the program REMTEST will find in its commandline:

check 0 9 DL9ZZZ

Some programmers had problems with the commandline. How is it set

up?  The  first  byte of the commandline is the length  of  valid

data in the line (as GEMDOS requires it).  All superseeding bytes

are cleared with ASCII 0. Do never expect the commandline to have

a fixed length!

6.6. The Language Files

DP is multilingual.  Depending on the users callsign,  a language

is selected. This mechanism works as follows:

The file DP\VERWALTG\WHOTALKS.LAN assigns callsigns to languages.

An example:

DU G

D DL

OE DL

HB DL

G G

F F

ON NL

3B8 F

PA NL

PB NL

PE NL

PI NL

The  first  word  is a prefix,  the  second  a  language.  If  no

assignment is found, language 'G' (english) is used.

Now,  for  all user dialogs of DP in packet  terminal  mode,  the

corresponding  file  DP\PACKET\LANGUAGE\XXX.PLN  is  used,  where

'XXX' is replaced by the selected language.

If no file is found, DP tries to load at least G.PLN (english).

You can easily create new language files,  what about a hungarian

one,  creating a file DP\PACKET\LANGUAGE\HA.PLN and assigning  HA

HA,  HG HA in WHOTALKS.LAN? As the language name in some cases is

used as a file extension, it may not exceed three digits.

DigiPoint Documentation                       16/05/94 v3.60 - Page 31

The other files in the folder LANGUAGE\ (HELP,  INFO,  NEWS)  are

the  corresponding answers to the remote commands of  this  name,

existing  in every available language,  the extension must be  of

the type of the user language.

6.7. The Text Convert Routine

DP  has  a  simple  text convert  routine  used  by  some  output

routines. More precise, it is used for the output of:

the HELP - file

the INFO - file

the NEWS - file

the GREETING - file

the WELCOME - file

the remote-IDENT

the //ECHO - command

the BATCH - processor

Converted Sequences:

\r   ->   CR LF (end of line)

\7   ->   CTRL-G (bell)

\z   ->   CTRL-Z

\D   ->   date

\T   ->   time (with seconds)

\Z   ->   time (without seconds)

\L   ->   logintime

\C   ->   call of remote user

\M   ->   own call

\N   ->   name of remote user (if defined in GREETING.DP)

\K   ->   TNC channel

\k   ->   internal channel number (TNC 1 chan 1 e.g. 12)

\d   ->   TNC number

\V   ->   version number of DP

\Q   ->   frequency of users TNC

\X   ->   this  is  special.  \X  has to be  followed  by  two

signs,  these  are  interpreted as  a  hexadecimal

value of a sign. \X4A is transformed to 'J'

\\   ->   \

The  standard  of this conversion was set by dl1men for  his  SP-

program.

6.8. The File GREETING.DP

The  file  DP\PACKET\GREETING.DP is used to define  user  levels,

user names and personal welcome messages.

It is a set of entries separated by '@'.

DigiPoint Documentation                       16/05/94 v3.60 - Page 32

Example:

@DL8HBS:9 Joachim

Welcome home... (CH \K, \Q Mhz)

\C de \M (\Z UTC)>

@DL1HAZ:7 Walter, DL1HAZ

@DB0GR:3

@DL9ZZZ:*

Immediately following the callsign we have the definition of  the

user level, DL8HBS has level 9, DL1HAZ level 7 and so on.

The  following  words in the same line are displayed in  an  info

text when this user connects DP. The optional lines following the

first line of an entry are sent as personal welcome message.

A user level definition of :* forces DP to immediately disconnect

the channel, the user is suspended.

The default user level is 1 (if no information was found).

Some user levels have special meanings (marked with '-':

- *  suspended

- 0  remote commands inhibited, error message occurs

1  level 1

- 2  remote commands inhibited, error message occurs

- 3  remote commands inhibited, no error message occurs

4  level 4

5  level 5

6  level 6

7  level 7

8  level 8

9  level 9

This seems to be a little complicated,  in fact it was a standard

of dl1men's SP, I took the same syntax.

User level 3 is conveniant for bbs callsigns.  If someone added a

useless //Q in a message you are reading out, DP will ignore that

command instead of disconnecting. Also, this generates a slightly

different reaction of DP when using the //WPRG - //RPRG  commands

for  TheBox - binary up/download.  So,  define every BBS you  are

using with a user level of 3 in GREETING.DP to avoid problems!

The minimum level numbers required to execute a //REMOTE  command

are  defined in a dialog within DP.  Refer to section  6.10.  You

can,  for example,  allow only users with a level > 4 to use  the

crossconnect feature, > 5 to enter the built-in BBS and so on.

DigiPoint Documentation                       16/05/94 v3.60 - Page 33

6.9. The Menu PARAMETERS -> PACKET

When  selecting  the menu entry Parameters ->  Packet  an  option

dialog for the packet radio terminal appears. Some of the options

are not worth explaining them.

The section MONITOR contains the following elements:

QRG-Check                Normally,  this should be  selected.  If

QRG-Check  is OFF,  the QRGLIST.DP  will

not be compared with the monitored frame

headers.

update heardlist         I have forgotten why I made this  switch

accessible. Let it switched ON.

short headers            If selected,  all monitor frame  headers

are shortened to the absolute minimum.

show Flexnet-frames      If selected,  the FlexNet routing frames

are displayed (but not decoded).

show TheNet-frames       If  selected,   the  TheNet  /   Net/Rom

interlink   and  broadcast  frames   are

displayed (and decoded).

show TCP/IP-frames       If selected, TCP/IP frames are displayed

(but not decoded).

show binary frames       If  selected,  binary  data  frames  are

displayed.

show pacsat broadcast    If not selected, pacsat broadcast frames

are suppressed. Note: I am experimenting

with the pacsat broadcast protocol,  but

the  current  versions  of  DP  do   not

include the decode/broadcast feature.

show huffman-frames      if selected,  huffman compressed  frames

(up to now generated by DP and SP v9.00)

are displayed decoded in the monitor. As

the detection of this kind of frames  is

not 100% sure,  from time to time it may

happen that normal frames are  treatened

by   the   decoder  (what   results   in

rubbish). Also, the decode check takes a

good amount of time.  This is no problem

with  data rates of 1200 baud,  but  you

will  remark  it  at  9600   baud.   So,

normally,  set  the  parameter  OFF  and

switch  it on if you want to  monitor  a

huffman compressed connection.  SP has a

protocol   behavior  that  causes   some

troubles:  it  also encodes frames  with

huffman  if  the result will  be  bigger

DigiPoint Documentation                       16/05/94 v3.60 - Page 34

than the input - DP passes those  frames

in  plain ASCII to gain data  throughput

(what  is  the  main  ambiance  of  this

protocol).  Such SP-generated frames are

not  accepted by the DP-decoder  because

of  the obviosity to crypt  transmission

instead    of    gaining     throughput.

Fortunately,  this happens very  seldom.

Write a letter to DK4NB @ DB0AAB to stop

this.

The section ADD ON'S contains the following elements:

startup in hostmode      Per  default,  DP  asks you  on  startup

wether  the TNC's should be  started  or

not.  If this parameter is selected, the

question is supressed and the TNC's  are

started automatically.

auto-compression         This switch determines, wether DP should

compress binary file transfers or not.

auto-parameters          DP   is  able  to  set  the   parameters

P(ersistence)  and O (MaxFrame) of  your

TNC's automatically corresponding to the

traffic load of the hf channel.  Test it

and  find  out  if  this  will  generate

proper  parms  for  your  local   packet

situation.

announcement of call     If TALK.TTP (author DL1XAO) is copied in

the folder DP\VERWALTG\,  DP generates a

synthetic speech announcement of a newly

logged  in  callsign  (all  in  german).

TALK.TTP  was too big to put it  on  the

DP-distribution disk.

connect pop-up-box       If  selected,   a  user  connecting  the

terminal  of DP will be displayed  in  a

pop-up-dialog announcing its call,  name

and channel.  On the FALCON,  the pop-up

did  not  disappear after  the  selected

time delay.  As I do no understand  why,

FALCON users are recommended to set this

switch OFF.

The section TIMING contans the element

frames per loop          this value determines the maximum amount

of frames read by one main program  loop

per  channel.  If the channel is 0  (the

monitor),  the  amount is multiplied  by

two.  In 1k2 operation, you can set this

value  to 1,  in 9k6 set it to  a  value

DigiPoint Documentation                       16/05/94 v3.60 - Page 35

between 5 and 15.  The lower the  value,

the  more time is left for keyboard  and

mouse.  This depends on the traffic load

of your local digipeater.

6.10. The Packet Terminal Pop-Up Menu

When  clicking in the middle of a packet radio window,  a  pop-up

menu appears. You can click on one of the following items:

connect        Opens  a select - window if  PATHLIST.DP  contains

valid information for this channel.  You can click

on  a line of this window to start a connect to  a

station.

disconnect     Same as ALT-D, ends a current connect.

load text      A  file select box appears and the  selected  TEXT

will appear in the pretype buffer.

send text      A  file select box appears and the  selected  TEXT

will be immediately sent on the actual channel.

send binary    A  file select box appears and the  selected  file

will  be  immediately sent in binary mode  on  the

actual channel.  Of course, you can also send TEXT

files by this mechanism. When connected to another

DP   station,   this   has  the   advantage   that

transmission will be compressed with LZH.

flags          A  second pop-up menu appears,  the entries  SAVE,

PRINT  and ECHO toggle the corresponding  terminal

functions.  ECHO  means,  that the  pretyped  text

appears in the connection window when sent.

TNC-parms      A  second  pop-up  menu appears and  shows  you  a

subset  of  the current TNC - parameters  of  this

channel.  If you click on the items of the  second

pop-up,  a  dialog appears and you can  alter  the

desired value.

userlevel      A  dialog appears informing you about the  userle-

vels  that are required to use a  remote  command.

You can alter the settings in this dialog.

heardlist      A  dialog  appears informing you  about  the  last

heard stations.  When clicking on one of the lines

the  QSO-monitor is started for the  desired  con-

nection.  A second pop-up menu appears, asking you

if  you  want  to monitor  'only  text'  or  'also

headers'.  The second is favourable to check  pro-

tocol or parameter problems of the monitored  sta-

tions.  DP  will search from the beginning of  the

backscroll   buffer  for  older  packets  of   the

DigiPoint Documentation                       16/05/94 v3.60 - Page 36

selected  connect  and  write them  'raw'  on  the

screen.  If in 'only text' - mode,  all  following

monitored frames are filtered for repetitions  and

missing frames will be indicated.

close          Closes this window.

6.11. The Packet Terminal Read Generator For Mailboxes

The read generator serves you to generate read commands out of  a

received  mail list of a BBS you are connected to (the list  that

appears  when  you enter 'L' in a F6FBB-bbs or 'C' in  a  TheBox-

bbs).

Clicking on a line of such a list with the RIGHT mouse button and

pressed  ALT-key  will generate and send immediately  the  corre-

sponding read command to the bbs.

Doing the same without pressing the ALT-key will result in a pop-

up menu where you can select what to do with the mail:

set lifetime   Only for remote sysops of a TheBox / DP  bbs:

You  can set a new lifetime of the mail in  a

dialog.

delete         A delete or kill command is generated.

read           A read command is generated.

transfer       Only with TheBox bbs:  a transfer command  is

generated,  you  will  be asked for  the  new

destination in a dialog.

export         Not supported here.  The same pop-up is  used

in your own bbs, there it has an effect.

The  result  of your operations is not immediately  sent,  it  is

written  in  the  pretype buffer,  you have to  confirm  it  with

<RETURN>.

When clicking on a line that contains no mail list,  the line  is

copied  in  the prescribe buffer,  this function  allows  you  to

manually echo received text.

6.12. The Automatic Password Generation

DigiPoint Documentation                       16/05/94 v3.60 - Page 37

DP offers sysops a highly automized feature to priviledge them at

their  remote controlled system.  For the following  systems  the

login procedure is done by DP:

- TheNetNode and compatible (TheNet,NET/ROM,BaycomNode,DP)

- F6FBB, BaycomBox and others using the 5-char-method

- RMNC

- TheBox

The  onliest you have to do is to write your password in  one  of

the following files (folder DP\PACKET\) :

- THENETPW.DP       for TheNetNode, NET/ROM and BaycomNode

- OTHER_PW.DP       for F6FBB, BaycomBox and others

- RMNCPW.DP         for RMNC

- DB0XYZ.DBP        for  TheBox,  where  DB0XYZ  has  to  be

replaced by the callsign of your BBS

Note  that the entry in OTHER_PW.DP is analyzed  WITH  SSID.  The

other password files do not contain a SSID...

Then, you have to connect your digipeater or node and type either

sys

//sys

priv

pw

to start a login. Of course, you have to use the above keyword as

is defined for your digi/bbs...

Please refer to the dummy - password files distributed with DP to

find more information about the format of the files.

Except  with  TheBox,  the procedure sends  four  login-requests,

whereof only one is correctly replied.  This prevents making  the

life of hackers too easy.

What is the difference between THENETPW.DP and OTHER_PW.DP?

The  detection  of  the  password  prompt  is  more  secure  with

THENETPW. TheNet - systems always prompt with this syntax:

IDENT:CALL> 1 2 3 4 5

And this syntax is NOT changeable!

BaycomBox or F6FBB usually prompt this way:

CALL> 1 2 3 4 5

or

1 2 3 4 5

DigiPoint Documentation                       16/05/94 v3.60 - Page 38

or

(1)> 1 2 3 4 5

The scanning mechanism then only reacts on 5 numbers at the right

end of the string.

6.13. The File QRT.DP

The file DP\PACKET\QRT.DP contains logoff messages.  If more than

one message is defined,  one is randomly selected. The message is

sent when a remote user types //Quit.

To define more than one message,  you have to separate them  with

'\\' in a single line. The length of every single message and the

total  amount of messages is not restricted.  When defining  more

than one message,  the total amount of messages has to be  marked

in the first line of QRT.DP with '\372' (for  example),  followed

by '\\' in the second line.

6.14. The File WELCOME.DP

The  file DP\PACKET\WELCOME.DP contains a general logon  message.

Depending  on the switch CTE= in PRCONF.DP (see above) this  text

is sent to every connecting station or only to those that have no

personal defined ctext in GREETING.DP.

6.15. The File NODE.DP

The file DP\PACKET\NODE.DP contains a general logon message  that

is sent to connecting stations of a node channel of DP.

6.16. The Status Display

The  status display is the title line of the  packet  window.  It

looks as follows:

CH:1|DL8HBS|DB0BLO-8|438.300|200|IXF|00|00|00|<flags>| 1  ! #MB#1

|     |       |        |     |   |  |  |  |     |     |  |   |

1     2       3        4     5   6  7  8  9    10     11 12  13

The fields are:

DigiPoint Documentation                       16/05/94 v3.60 - Page 39

1    the tnc channel of this window

2    the MyCall of this channel

3    the call you are connected to

4    the QRG of this tnc

5    the packet length of this tnc

6    the connection state of this channel

7    the frame counter for unsent frames

8    the frame counter for sent but unacknowledged frames

9    the retry counter

10   the flag field

11   the userlevel of the remote user

12   sysop status (='!' is displayed) or not

13   amount of mails scanned at the moment in the monitor

The flag field knows the following flags:

A    pending autoconnect

B    own bbs is active on this channel

C    message is recorded (boxCut)

E    echo is on

F    S&F is active on this channel

H    hold timer is active

M    QSO monitor is on

P    printer is on

S    save is on

b    connected to a TheBox system

p    huffman data compression active

s    satellite calculations active on this channel

x    convers active

6.17. The Connect Status

The  connect  status informations are displayed in  the  inverted

line that separates the pretype screen from the receive screen.

The connect status of up to 10 channels is displayed:

|       |*DB0BLO|>DL7AND|.DB0GR|       | dl4ybg|       |       |

*    indicates that this channel is displayed

>    indicates that on this channel a QSO monitor is active

.    indicates  that  on  this  channel  are  unacknowledged

frames

a  lowercase call indicates that data was received  but

not yet displayed

DigiPoint Documentation                       16/05/94 v3.60 - Page 40

7. The Satellite Calculations

The satellite calculations are only available in DP.PRG. They are

excluded in DP_MINI.PRG.

This  module  offers  you the possibility to  track  and  display

satellite positions, to steer your antenna rotator and to display

packet radio network maps.

7.1. The Available Satellite Calculations Commands

The set of commands depends on wether the satellite  calculations

are called in grafics mode (directly at the DP - console) or  via

packet radio (what results in text mode output).

The commands in text mode are:

<name>              shows   actual  position  of   satellite

<name>

NEXT <name>         shows   next  visibility  of   satellite

<name>

KEPLER <name>       shows the keplerian elements for  satel-

lite <name>

LIST                shows all stored satellites

VISIBLE             shows all current visible satellites

PROGNOSE <name> <h> the same as NEXT,  but all orbits in the

window <h> are calculated,  not only the

next.  <h> counts in hours starting with

the  current time.  Called by  a  remote

user, the window is limited to a maximum

of 2 hours due to the enormous amount of

processing time of this command.

QTH <qthloc>        shows  or alters the own QTHLOC that  is

used for the calculations.  Per default,

it is always the QTH of the installation

procedure.

TRACK <name>        calculates  the position of a  satellite

in  a constant loop,  only aborted  when

entering the next command.

QUIT                ends the satellite calculations  module.

The remote user is reconnected to either

the packet terminal or the bbs.

DigiPoint Documentation                       16/05/94 v3.60 - Page 41

The command set for grafics mode is more powerful:

<name>              see above

NEXT <name>         see above

KEPLER <name>       see above

LIST                see above

VISIBLE             see above

PROGNOSE <name> <h> the  prognose  command is  different  in

grafics mode.  <name> could also be ALL,

what  checks all satellites (take  care,

depending   on  the  amount  of   stored

satellites  this takes a lot  of  time).

The parameter <h> is not limited.

QTH <qthloc>        see above.  If the world map is  active,

the qth blinks for a while.

TRACK <name> <+>    this  is  the main  command  in  grafics

mode.  The world map will be  displayed,

one orbit of the choosen satellite  will

be  calculated  and the  groundtrace  is

plotted on the map. The display is ended

when entering a new command.

The  option <+> forces DP to search  for

ROTARY.PRG in the SAT - folder, if found

this  program will be called every  some

seconds,  the  commandline of ROTARY  is

filled  with  the actual  elevation  and

azimut of the satellite.  Frank, DL6DBN,

wrote  a universal ROTARY.PRG that  con-

trols nearly every rotator system. Thank

you  Frank.  Read the  documentation  of

ROTARY for more details.

You  can  add up to 2  up/downlink  fre-

quencies,  they will be displayed inclu-

ding  the actual doppler shifts  instead

of   the  default  frequencies   145900,

436000  and 1269000  kHz.  (TRACK  UO-22

435120 145890 for example).

POS <name> <+>      the  POS command is very similar to  the

TRACK command. The difference is that no

groundtrace   will  be   plotted   (this

fastens the calculation).

RANGE ON/OFF        while  using the TRACK or  POS  command,

the   footprint  of  the  satellite   is

DigiPoint Documentation                       16/05/94 v3.60 - Page 42

displayed  as  an inverted area  on  the

world  map.  You can disable  or  enable

this  feature  with  the  RANGE  command

either typed before or while the TRACK /

POS  command is active.  Note  that  the

footprint  is  only displayed  when  the

displayed  map  covers  the  full  earth

surface (this is the default case).

REVOLUTION <name>   plots the groundtrace of a satellite  on

the screen,  the plot is stopped with  a

keypress or after 24 hours of  displayed

time.  I don't know why,  but often  you

have to press several keys to stop  this

function. The map is not deleted by this

command,   so  you  can  plot  different

groundtraces on the same map. Delete the

map with the command MAP.

MAP                 this displays the world map.

PRINT <command>     the PRINT command forces DP to print all

text  output of the  command  <command>.

PRINT NEXT UO-22 for example.

PREFIX <call>       the  position of a call/prefix  will  be

displayed on the world map.

DBASE <citie>       the   position  of  a  citie   will   be

displayed on the world map.

BBSMAP <prefix>     this  starts a very  complex  operation.

The aim is to display the actual routing

map  of the built-in bbs.  The DP -  bbs

saves informations about every mailbox a

mail came from.  This is done by  analy-

sing the mail headers.  The BBSMAP  com-

mand  tries  to display  these  internal

routing  informations  on  the  map.  To

calculate  the geografic position  of  a

bbs, several methods are used:

1) the best one:  a bbs marked  its

WW-QTHLOC in the mail header  (this

is the case with most german  bbs).

The  qth information can be  consi-

dered to be correct.

2) if no qth info was found in  the

header info,  a file DIGIMAP.STN is

used to look up the qthloc.  If  an

info was found, the qth information

can  be considered to  be  correct.

The  file  DIGIMAP.STN  is  of  the

standardised format for programs as

DigiPoint Documentation                       16/05/94 v3.60 - Page 43

HAMMAP  or DIGIINFO.  You  have  to

update this file manually.  Consult

your local bbs for new datas.

3)  if the both preceeding  methods

had  no result,  the position of  a

bbs is calculated relative to their

forwarding neighbours. If a bbs has

three  s&f - neighbours with  known

positions,      the      calculated

(estimated) position of this bbs is

in  the  geografic  center  of  the

three.  The qth information  cannot

be  considered  correct.   Problems

occur  when we have a large  amount

of  unknown  qth's,  the  estimated

positions  then  have  no  more   a

relation  to the physical  reality.

DP  has  some mechanisms  to  avoid

plotting  two  bbs's  on  the  same

screen position.  Thus it cannot be

avoided in every case. An estimated

bbs position results in the  lower-

case  output of the callsign and  a

smaller marker on the map.

Remark:  I cannot understand, why nearly

every  bbs worldwide transmits  its  ZIP

code in the mail headers,  ain't that  a

totally  useless  information?  Did  you

ever write a letter to a bbs?  Why don't

they transmit their qthloc instead?

The   BBSMAP   command   has   different

options. The prefix option allows you to

select  a specific subset of the  infor-

mations to be displayed.  BBSMAP F  will

display all french bbs.  Without prefix,

the  command  uses the first  letter  of

your call as selection. When appending a

whole  bbs call,  the s&f  path  between

your  station and the call will be  dis-

played (BBSMAP GB7XYZ).  An additional +

or  - could be appended to the  command.

The + means:  display only DP-bbs of the

selection,  the - excludes DP-bbs of the

display.  BBSMAP  D - will  display  the

german bbs net without the DP bbs.  Note

that the selection D excludes  automati-

cally  DU,  this is a special  trick  to

avoid  phillipinean bbs plottet  in  the

german net.  BBSMAP ALL shows all  known

bbs worldwide.

What is displayed by this command?  It's

DigiPoint Documentation                       16/05/94 v3.60 - Page 44

not the reality,  but it is the  routing

map that actually is valid for the  mail

router of the DP-bbs. The displayed path

doesn't  mean  that a mail  really  will

take  this way,  but that this would  be

the fastest way a mail could be  distri-

buted.  Unfortunately,  many sysops  use

fixed forwarding settings even if  their

system offers an autorouting feature (as

DP, F6FBB and TheBox do).

DIGIMAP <prefix>    The  DIGIMAP command refers on  the  two

files  DIGIMAP.STN and DIGIMAP.LNK  that

have  to  be placed in the  SAT  folder.

These files are not automatically  crea-

ted,  you have to update them  yourself!

The  file format is of the HAMMAP  data-

file type,  at least in european bbs you

will find updates from time to  time.  I

don't know if that kind of program is in

use in the united states.

DP  creates  an  intermediate   datafile

DIGIMAP.DP if this file is not found  in

the  SAT folder.  This file  contains  a

subset of the informations found in  the

.LNK  and .STN files.  It is sorted  and

indexed  with pointers for fast  access.

Only links and digis marked with 'OK' in

the source files are taken into account.

Do not delete the .STN file after  crea-

tion  of  DIGIMAP.DP,  it will  be  used

later   for  the  display  of   detailed

informations  about a selected  digipea-

ter.

When  using the DIGIMAP command  without

optional  parameters,  all  found  datas

will be plotted on the world map.  If  a

prefix  is  added,   the  datas  of  the

selected  prefix  area  are   displayed.

DIGIMAP  G  will  display  all   british

digipeaters.  DIGIMAP F6ABJ will concen-

trate  the  display on the  area  around

F6ABJ.

Note: Both, the BBSMAP and the DIGIMAP command, require

some 100 k of free memory.  If the available memory  is

lower than necessary, the processing will be aborted or

only partially finished.

While the BBSMAP or DIGIMAP command is active,  you can

click on the map.  A pop-up menu appears,  offering you

the options

DigiPoint Documentation                       16/05/94 v3.60 - Page 45

Callsigns      will display all callsigns

Info           point on a digi / bbs and click  on

it,  a  new window opens and  shows

you   more  detailed   informations

about this station.

Zoom in        define an area with the mouse.  The

scale   of  the  display  will   be

recalculated.

Zoom out       enlarges the scale by two.

Full           sets the scale for full display  of

all selected stations.

N W S E        moves  the  map  clipping  in   the

choosen direction.

QUIT                closes the window.

Whenever a map is displayed,  you can select a new viewport  with

the  mouse.  Click on the map and define an area to be  displayed

with  the  mouse.  Note  that the map  information  is  not  very

detailed. And, remind what was written in section 1., some screen

enhancers have problems with the map display.

7.2. How To Get Keplerian Elements

DP accepts two formats of keplerians,  the NASA two liner  format

and the more verbose AMSAT keplerian format.

The NASA two liners are easily read in by simply reading out such

a  file  of your local bbs.  The informations are stored  in  the

folder DP\SAT\KEPLER\ , where you have to delete them manually if

some of them are erroneous or what else.

Only elements younger than the stored ones are taken.

This automatic keplerian read even works when simply monitoring a

readout of someone other on the qrg,  refer to the bbs section of

this manual for the monitor cut feature of DP.

Alternately you can parse a text file,  containing either NASA or

AMSAT  format,  by selecting the main menu option 'READ  KEPLERI-

ANS'.

DigiPoint Documentation                       16/05/94 v3.60 - Page 46

7.3. The ROTARY Interface

Even though a ROTARY.PRG is distributed with DP you might want to

know how the interface works, e.g. for own developments.

If  selected with the '+' - option of the TRACK/POS  command,  DP

searches for a ROTARY.PRG in the folder SAT.  If found,  a set of

parameters is put in the commandline of this program. The program

is  called every time either the elevation or the azimut  of  the

selected satellite changed and the elevation is higher than  -10.

The  program  must  be  fast working,  as  DP  is  stopped  while

execution.

The parameters in the commandline are:

AZIMUT  ELEVATION MA doppler29 doppler145 doppler436  doppler1269

doppler2410 name_of_satellite

DigiPoint Documentation                       16/05/94 v3.60 - Page 47

8. The Mailbox

The  mailbox  is  only available in DP.PRG.  It  is  excluded  of

DP_MINI.PRG.

The DP mailbox is a synthesis of the in germany well known TheBox

system and the f6fbb system.  The user interface of DP's  mailbox

is  similar  to TheBox,  but the mail forwarding  mechanism  also

includes  the  highly improved methods of f6fbb.  DP  acts  as  a

perfect intermediator between both systems.

You  may  ask  yourself why to implement a bbs  into  a  terminal

program. Some 'DP-history': The first versions of DP were written

because  I was bothered about the big amount of files I read  out

of  the packet bbs saving them on my hard disk.  I  lost  control

about  what was in the files and my file system  exploded.  So  I

wrote  a simple (really,  very simple) 'mailbox' to  administrate

those infos. After some time I thought about why not implementing

a  simple  terminal  in the program to read  out  the  bbs  files

directly into my own 'mailbox'. I believe the most of you has the

same  problems.  This  was  the very start  of  the  DP  project.

Sometimes I think about if it won't have been better to stop  the

development at that point (especially when I think about the time

the  ongoing  took).  Unfortunately,  I had a  new  idea.  I  was

impressed  of the amount of similar message files,  read  out  by

dozens of users day for day via the same digipeater,  out of  the

same bbs.  And all data appeared in the monitor, once, twice, ...

How  inefficient  this was.  I knew about  the  PACSAT  broadcast

experiments  (today,  it's no more an experiment).  That was  the

moment  I decided to program a frame collector that  should  scan

all  monitored frames for mailbox files and save them in  my  own

bbs,  just by monitoring. Whow! It worked! After some weeks, even

errors,  caused by rejects and reception holes were banned. I had

the first bbs filled with files without transmitting itself! That

were the pioneer days of the program, all that went later on were

only kinky improvements,  concerning the surface, reliability and

new software protocols (unfortunately, this took the most time).

And  what is it today?  Hundreds of users in europe,  one of  the

most efficient bbs systems (as far as I know the onliest clone of

F6FBB's protocol),  some installed public bbs with DP,  portation

works to the IBM PC.  It is,  by the way,  no more my project but

also yours.

Maybe this answers your question WHY to implement such a  complex

bbs in a terminal program.  The aim was to minimize radio channel

load  with a new monitoring technique,  and what came out  was  a

complete bbs system.

By the way, you can use DP to write your personal letters OFFLINE

and  then  transmitting them compressed to the next F6FBB  or  DP

bbs.  Some  years after the beginnings of DP,  the  TPK  software

started the same for the IBM PC.

The bbs can be used for personal message storage, but as the step

DigiPoint Documentation                       16/05/94 v3.60 - Page 48

to a fully equipped bbs was not far, you also can install it as a

full time public bbs.

8.1. Board Oriented System vs. MSG Number Oriented System

The user interface is different to that well known command set of

the wa7mbl - oriented systems (as partially also f6fbb  is).  The

reason is simple:  in germany,  those bbs systems are not in use.

I, the author, was used to the TheBox syntax. A version including

the f6fbb syntax is planned,  but shall not be promised.  Try  to

habituate yourself to the TheBox syntax,  it has its  advantages,

too.

The  main  difference is,  that the message  system  is  strictly

'board-oriented'.  A  LIST command will not,  as with  f6fbb  and

others,  output all new files regardless of their destination but

all  new  files of a board to be  specified.  The  current  board

selection is always visible, it appears in brackets in the system

prompt.  If the brackets are empty,  no board is selected and all

board  oriented commands (such as LIST,  ERASE,  READ) will  only

work with the explicite specification of a new board.

Another difference is,  that the command names are more  verbose,

such as READ instead of R.  But,  you can abbreviate them if  you

like to.

The  f6fbb system is message number oriented.  In TheBox  syntax,

this  is unknown.  Of course,  all messages of one board  contain

message numbers to distinguish them,  but they count at every new

board  from 1.  This results in small numbers,  the  human  brain

likes them more than astronomous message numbers as #57293.

8.2. A Typical Session In TheBox / DP - Syntax

Let  me  describe a typical session  under  TheBox  syntax,  this

makes the philosophy visible.

1)   The user logs in.

2)   Automatically,  the selected board is that one of the  users

call sign.  A LIST command is generated,  automatically too.

All new messages to the users personal board since his  last

login are displayed.  If there are no new ones, the last one

is displayed. An example:

[DP-3.60-AB1DFHMR$]

DigiPoint Documentation                       16/05/94 v3.60 - Page 49

DigiPoint Box v3.60.00 operated by DL8HBS

Welcome Joachim!

Last login  : Monday, 15.11.93 21:27

Actual login: Monday, 22.11.93 10:11

info-file: DL8HBS

Nr.  of     date     UTC    bytes  title

1199 F6KVE  17.11.93 15:27   2750  DP3.5 ...

1200 DL7VC  17.11.93 15:27    721  Bin wieder qrv.

1201 DG2GIW 17.11.93 17:35    558  Was ich vergessen hatte...

1202 DK5AD  18.11.93 15:39    950  FRAGEN MAGIC!X

1203 DL1MCX 18.11.93 19:21   1299  Anfrage

(DL8HBS) DL8HBS de DL8HBS>

Now, I would like to read my messages. This can be performed

in three ways.  The most simple is typing R.  This reads all

new messages of the selected board. Look at the above promt,

the selected board is (DL8HBS).  As I am connected to my own

bbs, the DL8HBS appears two more times hi.

I could also type R 1199- to read all messages starting with

1199.  Also,  I  could type R DL8HBS 1199- ,  but the  board

DL8HBS  is  selected,  so this  is  not  necessary.  Another

possibility  would be to type R 1200-1202 ,  this selects  a

subset of the new messages.

3)   Now I read my mails and would like to erase them.  I type  E

1199- . The E is the abbreviation for ERASE.

4)   I want to know which new bulletins are in the bbs.  I type C

(this is an abbreviation of CHECK).

The result is a list of the following type:

Check: ()

Nr Call     Area      Nr date     @mbx   bytes #LT title

1 G7MQD  > DIVERSES...4 22.11.93 WW      2001   2 CHILDREN IN NE

2 G0MTQ  > SAT.......11 22.11.93 WW      1921 365 HELP QUIKTRAK

3 DG0FT  > BAYCOM.....1 22.11.93 BB      3847  10 NEU: TFPCX v2.

4 DL7OL  > KEPLER....10 21.11.93 DL      3725  30 NASA 2Line day

5 DL7ATG > PACTOR....58 21.11.93 DB0BLO  4428 365 PTC und FSK

(the  list  is shortened in line length to fit  in  the  printing

form)

5)   Now,  I  would  like  to read the bulletin  from  DL7ATG  to

PACTOR.  I type R PACTOR 58 .  This sets the board PACTOR as

the  current board and tells the system that I want to  read

the 58th message of that board.

6)   I would like to see the older messages for PACTOR,  I type L

DigiPoint Documentation                       16/05/94 v3.60 - Page 50

1- . A list (shortened) appears:

info-file: PACTOR

Nr. of     date     UTC    bytes  title

1 DL1BIR 21.02.93 00:19   1956  KAM und Pactor Befehle?

2 G3ZYP  25.02.93 20:57    973  ** CQ PACTOR & PACTOR MBO's **

3 DU9BC  27.02.93 21:50   2442  DU-PACTOR STATION!!!

(...)

57 DF7ML  07.11.93 14:43   3584  Mark-info aus LA

58 DL7ATG 21.11.93 09:08   4428  PTC und FSK

(PACTOR) DL8HBS de DL8HBS>

7)   I type Q (for QUIT), the connection is terminated.

8.3. The Internal Commands Of The BBS (COMMANDS.BOX)

The  DP - BBS has a fixed set of internal commands.  You can  add

an unlimited number of external commands,  this will be described

later on.

The command processor requires a file COMMANDS.BOX in the  folder

DP\BOX\SYSTEM\ . This file contains the command names, equated to

one  of  the internal command numbers.  This means that  you  can

change  the  command  names.  I  beg you to  let  the  basic  set

unchanged.

The  command processor works with the 'first fit'  -  method.  As

said  above,  commands can be abbreviated.  An 'E' will be  reco-

gnized  as  'ERASE' and not as 'EXPORT' because the  position  of

'ERASE' in the COMMANDS.BOX is first.  The commands are not  case

sensitive.

In  the  COMMANDS.BOX,  a flag determines if a  command  is  only

available for sysops. I propose you not to alter this flag.

Several  commands require optional parameters to select a  subset

of  the messages within a selected board.  The syntax  for  these

selections is as follows:

1              msg 1 is selected

1-5            msgs 1 to 5 are selected

1-             all msgs are selected

-5             msgs 1 to 5 are selected

The  following two are only valid with  the  READ,  LIST,  ERASE,

EXPORT and CHECK command:

< Search       all  msgs having 'Search' in the title or  senders

DigiPoint Documentation                       16/05/94 v3.60 - Page 51

field are selected

1-5 < Search   all msgs from 1 to 5 having 'Search' in the  title

or senders field are selected

If no selection was made, all new msgs are selected.

The file COMMANDS.BOX:

Name                is the name of the command

Internal command nr is  the fixed number of the  command  in

the  bbs (the key to  the  corresponding

command)

Sysop               0 = No, 1 = Sysop only command

Userlevel           minimal  required  level of  a  user  to

execute this command (do not mix that up

with the userlevels in the packet  radio

terminal)

Name Internal_command_number Sysop Userlevel

LIST 1 0 1

READ 2 0 1

IMPORT 3 1 1

SEND 4 0 1

ERASE 5 0 1

TELL 6 0 1

CHECK 7 0 1

USAGE 8 0 1

USER 8 0 1

DIRECTORY 9 0 1

HELP 10 0 1

HIDDEN 11 1 1

LOGINTIME 12 0 1

QUIT 13 0 1

BYE 13 0 1

ENDE 13 0 1

EXIT 13 0 1

ABORT 14 0 1

STATISTIK 15 1 1

VERSION 16 0 1

PRINT 17 1 1

TIME 18 0 1

TRANSFER 19 0 1

CP 19 0 1

NAME 20 0 1

BBS 21 0 1

FIND 21 0 1

SPEAK 22 0 1

SF 23 1 1

SFLEVEL 25 1 1

EDITOR 26 1 1

DigiPoint Documentation                       16/05/94 v3.60 - Page 52

EXPORT 27 1 1

MYBBS 28 0 1

SETUSR 28 1 1

LEVEL 29 1 1

MBX 30 1 1

LIFETIME 32 1 1

LT 32 1 1

SETL 32 1 1

GARBAGE 33 1 1

FHEADER 34 1 1

HNAME 35 1 1

SFPARMS 37 1 1

FORWARD 38 1 1

RELOAD 39 1 1

BOXLOG 40 1 1

SYSTEM 41 0 1

SYSCHK 42 1 1

PASSWORD 43 1 1

DISCONNECT 44 1 1

LANGUAGES 46 0 1

KILL 47 1 1

REPLY 48 0 1

COMMENT 49 0 1

REBUILD 50 1 1

BULLID 51 1 1

MSG 52 0 1

BATCH 54 1 1

SETSF 55 1 1

RESET 56 1 1

TOSKILL 57 1 1

SAT 58 0 1

DBASE 59 0 1

PREFIX 60 0 1

SYSOP 61 0 1

CHAT 61 0 1

NODE 62 0 1

CTEXT 63 1 1

TALK 64 0 1

COMP 65 0 1

//COMP 65 0 1

PWMODE 66 1 1

SETTIME 67 1 1

SETDATE 68 1 1

SFTEST 69 0 1

CLEARBUF 70 1 1

DigiPoint Documentation                       16/05/94 v3.60 - Page 53

8.4. Command Description

ABORT     Ends the connection with the bbs.  The logindate is not

updated.  The  same is performed with a simple  discon-

nect.

BATCH     Sysop  only.  You  can start a batch  file.  This  file

contains text lines that will be typed as if you  would

type them manually.  Please take care to use a  correct

syntax in a batch file,  it is not aborted when  errors

are detected. Example:

BATCH D:\DP\BOX\CLEANUP.BAT

BBS

or

FIND      Outputs   informations  about  other   bbs'.   Possible

options:

BBS DB0XYZ     displays informations about the bbs

db0xyz  and shows you the path  the

mail  router  of DP would  take  to

send a mail to that station.

BBS D          displays  a list of all bbs  having

'D' as first letter in their  call-

sign.

BBS < Berlin   searches  for the bbs that has  the

string 'Berlin' in its mailheader.

BOXLOG    Sysop only.  Recreates the file BOXLOG.DP (the base  of

the checklist output).  Normally, you won't have to use

this command as GARBAGE includes this operation.

BULLID    Sysop  only.  You can check the state of the BID  pool.

Without  parameters  you get  some  informations  about

size,  maximum size and the seek pointer.  If the  seek

pointer is lower than the size the pool was  completely

filled and will be overwritten from the beginning. This

is normal.  If you add a BID as parameter,  the BID  is

searched and the position in the BID pool is displayed.

The added option '-' deletes the BID in the  pool,  the

option '+' appends the BID to the pool.

Examples:

BULLID

BULLID 12635_F6ABJ

BULLID 12635_F6ABJ -

BULLID 12635_F6ABJ +

For normal bbs operation you should not delete BIDs.

CHECK     The  CHECK  command is similar to L in  f6fbb  bbs.  It

shows you either

DigiPoint Documentation                       16/05/94 v3.60 - Page 54

-    all  new  bulletins  in the  bbs  (C  without

parameters)

-    all  bulletins  of  a special  subject  (C  <

search,  where  'search' is searched  in  the

title and sender field of all existing mails)

-    all new bulletins of a board (C ATARI)

-    a  subset of all bulletins (C  200-300,  this

shows you the last 200 to 300 bulletins; or C

50, this shows you the last 50 bulletins).

The CHECK list is aborted after 600 displayed lines  to

prevent heavy hf load.

Examples:

C

C 50

C 1000-1200

C < DL8HBS

C < FT-757

C ATARI

The  CHECK command has the optional parameter  '+'.  If

this is added to the command, instead of bulletins only

private mails are examined.  This is a sysop-only  com-

mand. Examples:

C +

C < DL8HBS +

C DL8HBS +

CLEARBUF  Sysop only.  Disposes all disposable RAM, same as ALT-2

via keyboard.

COMMENT   is  very similar to REPLY,  the difference is  that  no

private  mail is generated but a message with the  same

destination and @mbx field as of the read mail.

COMP or

//COMP    COMP  ON activates the huffman compression of the  bbs.

It will send a //COMP 1 answer and start compression at

that time.  COMP OFF switches the compression off,  the

answer is //COMP 0.

CTEXT     Sysop  only.  You  can  enter  a  logon  text  that  is

displayed to all users.  An old CTEXT is displayed  and

then deleted.

DBASE     Refer  to the packet terminal command  //DBASE  .  This

does not work at the console!

DIRECTORY The DIRECTORY command displays a board list of the bbs.

DigiPoint Documentation                       16/05/94 v3.60 - Page 55

It has four different options:

D    displays  the  bulletin  boards,  the  number

after  the name is the maximum  lifetime  for

this  board  as  set  in  RUBRIKEN.BOX   (see

below).

D I  also  displays the bulletin boards,  but  the

number  is the number of the last message  in

this board (which has,  due to erased  files,

not to be the total amount of messages).

D U  displays the user boards,  the number is  the

number  of  the  last message  in  the  users

board.

D A  displays the bulletin boards as 'D' would  do

it AND all user boards without message count.

I  is an abbreviation for INFO,  U for USER and  A  for

ALL.

Only  those boards are displayed that have a  lower  or

same access level as the user who typed the command.

To  speed up the processing,  the check if a  board  is

completely  erased is only performed with  user  boards

and  bulletin  boards with 3 or  less  messages.  If  a

bulletin  board  contains 100 messages that  just  have

been deleted by the sysop,  DIR will display that board

until the next GARBAGE (see below).

DISCONNECT

Sysop only.  You can disconnect either one or all users

of your bbs. The options:

DISC ALL       disconnects all users

DISC DL9XYZ    disconnects DL9XYZ

DISC 6         disconnects the user on boxport 6

A  text  may  be added that will  be  displayed  before

disconnecting. Example:

DISC ALL sri, system maintenance works.

EDITOR    Starts the text editor defined in the install  procedu-

re. Only effective at the console.

ERASE     Erases the selected files of a board.  Note:  the ERASE

command of DP does not work immediately.  It only  sets

an erase-flag in the file header.  As a sysop,  you can

unerase erased files up to the next GARBAGE.  Refer  to

the commands HIDDEN and GARBAGE.

Of course,  a non-sysop can only erase files to or from

him. Examples:

DigiPoint Documentation                       16/05/94 v3.60 - Page 56

E DL9XYZ 7-10

E 9

E ATARI 12

EXPORT    Sysop only. Exports a specified message (or a number of

messages) on disk.  Syntax as READ,  EXP ATARI 7-10 for

example.

You  are  requested  for a output  file  name.  If  the

command  is sent via packet,  the output file  name  is

EXPORT.BOX in the folder DP\SAVE\ .

FHEADER   Sysop only.  Usually,  all forwarding bbs add a forward

header of the type

R:931124/1714z @:DL8HBS.#BLN.DEU.EU [Berlin]

to  a  forwarded mail.  The FHEADER  command  sets  the

information  between the  brackets.  Per  default,  the

string  is:  [WW-LOC OP:<yourcall> DP-3.50].  The  'DP-

3.50' is always added to your definion. So, typing

FHEADER Berlin OP:DL8HBS JO62QL

will set the header to

[Berlin OP:DL8HBS JO62QL DP-3.50]

FHEADER  without parameters displays the  current  set-

ting.

FORWARD   Sysop  only.  The  FORWARD command allows you  to  mark

manually messages in the forward list. If for example a

new neighbour of yours started operation and it  wishes

to  get all messages of the board ATARI from  you,  you

can type FOR ATARI 1- DB0XYZ (where DB0XYZ is your  new

neighbour).  The @mbx field is set to @DB0XYZ, but this

is  in  effect  the onliest change  compared  with  the

automatic forwarding. If you want to avoid this change,

add a '+' to the command (but think about what you  are

doing!,  this could cause reforwardings of old messages

if  the  BID  pools of the bbs' in your  area  are  too

small).  Usually,  omit  the  '+'.

To  readress  wrong delivered usermails,  use  the  MBX

command.

GARBAGE   Sysop  only.  This starts the house keeping routine  of

the bbs.  It is a very complex routine that checks  all

entries  in the bbs and deletes physically  the  erased

messages and those where the lifetime expired.  If this

is  done,  the checklist is recreated (the base of  the

CHECK output).  Then the stored BBS route  informations

(file  HPATH.BOX in folder DP\BOX\SYSTEM\) is  checked,

DigiPoint Documentation                       16/05/94 v3.60 - Page 57

entries  with a 'lasttx' of older than half a year  are

deleted.

Whenever you have an inconsistency in any kind of datas

of the bbs,  call this routine.

The routine is started once a day at a defined time  to

delete the erased bulletins.  You should set this  time

to  local  0300  or 0400 (this is done  in  the  dialog

PARAMETERS -> BOX 1), it takes between 5 and 20 minutes

of time.  Of course,  if you are using DP as a personal

message  system,  it will not be running 24  hours  per

day.  In that case,  call the GARBAGE command  manually

every some days to clean up the bbs. THIS IS IMPORTANT,

SO DO IT!

HELP      The  HELP  command displays a file to be found  in  the

folder  DP\BOX\LANGUAGE\  .   The  file  has  the  name

'HELP.LAN' where .LAN is replaced by the users language

(HELP.G for english help).  When in sysop mode,  also a

file 'HELP_SYS.LAN' is displayed.  If one or both files

are missing,  a short message informing about this fact

appears.

HELP called with a parameter tries to find a file  with

the parameters name in the folder DP\BOX\LANGUAGE\ .

You might for example type H DIR,  this will search for

the file DIR*.DL ,  if your user language is german. If

no file is found, a second search for DIR*.G is started

(the english help).

HIDDEN    This is a switch (sysop only). Normally, only undeleted

messages  are displayed.  Typing HID ON ,  all  message

operations are working with erased messages.  The  LIST

command shows only erased messages,  as does the  CHECK

command  and  so on.  HID OFF switches back  to  normal

mode.  A [H] in the prompt indicates this mode. When in

hidden mode, you can unerase erased messages by erasing

them (all erase logic is inverted hi).  Note that  this

only works until the next GARBAGE.

HNAME     Sysop only.  Usually, all forwarding bbs' add a forward

header of the type

R:931124/1714z @:DL8HBS.#BLN.DEU.EU [Berlin]

to a forwarded mail.  The HNAME command is  responsible

for  the hierarchical routing info appended to the  bbs

call.  HNAME DEU.EU will add .DEU.EU to your bbs  call-

sign. Per default, there is no hierarchical path added.

HNAME without parameters deletes the hierarchical path.

IMPORT    Imports  a file into the bbs.  This could either  be  a

binary  file  (this  is detected  automatically)  or  a

DigiPoint Documentation                       16/05/94 v3.60 - Page 58

text/7plus file.  You are requested for the file  name,

the destination and title.  This command works only  at

the  console  and  should  be  a  sysop  only  command.

EXAMPLE:

IMP

KILL      Sysop  only.  The same as ERASE,  but additionally  the

Bulletin ID of the erased message is deleted in the BID

pool.  The bbs will not defer a resend of that  message

in s&f.

LANGUAGES Displays all available languages of the bbs.

LEVEL     Sysop only. Sets the userlevel of a user. Example:

LEV DL9XYZ 2

The  default userlevel of every new user  is  1.  Valid

levels are 0 to 127. A level of 0 inhibits this user to

log in the bbs.

The  levels CAN be used to define special  board  areas

where not everyone has access to.  But beware, at least

in germany, this is not legal.

A maybe more useful application of the level command is

to  define special bbs commands (as SEND,  ERASE)  with

higher  levels than the standard 1,  so a new  user  is

unable to SEND and ERASE mails until the sysop gave him

a higher userlevel. This is legal.

LIFETIME

or

LT

or

SETL      Sysop only.  You can correct the lifetime of a message.

This  does  not effect the outgoing s&f  but  only  the

administration in your own bbs. Example:

LT ATARI 1- 0

LT ATARI 5 #365

LIST      Lists the content of a board. Examples:

L ATARI 1-4

L < DL8HBS

An option '+' is available.  This displays more  infor-

mations about the listed files. Example:

L ATARI 1- +

LOGINTIME Displays or sets the last login date/time. Examples:

LOG            displays last logintime

LOG 1.10.      sets last login on the 1.10. of the

current year, 00:00 UTC

LOG 1.10.92    sets  last  login  on  the  1.10.92

DigiPoint Documentation                       16/05/94 v3.60 - Page 59

00:00 UTC

MBX       Sysop only.  When a user sent a mail with a wrong  @mbx

field, you can correct it with this command. Syntax:

MBX ATARI 8 EU

MBX ATARI 1-3 ALL

MYBBS

or

SETUSR    With  this  command a user enters his  home  bbs.  This

information is forwarded to M@THEBOX . Example:

MY DB0XYZ

If  the user added the option '+',  the information  is

NOT forwarded to other bbs. Example:

MY DB0XYZ +

A sysop can set a users mybbs,  then the information is

not forwarded. Syntax:

MY DL9XYZ DB0XYZ

MY DL9XYZ DB0XYZ 2       This  sets   additionally

the  userlevel of  DL9XYZ

to 2 (default 1).

MSG       The MSG command transfers a single text line to another

user of the bbs. Examples:

MSG DL9XYZ Hi Peter      Sends the line 'Hi Peter'

to DL9XYZ (if logged on).

MSG 6 Hi Peter           Sends the line 'Hi Peter'

to the user on port 6 (if

connected).

Of  course,  it is impossible to send a MSG to  a  user

writing a Mail or to a forwarding bbs.

You  can  send yourself a message,  this  seems  to  be

necessary for some batch generators of other  programs.

If the self-send text starts with a '&',  no prompt  is

echoed, simply the pure text (without '&').

NAME      Required parameter: the users name. Example:

N Joachim

In sysop mode, the sysop can enter data for other user,

example:

N DL9XYZ Peter

DigiPoint Documentation                       16/05/94 v3.60 - Page 60

NODE      Starts a (virtual) connect to the packet node. The user

is connected with the node as if he directly  connected

a node port. This does not work at the console!

PASSWORD  Sysop  only.  You can define a password to be used  for

the SYSTEM command of a specific user.  Note that  this

password  may not contain a semicolon (';') as this  is

the command split char... The password has to be longer

than 20 signs,  use long passwords with random signs to

avoid sysop logins of hackers. Example:

PASS DL8HBS DJGseut7k4088904fbvvju ghrfw6d3jgdcjh

PREFIX    Refer  to the packet terminal command //PREFIX  .  This

does not work at the console!

PRINT     Sysop  only.  PRINT ON switches the printer on for  all

output  of  the fowllowing commands,  PRINT  OFF  stops

printing.

PRINT  <command>  only prints the  output  of  command.

Example:

PRINT R ATARI 4

PWMODE    Sysop only. This command defines the priviledge level a

user has after responding to the password request.

Levels are: 0,1,2,10

Where

0    means that the user may connect via the modem

port, no write access (this is the default).

1    means that the user may connect via the modem

port,  with write access.  Also, this enables

write  access via a radio port if the  global

parameter 'SEND/ERASE with PW' is selected.

2    this is as 1,  except that the user is always

kept to enter the password for write  access,

independent of the global switch.

10   enables full sysop mode,  either via radio or

modem.

Take  care:  the default level of older versions of  DP

was  0,  so renew that setting for your remote  sysops!

Right now they require a setting of 10.

Example: PWM DL9XYZ 1

QUIT

DigiPoint Documentation                       16/05/94 v3.60 - Page 61

or

BYE

or

ENDE

or

EXIT      Ends the connection with the bbs.  The last login  date

is updated to the login date of this connection.

READ      Reads the content of a board. Examples:

R ATARI 1-4

R ATARI < DL8HBS

An option '+' is available. This displays the full file

header (S&F-header) of a file. Example:

R ATARI 9 +

REBUILD   Sysop  only.  This  deletes the BID pool of  your  bbs.

Then, it is refilled with all BID's currently active in

your bbs.  Normally, the BID pool contains informations

about messages that have been erased but were  received

by your bbs once. The effect of the REBUILD command is,

that  you can rereceive erased mails.  For  normal  bbs

operation this is not useful.

RELOAD    Sysop  only.  RELOAD  SF forces DP to reload all  SF  -

definition files.  RELOAD ALL reloads all files concer-

ning  the  bbs.  Unfortunately,  due to a funny  bug  I

cannot find since months,  you have to type the  RELOAD

ALL twice,  because after the first RELOAD all language

files are unload. After the second RELOAD they are back

again... funny, indeed.

REPLY     The  REPLY  command will create a private mail  to  the

sender of a message that was the last one read with the

READ command.  Note that this only works with the  READ

command without the option '+'.

The title of the reply mail is the original title  with

a  leading RE:  ,  the @bbs is first looked up  in  the

internal homebbs storage files,  only if no  definition

for  the destination was found,  the read mail will  be

scanned  and the last found bbs in the mail  header  is

used as @bbs field.

RESET     Refer to the packet terminal command //RESET.

SAT       The  satellite calculations are called.  This does  not

work at the console!

SEND      Creates a new file.  You are requested for all  omitted

parameters.  The  parameters for the SEND  command  are

numerous:

S DL9XYZ @DB0ZZZ <Sender $BID #Lifetime Title

The $BID,  #Lifetime,  <Sender and @mbx-field is optio-

DigiPoint Documentation                       16/05/94 v3.60 - Page 62

nal. Per default, DP sets the @mbx to your own bbs call

(the mail will not be forwarded),  the $BID (= Bulletin

/ Message ID) is generated automatically, the #Lifetime

is set to 0 (= infinite) and the <Sender is set to  the

callsign of the user that typed the SEND command.

If the destination is a callsign,  DP looks up the @mbx

field (if omitted) of the forwarded mybbs  informations

or  sysop/user setted local mybbs infos (this is  some-

thing similar to White Pages).

The lifetime determines,  after how many days the  mes-

sage shall be automatically deleted.  Only supported by

DP / TheBox.

Even if the <Sender field is present,  the callsign  of

that one sending the SEND command is always visible  in

the  mail itself.  The first two lines of a  mail  sent

with DP always contain an info as:

de DL8HBS   @ DL8HBS.#BLN.DEU.EU

to ATARI    @ EU

The call in the first line is not corruptable.  You may

ask  yourself why this feature  is  implemented.  Well,

with  the '<' option you can locally resend  msgs  with

the correct senders call.  And,  of course,  it  exists

because of the compatibility with TheBox.

Note:  there is no different command for private  mails

and bulletins.  The distinction is made by the destina-

tion. If it is a callsign, it is a private mail.

Examples:

S

S DL8HBS how are you

S ATARI @ EU problems with hard disk

S SYSOP @F

S ATARI @WW $123456DL9XYZ #30 new compression alg.

SETSF     Sysop only.  The SETSF command manipulates the  forward

list (that is,  by the way,  held in the pseudo - board

'X'). The command has three options:

SETSF DB0XYZ type DB0ZZZ      All  entries in  the

forward list of  the

type   'type'   that

shall  be  forwarded

to  DB0XYZ  will  be

sent to DB0ZZZ.

SETSF 3-6 DB0ZZZ              The  entries 3 to  6

of the forward  list

will   be  sent   to

DB0ZZZ.

SETSF type 1- DB0ZZZ          All  entries in  the

DigiPoint Documentation                       16/05/94 v3.60 - Page 63

forwward   list   of

type 'type' will  be

sent to DB0ZZZ.

Where 'type' is one of the following:

A         = all

B         = bulletin

P         = private

S         = system (erase and mybbs)

Note  that this command does not change  the  forwarded

mail itself. Only the neighbour bbs used for routing is

changed.

SETTIME   Sysop only. Sets the system clock. Take care, no UTC= -

correction is made. Example: SETTIME 21:10:12

SETDATE   Sysop only. Sets the system date.

Example: SETDATE 18.5.94

SF        Sysop  only.  Starts manually the S&F to  another  bbs.

Examples:

SF DB0XYZ           starts  the s&f to db0xyz  (on

TNC #0).

SF DB0XYZ #1        starts  the s&f to db0xyz  (on

TNC #1).

SF DB0XYZ 430900 #1 starts  the s&f to db0xyz  (on

TNC  #1,   frequency  set   to

430900).

This only works,  if there is a s&f definition file  in

the folder DP\BOX\SF for DB0XYZ (DB0XYZ.SF).

The command has two other options:

SF OFF              stops the s&f timer.  No  out-

going s&f connects are  estab-

lished,   incoming  s&f   con-

nections are defered.

SF ON               allows automatic s&f  operati-

ons.

The default is SF ON.  This switch is NOT  saved.  When

restarting DP,  it is always set to SF ON.  The SF  OFF

case is indicated by a [no S&F] - prompt.

SFLEVEL   Sysop only.  The SFLEVEL command defines how the SID (=

station identification) looks when being connected from

another bbs. DP switches between two SID's:

1)   [DP-3.60-AB1DFHMR$] (DP's own SID)

2)   [THEBOX-1.9-H$]     (a fake hi)

DigiPoint Documentation                       16/05/94 v3.60 - Page 64

This   is  necessary  because  the  TheBox  system   is

incompatible  to the internationally used  SID  format.

The  special functions of TheBox (binary  files,  mybbs

forwarding)  are only available if its partenaire  also

prompts with a [THEBOX-...], what is a very regrettable

solution of the programs author.

OK, here are the facts. An SFLEVEL of

0    results  in  the normal logon screen  of  the

bbs.  Note  that the SID No 1) is  sent,  but

this is the normal (and verbose) user login.

1    SID No 1) is sent,  directly followed by  the

S&F prompt in a new line ('>').

2    SID No 2) is sent,  directly followed by  the

S&F  prompt in a new line ('>').  To be  used

for THEBOX neighbours.

3    SID  No  1) is sent,  followed  by  a  little

welcome  text  and the S&F prompt  in  a  new

line. Only slightly different from 1 ...

Note that the SFLEVEL command only affects the reaction

of DP when answering an incoming connect. For the other

direction,  DP  switches automatically the SID.  If  an

outgoing  connect gets prompted with a THEBOX  SID,  it

gets answered with the same.  DP uses automatically the

fastest protocol possible.

Examples:

SFLEVEL DB0XYZ 2

SFLEVEL DB0XXX 3

SFLEVEL DB0YYY 1

SFPARMS   Sysop only.  Displays all SFPARMS - definitions of  all

defined  forwarding  neighbours (this is  done  in  the

files of the folder DP\BOX\SF\).  At the last  position

is  marked the UTC when your bbs will start s&f to  the

neighbour bbs. The command has different options:

SFP                      shows all SFPARMS.

SFP -                    deletes all SFPARMS  (has

about the same result  as

SF OFF).

SFP DB0XYZ 3 1           sets the third  parameter

of the sfparms of  DB0XYZ

to 1.

SFP DB0XYZ _all_parms_   sets   all  sfparms   for

DB0XYZ  (_all_parms_  has

to be replaced with ALL 9

DigiPoint Documentation                       16/05/94 v3.60 - Page 65

parameters!).

SFTEST    Checks the routing definitions.  A user can request the

forwarding  paths for a specific SEND  command.  SFTEST

accepts all parameters as SEND, of course no message is

created. The result is a list of bbs' the message would

be forwarded to or an error message.

Example: SFT DP @ ALL

SPEAK     sets a user language.  Example:  SP DL .  SPEAK without

parameters shows all languages.  In sysop mode, you can

set a language for another user, example: SP DL9XYZ DL

Per default,  a users language is defined according  to

the   entries  in  DP\VERWALTG\WHOTALKS.LAN   (you   do

remember  that  item?  It was explained in  the  packet

terminal section).

STATISTIK Sysop only command.  Called without parameters,  a list

with   extended  informations  about  every  board   is

displayed.  If  the parameter is a  boards  name,  only

informations  about the selected board  are  displayed.

Also,  the parameters -s , -u, -i are known, they force

a 'quick display mode',  only summary informations  are

displayed for either user boards (-u),  bulletin boards

(-i) or both (-s). Examples:

STAT

STAT ATARI

STAT -s

STAT -i

STAT -u

SYSOP

or

CHAT      Starts a (virtual) connect to the packet terminal.  The

user  is connected with you as if he connected  one  of

your terminal ports. This does not work at the console!

SYSTEM    Starts the sysop login procedure.  Refer to the command

//SYSTEM of the packet radio terminal,  the  proceeding

is similar.

SYSCHK    Responses an 'OK' if you are sysop,  if not,  an  error

message appears.

TALK      Starts a connect to anoher logged in user.  The  connet

is  not fully transparent,  binary data  transfer  will

fail. It is text oriented. The connection is ended with

a CTRL-Z.

Example:

TALK DL1XYZ

TALK 4

DigiPoint Documentation                       16/05/94 v3.60 - Page 66

TIME      Displays current date and time (in UTC).

TELL      Generates  a command that is forwarded to another  bbs.

The destination bbs executes the command and sends back

the   resulting  answer.   Only  TheBox  and   DP   bbs

understand that syntax,  it has no effect with  others.

This is a little bit more powerful than the REQDIR  and

REQFILE syntax of other systems. By the way: the REQ* -

commands of f6fbb and others are not supported (but are

correctly  forwarded  as private mails if DP  is  in  a

forwarding  chain).  As arguments of the TELL file  you

can   add  all  bbs  commands  that  do   not   require

interactive answers (as NAME, MYBBS do). Examples:

TE DB0XYZ L ATARI 1-

TE DB0XYZ R ATARI < DL8HBS

where  DB0XYZ  is the bbs where the  command  shall  be

executed.

Sysop information: the TELL command is forwarded in the

board 'T'.  It is a file generated with 'S T @ DB0XYZ <

sender_callsign  title',  where title is the  requested

command  added with '@ sender_bbs'.  If  the  resulting

output  of  a TELL command is bigger than  50k,  it  is

discarded  and  an  error message is sent  to  the  re-

quester.

TOSKILL   Refer to the packet terminal command //KILL

TRANSFER

or

CP        Transfers  a  bulletin  from one board  to  another  or

copies a user mail from one user board to  another.  If

not  in  sysop  mode,  only bulletins  directed  to  or

written by the current user are transfered. Examples:

TRA DL8HBS 12-13 > DL9XYZ     (the   '>'  is   not

necessary)

TRA DL8HBS 12 > DL9XYZ @ DB0XYZ

TRA ATARI 7 SOFTWARE

TRA IBM 9-10 > SOFTWARE

USAGE

or

USER      Without optional parameters,  this command displays all

current  users of the bbs.  With a callsign as  option,

all known informations about a user (as homebbs,  name,

language)  are  displayed.  When in  sysop  mode,  more

informations  as level and SF-level are  displayed.  If

the  user  is currently logged in,  the  sysop  gets  a

display of received and sent bytes,  used cpu time  and

logintime over all.

Examples:

U

DigiPoint Documentation                       16/05/94 v3.60 - Page 67

U DL8HBS

VERSION   Displays the version number of the software.  Also, the

free space in RAM is displayed.  If in sysop mode, also

the  free space on the DP disk drive is displayed  and,

if  installed,  the free space on the RAMDISK for  DP's

temporary files (see below).

8.5. The Configuration Files

8.5.1 RUBRIKEN.BOX

The  german word 'Rubrik' means the same as the english  'board'.

In this file,  the fixed boards of the bbs are defined.  You  can

create  other (undefined) boards by simply sending a mail to  it,

but  the difference between defined and undefined boards is  that

the  defined  usually have a longer lifetime.  The  lifetime  for

undefined boards is a general parameter in the dialog  PARAMETERS

->  BOX  1.  You do not have to define  user  boards.  They  have

automatically  an infinite lifetime.  But you can define them  to

provide them with a higher level.

The file RUBRIKEN.BOX (in folder DP\BOX\SYSTEM\ ) may contain  up

to 100 board definitions of the following kind:

ATARI:140:1

This defines the board ATARI with a maximum lifetime of 140  days

and a minimal required userlevel of 1 to have access to it.

Boards  under  DP and TheBox may have up to  8  signs.  They  are

automatically cut to 6 signs when being forwarded to  w0rli/f6fbb

systems and reexpanded to 8 signs when received in s&f.

Note that two special boards are necessary for the system itself:

M  and X.  The M board contains user informations as last  login,

level,  homebbs.  The X board holds the forward list.  You should

set the lifetime of X to 0.

Boards  with only one single letter are sysop  only  boards.  The

board T is used for the forward of TELL requests. In germany, the

board F is used for inter-sysop communication.

8.5.2. TRANSFER.BOX

DigiPoint Documentation                       16/05/94 v3.60 - Page 68

The   board  names  are  not  standardized.   For   example,   DX

announcements  may be found in DXNS,  DXNWS,  DX-NEWS and  other.

Let's  assume  that  your  bbs  has  a  board  DXNEWS.  The  file

TRANSFER.BOX  (in the DP\BOX\SYSTEM\ folder) should  contain  the

lines:

DXNS DXNEWS

DXNWS DXNEWS

DX-NEWS DXNEWS

All incoming mail to these boards is sorted in DXNEWS.  Note that

this  only affects your own bbs.  DP NEVER changes  any  relevant

data in outgoing s&f (unfortunately,  most other bbs systems  do,

swapping @mbx fields and destinations,  what causes a lot, really

a lot of trouble).

The total amount of definitions in TRANSFER.BOX is unlimited.

8.5.3 BADNAMES.BOX

The board names defined in this file are defered in incoming s&f.

I propose you to define at least

IMAGE

IMAGES

GIF

PIC

to  prevent  that  mess being  forwarded  to  you.  The  TRANSFER

definitions are executed before the BADNAME lookup.  There is  no

limit of the length of this file. (to be found in DP\BOX\SYSTEM\)

8.5.4. TCPIP.BOX

If one of the bbs' you are forwarding to is a TCPIP  system,  you

usually have the problem that this one is waiting for a 'protocol

identification frame' to switch to ax.25 and send its CTEXT.

In TCPIP.BOX you can define those calls.  When connecting such  a

station, DP sends a single <CR> to switch them to ax.25.

Other systems require more verbose login commands. For those, you

can use this file, too. All text typed after the call is sent. If

a call appears several times, several lines are sent.

Example:

DigiPoint Documentation                       16/05/94 v3.60 - Page 69

DB0SAO

DB0TUD bbs

DB0XYZ logon

DB0XYZ bbs

(to be found in DP\BOX\SYSTEM\ )

8.5.5. BEACON.BOX

To run a mail beacon,  you have to set up this file.  If defined,

all user board names with new mail are sent as unproto frames  in

a specified interval. Please, only use this feature if you really

install a public bbs. Don't load the hf with useless beacons!

Please  refer  to  the  explanations  in  the  file  itself.  The

processing  takes some time,  do not set the beacon interval  too

low.

To  enable the beacon,  the corresponding  switch in  the  dialog

PARAMETERS -> BOX 1 has to be ON.

(to be found in DP\BOX\SYSTEM\ )

8.5.6 AUTOBOX.DIR

The  file AUTOBOX.DIR (in DP\BOX\SYSTEM\) is responsible for  the

monitor cut features of DP.  As written above, DP can fill up its

own  bbs  by simply monitoring a hf channel.  If  DP  monitors  a

message  to  a board defined in this file,  the  bbs  monitor  is

started. The file header formats detected by this routine are:

-    W0RLI - S&F

-    TheBox / DP

-    BaycomBox

-    WAMPES (dk5sg-bbs)

-    F6FBB (this needs your assistance)

-    and all others that are compatible to one of the above

Note that monitoring f6fbb type s&f is impossible due to protocol

reasons.

Refer to the example of the AUTOBOX.DIR for the syntax.

8.5.7. F6FBB.BOX

DigiPoint Documentation                       16/05/94 v3.60 - Page 70

To  monitor  files of F6FBB type bbs you have to set  up  a  file

F6FBB.BOX (in the folder DP\BOX\SYSTEM\).

The reason is,  that every sysop can change the file  header.  In

the  distributed  F6FBB.BOX,   two  different  file  headers  are

defined.  Note  that  this file does not  contain  keywords.  The

definitions  are read in the first 16 uncommented lines  of  this

file. Do not add empty lines...

Read the comments in the file.  The best way to configurate it is

to  read out a file of your local f6fbb bbs and save it on  disk.

So  you can easily copy the required parts of the  fileheader  in

the  configuration file.  When you did this,  restart DP and  try

again  to read a file of the f6fbb bbs.  Please ensure  that  the

switch 'record files (connect)' is ON (PARAMETER -> BOX 1).

At  the moment the file appears on your terminal  window,  a  'C'

should appear in the status line of the terminal window.  If not,

you made an error in the definitions.

Note that there is no Bulletin ID marked in f6fbb headers. DP has

to  create  an  own to prevent  double  monitoring.  This  is  no

problem, BUT: never force DP to forward such a file to others, as

the  BID is no more the original one.  DP itself  never  forwards

monitored files, but you can force that with the FORWARD command.

I hope you won't do that except of special reasons.

8.5.8. The .SF Files

This is a complex materia.  First try to imagine,  how a  message

distribution system works:

We get a message (from elsewhere).  In the header of the message,

we have two relevant routing informations:

1)   the destination (callsign or board)

2)   the  @mbx field (bbs callsign or distribution  area  as

@WW)

Another  relevant  information is the call of the bbs  that  sent

that message.

Now,  the router has to decide what to do with the message. There

are three cases:

1)   don't forward it, it is destinated to the own bbs

2)   forward it to ONE neighbour,  it is a private mail or a

bulletin destinated to only one bbs.

3)   forward it to several neighbours, it is a bulletin with

a distribution area (@EU , @WW ...)

DigiPoint Documentation                       16/05/94 v3.60 - Page 71

Case 1) is very easy to detect.  Just find out if the @mbx  field

is the call of your own bbs or if the @mbx field is empty.

Case 2) is the most difficult.  Find out,  which of your  several

neighbours is the next to the target bbs.  Not that simple,  huh?

If you found out,  mark the message in a list to be forwarded  to

that neighbour.

Case 3) is not that difficult,  because it only depends on  sysop

settings.  Find out,  for which of your neighbours the sysop  has

defined  this  @mbx to be forwarded.  Mark the  message  in  your

forward list to be forwarded to those neighbours.

In reality, the routing is much more complex than described. Many

exceptions  may occur.  What,  for example,  should happen  if  a

message  was  forwarded  FROM a neighbour to  your  system  that,

according to your knowledge about the net,  has to be routet BACK

via that neighbour...

All ok until now?  Well,  the problem is,  the sysop is YOU.  You

have  to  set  up the forward  definition  files  very  accurate,

because  many routing problems can be avoided if the  definitions

are all right. The same is valid for the sysops of your neighbour

bbs'.  Get in contact to avoid wrong routing. The DP system stops

ping-pong mailing immediately,  but this does not stop the  wrong

forward definition either in your system,  your neighbours one or

both.

Every  unrouteable  mail creates a warning message  sent  to  the

sysop.

Now,  after that long preliminary,  you should take a look in the

.SF example distributed with DP.  The file DB0GR.SF in the folder

DP\BOX\ is a simple ASCII file,  and you have to create a similar

one for every one of your forwarding neighbours.  The files  have

to be stored in the folder DP\BOX\SF\ to be effective.

The file contains several keywords:

DEF

IFQRG

SFPARMS

FOR

NOT

NOTRUBRIK

NOTFROM

RUBRIK

The first three control the connect sequence and the timing.  The

others are responsible for the router.

DEF       here  you  have to repeat  the  callsign  (without

SSID) this definition is made for. Example:

DigiPoint Documentation                       16/05/94 v3.60 - Page 72

DEF DB0XYZ

IFQRG     this keyword may appear several times.  It defines

connect paths for a specific QRG.  Assure that the

file  DP\PACKET\QRGLIST.DP is all right!  You  can

connect  successively  via  several   digipeaters,

every  connect  command  is seperated  by  a  ','.

Example:

IFQRG 438300 C DB0BLO,C DB0BER,C DB0XYZ

IFQRG 430200 C DL9XYZ,//C DB0XYZ

SFPARMS   this keyword controls the timer that generates  an

outgoing  connect to a  neighbour  bbs.  Also,  it

defines  the maximum size of messages to  be  for-

warded. It has nine parameters, set them all:

minutes        the timer interval in  minutes

the outgoing forward is to  be

initialised if mails are  wai-

ting.

tnc            the tnc that is to be used for

the connect.

if             defines  the case the  forward

is started:

0 =  never

1 =  if mails are waiting

2 =  if mails are waiting

OR

if minutes2  expired

(reverse     forward

will be started)

maxuser        defines the maximum size of  a

user mail to be forwarded  (in

bytes).

maxbulletins   defines the maximum size of  a

bulletin  to be forwarded  (in

bytes).

maxproposals   defines the maximum size of  a

multiple  proposal  in   f6fbb

type forward (in bytes).

minutes2       the timer interval in  minutes

if case 2 of the 'if'  defini-

tion is valid.  More exact  it

is  the interval  the  reverse

forwarding  is started  if  no

mails are waiting.  Never  set

this value too low.

DigiPoint Documentation                       16/05/94 v3.60 - Page 73

start UTC      defines  the  start  time  any

forward   action   is   valid.

Example: 0000

end UTC        defines the end time any  for-

ward action is valid. Example:

2359

Overall example:

SFPARMS 15 0 1 30000 20000 40000 300 0000 2359

By  the way,  this is what you get displayed  when

entering the SFPARMS command in the bbs.

Now  you must define the routing relevant  datas.  The  following

keywords may appear several times in the file.

FOR       defines the @mbx field that is to be routed to the

neighbour bbs this file is made for.

NOT       excludes a @mbx field to be routed to the bbs.

NOTRUBRIK well,  this  is  germish.  Excludes boards  to  be

routed to the bbs.

NOTFROM   excludes messages received by a specific bbs to be

routed to the neighbour bbs.

RUBRIK    rubrik is the german word for board.  All messages

sent  to thiswise defined boards are forwarded  to

the neighbour bbs. This is not a proper forwarding

feature,  so  do  use it only  with  user  forward

definitions, not in bbs to bbs definitions.

The FOR and NOT keywords accept hierarchical definitions.

Some examples:

FOR *.AS,*.NA,*.NOAM,*.SA,*.AF

defines all messages to asean,  american and african bbs  to

be forwarded to this neighbour.

FOR DB0XYZ DB0XXX

defines all messages for @DB0XYZ and @DB0XXX to be forwarded

to this neighbour.

FOR *.FRA.EU

defines all messages for french bbs to be forwarded to  this

neighbour.

DigiPoint Documentation                       16/05/94 v3.60 - Page 74

FOR F*

defines  all messages for bbs with the first letter  'F'  in

the call to be forwarded to this neighbour.

FOR EU WW SCAN DL

defines all bulletins with the @mbx field @EU @WW @SCAN  and

@DL to be forwarded to this neighbour.

NOT *.#BLN.DEU.EU

excludes  all  messages for bbs in the berlin region  to  be

forwarded to this neighbour.

NOT F6KLM

excludes  all  messages for @F6KLM to be forwarded  to  this

neighbour.

NOTRUBRIK GIF SALES

inhibits  the  forward of the boards GIF and SALES  to  this

neighbour.

NOTFROM DB0ZZZ

inhibits  all messages received FROM DB0ZZZ to be  forwarded

to this neighbour.

RUBRIK ATARI

forwards  all arriving messages of the board ATARI  to  this

neighbour.

8.6. The Language Files

The  bbs is multilingual.  The language files are in  the  folder

DP\BOX\LANGUAGE\ .

To define a language, a file XXX.LAN has to be created, where XXX

is  replaced  with the abbreviation of the language (as  'G'  for

english,  'DL'  for german...).  The structure of these files  is

easy, revise one and you understand all.

It  is  proposed  to create a set of help  files  for  every  new

language.  Help  files have the extension .XXX (replace XXX  with

the  language abbreviation).  The english help file for NAME  has

the  name NAME.G ,  for example.  Such a file is read if  a  user

types H NAME .

DigiPoint Documentation                       16/05/94 v3.60 - Page 75

8.7. The Autorouting Feature

The bbs has an autorouter feature.  You cannot switch it off. How

does it work?

DP analyses the forward headers of every incoming mail (depending

on the switch 'analyse hierarchicals' in the PARAMETERS -> BOX  1

dialog).  A  file HPATH.BOX in DP\BOX\SYSTEM\ is feeded with  the

informations. The bbs compares the times in the forwarding header

to  find  out  the  fastest way to a  bbs.  The  fastest  way  is

overwritten  after a specific,  dynamically calculated  time,  so

that an enormous time record caused by extraordinary  circumstan-

ces cannot block a maybe better, newer way for too long.

If  a  mail  is  to be forwarded,  DP  first  looks  in  the  .SF

definition  files if there is a valid path to that  bbs.  If  not

(and ONLY in that case),  the best forward neighbour is  computed

by the informations in HPATH.BOX.

What does that mean in practice?

The  autorouter is not used very often,  because for most bbs  an

entry in the .SF files is existing because of the prefix lists. I

am not quite sure wether this is all right or it would be  better

to  use  always  the autorouter.  You can  experiment  with  that

feature,  if, after a time of operation, the autorouter is feeded

with enough data,  by deleting most of the single bbs definitions

in the .SF files. If it works fine for your purposes then use it.

Why am I in doubt?

Because most bbs use fixed forward paths. They could be irritated

by otherwise routed mails.

8.8. The Bulletin ID System

Bulletin  ID's  (BID's) are used to prevent double  reception  of

messages. They are a unique signature of a message. The bbs saves

every  received BID in the file DP\BOX\SYSTEM\MSGIDLOG.DP  .  The

BID remains 'known' even when the file is deleted in your bbs. Of

course,  the  file may not grow endless.  If its defined  maximum

size is reached, the file is overwritten from the begin.

You can realign the file to the current content of your bbs  with

the command REBUILD.

DP  generates BIDs of the form 123456CALL where 12 is the day  of

DigiPoint Documentation                       16/05/94 v3.60 - Page 76

month, 3 is the month (hexadecimal), 4 is the year (0-9), 56 is a

upcounting  number  from  00  to ZZ and  CALL  is  your  stations

callsign.  TheBox and BaycomBox are using the same or nearly same

syntax.  By the way,  this should be totally unimportant, because

BID specifications only define a string of up to 12 chars for the

BID.  Unfortunately, some BBS implementations seem to expect BIDs

of the form 12345_CALL. This is WRONG.

8.9. The Read Generator (II)

If you click on a CHECK or LIST output of your own bbs,  the same

pop-up menu as described in the packet terminal section  appears.

Refer to the above description. All commands are possible, as you

are the sysop of your bbs.

If  you  click with the RIGHT mouse button,  immediately  a  READ

command is generated.

A  read  message is displayed in an own window.  You  can  easily

scroll  it up or down,  pressing the left or right mouse  button.

Pressing both buttons closes the window.

By the way: the mouse is also useful to react on the

>>> Press a key for more <<<

message, that appears at long list outputs. Clicked with the left

button on that line will continue scrolling, the right will abort

the output (as if you typed return).

8.10. The Dialog BOX 1

This is the main configuration dialog for your bbs.

The section SPACING contains the elements

left marg.     enter  the  left margin for the  text  format

routine. This routine is active when you type

a  text in your bbs (with the SEND  command).

Of  course,   this  is  only  active  at  the

console.

right marg.    enter   the  right  margin  for  the   format

routine.

These  settings  are only effective when the switch  in  the

left upper corner is selected.

The section TIMING controls the GARBAGE command

DigiPoint Documentation                       16/05/94 v3.60 - Page 77

set  the time in UTC the GARBAGE routine is started  automa-

tically if the switch in the upper left corner is selected.

pack after     controls  the disk space saver of  DP.  After

the selected count of days a received message

is compressed.  When reading such a  message,

it  is  online  uncompressed,  that  takes  a

second or so. I have the delay set to 0 days,

this means that incoming messages are  packed

AT  ONCE.  That  takes a little bit  of  time

while  reception,  but  saves a lot  of  disk

space  (I only have a 30 MB hard  disk).  For

full time bbs operations,  a value of 2 or  3

days  is proposed.  The messages  are  packed

within the GARBAGE routine.

The section LIFETIMES contains the elements

unknown boards sets  the  standard lifetime for  boards  not

defined in RUBRIKEN.BOX. Normally 1..10 days,

depending on your disk space. 0 = infinite.

userfiles      sets the standard lifetime for userfiles. 0 =

infinite.

The section SYSTEM MEMORY contains the element

BullID-Mem     The  numerical value is the amount  of  BID's

after  which  the file MSGIDLOG.DP  is  over-

written from its beginning.  Its not a  fault

to  use values between 10000 and  20000,  but

note:  The bigger the file is,  the longer it

takes to check if a message is double.

The section INPUT contains the elements

record files (connect)   whenever you are manually connected

to  a  bbs and you  read  out  some

files, they are automatically saved

in the bbs.  This is a useful  fea-

ture  when installing DP as a  per-

sonal terminal / bbs system and NOT

as a public bbs.  If installed as a

public bbs,  this switch has to  be

off!

record files (monitor)   maybe the most stunning feature  of

DP.  Without transmitting a  single

byte you can fill up your bbs  with

all  new  mails.  DP  monitors  the

transmissions  of  a bbs  to  other

users  and  records  the  monitored

files  into your own bbs.  This  is

error protected. Try that, you will

DigiPoint Documentation                       16/05/94 v3.60 - Page 78

ask yourself why you read out those

MegaBytes   before  you  used   DP.

Responsible  for this type of  ope-

ration is the file AUTOBOX.DIR.  Of

course,  this feature has to be OFF

when using DP as a public bbs!

undefined boards         If the above function is  selected,

you can monitor boards that are not

defined in AUTOBOX.DIR.  Note:  The

file BADNAME.BOX remains effective.

accept remote erase      defines wether monitored or in  s&f

received  remote erase messages  (a

special   feature  of  the   TheBox

system) will be accepted.

analyse hierar. path     normally,  set this switch  active.

Only if heavy problems by monitored

distorted   mail   headers   apper,

switch  it  off and check  if  that

helps. Some Unix mail systems cause

trouble  in conjunction with  older

bbs programs.

send/erase with pw       if   this   global   parameter   is

selected,  all SEND/ERSE -  related

operations  in  the  BBS  are  only

allowed after proper password login

of  the  user.  If a  user  has  no

password defined,  he only has read

access.  Remember to set the PWMODE

of  a user to 1 (refer to  the  bbs

command list).

The section OUTPUT contains the elements:

adding R:-Line in S&F    You  can suppress the  transmission

of  the forward header of your  own

bbs.

compressed S&F           Normally this should be active.

mail beacon              Enables the mail beacon.  You  have

to config the BEACON.BOX file.  Use

this only if you installed a public

bbs!

hold View-file on disk   this  is a feature ONE user  of  DP

asked for (hi Michael).  When  rea-

ding  a  bbs entry  with  the  read

generator,  an intermediate file is

created on disk.  The name of  this

file  is displayed in the title  of

DigiPoint Documentation                       16/05/94 v3.60 - Page 79

the ViewIt  window.  Normally,  the

file is deleted after a moment.  If

this  switch  is ON,  the  file  is

present until you leave DP.  So you

can copy it or what else you  wanna

do.

The section RAM DISK contains the element:

Size      DP has a built-in RAM disk.  This item defines the

space  to be reserved.  DP makes a  proposal,  but

don't take it too serious.  Only use this  feature

if you have at least 2 or 3 MB of RAM.  A proposed

size  is about 500 kByte.  The RAM disk speeds  up

the GARBAGE routine and all read and write  opera-

tions into the mailbox. The RAM disk is temporary,

it is deactivated when ending DP.  CAUTION:  If DP

crashes with an active RAM disk,  RESET the compu-

ter!  DO IT!  By the way,  you should always RESET

the computer if DP crashes.

8.11. The Dialog BOX 2

In this dialog you can insert two types of informations used  for

the message scanner of DP:

Additional prompts for end of mails

DP detects the end of a scanned mail by three methods:

1) the beginning of a new one

2) the standard system prompt of some bbs systems

3) prompts to be defined here

Enter the prompt(s) of your local bbs system that you mostly

monitor. F6FBB bbs often send a last line as

[End of Msg #xyz]

This is a good end indicator.  As the #xyz is not  constant,

enter only '[End of Msg'

Prompts for cancelling monitor recording

Some  bbs systems offer the possibility to abort a  readout.

This would cause a too short mail while monitoring.  If  the

system prompts a special string when aborting,  enter it  in

this section. TheBox systems often send

H D C L R S ?>

DigiPoint Documentation                       16/05/94 v3.60 - Page 80

Take care that the entered string does not appear in  normal

bbs output but only in abort condition.

8.12. Message Filtering

DP  offers  you an interface to investigate every  incoming  mail

before sorting it in the bbs.  Your program can copy the mail for

special purposes,  e.g.  7plus extraction or force DP not to take

the  mail.  A third possibility is to change boardname and  life-

time.  You can even alter the content of the message itself. I do

not know why you should do this,  but you have all possibilities.

For  this  reason,  always reflect on what you are doing  if  you

alter the content of a mail. Normally, a message system should be

transparent  and NEVER change anything of the  mails.  You  could

cause  a lot of trouble in the net,  and that's not where DP  was

made for.

How does the interface work?

If you putted a program M_FILTER.PRG in the folder DP\BOX\RUN\  ,

this program is called every time a mail is decoded.  The program

is  called  with only one parameter in the  commandline,  a  full

filename (including path) of a file that contains the new mail.

Note  that  this slows down the execution time of the  mail  sort

routine.  Normally,  no intermediate file is created on disk when

sorting  a  mail.  If installed,  the RAM disk is used  for  this

operation.

In  the  first  seven  lines of the  file  you  find  some  ASCII

informations,  followed by the mail itself.  Take care,  the mail

could be a binary file!  DP supports full binary mode in s&f  and

user in/output. Your program also should...

The first seven lines contain:

-    the callsign of the box the mail was received from

-    the senders callsign

-    the board name (destination field)

-    the @mbx field

-    the lifetime

-    the bulletin id

-    the title of the message

You  can  alter the board name and the lifetime,  but  note  that

these  changes  are  only effective for your  own  bbs,  not  for

outgoing s&f.  The other parameters cannot be altered,  they  are

only present for information.

Following the seventh line,  you find the complete mail including

DigiPoint Documentation                       16/05/94 v3.60 - Page 81

forward headers.

Your program must return a return value. A value of

0    indicates that nothing was changed

1    indicates that the mail has to be deleted

2    indicates that the mail shall not be forwarded

3    indicates  that  your  program  changed  either  board,

lifetime  or mail content (or all) and that DP  has  to

accept the altered mail

4    indicates  that  your  program  changed  either  board,

lifetime  or mail content (or all) and that DP  has  to

accept the altered mail but shall not forward it.

If returning the values 3 or 4,  take care that the format of the

altered file is exactly the same of the file DP created.

A little source code example written in PASCAL is present on  the

distribution disk.

8.13. External Commands

The  internal set of commands can be expanded with  an  unlimited

number  of external commands.  For every new command you have  to

write  a  small (and fast working) program to be  placed  in  the

folder  DP\BOX\RUN\.  The program has the commands name  and  the

extension .PRG .

To make the command available,  you have to note its name in  the

COMMANDS.BOX  file  using the syntax of that  file  as  discussed

above.  The  internal_command_number  must be either  99  or  98,

depending  on wether the program will use the full GEM  interface

of the screen or not.

Examples of an entry in COMMANDS.BOX:

RUNTEST 98 0 1      defines the external command RUNTEST  as

a TOS application that will not use  the

screen,  no sysop flag,  requires  user-

level 1.

RUNTEST 99 1 1      defines the external command RUNTEST  as

a  GEM application with full  access  to

the screen,  sysop flag required,  user-

level 1.

If  defined as a GEM application,  DP closes all  windows  before

execution.

The program must terminate itself.  If the DP watchdog is active,

the timeout is set to 45 minutes,  this should be enough for hard

disk reorganizers or similar.

DigiPoint Documentation                       16/05/94 v3.60 - Page 82

You can define as many external commands as you want.

The interface:

We have to distinguish three cases:

1) the user types 'RUNTEST' without any parameters.

This  results  in the execution  of  RUNTEST.PRG.  The  com-

mandline  contains  two parameters.  The first is  the  user

callsign,  the second is an adress in decimal notation. This

adress is a pointer.  A record with user specific parameters

is  found at this memory position.  See Appendix C  for  the

structure of that record. Your program may alter values, but

think about what you are doing...

2) the user types 'RUNTEST' with additional parameters,  for

example 'RUNTEST show all'.

The proceeding is as with 1),  but the parameters 'show' and

'all'  are  appended to the standard two parameters  of  the

commandline.

3)  the user types 'RUNTEST -' with  additional  parameters,

for example 'RUNTEST - show all'.  The '-' forces DP not  to

put  user callsign and adress in the commandline,  the  com-

mandline only contains 'show all'. This is necessary to call

programs that were not written for the DP interface.

The third case creates some problems to ensure that the interface

cannot be hacked.  If a user typed 'RUNTEST - DL9XYZ 452191', the

commandline looks exactly the same as if the first two  arguments

were  created  by DP.  But what happens if your  RUNTEST  program

accesses  to  the structure at adress 452191?  An  adress  error,

because it is odd.  To prevent such mess, your program always has

to  check the validity of the first two  parameters.  How?  First

check,  if  the adress is even.  Then find out if it points to  a

memory  location that is truly available in  the  computer.  Then

compare  the callsign in the commandline with that one  found  at

the memory location the adress points to.  If they are equal, you

can proceed,  if not,  the user tried something illegal.  Please,

provide this security check!

Your program can return ASCII text to be transmitted to the user.

If a file 'OUT.TXT' is found after execution of your program,  DP

sends the content to the user and deletes the file.

You find a small source code example on the distribution disk.

8.14. The Folder NEWMAIL

DigiPoint Documentation                       16/05/94 v3.60 - Page 83

On  startup,  DP checks if this folder contains some  files.  The

found files are read and sorted into the mailbox.  The files must

be of a special format.  In general,  one could say they have  to

look exactly as outputted files of DP bbs,  TheBox,  BaycomBox or

F6FBB bbs.  They MUST contain a file header of one of the  listed

systems. Those files are not forwarded.

Check from time to time if there are files in the DP\BOX\NEWMAIL\

folder.  If they are not sorted and deleted by DP,  they  contain

erroneous headers. Delete them manually.

8.15. Timeouts

After 20 minutes of no activity, a user is disconnected.

A s&f connect is cancelled after 15 minutes of no activity.

8.16. Multiple Commands

You  can enter multiple bbs commands in one line,  separate  them

with a semicolon (';'). Example:

u;v;r dl8hbs 1;c

This  is  the  reason why no semicolon is  accepted  in  password

strings and message title input.

8.17. Output Abort

You  can  abort the output of the bbs at every time by  typing  a

single <RETURN> (an empty line).

8.18. Erase / MyBBS Forwarding

DP does, as TheBox and BaycomBox, erase and mybbs forwarding.

Whenever  a  user enters his mybbs,  a message to  M @ THEBOX  is

generated,  with a subject of HomeBBS + UnixTime,  separated by a

space.

DigiPoint Documentation                       16/05/94 v3.60 - Page 84

Whenever  a  user deletes a message that was written by  him  and

that  was forwarded before or received in forward,  a message  to

E @ THEBOX is generated, with a subject of the BID of the deleted

message.

The forwarding neighbours that have an entry FOR THEBOX in  their

.SF definition files will receive those messages.

8.19. The File CONFIG.BOX

To be found in DP\BOX\SYSTEM\

This file defines three additional parameters for the mailbox:

# If UNDEFBOARD is defined, all messages to boardnames not

# defined in RUBRIKEN.BOX will be transferred to the specified

# board, in this case to DIV. This is an optional feature.

UNDEFBOARD DIV

# SYSFORWARD defines the distribution area of generated Erase-

# and MyBBS - forwarding. Usually, this is @THEBOX, but you can

# alter it to @DPNET or what else...

SYSFORWARD THEBOX

# starting with v 3.60, DP supports resume of S&F in case of link

# failures. The parameter RESUMELT defines the count of days a

# message fragment is held on disk before it is deleted.

RESUMELT 3

8.20. The Auto Import Feature

All 10 minutes,  the NEWMAIL folder will be checked for files  of

the type IMPORT.*

They have to match the expected file format (full status  header)

and are imported in the bbs as if they were sent manually.

8.21. The S&F - Resume Mode

Starting  with v 3.60,  DP supports a crash recovery in  S&F  (as

does f6fbb starting with his version 5.15c).  This means, that an

aborted  s&f  transmission  (due to linkfailures  etc.)  will  be

resumed the next time at the point the transmission was  stopped.

So,  if you forwarded a 300 kByte file and the link failed  after

299 kByte,  you will only have to send the missing 1  kByte,  not

all  300 kByte again.  This is a real improvement of the  amateur

radio forwarding system.

DigiPoint Documentation                       16/05/94 v3.60 - Page 85

9. The MODEM Port

Starting  with  version 3.60,  DP has an integrated  modem  port.

A user may connect the mailbox via any one of the serial links of

the atari, to be defined in a file MODEM.BOX. This connection may

take place via telephone lines including automatic answer of  the

modem or without modem simply by connecting a terminal or another

computer  to  the  defined serial port.  Note that  you  have  NO

protocol  on the line so you must run an error  correcting  modem

protocol (such as MNP 3-5) or use a noise free line.  DP switches

RTS/CTS  on  if  available on that  port.  Also,  you  will  have

problems in exchanging binary data, as no X/Y/Z-Modem protocol is

available. 7PLUS works, of course.

There is no S&F possible on the modem port.  To enable SWLs  just

to  read messages,  assign them a fantasy callsign (with a  usual

callsign syntax) and give them read permission on the modem  port

(PWMODE 0).

So, how is it configurated?

9.1. The File MODEM.BOX

On  startup,  DP  tries to read a file MODEM.BOX  in  the  folder

DP\BOX\SYSTEM\

An example:

# the PORT keyword selects the BIOS port to be used for the modem

PORT 1

# BAUD selects the baud rate between Computer and modem

BAUD 2400

# SWITCHBAUD ON/OFF defines if the serial baud rate between

# computer and modem should be altered if a msg as 'CONNECT 300'

# occurs. This is usual with old modems, newer ones handle the

# speed change stand alone.

SWITCHBAUD OFF

# The string that is to be sent to the modem to perform a reset

RESET ATZ

# The init string, in this example: set auto answer on afer two

# rings

INIT ATS0=2

# The dial string. Not yet used. Maybe in future...

DIAL ATDP

# The connect detect string. Defines the message your modem

# prompts if a connection was made.

CONNECT CONNECT

DigiPoint Documentation                       16/05/94 v3.60 - Page 86

# The error detect string. Blank it or enter the error message

# of your modem. Take care: If this string appears in user input,

# connection will be regarded as failed...

ERROR NO CARRIER

# defines the command to switch the modem in the offhook state.

# Note that the escape sequence '+++' must be accepted by your

# modem to leave converse mode ...

OFFHOOK ATH0

9.2. Starting Modem Operation

If the modem port is not occupied by a tnc (as it mostly is  with

the  older Atari ST's),  the modem is initialized on startup  (if

MODEM.BOX  is  found) and reset on exit of DP.

If the modem is defined for the same port as a tnc is,  you  have

to enter the box (only at the console...) and type MODEM ON.  The

TNCs are deinitialized and you will be prompted to plug the modem

in the serial port,  then the modem is initialized.  To end modem

operation,  you  simply  type  MODEM OFF (in  the  BBS)  and  the

procedure is inverted.

Now you are called by a modem user.  After the defined ring delay

time the modem goes onhook and prompts a string:

You are connected to the <CALLSIGN> PBBS. Please enter your Call:

The user then enters his call. (5 retries)

Then he is prompted for his personal password (3 retries).

The  password has to be typed in full length,  not only 5  random

chars as via the radio.

Now,  he is connected as if he entered the bbs via  radio,  write

access  according to his PWMODE (see BBS commands).  The  timeout

for  the  modem port is three minutes (if  no  input  came),  the

connection is ended with QUIT or ABORT.

A modem user appears with a lowercase first char in his  callsign

in the bbs user list.

Of  course,  the  usuall  telephone bbs  systems  are  much  more

perfect.  Always take in mind that we are doing amateur radio and

that  the  modem port is only an additional feature  for  special

usage in the DP system.

If  you want to use it to log in via a simple  terminal  (without

modem)  you  have to generate the defined  modem  connect  string

manually, for example you have to enter 'CONNECT <RETURN>'.

DigiPoint Documentation                       16/05/94 v3.60 - Page 87

APPENDIX

A. The German Keytable

The german keyboard looks as follows:

1 2 3 4 5 6 7 8 9 0  ' #

Q W E R T Z U I O P  +

A S D F G H J K L

< Y X C V B N M , . -

B. Station Identifiers (SID)

A  station identifier for forwarding systems is created with  the

following syntax:

[<system_name>-<version_number>-<command_set>]

The most important field is the <command_set>. The existing chars

indicate  a protocol feature.  They may be followed by a  number,

this indicates the protocol revision.

Lets investigate DP's SID:

[DP-3.60-AB1DFHMR$]

A    =    ACK messages known

B    =    hufman compression, only in conjunction with F

B1   =    extended  compressed  protocol,   resume  and  crc

added.

D    =    extended boardnames (8 chars), improved CRC

F    =    message exchange in blocks of 5, syntax of f6fbb

H    =    hierarchical adresses accepted

M    =    Message ID's accepted (BID's of private mails)

R    =    extended reject messages as defined by aa4re

$    =    Bulletin ID's accepted

C. External BBS Commands: The DP User Record Structure

As  written  above,  a  pointer to a  structure  containing  more

informations  about  a  user is found in the  commandline  of  an

external command.  The structure is listet below.  Note that  all

strings  are in PASCAL notation and not in C,  the difference  is

that  they  contain a length byte at the first  position  and  no

ASCII  0  at the end.  When altering such a  string,  always  use

PASCAL  syntax!  BOOLEAN  values are represented by  0=FALSE  and

1=TRUE.

DigiPoint Documentation                       16/05/94 v3.60 - Page 88

type

boxparmpointer= ^boxparms;

boxparms=record                    { Offset Function           }

call    :   string[6];  {  0    Callsign            }

mybbs   :   string[40]; {  8    mybbs               }

language:   string[3];  { 50    language            }

name    :   string[15]; { 54    name                }

board   :   string[8];  { 70    actual board        }

reply_board:string[8];  { 80    board for REPLY-cmd }

reply_call: string[6];  { 90    call for REPLY-cmd  }

reply_box:  string[40]; {132    @mbx for REPLY-cmd  }

reply_mbx:  string[40]; {174    @mbx for REPLY-cmd  }

reply_title:string[80]; {216    title for REPLY-cmd }

password:   string[80]; {298    users password      }

lastdate:   integer;    {380    last logindate      }

lasttime:   integer;    {382    last logintime      }

level   :   integer;    {384    users level         }

logindate:  integer;    {386    actual logindate    }

logintime:  integer;    {388    actual logintime    }

lasttouch:  integer;    {390    last activity (min) }

s_e_ok  :   boolean;    {392    send/erase allowed  }

hidden  :   boolean;    {394    hidden flag         }

console :   boolean;    {396    user at the console?}

sysop   :   boolean;    {398    sysop flag          }

fbbmode :   boolean;    {400    fbb mode flag (unused)}

cputime :   long_integer;{402   used time in 200Hz  }

txbytes :   long_integer;{406   sent bytes          }

rxbytes :   long_integer;{410   received bytes      }

tncchan :   integer;     {414   0 or the tnc-channel}

end;                        {416   over all            }

D. The Index Files (.IDX)

The format of the index files of the bbs is a continual array  of

the following record:

indexstruct=packed record               { offset desc.      }

bid         :  string[12]; {  0 BID            }

destination :  string[8];  { 14 original board }

sender      :  string[6];  { 24 senders call   }

mbx_field   :  string[40]; { 32 @mbx           }

title       :  string[80]; { 74 subject        }

date        :  integer;    {156 rx date        }

time        :  integer;    {158 rx time        }

txdate      :  integer;    {160 original tx date }

txtime      :  integer;    {162 original tx time }

lifetime    :  integer;    {164 lifetime       }

txlifetime  :  integer;    {166 original lifetime }

bodycrc     :  integer;    {168 for PACSAT broadc.}

start       :  long_integer;{170 offset in .IFO}

DigiPoint Documentation                       16/05/94 v3.60 - Page 89

packsize    :  long_integer;{174 stored size   }

size        :  long_integer;{178 original size }

rxqrg       :  long_integer;{182 receive QRG   }

rxfrom      :  string[6];  {186 received from  }

deleted     :  boolean;    {194 delete flag    }

mode        :  byte;       {195 binary, comp.type}

level       :  byte;       {196 min. level for read}

fwdct       :  byte;       {197 n * forwarded  }

gate        :  byte;       {198 via tnc (unused)}

msgflags    :  byte;       {199 special        }

end;                         { total size 200 bytes }

Strings are (of course) of PASCAL type!

E. The Checklist Format (B_BOXLOG.DP and U_BOXLOG.DP)

The checklist is a continual array of the following record:

boxlogstruct=packed record

date       :  integer;    {  0 rx date        }

time       :  integer;    {  2 rx time        }

idxnr      :  integer;    {  4 index in board }

size       :  long_integer;{ 6 size           }

lifetime   :  integer;    { 10 lifetime       }

deleted    :  boolean;    { 12 delete flag    }

unused     :  boolean;    { 13                }

mode       :  byte;       { 14 binary, compmode }

level      :  byte;       { 15 userlevel      }

board      :  string[8];  { 16 board in my bbs}

bid        :  string[12]; { 26 BID            }

sender     :  string[6];  { 40 senders call   }

mbx_field  :  string[6];  { 48 @mbx           }

title      :  string[25]; { 56 subject        }

end;                       { 82 bytes over all }

Strings are (of course) of PASCAL type!

F. The File HPATH.BOX

This  file contains the result of the header analysis.  It  is  a

continual array of the following type:

hboxtyp=record

call     :  string[6];

hpath    :  string[40];

bestfrom :  string[6];

rxfrom   :  string[6];

desc     :  string[40];

lasttxd  :  integer;

DigiPoint Documentation                       16/05/94 v3.60 - Page 90

lasttxt  :  integer;

msgct    :  long_integer;

best     :  long_integer;

aver     :  long_integer;

bytect   :  long_integer;

atdate   :  integer;

attime   :  integer;

end;

Strings are (of course) of PASCAL type!

G. The Extended Forward Protocol

DP uses six different forward protocols

1) W0RLI

2) TheBox

3) F6FBB version 1

4) DP version 1

5) F6FBB version 2 (resume mode)

6) DP version 2 (resume mode)

The  differences between 3) and 4) are some extensions  made  for

proper use in the german network.

The DP forward protocol compared with F6FBB compressed protocol:

It is indicated by the presence of the letter 'D' in the SID.

Proposals are started with FD instead FA / FB.

The  checksum  of  the  proposals  is  a  CRC-16  in  hexadecimal

notation,  sent LSB -> MSB.  For computation of the CRC, only the

text content of the proposals without <CR> is regarded.  The  CRC

is preloaded with 0.

Example:

FD P DL8HBS DB0EAM.#HES.DEU.EU DL3AAA 31310ADL8HBS 700

FD B DL8HBS DL SYSOP 32670BDL8HBS 1508 60

FD B DL8HBS EU STANDARD 32670CDL8HBS 6987 365

(... up to five ...)

F> A7CF

Boards may count 8 signs.  Optionally,  a lifetime is added (last

parameter of proposals 2 and 3).

The message exchange is using YAPP,  as F6FBB does.  The  onliest

difference  is the end_of_transmission - block:  Instead of  a  1

byte checksum it contains a 2 byte CRC-16, LSB -> MSB. The CRC is

computed  over  the  content of all  blocks  transmitted  before,

including the title block. It is preloaded with 0.

DigiPoint Documentation                       16/05/94 v3.60 - Page 91

The difference between 5) and 6) is the altered proposal protocol

for 6) as in 4),  additionally, the CRC of 6) is extended via the

message  title.  5) only CRCs the message body.  Note that  F6FBB

uses a very special CRC calculation method.

H. The Static Huffman Compression Table

This table was set up by DK4NB for SP 9.00

ASCII:  HUFFMAN

<  0>:  101010110010110

<  1>:  101010101000010

<  2>:  100111111100010

<  3>:  101010110011110

<  4>:  101010110001110

<  5>:  101010101111110

<  6>:  101010101110110

<  7>:  101010101101010

<  8>:  101010101011010

<  9>:  1111001101

< 10>:  101010101010010

< 11>:  011111010110010

< 12>:  101010101101110

< 13>:  1111010

< 14>:  101010101001010

< 15>:  100111111111010

< 16>:  100111111101010

< 17>:  011111010111010

< 18>:  101010110100010

< 19>:  101010110011010

< 20>:  101010110010010

< 21>:  101010110001010

< 22>:  101010110000010

< 23>:  101010101111010

< 24>:  101010101110010

< 25>:  10101011011000

< 26>:  101010110000110

< 27>:  101010101100110

< 28>:  101010101011110

< 29>:  101010101010110

< 30>:  101010101001110

< 31>:  101010101000110

:  110

!:  001110101

":  1010101111

#:  00000000011

$:  011111010100

%:  10101011010111

&:  000000000000

':  10101011010110

(:  011111000

DigiPoint Documentation                       16/05/94 v3.60 - Page 92

):  001111001

*:  01111101000

+:  000000000001

,:  0001001

-:  0111101

.:  101110

/:  0011001

0:  0010001

1:  11110110

2:  00111101

3:  100111100

4:  101111011

5:  011111001

6:  000000001

7:  101010100

8:  101111010

9:  100111110

::  00000011

;:  1010101101111

<:  1010101101101

=:  001111000

>:  001100000

?:  00000000001

@:  011111010101

A:  0011100

B:  0111100

C:  1001110

D:  1111111

E:  001001

F:  10111100

G:  00000010

H:  00000001

I:  11110001

J:  00000000010

K:  00110001

L:  11110010

M:  0011010

N:  0001110

O:  0001111

P:  1011111

Q:  10101011101

R:  0011111

S:  000101

T:  0011011

U:  111100111

V:  111100001

W:  00100000

X:  11111100

Y:  1001111110

Z:  100111101

[:  101010111001

\:  001110111

]:  101010111000

^:  10101011010101

_:  0011101001010

DigiPoint Documentation                       16/05/94 v3.60 - Page 93

`:  10101011010100

a:  10100

b:  000110

c:  100110

d:  01110

e:  010

f:  000001

g:  101011

h:  111110

i:  0110

j:  0011101000

k:  11111101

l:  00101

m:  101100

n:  1000

o:  101101

p:  0001000

q:  011111010010

r:  11100

s:  10010

t:  11101

u:  00001

v:  11110111

w:  1010100

x:  011111011

y:  1111001100

z:  0111111

{:  10101011010010

|:  0011101001001

}:  10101011010011

~:  001110100110

<127>:  100111111111110

<128>:  100111111110110

<129>:  00100001

<130>:  100111111101110

<131>:  100111111100110

<132>:  111100000

<133>:  011111010111110

<134>:  011111010110110

<135>:  00111010010000

<136>:  101010110100000

<137>:  101010110011100

<138>:  101010110011000

<139>:  101010110010100

<140>:  101010110010000

<141>:  101010110001100

<142>:  1010101101110

<143>:  101010110001000

<144>:  101010110000100

<145>:  101010110000000

<146>:  101010101111100

<147>:  101010101111000

<148>:  001110110

<149>:  101010101110100

<150>:  101010101110000

DigiPoint Documentation                       16/05/94 v3.60 - Page 94

<151>:  101010101101100

<152>:  101010101101000

<153>:  10101011011001

<154>:  011111010011

<155>:  101010101100100

<156>:  101010101100000

<157>:  101010101011100

<158>:  101010101011000

<159>:  101010101010100

<160>:  101010101010000

<161>:  101010101001100

<162>:  101010101001000

<163>:  101010101000100

<164>:  101010101000000

<165>:  100111111111100

<166>:  100111111111000

<167>:  100111111110100

<168>:  100111111110000

<169>:  100111111101100

<170>:  100111111101000

<171>:  100111111100100

<172>:  100111111100000

<173>:  011111010111100

<174>:  011111010111000

<175>:  0011101001011

<176>:  011111010110100

<177>:  011111010110000

<178>:  101010110100011

<179>:  101010110100001

<180>:  101010110011111

<181>:  101010110011101

<182>:  101010110011011

<183>:  101010110011001

<184>:  101010110010111

<185>:  101010110010101

<186>:  101010110010011

<187>:  101010110010001

<188>:  101010110001111

<189>:  101010110001101

<190>:  101010110001011

<191>:  101010110001001

<192>:  101010110000111

<193>:  101010110000101

<194>:  101010110000011

<195>:  101010110000001

<196>:  101010101111111

<197>:  101010101111101

<198>:  101010101111011

<199>:  101010101111001

<200>:  101010101110111

<201>:  101010101110101

<202>:  101010101110011

<203>:  101010101110001

<204>:  101010101101111

<205>:  101010101101101

DigiPoint Documentation                       16/05/94 v3.60 - Page 95

<206>:  101010101101011

<207>:  101010101101001

<208>:  101010101100111

<209>:  101010101100101

<210>:  101010101100011

<211>:  101010101100001

<212>:  101010101011111

<213>:  101010101011101

<214>:  101010101011011

<215>:  101010101011001

<216>:  101010101010111

<217>:  101010101010101

<218>:  101010101010011

<219>:  101010101010001

<220>:  101010101001111

<221>:  001110100111

<222>:  101010101001101

<223>:  101010101001011

<224>:  101010101001001

<225>:  001100001

<226>:  101010101000111

<227>:  101010101000101

<228>:  101010101000011

<229>:  101010101000001

<230>:  100111111111111

<231>:  100111111111101

<232>:  100111111111011

<233>:  100111111111001

<234>:  100111111110111

<235>:  100111111110101

<236>:  100111111110011

<237>:  100111111110001

<238>:  100111111101111

<239>:  100111111101101

<240>:  100111111101011

<241>:  100111111101001

<242>:  100111111100111

<243>:  100111111100101

<244>:  100111111100011

<245>:  100111111100001

<246>:  011111010111111

<247>:  011111010111101

<248>:  011111010111011

<249>:  011111010111001

<250>:  011111010110111

<251>:  011111010110101

<252>:  011111010110011

<253>:  011111010110001

<254>:  001110100100011

<255>:  001110100100010