ATARIO

ATARIO is a TURBO Pascal program to read on an IBM PC true double

density diskettes from an ATARI double density drive.  That is to say

it will not read diskettes created on an ATARI 1050 drive in "density

and a half" mode.  This restriction (and the inability to read single

density diskettes) is due to the IBM PC diskette adapter.

This program assumes that the ATARI diskette is in drive A and that the

current IBM drive is other than A:.  Since source code is provided, you

may recode for any other assumptions or generality that you prefer.

When offered a numbered menu choice, you may either enter the digit

specified, or the corresponding Function key.  For example the digit 1

and F1 are equivalent, as are 0 and F10.

Main Menu:

1: Dump Sectors

2: Copy File

3: Directory List

4: Configure

0: Exit

Configuring ATARIO:

Choosing option 4 from the main menu brings up the configuration menu.

Three items are offered:

1: Side 1

2: DOS 2.x

3: Atari XF551

4: Drive A

5: Text Color Change

6: Text Background Change

0: Main Menu

Choice 1 acts as a toggle between Side 1 and Side 2 of a diskette

formatted on a double side drive.  THIS WILL NOT READ THE THE BACK SIDE

OF A "FLIPPY" DISKETTE WITHOUT FLIPPING IT.  Furthermore, this was

written using the assumption that the diskette was formatted in the

Percom style, where sectors 1-720 are on side one and 721-1440 are on

side 2. Furthermore, it is assumed that side 2 is logical diskette by

itself (as Dave's DOS does).  If you have a two sided diskette and your

DOS (such as MYDOS) treats the diskette as one large filespace (i.e.

sectors 1-1440), then you don't need this paramter at all.

Choice 2 acts as a toggle between ATARI DOS 2.1 and LJK Dos.  The

default is DOS 2.1.

Choice 3 acts as a toggle that affects how the program reads the second

side of a DS/DD disk.  It is set up to work with the Atari XF551.  If

you are having problems reading the second side of a disk and are not

using an XF551, try changing this parameter.

Choice 4 allows you to use another drive on your PC.  It uses DOS absolute

drive letters, which can be different from what you are used to, especially

if you have an external drive.  Try a directory after changing this to

see if the right drive is selected.

Choices 5 and 6 are self-explanatory, and are, I admit, a bit frivilous.

Dumping Sectors:

Choosing option 1 from the main menu allows one to dump in ASCII and

HEX format sectors from the ATARI diskette to the screen.  You are

prompted for a sector number (in decimal) to enter.  Simply pushing

enter will bump the current sector number by 1. Enter a value of 0 will

return you to the main menu.

Copying Files:

Choosing option 3 from the main menu brings you to the file copy menu.

You are offered 3 choices:

1: Text Copy

2: Image Copy

0: Main Menu

Whether option 1 or 2 is chosen, you will be prompted for an ATARI file

name.  An file of the same name will then be created on the current IBM

drive (which means that the current drive should not be the A: drive).

Text Copy:

Choosing option 1 invokes the Text Copy.  A Text Copy treats logical

records on the ATARI disk (a string of bytes terminated by a 9B) as a

logical record on the IBM file (a string of bytes terminated by a

CR-LF).

Image Copy:

Choosing option 2 invokes the Image Copy.  An Image Copy transfer the

ATARI file to the IBM file as is with no interpretation of the data.

Directory List:

Choosing option 3 from the main menu will give you a directory list of

the ATARI diskette.  Pressing any key will return you to the main menu.

Should a physical I/O error on the ATARI diskette occur during any

operation, the partial file will be deleted from the IBM drive and

an error display will appear with the following heading:

Dos_Sector  Bios_Side   Bios_Track   Bios_Sector   Return_Code

Where  Dos_Sector  : Atari sector number

Bios_Side   : Physical Side of the Disk

Bios_Track  : Physical Track on the Side

Bios_Secotr : Physical Sector on the Track

Return_Code : IBM BIOS return code

80 : Time out

40 : Seek operation failed

20 : NEC controller has failed

10 : Bad CRC on diskette read

09 : Attempt to DMA across 64K boundary

08 : DMA overrun on operation

04 : Requested sector not found

03 : Write attempted on write protected disk

02 : Address mark not found

01 : Bad command passed to disketted I/O

Return codes of 40, 10, 04, and 02 should be the most common error

codes, probably from copy protected diskettes.  If the physical error

occurs while copying a file, you will be given the option to delete

the incomplete file on the IBM disk.

Dave Brandman

805 Rotherham Drive

Manchester, MO  63011

Modified to add support for the Atari XF551, other drives on a PC,

changing colors, and deleting incomplete files on September 6, 1991 by

Kevin White

235 Race St.

Berea, OH 44017