________________________________________________________________________

#p           OCTALYS�R� �CHRISTMAS PR�VI�W� � Reference manual            #q

������������������������������������������������������������������������

Congratulations, You are now a proud owner of a preview version of the

almighty Octalyser�, a 4/6/8 channel soundtracker compatible with the

ProTracker, and the first 8 channel TRACKER for the STE-computers.

Replay Preferences:    Four, Six or Eight independed channels

 12.5/15.5/18.9/25Khz replay frequencies

 Full Volume-controll on each channel

 Compatible with all ProTracker commands

 Finetune and sample-looping

Credits:  COD� of TBL           Main coding and design

 Currently used graphics

 Project initiator

 Project leader

ALAN F. of R�WARD     Co-coding and main beta-testing

 16Mhz STE testing and support

 Public Relations and spreading

CHAOS of R�WARD       Graphics design (not used in this

preview version)

� This manual is written by Alan F. of Reward �

________________________________________________________________________

#p                           Using the pattern editor                     #q

������������������������������������������������������������������������

The pattern editor of the Octalyser is very similar to the one included

in the ProTracker ST(e).  To move between the events in a pattern, you

can use UP/DOWN arrows, or click the mouse onto the pattern position.

You can also select the event-data to edit by clicking on its position.

�Only� 4 channels are shown at the same time. You can scroll channels

by either using the arrows on the keyboard, or pressing the left/right

arrows at the edges of the screen.  Above the patterndata of each

channel is a number, that shows which channel that is.

To be able to change the event-data, you must enter the EDIT mode,

either by clicking at the EDIT-button, or pressing the SPACE on the

keyboard.  If you are familiar with the standard event-data format and

the ProTracker commands, then you know exactly what to do. :)

If you are not, then wait until the real version of the Octalyser. The

documentation there will be much more helpfull. Remember that this is

only a Christmas-preview!

Several keyboard shortcuts are available, here is a list of them:

F1 ���������������������� Toggle octaves

F2 ���������������������� Toggle write mode (Replace/Overwrite)

F7 ���������������������� Position $10

F8 ���������������������� Position $20

F9 ���������������������� Position $30

RETURN ������������������ Play module

shift RETURN ������������ Play pattern

TAB ��������������������� Cursor to the next channel

shift TAB ��������������� Cursor to the previous channel

control #p > #q ������������� Next pattern

control #p < #q ������������� Previous pattern

SPACE ������������������� STOP / Toggle edit-mode

ESC ��������������������� Quit to the DeskTop (or whatever)

If you hold shift while moving up or down through the pattern, then

the currect value of the pattern-step will be used as a jump offset.

More keyboard shortcuts will be available in the final version.

________________________________________________________________________

#p                 Replay modes, frequencies, channels etc.               #q

������������������������������������������������������������������������

Octalyser offers several replay modes, and the quality of the replay

depends on the power of you computer. That means that you will get

better quality on an 16Mhz STE compared with a normal 8Mhz STE.

You will not notice that in 4 channel mode however. This is what a

normal 8MHz STE can perform:

4 channels mode          12.5/15.5/18.9/25  kHz

6 channels mode          12.5/15.5/18.9     kHz

8 channels mode          12.5/15.5          kHz

You probably won�t notice the difference between 18.9 and 25kHz in the

6 channel mode. In the 8 channels mode high-hats, snares sampled at

high frequencies and crashs will sound poor if they�ve not been sampled

with a good sampler.  Note that the DMA-chip is set to 25Khz, so it

gives the sound somewhat better quality in 15.5 and 18.9kHz modes.

On an 16MHz STE (without cache), you will be able to use these

modes:

4 channels mode          12.5/15.5/18.9/25  kHz

6 channels mode          12.5/15.5/18.9/25  kHz

8 channels mode          12.5/15.5/18.9     kHz

As you can see, the quality of the replay is a bit better if you have

a 16MHz STE.

With the 16MHz and cache, you�ll be able to use 25Khz in 8 channel

mode.

The CPU-power of you computer is tested, and the pattern-update is

adjusted to it. Than means that you�ll be able to see more of the

pattern data on a faster CPU, depending on which mode you use.

This is quite a neat thing, especialy if you have a faster STE or

a megaSTE, most other trackers don�t use the extra CPU-time left

after having calculated the sound data! You can of course change

the number of NOPs, so that the border rutine will work on your

computer, in case that you have a computer with other timing.

Actualy, you probably won�t need to do that, as the routine checks

the speed of you computer and adjusts even the number of NOPs

automaticaly.

Please notice that the full volume control is available on all

channels indepentently of each other, and that the volume control

is processed the whole time. Without the volume control, a better

replay quality could be achived, but we thought that the lack of

volume control would be a major drawback, so you�ll have to write your

own 8 channel routs without volume control! Hehe. However, there might

be some extra modes in the final version... We do not promise

anything...

The sound is created by mixing the channels. The 4 channel mode sounds

thereafter loudest. In 8 channel mode more channels have to mixed into

one channel, and that lowers the loudness of the sound.  When you

pump up the bass, the peak will jump quite a lot, often creating the

distortion in the sound. To avoid this, you should lower the main

volume of the DMA-chip, and pump up the volume of your ghetto-blaster

or whatever sound system you use. When using 8 channel mode, this

isn�t necessery, so you may pump the DMA-mainvolume up again, and the

loudness will be just as when using 4 channels with lowered

DMA-mainvolume.

The Octalyser is compatible with most trackers (not TCB or such stuff),

but will NOT accept 15-intrument-modules. That format is used with

old modules, and is not really common nowadays.

The Octalyser has been tested with several modules, and they all

sounded like they should. The slides, dists, vibratos and all other

effects used in some modules (especialy ones using �chip� looped

sounds) sound just like the original ProTracker.  If you find a module

that fails to work, and you are sure that it�s not crunched or

something like that, then contact us!

So, compared to the EPSS by Unit 17, how good is Octalyser?

For you that are used to usual trackers, and the wonders of the

sample commands, and that think that volume control is important,

the Octalyser is the perfect product. However, if you have a

slow STE, then the sound quality will suffer a bit.  EPSS is

quite fast, thanks to the non-tracker principe used there, but

it cannot do volume control on all channels! Furthermore, it�s quite

unusable if you are not used to CuBase or some other sequencer.

________________________________________________________________________

#p                                Options                                 #q

������������������������������������������������������������������������

The options submenu offers you the posibility to adjust the DMA-chip,

set the key repeat and delay rate, replay frequencies etc.

In case that you set up a configuration that takes more than 100% of

the relative CPU-time, you will notice it when the mouse pointer is

being updated every second VBL. By using this method, the Octalyser

won�t crash or get stuck if it gets �overheated�. If you are playing

the module at that time, than it will sound quite strange.

You can also change to number of NOPs used in the border rutine, in

case that the lower border is flickering or you are have some other

problems with it.  To avoid the eventual bitplane errors, which

sometimes happen while flipping the NOPs, the screen frequency is

toggled to 60Hz and then back to the 50Hz.

________________________________________________________________________

#p                    Octalyser, the finished version                     #q

������������������������������������������������������������������������

The official finished version will be released when the block-edit

functions, sample-edit functions and a few more things are finished

and included in the Octalyser.

The final version will come as a complete package including the

Octalyser itself, documentation (probably in several formats, like

Calamus, Script and Raw-ASCII), demonstration modules, demonstration

samples, probably a player for 4/6/8 channel modules etc.

There might be a MIDI-IN support capable of polyphonic sample-replay

and polyphonic midi-recording. No more troubles with chords, strings,

etc. Now you have 8 channels!

We hope to make a fully functional product that you�ll have a

pleasure in using it.

As I don�t want to reveal all our future plans yet, you will just

have to wait and see!    We do hope that this Christmas preview

made you curious about the final product and that you will spread

this preview to all you contacts. (Think about all the credits you�ll

get for spreading this wonderful product!! hehe)

#p                                                                       #q

#p  Please note that we do not take any responsibility if the Octalyser  #q

#p  causes directly or indirectly any damage to your equipment. You are  #q

#p    using this application at your own risk.  Nor will we take any     #q

#p   responsibility for eventual heart attacks caused by this almighty   #q

#p                               product!                                #q

#p                                                                       #q

________________________________________________________________________

#p                      Contact adresses, BBSes etc...                    #q

������������������������������������������������������������������������

For contacting, write to these adresses:

CODE / Christian Dahl                     ALAN F. / Slutej

Fotbollsv�gen 11                          S�dra All�gatan 18

735 37 Surahammar                         722 14 V�ster�s

Sweden                                    Sweden

Or call these BBSes:

Vodka BBS (Alan F.) - 021-112076 - NonStop - 300/1200/2400BPS

2:206/209, 7:100/102, 15:4040/413, 90:1102/102, 66:106/113.1, 69:4606/116.1

(FidoNet)  (FujiNet)  (DataNETL.)  (NeST)       (BlackNet)    (SexNet)

Chaotic BBS (Chaos) - 0456-25134 - 22-08 - 300/1200/2400BPS

How about donating a Kurzweil K-2000?

________________________________________________________________________

#p                                Greetings                               #q

������������������������������������������������������������������������

Some personal greetings are comming your way.

Greets from Alan F. / REWARD go to:

Unit Seventeen            - I hope to see EPSS for Falcon soon. hehe.

Spec. greets to Elof and Jason

Omega                     - Grotesque was great, especialy on the big

screen in Lidk�ping!

Aura                      - I realy like your products!

AN COOL                   - No comments needed... :)

Best Babies               - It�s time for you to release something!!!

Zechariah                 - Take it easy with the booze. I sure am

trying.. Hehe, see you at the Die Krupps

concert, I hope!

All my BBS-contacts and users at Vodka BBS...

Special greets to Zeus, Buraz, Deity, Dux and rest of the gang...

#p The team of Octalyser... WE MAKE YOU A BELIVER... #q