These two modules simply allow a programmer to access the PCS

screen structure, retrieving the 8-bit RED,GREEN and BLUE values of any

defined pixel on the screen.

Notes...

READ_SPU only needs TWO addresses passed. DO NOT alter the second

two since it will cause a crash. READ_PCS can use all four addresses,

but that is assuming you want to work from an interlaced screen. To read

from a PCS-ST/STE screen, set up the first two and ZERO the second two,

this informs the module you are only interested in screen 1.

All returned values are 8-bit standard (0-255) and need to be divided

by 16 to be used on an STE/AMIGA, or by 32 for an ST.

INPUTS:	*-----------------------------------------------------*

CODE+30	interlace screen 1

CODE+34	interlace palette 1

CODE+38	interlace screen 2	\ Leave alone for READ_SPU.BIN

CODE+42	interlace palette 2	/ since only 1 screen is used.

D1		x-coordinate of pixel to read

D2		y-coordinate of pixel to read

OUTPUTS:	*-----------------------------------------------------*

D0		red value of pixel (0-255)

D1		green value of pixel (0-255)

D2		blue value of pixel (0-255)