KERMIT-09 Users guide

FLEX-09 KERMIT

--------------

Author:      Jur van der Burg

Nettelhorst 56

2402 LS  Alphen aan den Rijn

The Netherlands

Language:    C (Compiled with Introl (c) compiler)

Version:     2.3

Date:        November 1985

KERMIT for FLEX is derived from the UNIX version.

It is enhanced in several ways, such as data logging, server

mode etc.

It  should  run  on  about  any  version of the FLEX-09 (tm)

operating  system. Hardware dependent things are kept in the

files FLK.H and FLIO.C .

KERMIT-09 Users guide

Command summary:

CONNECT

Format:   CONNECT

The  CONNECT  command will allow you to make a connection to

the  remote  system  over the line that was specified by the

SET  LINE  command.  If a log file was opened (SET LOG) then

the  data  will be buffered in memory. If this becomes full,

it will be written to disk. Handshaking is provided (see SET

HANDSHAKE).

While connected, several sub-commands are possible.

These are the escape character arguments:

C            Close connection, "escape" to command mode

S            Show status of connection

Q            Quit logging

R            Resume logging

H            Show availability

B            Send 'BREAK' signal

F            Execute FLEX command

0            Send null

?            Show escape character arguments

escape char  Send escape character itself

Other        Rings the bell

SEND

Format:    SEND file [,file [,file...]]

The  SEND  command  will  allow you to send a file(s) to the

other   Kermit.  The  drive  number  specification  will  be

stripped off the filespec.

The current transfer can be aborted with control-C.

RECEIVE

Format:    RECEIVE

The RECEIVE command will allow you to receive a file(s) from

the  other  Kermit.  The  filenames will be specified by the

sending (remote) Kermit.

The current transfer can be aborted with control-C.

GET

Format:    GET file [,file [,file...]]

The  GET  command  will  allow  you  to get a file(s) from a

remote server by specifying their names.

The current transfer can be aborted with control-C.

KERMIT-09 Users guide

SERVER

Format:    SERVER

This  command  will  cause  Kermit to enter server mode. The

other  Kermit  can  then  issue  server commands to send and

receive  files  without  having  to  give  SEND  or  RECEIVE

commands  to  Kermit-09. Type a Control-C to quit the server

mode. After a timeout the server will quit.

FINISH

Format:    FINISH

This  command  will  case Kermit-09 to tell the other Kermit

(Which  should be in server mode) to exit from Kermit. After

receiving  acknowledgement  that  this is being done, Kermit

will prompt for another command.

BYE

Format:    BYE

This  command  will cause Kermit-09 to tell the other Kermit

(which should be in server mode) to exit from Kermit and, if

applicable,  terminate  its  job  (or  process, etc.). After

receiving  acknowledgement  that  this is being done, Kermit

will exit to FLEX.

HELP

Format:    HELP

This  command  will  give  a  short display of the available

commands.

LOAD

Format:    LOAD file

This command will send a textfile to the current line.

It  can  be  used  to  send  a file of parameters to a smart

modem, or to send bare text to the remote system.

FLEX

Format:    FLEX [command]

This  command  allows  you  to  execute  a FLEX command from

within Kermit. When no command is given, it will be prompted

for.  Be  careful only to use FLEX commands which run in the

utility command space!

KERMIT-09 Users guide

STATUS

Format:    STATUS

This  command  shows  the  current status of Kermit-09. This

includes  the  number  of characters that have been send and

received  from  the remote Kermit. When a real-time clock is

available,  an  estimate  of  the effective baud rate of the

transfer is included.

EXIT

Format:    EXIT

This  command  will  exit  Kermit-09. If a log file was used

with  CONNECT,  and the buffer contains still data, then the

buffer will be written to disk before terminating Kermit-09.

KERMIT-09 Users guide

The SET command is used to set various parameters in Kermit.

SET

Format:    SET command

LINE

Format:    SET LINE address

This  command  will the the address of the interface you are

using  for  the transfer. A check will be made to see if the

interface is available.

BAUD

Format:    SET BAUD speed

This   command   will  set  the  desired  baudrate  for  the

communication.  In  simple  FLEX  systems the only baudrates

allowed are 300 and 1200. This is switched by the divider of

the ACIA interface.

CONFIGURATION

Format:    SET CONFIGURATION number

This  command  will  set  the  configuration  to use for the

communication. The following values are possible:

0 - 7 bits, even parity, 2 stop bits

1 - 7 bits, odd parity, 2 stop bits

2 - 7 bits, even parity, 1 stop bit

3 - 7 bits, odd parity, 1 stop bit

4 - 8 bits, no parity, 2 stop bits

5 - 8 bits, no parity, 1 stop bit

6 - 8 bits, even parity, 1 stop bit

7 - 8 bits, odd parity, 1 stop bit

ESCAPE

Format:    SET ESCAPE character

This  command allows you to set the ESCAPE character for the

CONNECT processing.

DEBUG

Format:    SET DEBUG on/off

This  command  will  make the transfer more verbose. It will

show the received and transmitted packets, and various other

things of interest.      KERMIT-09 Users guide

TIMEOUT

Format:    SET TIMEOUT value

This command will set the number of seconds before Kermit-09

will  time out to attempt to receive a message. This timeout

is  used  to handle transmission errors which totally lose a

message. The default value is five seconds.

IMAGEMODE

Format:    SET IMAGEMODE on/off

This  command will set the image mode transfer flag. When it

is  set,  the  data  will  be  transferred as it is. This is

useful  for transfer between FLEX systems. When clear, space

compression  will take place, as well the expansion of tabs.

Carriage  returns  will  not  be processed, while a linefeed

will be converted to a carriage return.

DUPLEX

Format:    SET DUPLEX full/half

This  command  will control the CONNECT command, in that the

locally  typed  characters will be echoed to te local system

when duplex is set to HALF.

LOG

Format:    SET LOG file

This  command  will  specify  a  file  to  capture  the data

received from the remote system within the CONNECT command.

The file will be closed if the filespec is a dash ('-').

HANDSHAKE

Format:    SET HANDSHAKE start stop

This  command  will  set  the  handshake characters used for

controlling  data  within  the CONNECT command. The defaults

are  XON and XOFF. When the capture buffer becomes full, the

stop  character is sent to the remote system. If that system

stops  sending  data,  the  buffer  will be written to disk.

After that the start character will be sent.

The  SHOW  command will show all parameters set with the SET

command, with one extension: SHOW ALL.

Format:    SHOW command