*************************************************************

*                                                           *

* << Ametyste 4.6 : >>                                     *

* ---------------------                                     *

*                                                           *

* << Code : GT Turbo (GT) >>                                *

*                                                           *

* << Mouse Sprite, Font : RAZ(TheLAB) >>                    *

*                                                           *

*                      << C.V.S.D. 1995 >>                  *

*                                                           *

* Little revision : 14/04/2004                              *

*                                                           *

* What's Ametyste ?                                         *

*                                                           *

*                                                           *

* Asm coders, knows that it can be dangerous to code demos  *

* or games, coz you need speed and to do that, only one     *

* thing : killing all systems functions or calls ! But the  *

* little problem is when you have an error, the PRG doesn't *

* come back to Devpac (Turbo-asm, etc....) coz no           *

* interrupt, screen not at good address, etc....            *

*                                                           *

* So here come a little solution : Ametyste                 *

*                                                           *

* Ametyste will modify the bus, adress, illegal, div by     *

* zero, CHK vector to avoiding the non come back.           *

*                                                           *

* When you have an error, Ametyste will restore some        *

* registers (MFP, video, Yamaha, etc...), display some      *

* infos (Error number, 68000 registers), wait some time     *

* and come back to your editor.                             *

* The reset vector is modified too, for neverending loop !  *

*                                                           *

* The only thing to do is to put Ametyste on the root       *

* of the boot disk (A: for drive disk, C: for hard drive)   *

*                                                           *

* When you open the .ACC, you have some choices :           *

*                                                           *

* - State : Installed means the vectors errors are the      *

*           Ametyste one.                                   *

*           Removed means the vectors errors are the        *

*           original one (Same at the boot)                 *

*                                                           *

* - Reset : same thing as State                             *

*                                                           *

* - Resave state : If you have modified some registers      *

*                  and you want Ametyste to resave it       *

*                                                           *

* Shortcut keys are between brackets (S) means Ctrl+S       *

* you can close Ametyste with (C) (Ctrl+c)                  *

*                                                           *

* The last thing, you can send an adress rout to Ametyste.  *

* After an error, Ametyste will display infos, wait a key   *

* and after jump to your rout.                              *

*                                                           *

* To do that : just put our adress rout in a0.l             *

*              and calling trap #4                          *

*              But don't forget to quit in your rout !      *

*                                                           *

* Display infos : when you do an error, on the screen are   *

*                 displayed the 68000 registers (Dx, Ax)    *

*                 and the error number by some little cars  *

*                 in the end of display.                    *

*                 2 Cars means Bus error                    *

*                 3 Cars means Adress error                 *

*                 ..........................                *

*                                                           *

* The display rout is only med rez, sorry for low res       *

* coders, display will be a little bit hot !!               *

*                                                           *

* This version run only on STF/STE, i've got a falcon one   *

* but this doesn't run 100% correctly. When i got a little  *

* bit of time, i will do it better. Or you can do one with  *

* this, coz the source are given. If you do a Falcon one    *

* contact me (GT Turbo).                                    *

*                                                           *

*                                                           *

* C U Soon you on : CEREBRAL-VORTEX.NET or ATARI-FORUM.COM  *

*                                                           *

*                                                           *

*                     The < Cerebral Vortex > Team          *

*                                                           *

*************************************************************