This is a plaintext dump of the HTML version available at

http://www.nacs.net/~classics.   The resulting format is sometimes hard

to read.  For a well formatted and illustrated version see the URL above.

----------------------------------------------------------------------------

THE ATARI PERIPHERAL EMULATOR

USERS GUIDE

-----------------

_Version 1.18_

To make sure you have the _NEWEST_ version of APE, check the APE HOME

PAGE at _http://www.nacs.net/~classics/_

Also see the ProSystem Users Manual and the APE Warp+ OS InfoPage for

current information!

July 30, 1998

_Steven J Tucker

3978 Brookside Blvd

Cleveland, Ohio 44111

_U.S.A

Written by Steven J Tucker

Copyright (c) 1998 by Steven J Tucker

---------------------

The Atari Peripheral Emulator (APE) is a advanced serial application

that emulates an entire

subsystem of atari 8 bit peripherals using your existing or purchased

APE cable.

APE has been created in the idea that _anything_ a real, physical SIO

device can do,

APE can do better, faster, and at _far less cost_ than the real

peripheral!

__________________________________________________________________

ORDERING INFORMATION

For additional information on licensing the Atari Peripheral Emulator,

see the end of this document,

the Ape Online Order Form, ORDER.DOC (included in the APExxxD.ZIP

distribution), or contact:

_Steven J Tucker_

Voice (216) 688-0001

3978 Brookside Blvd

Cleveland, Ohio 44111 USA

Electronic Mail to:

_classics@nacs.net _

(if mail to this address fails, please e-mail

_ai123@freenet.carleton.ca_)

_______________________________________________________________

APE DISTRIBUTION AND SOFTWARE LICENSE TERMS

_The Atari Peripheral Emulator, the Ape Pro System program and image

format,

this document and all other portions of the APE archive not the

property of

third parties are the copyrighted property of Steven J Tucker_.

Distribution of registered versions of this product is a violation

of copyright law. You may use the unregistered version of this

program for a 30 day free trial period. If after 30 days you wish

to continue using APE you MUST purchase it or remove all copies of

the software from your system.

The author makes no representations or warranties with respect to

the contents hereof and specifically disclaims any implied

warranties of merchantability or fitness for any particular

purpose. Further, the author reserves the right to revise this

publication and to make changes from time to time in the content

hereof without obligation of the author to notify any person or

organization of such revision or changes.

PROGRAM DISTRIBUTION

Without my written permission, no version of APE, past or present,

may be included on any CDROM or other software compilation that is

intended for sale, with or without profit to the seller. The

shareware version of APE may not be sold for any amount, except a

media charge not to exceed $5 US, without my written permission.

The registered version may never be distributed in any form, and

any licences received are non-transferable.

Written permission can be obtained only by submitting your request

along with your product content listing, and sale price to:

Steven Tucker

3978 Brookside Blvd

Cleveland, Ohio 44111

The shareware version of APE may continue to be freely distributed

between private parties without charge (including personal web

pages), and by online archives and BBS systems, without charge.

_______________________________________________________________

WHAT IS APE?

Thank you for trying the Atari Peripheral Emulator! You've taken the

first step to making your Atari 8 Bit computer more enjoyable and

useful!

APE is an advanced serial application capable of emulating an entire

subsystem of devices on the Atari 8 Bit SIO bus.

---------------------------

WHAT HARDWARE IS REQUIRED?

APE talks to the SIO bus of your Atari 8 bit home computer using the

Ape Interface Cable (or SIO2PC cable). This serial device provides the

hardware required to convert the voltage levels present on the Atari

SIO bus to levels compatible with the PC serial port.

This device hooks _directly to the SIO port of your atari computer_,

and to the serial port of your PC compatible computer. No interface or

other additional hardware is required!

Plans for building this device yourself are found at the end of this

document. The 2 chip version requires $20-25 of parts, some not

commonly available. The newer _one chip_ version of this device

requires only commonly available parts and can cost between $10-20,

depending again on the cost of parts in your area.

Assembly and testing of this device takes 2-3 hours. If you have no

soldering experience I suggest you pick up a book on electronics and

practice before attempting final construction.

If you do not wish to make the interface yourself, I can provide a

completely assembled and pre-tested serial unit for you. See ORDER.DOC

for details, or ORDERING INFORMATION at the end of this document.

Also, anyone purchasing the hardware from me will receive a _$5

discount_ on the more robust REGISTERED version of this software.

PC Hardware Required

APE is compiled using 808x compatible instructions, and will run on

most any machine from an _XT to a Pentium II_ with a serial port and

at least 200k of free DOS memory. A 286+ class machine with a hard

drive and disk caching software (Norton Speed Drive, MS Smart Drive,

etc) are recommended to get the very best performance. You should have

WRITE BEHIND CACHING turned ON when using APE w/ disk cache software.

_______________________________________________________________

CAN APE BE MULTITASKED?

APE is designed to operate under MSDOS or MSDOS compatible operating

systems that allow _exclusive control of the system by a single task

_. APE requires very precise timing for its serial core functions, and

may fail to function if operated under 'time sharing' operating

systems such as Desqview, Windows 95, etc. Some users however have

reported success under these operating systems with high end pentium

class machines.

_This is not a shortcoming in APE_. The Atari SIO bus was designed to

talk to devices that were always ready to respond, and to respond

within a strict time frame. When you system takes time away from APE

for other applications, the time critical handshaking operations

between APE and your Atari become desynced, and IO operations may

fail.

Even systems like Windows 95, which claim to give 'idle sensitive'

control of the system to DOS applications can still fail. This is

because the OS layer of these applications continues to slice time

away from APE for its own housekeeping functions.

If you wish to try running APE under Windows95, first run APE under a

clean dos boot to see it work _properly._ After seeing how it should

work, you will have better luck evaluating your success under windows.

To boot to a real dos session with Win95 installed, press F8 _BEFORE_

your system boots and select _Command Prompt Only_. When you do try

running APE in a Win95 dos box, set _idle sensitivity_ under

_Properties_ to its _lowest setting_, and _window type_ to Full

Screen.

_APE can also be run under Linux DOSEMU._

_Instructions for running APE under dosemu 0.60:_

1. In the dosemu-60.x/serial directory, edit the file ser_defs.h and

change the following values:

2. The line #RX_READ_FREQ. Change the value shown to 19L.

3. The line #MS_MIN_FREQ. Change the value shown to 19L.

4. This will change how often dosemu updates the serial port status

registers for your dos box. We need FAST updates.

5. In /etc/dosemu.conf, emulate ONLY ONE SERIAL PORT. Comment all the

other ports out. If you enable 2 or more ports things get choppy

as it takes longer to update the serial registers.

6. (Re)compile dosemu and install it, run it, and run APE.

APE should work OK for you. I have /dev/ttyS0 emulated, with

'sillyint' set to off. Setting sillyint to the IRQ of your serial

port might yield some improvement if you still have trouble. The

higher the priority you give 'dos', the WORSE ape runs. If you set

the priority with renice to say '14' then APE runs so good even

APE+ 56kbps 3X ultraspeed works.

_______________________________________________________________

WHAT _SOFTWARE SERVICE_S & _HARDWARE EMULATION_ DOES APE PROVIDE?

_APE emulates the following devices, commands and addressing modes. _

* Up-To 8 Hard Drives & Floppy Drives

* ANY DENSITY: Single, Double, Quad, and 1050 Enhanced Density for

ANY floppy drive.

* Disk images of up to 16 megabytes in size, up to 8 hard drives at

once!

* SpartaDOS ULTRASPEED I/O, or APE+ 3X SIO in any drive & density

combination!

* Full cooperation with REAL sio devices, such as disk drives,

modems, printers, etc.

* PC-MIRROR's new _ApeLoader_ software allows you to load EXE files

directly from your PC Hard Drive!

_Don't waste your time with boot loaders, ATR images, or menu

programs anymore! _

* Special _APE+ 3x SIO mode!_

APE+ Ultraspeed is 1.5x faster than Happy Warp Speed, and 3X

FASTER THAN STANDARD DISK I/O!

_Functions transparently under Spartados, or ANY dos with the APE

Warp+ OS. No patch or loader required!_

* _PC MIRROR drive gives access to files on your PC hard drive &

MSDOS filesystem with any dos and at all SIO speeds, including

APE+ 3X SIO! _

* REGISTERED users may now WRITE files, as well as read them from

the PC mirror, making file transfers to your PC as easy as using

the _Copy_ command of your favorite Atari DOS.

* As of version 1.09, APE can emulate _ALL FORMS OF COPY PROTECTION

_using images created using the AMAZING NEW APE PRO SYSTEM!

APE can emulate even the most complex, heavily protected disks by

companies such as Electronic Arts, Datasoft, Broderbund, etc!

Ape's ProSystem can easily copy disks protected with so-called

'impossible to copy' schemes such as the Electronic Arts

super-track games (Realm of Impossibility, Archon II, etc),

Synapse super-track, etc.

_All using a STANDARD 1050 DISK DRIVE w/ ProSystem cable and

software. No expensive Archiver/Happy hardware required!_

* All your valuable copy protected diskettes can easily be converted

to the new .PRO format using a stock 1050 (or 100% compatible)

disk drive, using the powerful _APE ProSystem_ software!

* FULL Emulation of the Atari 850 interface, allowing you to use

your PC modem and printer _from any atari application_ that can

use the 850 interface.

* _100%_ 850 stage 1 compliant, allowing loading of the Atari R:

handler from any communications program!

* _Hardware flow control for the PC modem using the R: device. _

A feature offered in NO OTHER ATARI 8 BIT SOFTWARE OR HARDWARE

INTERFACE!

_Basic SIO Commands Supported_

* Read Sector

* Write Sector

* Put Sector

* Disk Drive Status

* Controller Format Diskette

* Controller Format Enhanced Density Diskette

* Read Percom Extended drive configuration block

* Write Percom Extended drive configuration block

* Get 'Speed Byte'

* Ultraspeed HS I/O Sector Skew Format Diskette

* Happy HS I/O Skewed Sector Format Diskette

* Print to 850 interface

* Initialize 850 R: Interface and download custom R: handler

* Custom Stage 2 and 3 R: 850 Interface download code

* XF551/Speedy High Speed Read Sector

* XF551/Speedy High Speed Put Sector

* XF551/Speedy High Speed Write Sector

* XF551/Speedy HS Format Diskette

* All Spartados 3.2X flavors of the above XF551 commands

* Archiver Read Track (Beta)

* Archiver Write/Format Track (Beta)

* Happy Warp Speed Read Sector

* Happy Warp Speed Put Sector

* Happy Warp Speed Write Sector

* Happy 'Are You Happy?' Command

* Happy Read Status Command

* Happy Format Diskette

* Happy Init Controller

* Happy Reset Drive

* Happy Set Unhappy Command

* Happy Set Fast Write

* Happy Force Protect

* Happy Force Write

* Download Happy Tracer Data

* Happy Controller Option RAM Load/Unload

* Happy Drive Program Area RAM Load/Unload

* REMOTE.COM download disk name

* REMOTE.COM upload keystrokes

* And many more...

_______________________________________________________________

WHY USE APE?

_Buying or building this interface, and purchasing the REGISTERED

version of APE will be one of the the best investments you can

possibly make in your Atari computer._

APE is an investment in your Atari 8 bit, securing your data and

ability to make use of it for years to come. APE provides

extraordinary hardware emulation at a SMALL FRACTION of the cost of

real hardware devices!

_______________________________________________________________

THE PRICE FACTOR

_For less than the price of a SINGLE 'refurbished' 1050 disk drive

from one of the few remaining atari 8 bit dealers you will receive all

the following:_

* Emulation of 8 disk drives, each of which may hold a disk image of

up to 16 MEGABYTES.

_Thats 128 megabytes of information and programs at your

fingertips at all times!_

* Each emulated disk drive is also capable of emulating the

Spartados US Doubler ULTRASPEED (2x), XF551 Warp Speed I/O (2x),

Happy Warp Speed I/O (2x), APE+ Warp Speed I/O (3x) [Under ANY

DOS!]

This enables your disk access to be more than _three times_ as

fast as a standard 1050 disk drive using Spartados, Happy Warp

Speed Dos, Mydos, and other high speed operating/dos systems and

copy programs or just ANY dos using the Happy Warp Speed OS patch.

New!: As of 1.10, Ape fully supports the Ape Warp+ OS Upgrade! You

can now experience 3x SIO at all times, under any OS, and even in

games! Check out the Ape Warp+ OS InfoPage for information and

benchmarks.

* Using APE is like having a hard drive equal to that of your PC on

your Atari, and for a _SMALL FRACTION_ of what a similar system

would cost for your 8 bit.

* APE _preserves_ the value of your existing PC hardware by making

it available to use on your Atari also.

_Ape does ** NOT ** erase and reuse your PC hard disks, it stores

data in 'disk images'. Disk images are regular PC files (like this

one) that mimic the size and format of a real Atari disk. They may

be as small as 90k, the size of a Single Sided/Single Density 810

disk. And as large 16 megabytes, 65,535 DOUBLE DENSITY SECTORS._

* If you previously used the software 'SIO2PC', your old SIO2PC disk

images are 100% compatible with APE.

APE itself creates images that are backward compatible with the

SIO2PC .ATR format.

* _Transparent access to your PC printer_ from any atari 8 bit

application, without the use of any special atari software. APE

can redirect the P: printer output of all Atari applications and

send it directly to your PC Printer, or even to a file on your PC!

APE also is capable of performing translations of this data on the

fly to match the exact specifications of your PC printer!

* _Direct access to any file on your pc hard drive, from your Atari

using the powerful PC MIRROR function. _

_Transfer files to and from your MSDOS filesystem, including hard

drives, CD-ROM's, Floppy Disks, ZIP Drives and MORE using only the

Atari DOS copy function! No other software can offer this

revolutionary new feature!

You can even download small separate executable (EXE) files from

the major archives and run them right from the PC Mirror, without

even copying them into ATR images! _

* _Transparent access to your PC MODEM_ from most any Atari terminal

program that is compatible with the Atari 850 interface!

APE responds to stage 1 calls for the 850 interface R: (modem)

handler, making it 100% compatible with terminal programs like

_BobTerm _that use the device independent stage 1 loader!.

* APE continues to provide access to your disk drives even when

running in concurrent i/o.

This allows you to do things such as run a BBS, download files,

etc all without having ANY real atari disk drives or modems

attached to your computer! (or wallet :)

Note: _Concurrent I/O _is the special serial bus mode the atari

uses to communicate with the 850 interface, modems and other

streaming serial devices.

* The _registered_ version of APE provides automatic _HARDWARE_ flow

control, allowing you to access modems transmitting data faster

that 19.2k. No other Atari 8 Bit Hardware or software product can

provide this service!

* _Advanced copy protection emulation!_

_APE can emulate even the most complex copy protection schemes,

making the archival of your valuable copy-protected originals

possible._

APE can emulate even the _most complex_ forms of floppy disk

protection, such as '_Phantom Sector_' and '_Fuzzy Sector_'

schemes.

APEs _.PRO format_ has been able to archive even the most

notorious of the late issue titles, such as:_ Alternate Reality_,

_All Electronic Arts titles_, _Lucasfilm titles_, _First Star

titles _(Spy vs Spy), _Broderbund_ titles, and others.

The APE ProSystem is included in this distribution. It allows you

to create .PRO images of even the most complex copy protected

disks and use them with APE.

All _PHANTOM EMULATION_ modes are not available to _unregistered_

users. Modes 1,3,4,5,6,7,8, and 9 are reserved for users who

support the future development of APE by _REGISTERING_.

* _Reliability_. You will never fear your data being lost to a bad

floppy disk. Your files and data will rest in file on your PC hard

drive, and can be backed up to a more reliable storage medium such

as CD-ROM, tape streamer, etc.

We all know how difficult it can be to find good, working

replacement hardware for our atari computers at reasonable prices.

APE is designed to _eliminate_ these worries, providing you with a_

flexible software system_ that will continue to grow and become

more useful over time as the software is improved and upgraded.

_

And MUCH MORE! Read on to discover all the powerful features of APE.

____________________________________________________________

ABOUT SHAREWARE

The user supported software concept (also known as "Shareware") is

an attempt to provide quality software at low cost.

Both the user and author benefit from this plan. The user will

benefit by being able to "_test drive_" software thoroughly before

purchasing it. The author benefits by being able to enter the

commercial software market while avoiding the high cost of

commercial distribution.

This concept helps many independent authors and small companies

that otherwise would be discouraged from developing and promoting

their ideas. It can only work with your support. If you obtain a

user supported program from a friend and are still using it after a

few weeks, then it is obviously worth something to you, and a

registration fee should be paid. _

This software is distributed under the user supported software

concept. Though APE is copyrighted, you are encouraged to copy and

distribute the ** UNREGISTERED ** package to others. You may only

distribute the unregistered software package in its original

unaltered ZIP form.

Please do not steal APE, it is the product of THOUSANDS of hours of

coding and debugging. It also represents something quite rare at

this time, new software development for the Atari 8.

IF YOU USE APE LONGER THAN 30 DAYS YOU *MUST* REGISTER IT_

_______________________________________________________________

UNREGISTERED RESTRICTIONS AND REGISTRATION BENEFITS

_Unregistered_ evaluation users have ALL of APEs powerful features at

their disposal except the follow, which are available to _REGISTERED_

users only.

1. _UNREGISTERED_ users may not bypass the startup screen. Registered

users may bypass this screen by setting a configuration switch.

2. _UNREGISTERED_ users are allowed _complete working access_ to the

powerful _APELOADER_ software. UNREGISTERED users will see a

registration reminder, and a small delay before loading an EXE

file from the menu.

3. _UNREGISTERED_ users are allowed _*FULL ACCESS*_ to read files in

the _PC MIRROR_ drive. Only _registered_ users may WRITE files to

the _PC MIRROR_.

4. _UNREGISTERED_ users are allowed _FULL ACCESS_ to APE's advanced

_Pro System_, and can create .PRO images of even the most complex

copy protected diskettes. Only _REGISTERED_ users receive full

access to all _PHANTOM EMULATION_ modes in APE.

5. _UNREGISTERED_ users may not use sticky sessions.

6. _UNREGISTERED_ users will see a short delay when using the

APETIME.COM time downloader for the first time each session.

7. _UNREGISTERED_ users will see a short delay when mounting images

located with the [F]ind Image function.

8. Registered users are granted _full access_ to APE's powerful

ATR-FAST system. Once registered you will be able to see the

contents of an ATR image _as you browse them_ in the file

selector! A terrific time saving feature!

9. Only 1 F-Key macro. _REGISTERED_ users may define 10 custom F-Key

macros that allow any series of keystrokes to be entered

_automatically_.

10. Block Configurations. _REGISTERED_ users may create special _Block

Configurations_ that allow up to 8 disk images to be loaded with a

_single keystroke_. See later in this document for more

information on block configurations.

11. All of APE's enhanced SIO read modes are available to

_unregistered_ users. You may read your diskettes at any speed,

including the enhanced _3X APE+ SIO_ mode. Only _registered_ users

may perform disk WRITES at the following speeds: Happy 2x Warp,

APE+ 3x Warp, APE+ 3x Sparta.

_Unregistered_ users may still read AND write at the US DOUBLER

Spartados warp speed, and normal SIO speeds. The disk image

_453_WARP.ATR_ is included as a demonstration of Happy 2x Warp SIO

in MyDOS, and APE+ 3x SIO in MyDos. No special software or loaders

are required to use these enhanced speeds under Spartados.

12. _Unregistered_ users _ARE_ permitted to use APE's POWERFUL R:

_Atari Modem emulator_, but only at speeds up to 2400 baud.

_Registered_ users receive access to the concurrent i/o R:

emulator at 38.4K and above! Speeds of up to _56k bps_ are

possible using the enhanced _registered_ version of APE.

13. _Registered_ Users may start the ATASCII/ASCII/UNIX text browser

from the command line. _Unregistered_ users still have full access

to this powerful feature, but must start it from within APE.

14. _Unregistered_ users are limited to 2 command line switches. There

are no limitations (except as imposed by the DOS operating system)

on _registered_ users.

15. APE's startup screen may be bypassed by _registered_ users, and

there are no nag screens or timed waits.

16. Unlimited usage. You may use the _unregistered_ version of this

software for _only a 30 day trial period_. If you wish to continue

using APE after that time you _must_ pay for it. The program will

not eat itself or stop working after 30 days, I'm depending on

your honesty as a fellow atari 8 bit computer user to do your

part.

17. Limited tech support. _Registered_ users will receive priority

with regard to technical support issues, custom versions, and bug

fixes.

18. _Registered_ users will receive upgrades to the next revision of

the program (os specific) free or at a substantial discount. (so

far all upgrades have been free)

19. Future registered versions will include:

+ The ability to save browsed text in ibm, unix, or atascii

format.

+ Direct manipulation of files inside ATR images

+ Additional _PHANTOM EMULATION_ modes, as required.

+ _User Suggestions._ Over 50% of the new features in APE since

Beta 4 are the direct result of suggestions submitted by _APE

users._ Your comments, complaints, and suggestions are _very_

important to me. Please feel free to contact me for even the

smallest suggestion!

_______________________________________________________________

INSTALLING AND STARTING APE

The _SHAREWARE_ APE package is distributed as a ZIP'ed collection

under the filename APExxxD.ZIP Where 'xxx' represents the version

number of that archive. The archive contains the following files:

APExxx.EXE The Main APE Executable

PROxxx.EXE The FULL OPERATIONAL APE Pro System!

SIGNATR.EXE Tool to sign & authenticate ATR Archives

ORDER.DOC APE Registration and Order Form

APExxx.DOC The APE Users Manual in ASCII Form

PROxxx.DOC The ProSystem Users Manual

APE_F32G.ATR APE version of the Spartados 3.2g distribution

w/ APETIME.COM and REMOTE.COM

APE_WARP.ATR APE version of the Mydos 4.53 distribution

w/ REMOTE.COM, APE+ OS Patch and High Speed Sector Copier

*.PRO Any Example .PRO Protected Disk Images

*.CAS Any Example .CAS Digital Cassette Images

To install APE, change to the directory on your hard disk where you

wish to place APE and unzip the ZIP file by typing '_PKUNZIP

APExxxD.ZIP_'

_HINT_: The first thing you should do when you start APE is to go to

the _[Z]_ Configuration Menu and choose option _[D]_ to configure your

Com Ports.

This will allow APE to function if your computer uses a non standard

IRQ/BASE address for its comm ports.

_______________________________________________________________

Starting Ape

Once you have successfully installed APE, its time to start and

configure it for your hardware.

Typing 'APE /?' will give you a listing of command line options:

_APE Command Line Options_

Specifying any of these options on the command line will override any

settings you might store in your startup _APE.CFG_ configuration file.

These are all shortcuts for commands available in the main &

configuration menus. Their complete functions are covered in detail in

those sections.

_After processing of any command line options you may have specified

the emulator will start. You will then be presented with the status

screen._

Use of the '/?' switch will immediately return you to DOS without

starting the emulator.

_______________________________________________________________

The Status Screen

APE Status Screen

This is the main status screen you will see during all disk operations

with the Atari. All commands are carried out using the PC keyboard.

The status screen displays a wealth of information about the state of

your emulated peripheral system. The screen is as you would see it

with various disk images loaded, and the emulator _is ready_ to

accepts commands from you or the Atari.

The status line for bay 1 shows you have loaded 'DUNGEON1.PRO' and the

following information:

* _Sio Command_ - This field will display the last SIO COMMAND

issued by the Atari to this drive.

* _Sector_ - The sector number that was read

* _Total_ - The total number of sectors in this image.

In this case the image is that of a PROTECTED disk that contains

more sectors than a normal 720 sector diskette.

* _'P' - PRO System Emulation Mode_. '-' Indicates this image is

_NOT A PRO IMAGE_. Any other symbol indicates the _PRO SYSTEM_

copy protection emulation mode shown is currently active for that

image.

* _Dns_ - Represents the number of bytes per sector in this image,

also called its _Density_. Single density sectors contain 128

bytes, Double density sectors contain 256 bytes.

* _Protect_ - Indicates if APE will or will not protect your disk

image from writes by the atari. When you 'write protect' and disk

image, _no_ harmful action can be taken on it by the atari

* _Image Fname_: The real filename of the image as it resides on the

PC hard drive.

* '_Ultraspeed: APE+_' shows that the last command received on the

serial bus was answered using the incredible _APE+ 3X SIO!_

* '_Emulation: SMART_' shows that the emulator is currently handling

disk images in its 'SMART' mode. More information on this feature

is given in the KEYSTROKE COMMANDS section of this manual.

* Any drive that is currently tied to the _PC mirror _function will

have its drive number in inverse/red text as with drive 8.

* Any disk images currently running in the _Pro System Emulation_

mode (i.e. a .PRO file was loaded) will have the corresponding

drive number _highlighted in inverse purple_. Disk 1 above is a

copy protected disk image. The number listed indicates the current

_Pro System Copy Protection_ emulation mode.

_______________________________________________________________

Keystroke Commands

Pressing _'H' _from the main menu will display the following menu:

Ape Help Screen

Help Screen

* _[F1 - F10]_ - Execute Quick Macro. These keys will execute the

quick macro assigned, just as if the command had been typed at the

PC keyboard.

* _! _- Spawn your default shell. This command will launch a _DOS

shell _using the program specified in your _COMSPEC_ dos variable.

If _COMSPEC_ is not set, it will attempt to spawn_ C:\COMMAND.COM_

or _C:\DOS\COMMAND.COM_. You can set APE to swap itself to EMS or

DISK using the configuration menu [Z]. By default APE will try EMS

if it is available.

* _@_ - You can _force the program into its concurrent i/o emulation

mode_ with this command. This is only useful if while in your

terminal program you are forced to abort concurrent i/o on the PC

side to mount a new disk image, etc. More on modems, RS232

emulation and concurrent i/o later in this document.

* _1-8_ - You can use these keys to _swap disk images_ around in

their bays. Lets say for example you have _ENCHANT1.ATR_ loaded in

bay 1, and _ENCHANT2.ATR_ loaded in bay 2. After you load side one

of Enchanter (which is stored in _ENCHANT1.ATR_) it tells you to

please 'turn the disk over'. You can move_ ENCHANT2.ATR_ into bay

1 _by simply pressing '12' on your keyboard._

* _A_ - _DCM & XFD -> ATR Conversion_. Before you can use _Xformer

(.XFD) and Diskcomm3 (.DCM)_ images with APE, they need to be

converted from their format into a better ATR '_disk image_' thats

available for random access, and has a descriptive header.

When you select [A], you will be prompted for a filename or

filemask for the image to convert. If your specify no filename, a_

file selector will appear_ so you can pick a single XFD/DCM image

to convert.

If you sucessfully convert an image, you will then be asked if you

would like to mount the new ATR image automatically!

If you specify _wildcards_ (i.e. _C:\TEL\DOWN\*.DCM_) _all _the

files matching your filespec will be coverted in automatic _BATCH

mode._

* _B_ : _Block Configurations Menu_. Block configurations are files

that store the current state of all 8 bays of the disk emulator.

This can be a _TERRIFIC time saver_ if you often need to load

specific sets of disk images, for games, utilities, etc. With just

a single command you can mount 8 disk images and be ready to go!

To create your own block configuration file, use the 'L' command

to load your disk images into the drive configuration you want to

save. Then select the 'B' option. You will be asked if you want to

_Load or Save_ a block configuration. Press 'S' for save. You can

then choose a name for your block configuration, after which it

will be saved to disk for future use.

_Loading your newly created block configuration is now a breeze!

_Just press 'B' then 'L' for load, choose your configuration from

the menu and _all the images you specified previously will be

automatically mounted!_

* _C_ - _Create ATR Image_. This is possibly one of the most complex

(and often confusing to the newcomer) part of using emulated

disks.

As previously explained, _your new atari disks are not real floppy

disks at all_, but MSDOS files that match the storage capacity of

the _real physical media._

When you press 'C' to create a disk image, you will first be

prompted to choose the _'disk size'_ for your image.

_A number of pre-defined sizes are provided for your convenience.

_These sizes are _exact_ matches for real physical disks/drives

that are commonly used on the Atari 8 bit.

Even though you may want small disk images for copying your real

physical disks to (_for a perfect size match_), you can create

disks as large as _16 *MEGABYTES* _each!

Using advanced DOS systems like _MyDOS_ and _SpartaDOS_ you can

format and utilize disks up to the Atari addressing limit of 16

megabytes (_65,535 double density sectors_).

If you are using a early dos, such as atari dos 2, 2.5, or other

non-enhanced dos you should stick with the smaller sizes until you

are more familiar with the creation of images.

Once you have chosen a size for your disk image (_choose #1 if

this is your first time_) we have to pick out a_ name_ for your

'disk image' file on your PC.

I recommend you use a filename that describes the contents of the

disk image well, such as '_MYDOS.ATR_' for your MYDOS master disk,

etc.

If you wish to overwrite an existing disk image, you can simply

press RETURN at the filename prompt and_ a file selector box will

appear_. You may use this file selector to quickly pick the ATR

you want to overwrite.

_CAUTION: IF YOU OVERWRITE A DISK IMAGE USING THIS FUNCTION IT CAN

NEVER BE RECOVERED, USE EXTREME CAUTION WHEN ERASING OLD DISK

IMAGES! IF IN DOUBT CHOOSE A UNIQUE NAME FOR YOUR DISK._

Once you have selected a filename you will be prompted one last

time for final confirmation. You will also be shown various

information about the image that will be created, such as real

msdos file size, if the file exists, and its full pathname.

Pressing 'Yes' when prompted will begin the creation of the disk

image.

This is _very_ dependent on speed of your machine, its disk

caching, and your hard drive throughput. _A bar graph will display

the progress of the creation._

Small disk images are created very quickly, where as very very

large disk images such as 16 megabytes can take a few minutes

(_though this is rare, it takes 15 seconds on my 486/100_).

Installing a disk cache with '_write caching_' activated will

greatly increase the speed of this function, and also improve the

performance of the emulator greatly.

Once successfully created you will be asked if you wish to mount

the image you just created. A great time saver over having to

manually load the image after creation.

FORMATTING YOUR DISK IMAGE

Note also that a newly created 'disk image' is the same as a blank

diskette you would bring home from the supply shop. _Its totally

blank and must be 'formatted' using your favorite atari DOS before

it can be used._

_FORMATTING WITH ATARI DOS 2.x_

If you disk is a standard size, such as 720 sectors & single

density it can be formatted with any standard atari DOS. If you

are using Atari DOS 2.0, you may only use disks of 720

sectors/single density. Atari DOS 2.0 is rather stupid, and

refuses to believe that any other size disk might exist.

If you are using dos 2.5, you may create images of 1040 single

density sectors (called _ENHANCED DENSITY_) You may then format

them with Dos 2.5's '_I_' command.

To make use of _VERY large_ disks (virtual atari hard drives you

might say :) you will need an advanced DOS system such as

_SpartaDOS _or _MyDOS_. _I recommend if you use your atari more

than very infrequently that you get one of the DOSes_. They are

not only capable of using standard sizes (720, 1040, etc sectors),

they can manage images of _ANY_ size up to 16 megabytes.

Both of these fine products are _SHAREWARE,_ and can be downloaded

from gopher://gopherblue.umich.edu in the Software

Archives/Atari/8Bit/Os directory. If you use them please register

them. MyDos 4.53 is included in this APE distribution on the disk

image Warp_453.Atr

Lets assume you created the grand daddy of disk images, the 16 meg

monster :) Formatting procedures for both DOS systems are as

follows:

_FORMATTING WITH MYDOS 4.5x_

Boot MYDOS and mount your _mega disk image_ in bay 2. APE will

show the image as having 65535 DD sectors.

Choose option '_O_' from the _Mydos_ main menu, configure drive.

We need to tell MYDOS that the disk in 'drive' one is bigger than

your average disk (heh).

_Steps for getting MyDos ready for a LARGE disk image_

+ Enter 'O' at the main menu for Configure Drive.

+ Enter '2' for the drive number.

+ Enter 'Y' for 'is drive configurable'

+ Enter 'Y' for 'is drive a HIGH CAPACITY drive'

+ Enter '65535' for number of sectors

You should now be back at the main menu. Notice at the top of the

screen that it says '_2H_'. Mydos now understands drive 2 is a

_high capacity_ drive.

We now need to FORMAT the 'disk' to make it ready for DOS use.

Press '_I_' at the MYDOS main menu and format the disk (do _not_

use the ENHANCED density option).

To verify the disk was formatted Ok press '2' at the MYDOS main

menu for a listing of files on drive 2. It should read _well over

65,000 free sectors_.

Now choose option 'H' and write the MYDOS disk operating system to

your image. _Once completed you are ready to boot off your virtual

hard drive! Yeah!_

_FORMATTING WITH SPARTADOS 3.2g_

Boot SpartaDOS and mount your _mega disk_ image in bay 3. APE will

show the image as having 65535 DD sectors.

_Spartados_ is a modular DOS system, its utility functions stored

in separate executable files on the main system disk. To

initialize a disk image this large, you will require the Spartados

utility '_HDINIT_' (_Hard Disk Init_)

_Run HDINIT_. It will ask you which hard disk you want to

initialize, 3 or 4. The program apparently only thinks disks this

big would be on those drive numbers, so for now its important your

image be loaded in bay 3. (or 4)

It will ask you if you're sure, and once confirmed will proceed to

format and write the SpartaDOS FAT to your disk image.

Now that the basic format of the disk has been established, you

need to copy a dos file to the image so it can be booted directly.

Copy your favorite Spartados DOS file (the *.DOS file, like

X32G.DOS) from an existing disk (or disk image) to your new image.

Type 'BOOT X32G.DOS' where X32G.DOS is replaced by the name of the

DOS file you just copied onto the disk. Spartados will now

configure the boot sectors of your hard disk image to self boot

using the new DOS file!

* _F_ - _Find Image_.

_Find Image Screen_

Find Image Screen

The find image command will allow you to search any set of drives

specified, including CDROM drives, floppy drives, etc.

To specify which drives to search, use command [D]rives, and edit

the drive listing.

To speficy which files to search for, use the [F]ilespec command,

and edit the search filespec.

To start a search, just press [F]ind, and APE will start a quick

search for up to 10 images that match your request.

_Find Search Results_

Find Results

The screen above displays the results of a search for MyDOS

related images, using the search filespec "MYD*.*" across drives

A-G.

As shown above, oncce you have the results of your search, loading

the images into APE is easy!

Just press [F1 - F10] to select the image you want to mount, and

press [1 - 8] to select the APE bay you want to mount the image

in!

You can repeat the search / mount process as many times as you

need to.

While in the Find section, you can also perform some basic APE

utility commands, such as [S]wap drives, and [R]elease Image.

* _H_ - _Help_. Displays the quick keystroke reference chart.

* _L_ - _Load Image_.

Ape 1.12 Enhanced File Selector

Ape 1.12 Enhanced File Selector

This function is used to select an already created ATR image file

and mount it in the drive '_bay_' of your choice.

You will first be prompted to select the bay # for the image your

going to load. _If an image in already using that bay you will be

prompted to confirm before displacing it._

Once you have selected the drive bay, you will then be asked for

the filename of your disk image. If you know the filename and want

to type it in you may, or you can just _press RETURN for an easy

to use file selector with which you may browse your ATR images._

After you select the name of your image, APE will take care of

everything else such as reading the image configuration, etc, and

mount it for you in your selected bay.

If your image contains an Authentication Signature, it will be

evaluated and if the CRC does not match you will be notified.

PROTECTED DISK SOFTWARE

_APE can now emulate ALL copy protection, even the most complex

'Phantom' sector schemes._

Using a special program to read your protected disks, APE can

create _.PRO disk images_. _This is a new form of disk image,

incorporating timing, error and other information critical to

emulating the copy protection present on the orignal diskette._

APE can currently emulate defective sectors and the resultant

contents of the FD1771 status register when reading them._ APE can

also emulate protection schemes that do not produce controller

errors, such as the Phantom Sector protection used on Electronic

Arts titles._

APE has been tested with the following software originals and has

performed _perfectly_.

These PRO images are not '_cracked_' copies of games, but contain

information representing the _real copy protection_ on the

original diskette:

+ Alternate Reality, The City (Phantom Sector Protection)

+ Alternate Reality, The Dungeon (Fuzzy Sector Protection)

+ Realm Of Impossibility (EA 'Super Track' Protection)

+ Archon (Phantom Sector Protection)

+ Archon II: Adept (EA 'Super Track' Protection)

+ 7 Cities Of Gold (Phantom Sector Protection)

+ Agent USA (Standard Protection)

+ Spy vs Spy: Artic Antics (Phantom Sector Protection)

+ Ultima ][ (Standard Protection)

+ Jumpman (Phantom Sector Protection)

+ Koronis Rift (Phantom Sector Protection)

+ Apple Bandits (Standard Protection)

+ The Edilon (Phantom Sector Protection)

+ Spy vs Spy (Standard Protection)

+ Gumball (Standard Protection)

+ Karateka (Phantom Sector Protection)

As other images are reported to work by registered users, they

will be added to this list. _Based on the complexity of the copy

protection used by the above companies, I am confident that APE

can emulate 100% of existing software._

* _P_ - _Change Pro Emulation Mode_. Changes the current ProSystem

copy protection emulation mode for any currently loaded .PRO

image.

This function is available to _REGISTERED_ users only. You can now

change ProSystem modes easily from within APE to allow you to find

the best mode for your new .PRO images as quick as possible!

* _Q_ - _Quit program_. Unmounts all images/drives and exits to your

operating system.

* _R_ - _Release Image_. This function will unmount your ATR image

from the bay (drive) you select and make that slot then available

for other disk images.

* _S_ - _Set the PC MIRROR drive_.

_The PC MIRROR is by far one of the most powerful features

available to you in APE. _When you mount at drive as a 'PC MIRROR'

that drive acts like a window to the files that actually reside on

the MSDOS portion of your PC hard drive.

Where before it was very difficult to get files from your PC to

your Atari, using null modem cables, modems, terminal software,

etc; _You can now access these files DIRECTLY!!_

The new _APELOADER_ Software of the PC-MIRROR provides the

_ULTIMATE_ bootloader, by giving you access to all your EXE files

directly from the Atari keyboard, using a simple and functional

menu interface.

_You can access the APELOADER at any time by simply booting

directly from your PC-MIRROR drive!_

Lets try an example of using some PC-MIRROR services!

ApeLoader Services

_________________________________________________

_________________________________________________

To boot the APELOADER, set the PC-MIRROR drive to drive #1, then boot

your Atari from the PC-MIRROR as you would normally boot, depending on

if you want BASIC, etc.

_The ApeLoader will work with or without BASIC, on any machine type._

__________________________

ApeLoader KeyStroke Commands

(all commands are used directly from the Atari keyboard)

__________________________

+ _A-Z and 0-9_: These keys select files or directories,

depending on what is displayed on the screen.

If you select a directory, the ApeLoader will change the

directory on the PC, and automatically display the new

listing of files!

If you select an Atari executable file (EXE, COM, etc), APE

will verify it to be a valid Atari binary file, and load it

automatically!

The memory footprint of the ApeLoader in Atari memory is

_less than 128 bytes_! This makes the ApeLoader the _most

powerful and flexable binary boot loader EVER MADE_ for the

Atari. It can load _ANY_ file that does not REQUIRE dos.

Even if the binary file overwrites the ApeLoader, _Ape can

STILL LOAD THE FILE_, becuase the ApeLoader can be relocated

in memory!

The ApeLoader is also intellegent, and will try to start

binary files even if they have had extra data appended to

them via old style XMODEM transfers, etc!

+ _[Up Arrow]/[Down Arrow]_: Press these WITHOUT shift (easier

that way :-). These keys cause the menu to scroll to the

next/previous page of files. The ApeLoader can display up to

3000 (yes, thats really _three THOUSAND_) files per

directory.

+ _[Shift + Letter]_: Pressing SHIFT + any keyboard LETTER will

cause the ApeLoader to instantly ZOOM to the first file that

starts with that letter! This is a great timesaver, and even

if you have 1000 EXE files in a single directory, you can

file any one with just 2-3 simple keystrokes!

+ _[Control + Letter]_: Pressing CONTROL + any keyboard LETTER

fill cause the ApeLoader to change your PC DIRECTORY to the

drive letter you selected. e.g. press CONTROL+C for drive C:,

or CONTROL+E for drive E:.

+ _[TAB]_: This option will change the BASE ADDRESS of the

ApeLoader boot program.

The BASE ADDRESS is the area of memory inside the Atari

Computer used to hold the ApeLoader Boot Code.

If your game/program tries to overwrite the ApeLoader while

loading, just change the base address using this option and

you can still load your program!

Ape can relocate its loader to page 4 ($400) or to the high

stack ($110).

_________________________________________________

_PC-MIRROR File Services_

_________________________________________________

Let's try the PC-MIRROR's powerful file services!

Boot MYDOS 4.5x and once at the main menu choose APE option 'S'

and mount your PC mirror drive in bay 2.

Now, press '2' on the atari keyboard, you will get an atari

directory of the files in the root directory of your PC hard

drive! _APE takes the information from your PC files and makes

your atari THINK they are on a real atari disk!_

Lets try and copy one of your _MSDOS_ files to the atari. For this

example we will use _AUTOEXEC.BAT_ since most anyone with MSDOS

will have this file.

Choose MYDOS option 'C' (_Copy file_) and then enter

'2:AUTOEXEC.BAT,1'. This will tell MYDOS to copy file

'AUTOEXEC.BAT' off drive 2, and put it on drive 1.

_Its just that easy! The entire contents of your PC hard drive are

now at your fingertips!_

_Note: Files on the pc mirror are sequential access only! If you

wish random access to a file on the PC MIRROR for editing, etc,

simply copy it to an ATR drive._

NAVIGATING PC DIRECTORIES IN THE PC MIRROR

Getting a directory listing of the PC MIRROR directory of your PC

will show not only the files there, but also the _subdirectories_

on your hard drive.

One of my main goals when creating APE was to make it _DOS

independent_.

_SpartaDOS, MYDOS, etc all use separate methods for storing

information in subdirectories, but APE will work with them all._

_To change to a subdirectory on your PC MIRROR drive, all you have

to do is to make the atari access that 'file' somehow. Its just

that easy._

If you using_ MYDOS_, use the _'L'_ command and 'load' the

directory. You will receive an error, which is _*NORMAL* and

expected._ _APE is telling MYDOS that it cant load that file and

mydos goes back to its main menu. _Now press '2' once again. _WOW!_

APE has changed directories and your now looking at the

subdirectory you selected on your PC hard disk.

If your using _SPARTADOS_ all you need to do is type the name of

the directory _(you may need to add a '.' to the end of the name

if there is not extension, i.e. 'DOS.')_ and _APE will do the

rest_. After you receive the error message, type 'DIR' once again

and you will see APE has changed directories for you! :)

Also note that the subdirectory you are in contains a directory

called _'UP'_. _'L'oading or otherwise accessing this directory

will cause the PC MIRROR to go UP the MSDOS directory tree to the

parent directory. This is equivilant to the MSDOS command 'CD ..' _

WRITING FILES TO THE PC MIRROR

_Registered _users may now _WRITE_ files directly to the PC mirror

using MyDOS. To transfer a file to the current PC MIRROR directory

from your Atari all you need to do is use the MyDOS (C)opy command

and copy your file to the PC MIRROR drive.

_NO SPECIAL SOFTWARE OR DRIVER is required for this function! APE

takes care of EVERYTHING for you, once the copy operation is

complete, a file of the same name and size will exist on your PC

hard drive! :)_

This advanced feature will make file transfer problems between

your PC and atari a thing of the past!

* _T - Toggle Emulation. _This command switches APE between its

_'SMART'_ mode of emulation, and its _'STUPID'_ mode.

This toggle is useful for booting some programs that refuse to run

on _'enhanced' _drives. This switch may be obsolete since the

addition of full support for Happy Warp Speed I/O. It was really

only useful with SUPERDOS which now works perfectly in Happy Warp

Speed Mode.

SMART MODE

APE operates at full tilt, all _heuristics are enabled_ and APE

will respond to all special and enhanced forms of disk

communication it understands.

STUPID MODE

All Ultraspeed SIO _heuristics are disabled _and APE will refuse

any request to switch into an enhanced drive emulation mode. _APE

is most like a stock 1050 disk drive when used in this mode._

* _U - Ultraspeed Toggle._ This command will _FORCE _APE to switch

the method it uses to talk on the SIO bus. Normally APE uses

advanced heuristic methods to _automatically_ determine what speed

devices on the SIO bus are talking at, but if you encounter a

problem with SpartaDOS desyncing you can use this command to force

the switch.

This command is left over from the early beta test versions of the

program, and since I installed the new syncro code and switching

logic I've never had to use it again.

* _V - View Atascii/Ascii/Unix Text File. _You can browse any text

file with this command, _even native ATASCII text!_

Its is a completely integrated version of my Multitype file

browser. Once you select the file to view, APE will determine what

type of text is it automatically (atascii, etc). If APE fails to

correctly display your text you can toggle the format manually

with the _[F] _command inside the browser.

You may browse _multiple files_ once the browser has been started

by selecting the _[N]_ew filespec option. _Press ESC to exit the

browser._

_Registered users _may start the browser program from the command

line, as a stand alone program within APE.

* _W - Write Protect Image. _This function marks your ATR image as

write protected, and affords it the protection offered by real

physical write protection on floppies. Once write protected, any

writes to the image will be totally ignored. _The Atari cannot

damage or write to the image in any way._

Write protect is a toggle, and your images may be _unprotected_ by

using 'W' on them again.

NOTE: This does not protect your images from being damaged by the

MSDOS operating system, only by APE. As always, keep appropriate

backups of your data.

* _Y - Playback Cassette. _

APE now includes full support for _Ernest Schreurs_ excellent

digital cassette format. To obtain more information, and the

program used to create .CAS files, you currently need to contact

Ernest, as there is no public FTP site yet.

This feature is unrestricted and free for all users. An example

.CAS cassette file is included with APE.

Playing files is a simple process, simply enter the filename (or

use the lightbar selector to locate it), and APE will prompt you

to press return to start.

Boot your Atari the normal way you would load the cassette, and

press return at APE and the Atari at the same time. The cassette

will then playback.

* _Z - Configuration Menu. _

This is used to setup APE for proper operation with your system, and

to setup your quick Function Key macros.

_______________________________________________________________

General Configuration Menu

This screen controls the operation of most serial core functions, the

hardware flow control for concurrent i/o, the sio command window, the

verbose command monitor, sio command monitor filters, and registered

user keystroke definitions.

* _1 - Sio2PC device type_. This tells APE what kind of serial

conversion cable you are using. Older versions of the cable use a

2 IC setup which requires manual control over the serial bus

tri-state.

_Its very important you set this correctly._ If you have a 2 chip

device and you set it to 1 chip, it will fail to function

correctly or will cause interference with other, real SIO

peripherals.

* _2 - APE device port. _This is the Com port to which your serial

conversion device is attached. Com Port _IRQ/Base Address_ may be

set under configuration menu Z/C.

* _3 - Framing Threshold._ This is used by the heuristic ultraspeed

switching routines to decide how quickly to change modes when new

speeds are detected on the SIO bus. Normally you will not change

this setting.

If APE switches to ultraspeed when it should not, you may have the

framing threshold set _too low_. A setting of below 8 may cause

this.

If APE fails to switch into ultraspeed when using mixed speeds,

and you get command timeouts or drive errors, you may have the

framing threshold set _too high_.

This will depend _greatly_ on the speed of your machine. I

recommend a setting of 8 for most machines, but if this causes

problems you will need to experiment with this setting. Framing

thresholds of below 4 or above 15 are of no use and are not

allowed.

* _4 - Extra Safety Margin_. This setting should not be changed

unless you are having serious timing problems due to your machine

being _too fast_. Setting to other than '1' on machines which run

APE fine will result in a decrease in performance.

* _5 - Serial Control_. When building the standard APE Interface,

you will use the RI line on the serial port. Some people have

reported better performance under windows using the DSR line and

requested an option to support it, so here it is. If you set this

to DSR with a standard APE (RI) cable the system will not work.

CTS is also available at the request of MAC users.

* _% - Command filters_. Command filters are used to eliminate the

display of commands you dont wish to see from the scrolling SIO

command monitor. I.E. you may wish to filter out the read sector

command _[$52] _to be able to see the more unusual SIO commands

only.

All other switches in this field control the HEX value of the

filters you wish to use.

* _F - SIO Command monitor._ This switch controls the scrolling SIO

Command Monitor at the _bottom_ of the Status screen. _If your

machine is slow, turning this OFF will help increase the speed of

the emulator._

* _G - Verbose command monitor._ This switch controls the display of

the command names/sector numbers for each SIO command received.

_Turning it off can help speed up the emulator on very slow

machines._

* _H - File Selector._ As of APE 1.12 a new, better file selector is

available. It supports a number of new features: Atr-Fast Window

+ _Full Support for Windows 95 Long Filenames _(Only available

in the Windows 95 Dos Box, see notes on multitasking)

+ _Warp-to-Key._ Pressing the first letter of a file will warp

to it, and the next with the same letter if the key is

pressed again.

+ _ATR-Fast_. Pressing SHIFT will display a window with the

root directory of your Spartados/Atari DOS image. ATASCII is

supported.

+ _Fast drive switch_. Pressing Ctrl-Drive to quickly change

drives. i.e. Ctrl-D for drive D.

+ Extended use of Page Up/Down and Home/End keys.

You can change to the old file selector if you dont like the new

one using this option.

* _I - ATR-FAST Mode_. When using the old file selector, you can

control the method used to make the ATR-Fast window appear

(registered users only). Options are:

1. Never - The ATR-FAST window is disabled.

2. Shift-Driven - Pressing any SHIFT key brings up the window.

3. Always - The ATR-FAST window is always present.

Because the new file selector needs the whole screen for Win95

support, the ATR-FAST mode is always Shift-Driven when using the

new selector, regardless of this setting.

* _J - Swap out on Shell_. When spawning your default DOS shell with

the '_!_' command, APE can swap itself out to EMS or DISK. You can

set the default swap method here. If you don't want APE to ever

swap out, select 'NEVER'.

* _M - Happy/Syncromesh Detect Enable_. With some special

applications, like some games and Spartados X, you might want to

disable the detection of the Happy Enhancement and Syncromesh. You

can do so by selecting 'RESTRICT'.

* _N - ApeLoader Base Address_. With this option you can set the

_default_ base address for the ApeLoader Boot Code. This option

can be changed on the fly from the ApeLoader menu of course. See

the PC-MIRROR section for more details on the Apeloader.

* _O - Hard Write Protect_. As of 1.15, APE can use 2 different

methods of emulating diskette write protect.

+ Old (_SOFT_) Method: No error is reported to the Atari when

it tries to write to a protected ATR, yet the ATR remains

unchanged.

+ New (_HARD_) Method: Atari receives en error when trying to

write to a write protected ATR image. This mode operates

exactly as if a real write protected diskette were in a real

disk drive.

* _P - Persistent Sessions_. Persistent sessions allow registered

users to keep their current APE setup intact between sessions.

For example, with persistent sessions turned ON:

+ You have 'C:\MYDISK.ATR' loaded in drive 1, and the PC-MIRROR

set in drive 2.

+ You press 'Q' and quit APE.

+ You restart APE sometime later.

+ Your 'C:\MYDISK.ATR' is still loaded in drive 1, and the

PC-MIRROR is still in drive 2 at the same path.

With Persistent Sessions turned off, APE will start each session

with 8 empty drive bays.

This feature is not available to unregistered users.

* _Q - Bypass Startup Screen_. This switch allows registered users

to bypass the startup copyright and information screen.

This feature is not available to unregistered users.

* _T - FAT32 Long Filenames_. If you experience any problems with

APE attempting to obtain long filenames, such are garbage in the

file selector, you can manually disable the use of long filenames

using this setting.

* _U - Allow Large PC-MIRROR_. In APE 1.16, the PC-MIRROR has been

expanded to allow files as large as 7 megabytes to be written

directly to it. This change requires the PC-MIRROR to appear to

the Atari to be a large hard disk formatted w/ Atari DOS.

Spartados X for example, is not capable of reading from Atari DOS

formatted hard disks due to a problem in the ATARIDOS.SYS driver.

If you experience any problems reading or writing to the new

PC-MIRROR, you may set this option to 'NO' to force APE to use the

old, smaller, PC-MIRROR.

A rom patch for Spartados X is available.

* _L - Load APE Configuration_. Loads the configuration stored in

APExxx.CFG, in the path which you launched APE from. If APExxx.CFG

(where xxx is the version number) does not exist you can create

one with the _[S]_ save function. This file is automatically

loaded when you start APE.

* _R - Restore system defaults. _This loads the default parameters

that are _hard coded_ into APE. This should be used only if you

damage your configuration and connot figure out how to reset it.

* _S - Save APE Configuration_. Saves the configuration you have set

in _ALL APE MENUS_ to the file called _APE.CFG_ in the path from

which you launched APE.

_______________________________________________________________

_A_ - Printer Configuration

This screen controls the operation of the _Atari Printer Emulation_.

All commands sent from the atari to the_ P:_ printer device will

handled by APE or passed on to your Atari printer depending on your

configuration.

Values set in this section can be saved to APExxx.CFG.

* _A. Print Destination_. Select PRINTER to pass Atari print

commands directly to your PC Printer, FILE to save the print data

to a file, or NOWHERE to allow a real Atari Printer to answer the

request.

* _B. Printer Port_. If PRINTER is selected in option A, this option

controls the PC Printer port used

* _C. EOL Conversion_. When the atari prints data, it terminates

each line with the _ATASCII_ (Atari Ascii) EOL character which is

155. The PC terminates each line diffrently, with a _CR/LF_ pair.

Even though this is the standard for PC text, some printers

require_ only _the CR part be sent.

If your printer requires line feeds (such as the_ HP deskjet_

series) then select _YES_. If your printouts are all on one line,

text overlapping text then your printer requires line feeds and

CR/LF should be _selected_.

* _D. Atascii TAB conversion_. The Atascii character set uses a

different value for _TAB_ than the PC and PC printers. Enabling

this will _automatically_ convert these characters to PC

compatible ones.

* _E. Global conversions_. Enabling this will strip the high bit

from all characters sent to the printer. Useful with some printers

that may react badly to _ATASCII_ graphic characters, etc.

* _F. Printer data drive and filespec_. Specify in this box the file

in which you want printer data sent from the Atari stored in, if

you selected 'FILE' in option A.

* _G. Append or Overwrite_. Selecting APPEND will cause the file

selected in option F to be added to each time APE prints, if you

select OVERWRITE, the file will be replaced each time APE starts

_______________________________________________________________

_B_ - Modem (_R:_) Setup & Emulation

* _A. PC Modem Com-Port_. Com-Port of the PC MODEM to use when

emulating the R: modem/interface device on the Atari 8-bit.

* _B. PC Modem Baud Rate_. Baud rate at which the PC will

communicate with the PC-Modem. By default this is 57.6k BPS.

* _C. PC Modem DTR Line_. Setting of the DTR line when communicating

with your PC-Modem. Consult your modem manual for the proper

setting.

* _D. PC Modem RTS Line_. Setting of the RTS line when communicating

with your PC-Modem. Consult your modem manual for the proper

setting.

* _E. Hardware Flow Control Window_. Unlike all other serial

adapters and products, APE can provide real _HARDWARE FLOW CONTROL_

for your Atari.

This option sets the overflow queue size after which hardware flow

control will be used.

A setting of 700 is default.

* _F. Disable R: Emulation_. Disabling R: emulation will stop APE

from doing the following things.

+ Opening or reading from the specified com port.

+ Responding to requests for the 850 interface handler.

+ Doing any modem emulation.

It is reccomended that you use this switch if you do not have a

modem attached to your PC.

________________________________________________________

_C_ - APE Path, Macro, and Find System Configuration

Path Configuration Screen

This screen allows you to set the APE Search Paths for locating

images, the _PC-MIRROR_ path and filespec, the Find system defaults,

and the Quick Macros.

Values set in this screen may be saved to APExxx.CFG.

* _[1 - 0] - Define APE Quick Search Path._

The APE Quick Search Paths are a tool for convienent access to

your disk images.

Specify upto 10 locations on any device attached to your PC, and

APE will automatically look in those directories each time you

specify an image that is not located in the current directory.

For example: you press [L] to load an image, and type MYDOS.ATR in

the quick load box. If MYDOS.ATR is not found in the current DOS

directory, APE will look in all the paths specified here for it

automatically.

* _[F1-F10] - Define Quick Macro._ The quick macro feature of APE

can be a _very powerful _time saver. By allowing you to enter a

series of keystrokes to be executed on the function key of your

choice, most any feature of APE (_and even into the DOS shell!_)

can be automated!

All characters entered in your macro will be executed _EXACTLY_ as

if you had typed them from the PC keyboard. The only exception to

this is the '|' character which is used for a embedded carriage

return.

Any function that requires a _RETURN_ when done should be trailed

by the '|' character since APE will not press return for you be

default.

This feature can also execute functions _OUTSIDE APE!_ You can use

the '!' function to shell to DOS inside your macro and any

remaining keystrokes will be executed by your dos shell. (i.e.

'!EDIT AUTOEXEC.BAT|' would cause APE to shell to DOS and execute

_EDIT AUTOEXEC.BAT_)

_Example_: Setting F1 to the string '!C:|CD C:\TASM|MAKE|EXIT|'

and then pressing F1 at the APE main screen will:

1. Cause APE to shell out (command !)

2. Change to drive C (C: typed at the command prompt)

3. Change to directory 'C:\TASM\' (CD C:\TASM typed at the

command prompt)

4. Execute the program or batch file called 'MAKE' (MAKE typed

at the command prompt)

5. Exit back to APE (EXIT typed at the command prompt)

All quick macros are executed from the main status screen only. To

access deeper functions simply add them to your macro. (i.e.

'L1C:\TEST.ATR|' to tell APE you want to load an image, then in

Drive 1, then the filename and RETURN to load it)

* _D - Find Image Drives. _This is the _default_ list of drives that

will be presented to you each time you use the [F]ind Images

command. You may add or remove any drives you desire.

* _F - Find Image Filespec. _This is the _default_ filespec that

will be presented to you each time you use the [F]ind Images

command.

* _P - Default PC Mirror Path. _This is the _default_ path APE will

use for the PC-MIRROR each time APE is started.

* _M - Default Mirror Filespec. _This is the _default_ filespec that

will be used for the PC-MIRROR each time APE starts.

* _R - Remember last path used. _If this is set to YES, APE will

remember the last PC-MIRROR path and filespec you used between

sessions, and restore it each time APE is started.

For example: if you last accessed a file via the PC-MIRROR in

C:\MYFILES\ then APE would start in that directory next session.

If this is set to NO, APE will use the values entered here as the

default pathname, and filespec each time it starts.

_______________________________________________________________

_D - Com IRQ & Base Address

_This menu will allow you to confure APE to function with the

_IRQ/BASE address_ settings used on your computer. APE is pre

configured with the 'standard' AT values for these ports, so _you

should only need to use this function if APE fails to perform properly

for you._

To find the IRQ/BASE address of your com port use the_ MSD.EXE_

utility provided with MSDOS v5 and above, or consult your users

manual.

You may totally disable any Com ports you wont be using by setting its

BASE ADDRESS to_ $0000_. This is normally NOT required, but may help

if you are running TSR comm software, etc.

_______________________________________________________________

E - Configure Ultraspeed

The above menu will allow you to tell APE what speed it should operate

at with Spartados and other programs that use the US Doubler enhanced

warp speed mode.

_This option effects ONLY the operation of US DOUBLER type I/O and has

NO EFFECT on the operation of other high speed systems such as Happy

Warp Speed or XF551 mode._

* _1 - Force 1x SIO operation in Ultraspeed mode._ This will force

APE to *NEVER* switch into 2x or 3x ultraspeed mode, even when

asked to. _This option should NEVER be used unless you have very

serious problems with Ultraspeed I/O._ (Please contact me if you

do!)

* _2 - Set 2x SIO Ultraspeed operation._ This is the normal _US

DOUBLER ultraspeed mode_, and operates at 2x the standard SIO

rate. This is the speed you normally see with enhanced disk

peripherals such as the _USdoubler, Happy Enhancment_, etc.

* _3 - Set 3x APE+ Ultraspeed. _APE+ 3X SIO operation is a feature_

NEVER BEFORE_ available on the atari computer. It allows your

Atari computer to commuicate with APE at 3x the normal SIO rate.

_Thats 1.5x faster than Happy, XF551, and USDoubler drives, and 3x

the speed of a stock 1050!_

No special software or patch is required to use APE+ Ultraspeed.

Spartados 3.2g will _AUTOMATICALLY_ switch to this mode at bootup

when APE instructs it to.

_APE+ IO pushes Spartados to its serial speed limits. _You should

use_ FTE's Spartados 3.2g _(earlier versions may not be fast

enough) and have KEY turned OFF. Having KEY turned on slows the

Atari to the point it cannot keep up w/ APE+ 3X SIO.

To get the very most from APE and 3x SIO, check out the Ape Warp+

OS Upgrade.

USING YOUR PC MODEM WITH APE

Having your inexpensive PC modem do double duty on your your atari is

now possible!

_You may use any program that is compatible with the Atari 850

interface. No additional software or hardware is required!_

For this example we will use_ BOBTERM_. If you already have bobterm

make sure you use the bare version of the program that _does NOT_ have

an R: handler appended to it.

Load BOBTERM using your favorite DOS. Upon loading, BOBTERM will

'call' for the 850 interface handler. _APE will download its own

resident handler and bobterm will finish loading._

_Notice your APE screen now says 'Concurrent I/O running', it has been

commanded by BOBTERM to start talking to your PC modem!_

Make sure BOBTERM is set for 19.2k bps and then go ahead and enter

terminal mode. You should be able to type commands to your PC modem

just as if it was attached through an 850 interface!

If you cant seem to talk to your PC modem go back to the configuration

section and review the instructions for setting up your modem with

APE. Also, make sure bobterm is set for 19.2k bps.

_Unregistered Users Note: You also should set your terminal rate to

19.2k bps but your actual throughput will be restricted to 2400 bps

until you register._

_______________________________________________________________

USING YOUR PC PRINTER WITH APE

_First review the above configuration section on setting up your

printer for use with APE._ Once set up properly all commands sent

through the atari P: (printer) device will be intercepted by APE and

sent to the destination of yourc choice!

_______________________________________________________________

COMMON QUESTIONS AND ANSWERS

* Q: _How can I use ultraspeed and Ape Warp+ 3x SIO in games?_

A: While some dos systems, such as Spartados have native support

for speed-enhanced peripherals, no games or other software by

default support it. To get Ape Warp+ 3x sio at ALL times, even in

games, check out the Ape Warp+ OS InfoPage.

* Q: _What is the difference between the APE and ProSystem Cables?_

A: The APE Cable is designed to allow the PC to emulate Atari

peripherals. Using it, your PC pretends to be a wide range of 100%

SIO compliant Atari 8 Bit peripherals.

A: The _PRO System_ cable is designed to to allow Atari

PERIPHERALS such as the 1050 disk drive to be attached directly to

the PC. Using a special software package I have developed, called

the Pro System, you can use this system to create .PRO images of

even the most complex copy protected diskettes using only a stock

1050 disk drive. Or just ATR images at high speed if you choose.

A: The PRO Cable does _NOT_ perform the functions of the APE

cable. Once the .PRO images have been created, you can use your

REGISTERED Ape software to load the images and emulate the

peripheral behavior using the APE Cable.

A: Remember! The .PRO cable is used ONLY to _CREATE_ disk images.

* Q: _I can't find any documentation for the Prosystem!_

A: For the illustrated HTML version, check here. For a plaintext

version check PROSYS.DOC in the APE distribution.

* Q: _Why is APE 'crippled'?_

A: APE is provided in its current form for your evaluation. Even

in its UNREGISTERED form, it provides unmatched power, speed and

flexibility. You are given free of charge the ability to 'try

before you buy' all of APE's powerful features, such as the PC

MIRROR, Copy Protection Emulation, R: Modem emulation, 3X SIO

operation, unmatched hruristic Ultraspeed speed switching,

Diskcomm3 compatibility, and much more!

* Q: _I created a .PRO image of 'Game X' using the Pro System and it

will not run under APE._

A: If you are a registered user, and cannot get your title to run

under APE using all the available emulation modes, it it possible

I can add a new emulation mode that can deal with this new scheme.

Please send the original or 100% functional copy to me at the

address listed in REGISTER.DOC and I will do my best to make it

part of the next APE release.

* Q: _When I create disk images with APE, dos shows them all as

having 000 sectors free!_

A: When you create a 'disk image', it is the same as a real floppy

disk you bring home from the store. Its totally blank and needs to

be FORMATTED with your favorite DOS before it can be used. See the

above section on creating disk images for more assistance.

* Q: _How can I boot my Atari if I dont have a real disk drive?_

A: With the serial conversion device attached to the SIO port of

your Atari computer, the atari cannot tell APE from a real Atari

disk drive. If you mount an image in drive 1 and turn on the Atari

it will boot from that 'disk' just as if it were a real drive.

Ever better because APE is faster than a real disk drive.

* Q: _Will APE overwrite or format my PC hard drive?_

A: NO. Ape stores all its data and disk images in regular PC files

like every other MSDOS application and will not harm, alter or

otherwise interfere with the normal operation of your filesystem.

* Q: _Can I use real hardware in combination with APE?_

A: APE can cooperate fully with your existing SIO hardware such as

disk drives, cassette drives, etc. Just make sure you dont assign

an APE drive to the drive number of a REAL disk drive that is

turned on. If both APE *AND* the real disk drive try to answer on

the SIO bus at the same time it will confuse the atari and cause

an error. Your serial conversion cable should be the last device

on your SIO chain.

If you are using a 1 chip cable and are having interdevice

interference problems, consider upgrading to a 2-chip cable with a

manual tri-state. It may help alleviate problems w/ an

ultra-sensitive devices.

* Q: _All my data is on floppy disks, how can I convert them to ATR

images?_

A: If you wish to move your data to faster more reliable ATR

images all you need is a floppy drive capable of reading your

disks.

+ Set up your REAL floppy disk drive as Drive 1.

+ Create a new ATR image that matches the physical size and

density of your real disk using the 'C' function of APE.

Mount this image in bay #2.

+ Using a SECTOR COPIER program such as MYCOPIER, DISK WIZARD

or FASTCOPY copy the REAL diskette in drive 1 to the emulated

diskette in drive 2. (I suggest you use MYCOPIER since it

supports ULTRASPEED high speed I/O)

The disk image in bay #2 is now an exact copy of the real physical

disk in drive 1. You may use it in the same manner (except you

cant play Frisbee with it anymore :)

This can obviously be a time consuming process. Future updates to

the ProSystem software will allow the Pro cable to be used to

create .ATR images in a fraction of the time this requires, and at

a fraction of the time required to create the complex .PRO images.

* Q: _When I try to access drives higher than #2 I get error 160._

A: Atari DOS 2.0 and other early dos systems were not by default

capable of addressing disk drives higher than 2. (At $1000 each

who had more than 2 disk drives back then anyway? :) You can

attempt to patch your DOS.SYS file using the instructions

available on many BBS's, or you can switch to a superior dos

system. I suggest MYDOS because it is DOS 2 compatible.

* Q:_ After using the '!' command to exit to DOS, I cant run APE

again. I get an error message!_

A: When you use the '!' command to shell out to DOS, APE does not

actually terminate. APE places itself into the background and

launches a new copy of your command shell. To return to APE after

using the '!' command, type 'EXIT' then return.

* Q: _Do I need to save my disk images before I leave APE?_

A: No. Any time the Atari writes to a disk 'image' APE updates

that file automatically.

* Q: _Can I use my copy protected software on APE?_

A: Yes! _All copy protected software can now be emulated

successfully with APE! _Using the Pro System included and a stock

1050 (or 100% compatible) drive, you can create .PRO images of

even the _most complex_ diskettes.

See 'PROTECTED DISK EMULATION' in the APE users manual for

complete information on .PRO images.

_______________________________________________________________

TECHNICAL SUPPORT

Though technical support is not restricted to registered users, they

will be given top priority. Please review the documentation and common

questions sections also.

Internet E-mail: _classics@nacs.net_ [this is the preferred addr]

_dh395@cleveland.freenet.edu_ [emergency backup]

Telephone: _216-688-0001_.

Anytime you can catch me at home is pretty much ok. Just try and call

_before_ 11pm EST.

Snail Express

Steven J Tucker

3978 Brookside Blvd

Cleveland, Ohio 44111

USA_

_______________________________________________________________

PROGRAM DISTRIBUTION

_APE is user supported.

_This means that you may copy UNREGISTERED VERSIONS freely and give

the copies away to anyone you wish. They are in turn requested to send

in the registration if they decide to use it.

You can help me distribute APE to others who might find it useful by

uploading it local BBSs and by giving copies of it to friends, user

groups, and business associates.

_Thank you for supporting the Atari Peripheral Emulator and the

Shareware concept.

________________________________________________________________

NEW FEATURES/PROGRAM HISTORY

_This version of APE has many new features not available in the

previous beta test and public releases._

7/30/98 1.17. Added or improved since 1.16

*         o ProSystem emulation mode added for special Mindscape games

o Programs updated to run on fast machines (RTE 200 Bug)

o Incorrectly formed ATR's could cause program abort.  Fixed.

o Colors restored correctly upon program exit.

1/30/98 1.16. Added or improved since 1.15.

* _Summary of main program changes._

+ _APETIME.COM_ now included free with APE.

o Included on image APE_F32G.ATR

o Source code available.

o Downloads the PC time to the Atari and updates the

Spartados clock.

o Compatible w/ both Spartados and Spartados X.

o Atari 800 and XL/XE compatible.

+ _REMOTE.COM_ now included free with APE.

o Included on both APE_F32G.ATR and APE_WARP.ATR

o Allows user to perform all APE functions from the Atari

keyboard.

o Atari 800 and XL/XE compatible.

o Compatible w/ all DOS systems.

o See users manual for a complete description.

+ _Feature [F]ind Image_ implemented.

Can recursively search an entire set of drives for any

image(s)

+ Path configuration now supports 10 'spider' paths.

+ Configuration Option [_5_].

Cables using the CTS line now supported.

+ Configuration Option [_P_].

Registered users can now use persistent sessions.

+ Configuration Option [_Q_].

Registered users can now bypass the startup screen

permanently.

+ Configuration Option [_T_].

Attempts to use Windows95 long filenames can be manually

disabled.

+ Configuration Option [_U_].

Force use of old smaller PC-MIRROR. (Spartados X)

+ Configuration Option [_B/F_].

R: Atari Modem emulation can be manually disabled.

+ Configuration Option [_C/1-0_].

Spider path settings.

+ Configuration Option [_C/D_].

Default search drive list for find and REMOTE.COM

+ Configuration Option [_C/F_].

Default filespec for find function.

+ Configuration Option [_C/R_].

Persistent PC-MIRROR path switch.

* _Summary of PC-Mirror changes._

+ Maximum file write size is now 7,864,320 bytes.

Old limit was 93,000 bytes.

+ Maximum read size is now unlimited.

+ Configuration Setting allows switch back to old small image

method.

+ Special SpartadosX 4.20 rom patch available on request.

+ Mirror can now be swapped like any other image, using keys

1-8.

+ Mirror path and filespec are now separate items.

* _Summary of APELOADER changes._

+ Now moves up a directory when _backspace_ is pressed.

+ Now executes APE macros on PC when Shift 1-0 pressed.

+ Complete loader rewrite, now totally relocatable anywhere in

memory.

* _Bugs Fixed in APE_

+ Screen color corruption in menu Z/C fixed.

+ DCM Decompression code fixed.

Thanks to Ernest Schreurs (see IMAGIC.DOC).

+ Faster, more consistent, serial I/O routines written.

+ Keyboard buffer expansion code rewritten.

Problem with an international keyboard driver.

+ Work around for faults in Microsoft's Win95 serial interrupt

processing.

Could cause APE to stop responding when Win95 drops an

interrupt.

5/13/97 1.15. Added or improved since 1.11b/1.12b

* When a DOS shell is launched (Key _!_),

APE will now try and swap itself out to EMS or disk.

* Free's up about 300k of memory for shell usage.

* New Config Opt: Default swap method.

+ EMS

+ DISK

+ NEVER

* Various Screen Update Bugs Fixed

* Fixed READ-ONLY File Problem w/ PC-MIRROR

* New Printer Options

+ Print to file now supported

+ Print to any printer port supported

+ Turn off printer (allows real Atari printer to talk)

+ Print to file supports APPEND or OVERWRITE

+ (Next version will support multiple emulated printers e.g.

P2,etc)

* New Config Opt: Disable Happy/Syncromesh Detect

* Patch available for Spartados X 4.20 for Ape Warp+ (request)

* Added HARD Write protect, acts like a real drive (error returned).

* New Config Opt: Hard/Soft Write Protect (Soft allows old method)

* _APELOADER Software Created_

* APELOADER Added via direct boot from PC-MIRROR drive.

* (see Ape User's Manual for all the features!)

* APELOADER updated to run even broken EXE files

* APELOADER menu updated, allow base address switch from Atari

keyboard

* New Config Opt: Default APELOADER base address

* Full support for .CAS digital cassette images added (key _Y_)

* (see Ape User's Manual for all the features!)

* New PC modem configuration interface

* New configuration manager interface

2/7/97 1.12. Added or improved since APE 1.10

* APE now works 100% will all monochrome cards.

* Lockup bug fixed.

* Indus GT Syncromesh is now supported

* Many internal changes

* Crashes caused by file access violations are fixed.

* Printer emulation reworked. APE now supports many new printer

options.

* PC Mirror could be crashed by unexpected file permissions, fixed.

11/11/96 1.10. Added or improved since APE 1.09:

* Added initial ATR-FAST code.

* Buffers are allocated earlier in the initialization for computer

with low memory.

* ATR FAST system now supports multiple call methods and keystroke

activation

* Setting the framing threshold sometimes caused an infinite loop,

fixed.

* The TILDE character used in Windows95 long filenames caused

filenames in the PC MIRROR to be garbled. Files now use the

underscore character for long filename translation

* While writes to the PC mirror under spartados are not supported,

trying to do it caused a crash instead of a warning. Fixed.

* SpartaDOS X cartridge support for floppy formatting added

* SpartaDOS X cartridge support for PC Mirror added

* SpartaDOS X cartridge now sees the PC mirror and can read it.

* SDX COPY command now compatible with PC Mirror.

* Problem with last path used in ATR file selection fixed

* Ultraspeed and Ape Warp+ 3x SIO now compatible with more software,

Top Mag, Superdos, etc

* ATR FAST now displays ATASCII comments properly

* ATR FAST system now responds to shift driven option

* Extra SIO commands added for more support of the 1050 Speedy

upgrade. Bibocopy 1.5 now works at 56k

* Some functions optimized to allow faster performance with the APE

Warp+ OS

* Configuration menu totally revamped

* New configuration options: Safety Margin, Sio Control Method, file

selector choice.

* Cursor was sometimes left visible after a pick box was displayed.

Fixed.

* More robust configuration file checking installed

* Many new features added to the Win95 file selector

* APE sometimes left stray characters at the right margin. Fixed.

* Write protect no longer clobbers byte 15 of ATR header

* APE fully supports CRC32 ATR Authentication. ATR Authentication

standard and documentation available at the APE Home Page.

* Warp_453 from 1.09 now obsolete due to new driver in APE_WARP.ATR.

3x SIO patch for MyDOS/Atari DOS now much more reliable.

* New versions of MyCopier included on the APE_WARP.ATR. Old version

was also corrupted.

5/22/96 1.09. Added or improved since APE 1.07:

* Distribution Change: FTED32F.ATR contains the Shareware version of

Spartados 3.2f. Spartados is an advanced _high speed_ DOS system

from Fine Tooned Engineering.

* Distribution Change: 453_WARP.ATR contains MyDos 4.53, as well as

2x Happy High Speed I/O Patch, 3x APE+ Warp Speed Patch (modified

Happy patch), and MyCopier ultra high speed sector copier that is

compatible with APE+ 3x SIO.

* Ape 1.09 and ProSystem 1.50 now support images of enhanced density

and a extension of the .PRO format was changed to help this along.

You _must_ convert your old v2 .PRO images to v3 .PRO images or

they cannot be used with Ape 1.09. A fast and automated program,

PRO2TO3.EXE is included to convert all your old images. You can

give it a wildcard filespec so you should be able to convert all

your .PRO images in one pass :)

* Executable file now smaller because of internal changes in

allocation of dynamic memory vs. the old static method.

* Executable now uses overlays for routines that are not time

critical, freeing page space for more features in the main

program.

* 850 Emulation Fixed - A problem in 1.07 caused a 6502 vector in

the stage 3 handler to point to a BRK instruction (OOPS!).

* 850 Emulation Fixed - Problem with concurrent I/O dropping out the

instant it starts cured.

* Fixed DCM code to allow decompression of double density images,

bug has been in the DCM code since its was written.

* PC Mirror: Files written to the PC mirror would fail with a DISK

FULL message after about 400 sectors, fixed. Mirror now supports

capture of files up to 8 Megs.

* PC Mirror: Writing to a file that already existed in the PC Mirror

drive would cause the program to fail with a device timeout.

Mirror now supports writes to any file, even if it exists.

* PC Mirror: Files written to the PC Mirror would on rare occasions

be truncated to 125 byte intervals, or have an extra 125 bytes

appended. Fixed.

* Pc Mirror: Files written to the PC Mirror would appear in the

default DOS path instead of the current PC Mirror path. Fixed.

* Ape and ProSystem now be more monochrome friendly. If MDA adapters

are detected all screens are displayed in white on black _only_.

* Ape 1.07 could be crashed about 10 ways with runtime error 203.

This sloppy memory problem has been fixed and wont happen anymore.

* Spartados 3.2x (XF551 version) is now fully supported. Thanks to

the user who sent 3.2x for testing. Problems included failure to

format any disk floppy or HD). Unknown if this will effect the X

cart situation.

* When ATR images were created, they could then be mounted on the

same drive as the PC Mirror, caused some interesting effects but

is fixed.

* When creating ATR images they would appear in the default DOS path

if no path was specified, they now appear in the default ATR/PRO

path.

* APE ProSystem emulation code rewritten and streamlined, added 5

extra emulation modes (more coming). ProSystem now _requires_

version 3 format images. See the above instructions for converting

your existing .PRO images.

* Many improvements to file selector including better path handling,

type highlighting, drive change entries, visible marker (mono

friendly) and internal changes.

* Ape now includes an XFD conversion utility in addition to an

improved and hopefully now complete DCM conversion utility. Image

conversion is available to all users and is invoked with the 'A'

keystroke.

* Many users asked for APE to handle commands sent by Nick Kennedy's

program REMOTE.OBJ. Ape now handles these requests, but obviously

you must use APE keystrokes. Keystrokes accepted for registered

users only.

* REMOTE.OBJ was created by and is the property of another software

author and is therefore not included in this software package. I'm

not aware of any restrictions on its use, but if it requires some

type of payment for use of course please register it.

* You can now select a new ProSystem emulation mode for any image

directly from within APE as of 1.09. This _greatly_ speeds up the

PRO image testing process. Registered users only.

* Sometimes the screen was left 'dirty' from previous menus, little

dots at the right edge, etc. Fixed.

* The shareware version of APE could crash in the Atascii/Ascii

viewer. Fixed.

* The file selector has again changed and you should note a

significant change to the usage of the 'Additional Extension'

option of configuration menu 3. This option used to specify the

extension of the files to be displayed in the file browser.

This setting _no longer_ performs this function. It now specified

an _extra_ extension to be displayed in the file browser. ATR and

PRO extensions are now automatically displayed at the proper

times. If you have this option set to '*' I suggest you clear it

so you do not have to wade through tons of useless files to get to

your disk images.

11/20/95 1.07. Added or improved since APE 1.06:

* Most of the changes I had noted were lost when I had a file

corruption problem. So anything new thats not listed here... :)

* Screen blanker was causing many systems to crash after loading

APE. It has been removed. Other more stable freeware screen

blankers are available anyway.

* Old .PRO format totally scrapped. Old .PRO riders are no longer

compatible with APE and will not be supported in the future.

* New .PRO v2 image format created.

* Coded or re-coded about 1000 lines of code to install full dual

ATR/PRO compatibility. First public release in 1.07.

* Bug causing ignored writes to disk images for many users has

hopefully been eliminated.

* Beta .PRO image creator coded and made available to registered

users.

* APE now 100% functional in dual ATR/PRO mode.

* Added multiple compatibility modes for PRO images, now allows all

my copy protected software to run!!! :)

* Added compatibility timing to PRO v2 format. Disks which require

reads at normal 1050 speed (300 rpm) can now be emulated.

* Added sector overrides for all .PRO functions. Each sector can now

be modified individually (if desired) to bypass any of the global

.PRO settings.

* Minor improvments/bugfixes for the file selector

* Minor improvments/bugfixes for the text browser

* Misc improvements to the core routines

* Pro System 1.02b now available to the public.

* Plans for the PRO CABLE added to APE.DOC

9/2/95 Added or improved since APE 1.04:

* Titles added to file selector

* TAB can now be used to edit file selector path/filespec

* Misc HELP screen fixes

* Found memory leak in load disk image routine, fixed.

* Added first Diskcomm3 -> ATR conversion code

* DCM -> ATR converter now part of APE program, option [A]

* DCM -> ATR converter can now handle batch processing using

wildcards

* DCM -> ATR converter now asks if you want to mount the image you

just converted (if you convert only 1 image).

* Integrated new COMM routines, speed and reliability improved

* Verified HAPPY HS I/O w/ Diskcomm3, ok.

* Verified Ultraspeed I/O w/ Bitter Reality Demo, ok.

* APE now rejects corrupt/invalid ATR files

* HAPPY SECTOR COPY from the Happy System Disk verified to work ok.

* Bug in Happy write routine found and fixed.

* Bug in computation of high sector numbers found and squashed.

Nobody complained about this, anyone out there? :)

* Added UART/COMM configuration screen and code, now available in

[Z] Configuration, [4] Option 4.

* UART/COM settings now saved in APE.CFG

* Eliminated '.' from file selector, useless entry and its now

easier to navigate using the browser.

* Added '>' and ' PC Mirror updated to work a 2x speed w/ Happy HS

I/O

* * PC Mirror now compatible with Spartados. Can now copy files of

any size from the PC mirror into spartados using COPY, XCOPY, etc.

Spartados still has a bug in its i/o routine but i found a way

around it.

* Happy HS I/O writes were still causing jumps to ultraspeed on

occasion. Found a problem in the timing code and its now a very

solid routine.

* XF551 high speed I/O didn't work most of the time, again nobody

reported this monster bug. Fixed and verified with XFTOOLS.ZIP.

* Much of APE recoded and optimized. Many sections were in the

original beta 'just make it work' form and were real electronic

elephants. A few sections of non time-critical repetitive code

centralized. Code and data segment sizes reduced significantly and

program speed increased.

* Started recoding I/O routines for 3x SIO.

* Recoding of 3x SIO routines finished.

* Patched WARP.COM to allow 3x SIO speed, now renamed to

APE_WARP.COM to avoid confusion. Verified reads and writes with

MyDOS 4.53.

* * Added support for 3x SIO speed using Spartados 3.2g with NO

DRIVER REQUIRED. Spartados now boots in APE+ (3x SIO) mode

automatically.

* 'Force' in the help screed was spelled 'Forse', duh.

* Added Spartados Configuration Menu (Cfg [Z] Option [5]) to allow

selection of default Spartados speed for very slow PC machines

that cannot handle 3x SIO.

* 3x SIO verified NOT to work with Spartados versions 3.2d and

earlier. Spartados SIO routines in these versions cannot keep up

with APE+ SIO and get choppy.

* Cursor turned off, guess it annoyed some folks just sitting there

and blinking.

* Added 'APE+' display to indicate 3x sio running.

* APE+ SIO (3x) verified to work w/ 3.2g in single and double

density.

* APE+ SIO (3x) verified to work at 3x speed w/ bitter reality demo.

Any program that follows the Spartados model should work w/ APE+

IO.

* APE+ SIO (3x) verified to work with MyCOPIER at 3x SIO speed.

* APE+ SIO (3x) verified to work w/ Diskcomm3

* * Added support for 'simple' PROTECTED DISK images. Protection

information is stored in a 1k file of the same name with the

extension '.PRO'.

Sample Images

APPLE.ATR/PRO - Apple Bandits protected version SPY.ATR/PRO - Spy

vs Spy protected version

* Fixed a bug in the DCM->ATR conversion code, leftover junk from

the standalone version eliminated.

* Unregistered users restricted to only 2 command line switches.

* Fixed PC MIRROR to allow APE+ (3x) sio access.

* [The following code is complete, but is not included in APE 1.06

because without the ability to use the READ/WRITE TRACK commands

properly, they are not really useful except for show. When I get a

Happy/Archiver drive to test with, these may be included]

* Happy Controller Ram emulated.

* HAPPY System Disk now recognizes APE as HAPPY w/ Controller Option

installed.

* Happy Tracer Buffer emulated. HAPPY system disk now recognizes its

presence and displays it contents.

* Happy code buffer emulated and high speed transfers working. Able

to completely load 'BACKUP ANY DISK' and accept drive programming.

* Happy move head command recognized

* Archiver READ TRACK command recognized, only response figured out

so far indicates a totally empty track.

* Archiver FORMAT TRACK command accepted.

* Screen blanker added, now blanks screen after 2 minutes of

keyboard and SIO inactivity.

* Ultraspeed timing tightened, better performance.

Added or improved since public beta 8:

* Bugfix for 1050 formatting and status routines

* Happy high speed reads now supported

* New syncro code added, great improvement!

* Better swapping method, you no longer have to press the existing

image first, either is fine.

* Happy high speed write support added

* Some extended Happy commands now supported

* Write protection of images now effective

* Command line options added

* User now asked to mount image after creation

* 850 Interface serial emulation now working at 9600 baud

* Scrapped 850 interface code and starting rewriting some public

domain R: code.

* Automatic switching between command and concurrent i/o mode now

working

* PC MIRROR directories are now built on the fly

* Support for 43/50 line screens installed

* 850 Interface R: emulation now working at 19.2k bps

* Added General/Serial configuration screen

* Added command filters

* Added slow machine display options

* Added user definable framing value

* New streamlined core serial i/o routines. Much faster now.

* Now recognizes many more extended happy commands

* Added hardware flow control for PC modem, now works automatically.

* Added user definable flow control threshold

* Added the ability to load and save block configurations

* Added enhanced command line help and registered CL features

* Added Path/Macro configuration screen

* Added support for default paths and file extensions

* Added support for quick macros, and editing code

* New consolidated configuration menu, 'I' function for printer

configuration now in this menu.

* Improved and incorporated my multi-type text browser into APE. Can

also be called from the command line. (reg)

* Improved file selector, now has title and user can enter own

path/filespec using the TAB key.

----------------------------------------------------------------------------

REGISTRATION AND ORDER FORM

Send orders to:       Steven Tucker

3978 Brookside Blvd

Cleveland, Ohio  44111 USA

Program information:  Telephone:      216-688-0001

E-Mail:         classics@nacs.net (primary address)

ai123@freenet.carleton.ca

-----------------   Single Computer Licenses Includes   ----------------

1. Enhanced & Registered versions!

2. Priority Tech Support (E-Mail, Phone, Mail)

3. Notice of Major Upgrades

------------------------------------------------------------------------

Item Description                                  Price     Quantity

_Atari Peripheral Emulator_

Registered & Enhanced Version of APE              $39.99      [  ]

_Ape WARP+ OS Upgrade Option for XL/XE Computers

APE Warp+ XL/XE OS_ w/ Installation Kit           $49.99      [  ]

APE Warp+ XL/XE OS_ Installed (see shipping note) $79.99      [  ]

_Preassembled Interface Cables_

APE 1489 Serial Interface Cable                   $59.99      [  ]

ProSystem .PRO Imaging Cable                      $59.99      [  ]

_Imagic_

Imagic-95 Registered and Enhanced                 $10.00      [  ]

Imagic-DOS Registered and Enhanced                $10.00      [  ]

(GET BOTH IMAGIC-DOS and IMAGIC-95 for only $15)              [  ]

_Shipping_

Shipping ($5 USA, $6 Canada, $9 Abroad)           [    ]

Subtract Discount                                 [    ]

($5 discount _per interface cable_ purchased)

_Total + Shipping (less any discount)_            [              ]

-----------------------------------------------------------------------

_Shipping_: flat-rate, regardless of the size of your order. If you

choose to get the APE Warp+ OS upgrade with installation, return

shipping of your computer is FREE within the contential USA. Please

contact me for return shipping rates outside the USA. For more

shipping details see the Ape WARP+ OS Info Page.

Note on payment: Canadian and overseas users must pay by check or

money order _PAID IN US DOLLARS AND DRAWN DIRECTLY ON A US BANK._ In

Canada, a postal money order in US DOLLARS is acceptable.

Contact: ________________________________________________________

Company ________________________________________________________

Address ________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

Phone _________________________ FAX _________________________

How did you get your copy of APE/IMAGIC?

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

Suggestions or Comments:

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_______________________________________________________________

The APE 1489 Serial Conversion Interface

Click Image for large schematic

_How to construct the 1489 version of the APE interface_

Sio Orange (Sio Pin 3) -> Diode Clear End & Diode Band End -> 1489 Pin

#8

Sio Lt Blue (Sio Pin 10) -> 1489 Pin #14

Sio Black (Sio Pin 4) -> 1489 Pin #7

Sio Purple (Sio Pin 7) -> 1489 Pin #4

Sio Green (Sio Pin 5) -> 1489 Pin #1

PC Signal Ground (9p #5) (25p #7) -> 1489 Pin #7

PC Ring Indicator (9p #9) (25p #22) -> 1489 Pin #6

PC RXD (9p #2) (25p #3) -> 1489 Pin #3

PC TXD (9p #3) (25p #2) -> 1489 Pin #10

_______________________________________________________________