THE ATARI PERIPHERAL EMULATOR USERS GUIDE

Version 1.07

November 27, 1995

Steven J Tucker

9731 Sunrise Blvd #M33

North Royalton, Ohio 44133

U.S.A

Written by Steven J Tucker

Copyright (c) 1995 by Steven J Tucker

The Atari Perihperal Emulator (APE) is a advanced serial application that

emulates an entire subsystem of atari 8 bit peripherals using your

existing or purchased SIO2PC compatible serial conversion device.

APE is:

* Compatible

* Fast

* Powerful

* Flexible

* Affordable

ORDER INFORMATION

For additional information on licensing the Atari Peripheral

Emulator see the end of this document, the file ORDER.FRM that

was included in this distribution or contact:

Steven J Tucker                     Voice (216) 237-5308

9731 Sunrise Blvd #M-33

North Royalton, Ohio  44133         Electronic Mail to:

USA                                 classics@nacs.net

APE LICENSE AND DISCLAIMER

The Atari Peripheral Emulator is the copyrighted property of

Steven J Tucker.  Distribution of registered versions of

this product is a violation of copyright law.   You may

use the unregistered version of this program for a 30 day

free trial period.  If after 30 days you wish to continue using

APE you MUST purchase it or remove all copies of the software

from your system.

The author makes no representations or warranties with respect

to the contents hereof and specifically disclaims any implied

warranties of merchantability or fitness for any particular

purpose. Further, the author reserves the right to revise this

publication and to make changes from time to time in the

content hereof without obligation of the author to notify any

person or organization of such revision or changes.

TABLE OF CONTENTS

ORDER INFORMATION . . . . . . . . . . . . . . . . . . . . 2

DISCLAIMER  . . . . . . . . . . . . . . . . . . . . . . . 2

WHAT IS APE?  . . . . . . . . . . . . . . . . . . . . . . 5

WHAT HARDWARE IS REQUIRED?  . . . . . . . . . . . . . . . 5

CAN APE BE MULTITASKED? . . . . . . . . . . . . . . . . . 5

WHAT SIO HARDWARE DOES APE EMULATE? . . . . . . . . . . . 6

WHY USE APE . . . . . . . . . . . . . . . . . . . . . . . 7

THE PRICE FACTOR. . . . . . . . . . . . . . . . . . . . . 7

ABOUT SHAREWARE . . . . . . . . . . . . . . . . . . . . . 9

REGISTRATION BENEFITS AND UNREGISTERED RESTRICTIONS . . . 9

INSTALLING AND USING APE  . . . . . . . . . . . . . . . . 11

1) Installing APE . . . . . . . . . . . . . . . . . . . 11

2) Starting APE . . . . . . . . . . . . . . . . . . . . 11

THE STATUS SCREEN . . . . . . . . . . . . . . . . . . . . 12

KEYSTROKE COMMANDS AND EMULATOR FUNCTIONS . . . . . . . . 13

[1-8] Swap Disks . . . . . . . . . . . . . . . . . . . 14

[A] ATR/DCM Disk Conversion. . . . . . . . . . . . . 14

[B] Block Cfg Load/Save. . . . . . . . . . . . . . . 14

[C] Create Disk. . . . . . . . . . . . . . . . . . . 15

- Creating new disk images . . . . . . . . . . . . 15

- Formatting your disk images  . . . . . . . . . . 16

- Atari DOS . . . . . . . . . . . . . . . . . . 16

- Mydos . . . . . . . . . . . . . . . . . . . . 17

- Spartados . . . . . . . . . . . . . . . . . . 17

[H] Help . . . . . . . . . . . . . . . . . . . . . . 18

[L] Load Disk Image. . . . . . . . . . . . . . . . . 18

PROTECTED DISK SOFTWARE . . . . . . . . . . . . 18

[Q] Quit Program . . . . . . . . . . . . . . . . . . 19

[R] Release Image. . . . . . . . . . . . . . . . . . 19

[S] Set PC Mirror Drive. . . . . . . . . . . . . . . 20

- Navigating PC directories in the PC mirror . . . 20

[T] Toggle Emulation . . . . . . . . . . . . . . . . 20

- Smart Mode . . . . . . . . . . . . . . . . . . . 21

- Stupid Mode. . . . . . . . . . . . . . . . . . . 21

[U] Ultraspeed Toggle. . . . . . . . . . . . . . . . 21

[V] View file w/ multi-filetype browser. . . . . . . 21

[W] Write Protect Image. . . . . . . . . . . . . . . 21

CUSTOMIZING AND CONFIGURING APE

[Z] Configuration Menu . . . . . . . . . . . . . . . 22

- General/Serial Setup . . . . . . . . . . . . . . 22

- Printer Setup. . . . . . . . . . . . . . . . . . 25

- Path and Quick Macro Setup . . . . . . . . . . . 26

- Comm Port Setup. . . . . . . . . . . . . . . . . 27

- Ultraspeed/APE+ Setup. . . . . . . . . . . . . . 27

USING YOUR PC MODEM WITH APE. . . . . . . . . . . . . . . 29

USING YOUR PC PRINTER WITH APE. . . . . . . . . . . . . . 30

COMMON QUESTIONS AND ANSWERS. . . . . . . . . . . . . . . 30

TECHNICAL SUPPORT . . . . . . . . . . . . . . . . . . . . 31

NEW FEATURES / PROGRAM HISTORY. . . . . . . . . . . . . . 32

ORDER FORM. . . . . . . . . . . . . . . . . . . . . . . . 36

BUILDING THE PRO SYSTEM INTERFACE YOURSELF . . . . . . . .36

BUILDING THE INTERFACE YOURSELF . . . . . . . . . . . . . 37

WHAT IS APE?

Thank you for trying the Atari Peripheral Emulator! You've taken

the first step to making your Atari 8 Bit computer more

enjoyable and useful!

APE is an advanced serial application capable of emulating

an entire subsystem of devices on the Atari 8 Bit SIO bus.

WHAT HARDWARE IS REQUIRED?

APE talks to the SIO bus of your Atari 8 bit home computer using

an SIO2PC compatible serial cable.  This serial device provides

the hardware required to convert the voltage levels present on

the Atari SIO bus to levels compatible with the PC serial port.

This device hooks directly to the SIO port of your atari computer,

and to the serial port of your PC compatible computer.  No

interface or other additional hardware is required!

Plans for building this device yourself are found later in

this document.  The 2 chip version requires $20-25 of

parts, some not commonly available.  The newer one chip

version of this device requires only commonly available

parts and can cost between $10-20, depending again on the

cost of parts in your area.

Assembly and testing of this device takes 2-3 hours.  If you have

no soldering experience I suggest you pick up a book on

electronics and practice before attempting final construction.

If you do not wish to make the interface yourself, I can

provide a completely assembled and pre-tested serial unit

for you.  See ORDER.FRM for details, or ORDERING INFORMATION at the

end of this document.  Also, anyone purchasing the hardware

from me will receive a $5 discount on the more robust REGISTERED

version of this software.

APE is compiled using 808x compatible instructions, and will run

on most any machine from an XT to a Pentium with a serial port and

at least 200k of free DOS memory.   A 286+ class machine with a hard

drive and disk caching software (Norton Speed Drive, MS Smart Drive,

etc) are recommended to get the very best performance.   You should

have WRITE CACHING turned ON when using APE w/ disk cache software.

CAN APE BE MULTITASKED?

APE is designed to operate under MSDOS or MSDOS compatible

operating systems that allow exclusive control of the system

by a single task.  APE requires very precise timing for its

serial core functions, and may fail to function if operated

under 'time sharing' operating systems such as Desqview,

Windows, OS/2, etc.

This is not a shortcoming in APE.  The Atari SIO bus was designed

to talk to devices that were always ready to respond, and to

respond within a strict time frame.  When you system takes

time away from APE for other applications, the time critical

handshaking between APE and your PC becomes desynced and IO

operations may fail.

Even systems like Windows 95, which claim to give 'exclusive' control

of the system to DOS applications can still fail.  This is

because the OS layer of these applications continues to slice

time away from APE for its own housekeeping functions.

WHAT SIO HARDWARE DOES APE EMULATE?

The following SIO hardware and command systems are supported:

* Full cooperation with REAL sio devices, such as disk drives.

* Special APE+ 3x SIO mode!  1.5x faster than Happy Warp Speed

and US DOUBLER Ultraspeed.  Functions transparently under

Spartados, no patch or loader required!

* SpartaDOS ULTRASPEED I/O in any drive/density

* Upto 8 Diskette Drives

* Single, Double and 1050 Enhanced Density for any drive

* Disk images of upto 16 megabytes in size!

* PC MIRROR drive gives access to files on your PC hard drive with

any dos and at all SIO speeds, including APE+ 3X SIO!

* REGISTERED users may now WRITE files to the PC mirror, making

file transfers to your PC as easy as using the

DOS Copy command

* As of version 1.07, APE can emulate ALL FORMS OF COPY PROTECTION.

All your valuable copy protected diskettes can easily be

converted to the new .PRO format using a stock 1050 (or 100%

compatible) disk drive!

* Emulation of the Atari 850 interface, allowing you to use

your PC modem and printer from any atari application that

can use the 850 interface.  100% 850 stage 1 compliant.

* Hardware flow control for the PC modem using the R: device.

* Basic SIO Commands Supported

- Read Sector

- Write Sector

- Put Sector

- Disk Drive Status

- Controller Format Diskette

- Controller Format Enhanced Density Diskette

- Read Percom Extended drive configuration block

- Write Percom Extended drive configuration block

- Get 'Speed Byte'

- Ultraspeed I/O Sector Skew Format Diskette

- Print to 850 interface

* XF551 High Speed Read Sector

* XF551 High Speed Put Sector

* XF551 High Speed Write Sector

* Happy Warp Speed Read Sector

* Happy Warp Speed Put Sector

* Happy Warp Speed Write Sector

* Happy 'Are You Happy?' Command

* Happy Read Status Command

* Happy Format Diskette

* Happy Init Controller

* Happy Reset Drive

* Happy Set Unhappy Command

* Happy Set Fast Write

* Happy Force Protect

* Happy Force Write

[Extended Happy Stuff]

* Happy Tracer Data

* Happy Controller Option RAM

* Happy Drive Program Area

WHY USE APE?

Buying or building this interface, and purchasing the REGISTERED

version of APE will be one of the the best investments you can

possibly make in your Atari computer.

THE PRICE FACTOR

For less than the price of a SINGLE 'refurbished' 1050 disk

drive from one of the few remaining atari 8 bit dealers you

will receive all the following:

a) Emulation of 8 disk drives, each of which may hold a

disk image of upto 16 MEGABYTES.  Thats 128 megabytes

of information and programs at your fingertips at

all times!

b) Each emulated disk drive is also capable of emulating

the Spartados US Doubler ULTRASPEED (2x), XF551 Warp

Speed I/O (2x), Happy Warp Speed I/O (2x), APE+ Warp

Speed I/O (3x).

This enables your disk access to be more than three times

as fast as a standard 1050 disk drive using Spartados,

Happy Warp Speed Dos, Mydos, and other high speed

operating/dos systems and copy programs or just ANY dos

using the Happy Warp Speed OS patch.

c) Using APE is like having a hard drive equal to that of your

PC on your Atari, and for a SMALL FRACTION of what a

similar system would cost for your 8 bit.  APE also preserves

the value of your existing PC hardware by making it

available to use on your Atari also.

Ape does ** NOT ** erase and reuse your PC hard disks, it

stores data in 'disk images'.  Disk images are regular

PC files (like this one) that mimic the size and format

of a real Atari disk.  They may be as small as 90k, the

size of a Single Sided/Single Density 810 disk. And as large

16 megabytes, 65,535 DOUBLE DENSITY SECTORS.

If you previously used the software 'SIO2PC', your old SIO2PC

disk images are 100% compatible with APE. APE itself creates

images that are backward compatible with the SIO2PC .ATR

format.

d) Transparent access to your PC printer from any atari 8

bit application, without the use of any special atari

software.   APE intercepts and replies to all SIO commands

sent to the P: (printer) device and passes that data

on to your PC printer.  APE also is capable of preforming

translations of this data on the fly to match the

specifications of your PC printer!

e) Direct access to any file on your pc hard drive, from your

Atari using the powerful PC MIRROR function.  Transfer

files to your MSDOS filesystem using only the DOS

copy function!  No other software can offer this

revolutionary feature!

f) Transparent access to your PC *MODEM* from most any Atari

terminal program that is compatible with the Atari 850

interface!  APE responds to stage 1 calls for the 850

interface R: (modem) handler, making it virtually 100%

compatible with most terminal programs.

APE continues to provide access to your disk drives even

when running in concurrent i/o.  This allows you to do things

such as run a BBS, download files, etc all without having

ANY real atari disk drives or modems attached to your

computer! (or wallet :)

The registered version of APE provides automatic

HARDWARE flow control, allowing you to access modems

transmitting data faster that 19.2k.

Note: Concurrent I/O is the special serial bus mode the

atari uses to communicate with the 850 interface, modems

and other streaming serial devices.

g) Advanced copy protection emulation!  APE can emulate even

the most complex copy protection schemes, making the

archival of your valuable copy-protected originals possible.

APE can emulate even the most complex forms of floppy disk

protection, such as 'Phantom Sector' schemes.  APEs .PRO

format has been able to archive even the most notorious

of the late issue titles, such as: Alternate Reality,

All Electronic Arts titles, Lucasfilm titles, First Star

titles (Spy vs Spy), Broderbund titles, and others.

The APE Pro System is included in this distribution.  It

allows you to create .PRO images of even the most complex

copy protected disks and use them with APE.

All PHANTOM EMULATION modes are not available to

unregistered users.  Modes 1, 3, 4 and all future

emulation modes are reserved for users who support

the future development of APE by REGISTERING.

h) Reliability.  You will never fear your data being lost to

a bad floppy disk.  Your files and data will rest in file

on your PC hard drive, and can be backed up to a PC tape

drive, etc.  The same as your other valuable PC data.

We all know how difficult it can be to find good, working

replacment hardware for our atari computers at reasonable

prices.  APE is designed to eliminate these worries,

providing you with a flexible software system that will

contine to grow and become more useful over time as the

software is improved and upgraded.

And MUCH MORE!  Read on to discover all the powerful features

of APE.

ABOUT SHAREWARE

The user supported software concept (also known as "Shareware")

is an attempt to provide quality software at low cost. Both the

user and author benefit from this plan. The user will benefit by

being able to "test drive" software thoroughly before purchasing

it. The author benefits by being able to enter the commercial

software market while avoiding the high cost of commercial

distribution.

This concept helps many independent authors and small companies

that otherwise would be discouraged from developing and promoting

their ideas. It can only work with your support. If you obtain a

user supported program from a friend and are still using it after

a few weeks, then it is obviously worth something to you, and a

registration fee should be paid.

This software is distributed under the user supported software

concept. Though APE is copyrighted, you are encouraged to copy

and distribute the ** UNREGISTERED ** package to others.  You may

only distribute the unregistered software package in its original

unaltered ZIP form.

Please do not steal APE, it is the product of hundereds of hours

of coding and debugging. It also represents something quite

rare at this time, new software development for the Atari 8.

IF YOU USE APE LONGER THAN 30 DAYS YOU *MUST* REGISTER IT

UNREGISTERED RESTRICTIONS AND REGISTRATION BENIFITS

Unregistered evaluation users have ALL of APEs powerful

features at thier disposal except the follow, which are

available to REGISTERED users only.

A) UNREGISTERED users are allowed *FULL ACCESS* to read files

in the PC MIRROR drive.  Only registered users may WRITE

files to the PC MIRROR.

B) UNREGISTERED users are allowed FULL ACCESS to APE's advanced

Pro System, and can create .PRO images of even the most

complex copy protected diskettes.   Only REGISTERED users

receive full access to all PHANTOM EMULATION modes

in APE.EXE.

C) Only 1 F-Key macro.  REGISTERED users may define 10 custom

F-Key macros that allow any series of keystrokes to

be entered automatically.

D) Block Configurations.  REGISTERED users may create special

'Block Configurations' that allow upto 8 disk images to

be loaded with a single keystroke.  See later in this

document for more information on block configurations.

E) All of APE's enhanced SIO read modes are available to

unregistered users.  You may read your diskettes at

any speed, including the enhanced 3X APE+ SIO mode.  Only

registered users may perform disk WRITES at the

following speeds: Happy 2x Warp, APE+ 3x Warp, APE+ 3x Sparta.

Unregistered users may still read AND write at the

USdoubler Spartados warp speed, and normal SIO speeds.  The

disk image WARP.ATR is included as a demonstration of

Happy 2x Warp SIO in MyDOS.  The disk image APE_WARP.ATR is

included as a demonstration of APE+ 3x SIO in MyDOS.  No

special software or loaders are required to use these

enhanced speeds under Spartados.

F) Unregistered users ARE permitted to use APE's POWERFUL R:

Atari Modem emulator, but only at speeds upto 2400 baud.

Registered users receive access to the concurrent i/o R:

emulator at 19.2K and above!   Speeds of upto 56kbps are

possible using the enhanced registered version of APE.

G) Registered Users may start the ATASCII/ASCII/UNIX text

browser from the command line.  Unregistered users still

have full access to this powerful feature, but must start

it from within APE.

H) Unregistered users are limited to 2 command line switches. There

are no limitations (except as imposed by the DOS operating system)

on registered users.

I) APE's startup screen may be bypassed by registered users, and

there are no nag screens or timed waits.

J) Unlimited usage. You may use the unregistered version of

this software for only a 30 day trial period.  If you wish

to contine using APE after that time you must pay for it.

The program will not eat itself or stop working after 30 days,

I'm depending on your honesty as a fellow atari 8 bit

computer user to do your part.

K) Limited tech support.  Registered users will recieve priority

with regard to technical support issues, custom versions,

and bug fixes.

L) Registered users will recieve upgrades to future

revisions of the program (os specific) for only $3 to cover

postage and diskette.  If by e-mail the upgrade will be free.

Major upgrades, such as ports to other operating systems

are not covered.

M) Future registered versions will include:

1) The ability to save browsed text in ibm, unix, or

atascii format.

2) Direct manipulation of files inside ATR images

3) Backward compatibility with, and new advanced remote

control features.

4) Advanced timings menus with the ability to alter the

bus timings of all core functions.

5) Additional PHANTOM EMULATION modes, as required.

6) Lots of other user suggestions! :)

INSTALLING AND STARTING APE

The SHAREWARE APE package is distributed as a ZIP'ed collection

of the filename APEDxxx.ZIP  Where xxx represents the

version number of that archive.  The archive

contains the following files:

1) APE.EXE      [The main APE executable]

2) APE.DOC      [The APE users manual]

3) PROSYS.EXE   [The FULL OPERATIONAL APE Pro System]

4) ORDER.FRM    [A printable order form for your use]

5) WARP.ATR     [Example of Happy WARP SPEED I/O]

6) APE_WARP.ATR [Example of APE+ 3x WARP SPEED I/O]

7) MYDOS.ATR    [Starter ATR disk w/ MYDOS]

8) *.PRO        [Demonstration PRO Images]

To install APE, change to the directory on your hard disk

where you wish to place APE and unzip the ZIP file by typing

'pkunzip APExxxD.ZIP'

*** HINT: The first thing you should do when you start APE is to

goto the [Z] Configuration Menu and choose option [4]

to configure your Comm Ports.  This will allow APE to

function if your computer uses a non standard IRQ/BASE

address for its comm ports.

B) Starting Ape

Once you have successfully installed APE, its time to start and

configure it for your hardware.   Typing 'APE /?' will give

you a listing of command line options:

����������������������������������Ŀ

Steven J. Tucker    � Atari Peripheral Emulator v1.06S � ���#  E-Mail  #��

9731 Sunrise Blvd #M33 �  Copyright 1995 Steven J Tucker  �  classics@nacs.net

North Royalton, Ohio  ����������������������������������Ĵ ���#  Phone   #��

44133 USA        �       Command Line Switches      �    216-237-5308

�����������������������������������������������������������������������Ŀ

� ? � This Screen!      � APE /?                                        �

� C � Two/One Chip      � APE /C1 [Choose One Chip Operation]           �

� P � SIO2PC Port#      � APE /P2 [Choose Comm 2 for SIO2PC Device]     �

� M � Modem Port        � APE /M1 [Choose Comm 1 for PC Modem]          �

� D � DTR Status[H|L]   � APE /DL [Set DTR on Pc Modem to LOW]          �

� R � RTS Status[H|L]   � APE /RH [Set RTS on Pc Modem to HIGH]         �

� F � Framing Threshold � APE /F9 [Set Framing Threshold to 9]          �

� V � Verbose Monitor   � APE /VN [Turn Verbose Command Monitor OFF]    �

� W � Command Window    � APE /WY [Turn SIO Command Window ON]          �

� S � Set PC Mirror     � APE /S8 [Set Drive 8 to PC Mirror]            �

� Z � Command Filters   � APE /ZY [Turn Command Filters ON]             �

� I � Mount ATR Image   � APE /I2C:\TEST.ATR [Mount TEST.ATR in slot 1] �

� B � Browse Text File  � APE /BC:\ATASCII.TXT [View ATASCII.TXT] (reg) �

� N � No Startup Screen � APE /N  [Skips startup screen] (registered)   �

�������������������������������������������������������������������������

Specifying any of these options on the command line will override

any settings you might store in your startup configuration file.

These are all shortcuts for commands available in the main &

configuration menus.  Their complete functions are covered in

detail in those sections.

After processing of any command line options you may have specified

the emulator will start. You will then be presented with the

status screen.

The use of the '/?' switch will immediately return you to DOS without

starting the emulator.

THE STATUS SCREEN

Atari Peripheral Emulator 1.07

Copyright 1995 Steven J Tucker

9731 Sunrise Blvd M-33 - North Royalton Ohio 44133

�� Drive Activity ��������������������������������������������������������������

�����������������������������������������������������������������������������Ŀ

� � �     Sio Command     � Sector � Total � Dns � P � Protect � Img Filename �

�����������������������������������������������������������������������������Ĵ

� 1 � Happy Read Sector   �  361   � 1040  � Sng � - �   Yes   � HAPPYDOS.ATR �

� 2 � Set Configuration   �        �  720  � Dbl � - �    No   � MYDOS003.ATR �

�*3*� Read Sector         �    1   �  720  � Sng � 3 �   Yes   � APPLE.PRO    �

� 4 �                     �        �       �     �   �         �              �

� 5 �                     �        �       �     �   �         �              �

� 6 �                     �        �       �     �   �         �              �

� 7 �                     �        �       �     �   �         �              �

�*8*�                     �        �       �     �   �         �              �

�������������������������������������������������������������������������������

� Ultraspeed: OFF � Emulation: SMART  �

���������������������������������������

�� SIO Command Monitor ���������������������������������������������������������

This is the main status screen you will see during all disk

operations with the Atari.  All commands are carried out

using the PC keyboard.  Here is a listing of the keystrokes

available from the main menu:

The status screen displays a wealth of information about the

state of your emulated peripheral system.   The screen is as

you would see it with the disk image 'HAPPYDOS.ATR' loaded

in bay 1, and 'MYDOS003.ATR' loaded in bay 2.

The status line for bay 1 shows you have loaded 'HAPPYDOS.ATR'

and the following information:

* Sio Command: The last command to this drive was HAPPY READ SECTOR,

a high speed i/o command.

* Sector     : The sector number that was read

* Total      : The total number of sectors in this image.  In

this case the image is that of a 1050 enhanced

density disk, with a total of 1040 sectors.

* 'P'        : PRO System Emulation Mode.  '-' Indicates this

image is NOT A PRO IMAGE.

* Dns        : Represents the number of bytes per sector in this

image.  Single density sectors contain 128 bytes,

Double density sectors contain 256 bytes.

* Protect    : Indicates if APE will or will not protect your

disk image from writes by the atari.  When you

'write protect' and disk image, no harmful

action can be taken on it by the atari

* Image Fname: The real filename of the image as it resides

on the PC hard drive.

The last command to the disk image mounted in bay 2 was

the SET CONFIGURATION command, which is often used by the atari

in configuring disk drives.

There is no sector number displayed since no sector data was

transferred as a result of this command.

'Ultraspeed: OFF' shows that the last command received on the

serial bus was not sent using Spartados Ultraspeed i/o, or

APE+ Ultraspeed I/O.

'Emulation: SMART' shows that the emulator is currently

handling disk images in its 'SMART' mode.  More information

on this feature is given in the KEYSTROKE COMMANDS section

of this manual.

Any drive that is currently tied to the PC mirror function will

have its drive number in inverse/red text as with

drive 8. (shown here with asterisks only)

Any disk images currently running in the protected software mode

(i.e. a .PRO file was loaded) will have the corresponding drive

number highlighted in inverse purple.  Disk #3 above is a

copy protected version of Apple Bandits.  The number '3' indicates

the current PHANTOM EMULATION mode.

KEYSTROKE COMMANDS

Pressing 'H' from the main menu will display the quick keystroke

reference as shown below.

�����������������������������������������������������������͸

�                 Atari Peripheral Emulator                 �

�                Copyright 1995 Steven Tucker               �

�                                                           �

�     These are the DEFAULT program keystroke commands,     �

�      REGISTERED users may define thier own keystrokes     �

�                                                           �

�                   [ALT-H/H] This Screen                   �

�               [A] ATR/DCM Conversion Utility              �

�    [1-8] Swap Disk Positions  [S] Set PC-Mirror Drive     �

�    [B] Block Configurations   [C] Create New ATR Image    �

�    [L] Load ATR/PRO Image     [Q] Quit Program            �

�    [R] Release ATR Image      [T] Toggle Emulation        �

�    [U] Ultraspeed Toggle      [W] Write Protect Image     �

�    [!] Spawn Your Shell       [Z] Configuration Menu      �

�    [@] Force Concurrent I/O   [|] Force Filesystem Reset  �

�          [V] Browse Ascii/Unix/Atascii Text File          �

�����������������������������������������������������������;

[!]   : Spawn your default shell.  This command will launch

a DOS shell using the program specified in your

COMSPEC dos variable.  If COMSPEC is not set, it will

attempt to spawn C:\COMMAND.COM or C:\DOS\COMMAND.COM.

[|]   : Force Filesystem Reset.  Mostly for debugging.  Closes

all files, resets all pointers and remounts all

images.

[@]   : You can force the program into its concurrent i/o emulation

mode with this command.  This is only useful if while

in your terminal program you are forced to abort

concurrent i/o on the PC side to mount a new

disk image, etc.   More on modems, RS232 emulation and

concurrent i/o later in this document.

[1-8] : You can use these keys to swap disk images around

in their bays.   Lets say for example you have

ENCHANT1.ATR loaded in bay 1, and ENCHANT2.ATR loaded

in bay 2.  After you load side one of Enchanter (which

is stored in ENCHANT1.ATR) it tells you to please

'turn the disk over'.  You can move ENCHANT2.ATR into

bay 1 by simply pressing '12' on your keyboard.

[A]   : DCM -> ATR Conversion.  Before you can use Diskcomm3

images with APE, they need to be converted from thier

compressed archival format into a ATR 'disk image' thats

available for random access.

When your select [A], you will be prompted for a filename

or filemask for the image to convert.  If your specify

no filename, a file selector will appear so you can pick

a single DCM image to convert.

If you sucessfully convert a single DCM image, you will

then be asked if you would like to mount the new ATR

image.

If you specify wildcards (i.e. C:\TEL\DOWN\*.DCM) all the

files matching your filespec will be coverted in BATCH

mode.

[B]   : Block Configurations Menu.  Block configurations are

files that store the current state of all 8 bays

of the disk emulator.  This can be a TERIFFIC time

saver if you often need to load specific sets of

disk images, for games, utilities, etc.  With just

a single command you can mount 8 disk images and

be ready to go!

To create your own block configuration file, use the 'L'

command to load your disk images into the drive

configuration you want to save.  Then select the

'B' option.  You will be asked if you want to Load or

Save a block configuration.  Press 'S' for save.  You

can then choose a name for your block configuration, after

which it will be saved to disk for future use.

Loading your newly created block configuration is now

a breeze!  Just press 'B' then 'L' for load, choose

your configuration from the menu and all the images

you specified previously will be automatically mounted!

[C]   : Create ATR Image.  This is possibly one of the most

complex (and often confusing to the newcomer) part

of using emulated disks.

As previously explained, your new atari disks are not

real floppy disks at all, but MSDOS files that match the

storage capacity of the real physical media.

When you press 'C' to create a disk image, you will first

be prompted to choose the 'disk size' for your image.

A number of pre-defined sizes are provided for your

convenience.  These sizes are exact matches for real

physical disks/drives that are commonly used on the

Atari 8 bit.

Even though you may want small disk images for copying

your real physical disks to (for a perfect size match),

you can create disks as large as 16 *MEGABYTES* each!

Using advanced DOS systems like MyDOS and SpartaDOS you

can format and utilize disks up to the Atari addressing

limit of 16 megabytes (65,535 double density sectors).

If you are using a early dos, such as atari dos 2, 2.5, or

other non-enhanced dos you should stick with the

smaller sizes until you are more familiar with the

creation of images.

Once you have chosen a size for your disk image (choose

#1 if this is your first time) we have to pick out

a name for your 'disk image' file on your PC.

I recommend you use a filename that describes the

contents of the disk image well, such as 'MYDOS.ATR' for

your MYDOS master disk, etc.

If you wish to overwrite an existing disk image, you can

simply press RETURN at the filename prompt and a file

selector box will appear.  You may use this file

selector to quickly pick the ATR you want to overwrite.

CAUTION: IF YOU OVERWRITE A DISK IMAGE USING THIS

FUNCTION IT CAN NEVER BE RECOVERED, USE EXTREME

CAUTION WHEN ERASING OLD DISK IMAGES!  IF IN DOUBT

CHOOSE A UNIQUE NAME FOR YOUR DISK.

Once you have selected a filename you will be prompted

one last time for final confirmation.  You will also

be shown various information about the image that

will be created, such as real msdos file size, if the

file exists, and its full pathname.

Pressing 'Yes' when prompted will begin the creation of

the disk image.  This is very dependent on speed

of your machine, its disk caching, and your hard

drive throughput.  A bar graph will display the progress

of the creation.

Small disk images are created very quickly, where as

very very large disk images such as 16 megabytes

can take a few minutes (though this is rare, it takes

15 seconds on my 486/100).  Installing a disk cache

with 'write caching' activated will greatly increase

the speed of this function, and also improve the

performance of the emulator greatly.

Once successfully created you will be asked if you wish

to mount the image you just created.  A great time saver

over having to manually load the image after creation.

FORMATTING YOUR DISK IMAGE

Note also that a newly created 'disk image' is the same

as a blank diskette you would bring home from the

supply shop.  Its totally blank and must be 'formatted'

using your favorite atari DOS before it can be used.

FORMATTING WITH ATARI DOS 2.x

If you disk is a standard size, such as 720 sectors &

single density it can be formatted with any standard

atari DOS.   If you are using Atari DOS 2.0, you may

only use disks of 720 sectors/single density.  Atari

DOS 2.0 is rather stupid, and refuses to believe that

any other size disk might exist.

If you are using dos 2.5, you may create images of

1040 single density sectors (called ENHANCED DENSITY by

most people).  You may then format them with Dos 2.5's

'I' command.

To make use of VERY large disks (virtual atari

hard drives you might say :) you will need an advanced

DOS system such as SpartaDOS or MyDOS.  I recommend

if you use your atari more than very infrequently that

you get one of the DOSes.  They are not only capable of

using standard sizes (720, 1040, etc sectors), they

can manage images of ANY size up to 16 megabytes.

Both of these fine products are SHAREWARE, and can

be downloaded from ftp://atari.archive.umich.edu in the

8Bit/Os directory (internet). If you use them please

register them.

Lets assume you created the grand daddy of disk images, the

16 meg monster :)   Formatting procedures for both DOS

systems are as follows:

FORMATTING WITH MYDOS 4.5x

Boot MYDOS and mount your mega disk image in bay 2.

APE will show the image as having 65535 DD sectors.

Choose option 'O' from the Mydos main menu, configure

drive.  We need to tell MYDOS that the disk in 'drive'

one is bigger than your average disk (heh).

Enter '2' for the drive number.

Enter 'Y' for 'is drive configurable'

Enter 'Y' for 'is drive a HIGH CAPACITY drive'

Enter '65535' for number of sectors

You should now be back at the main menu.  Notice at the

top of the screen that it says '2H'.  Mydos now

understands drive 2 is a high capacity drive.

We now need to FORMAT the 'disk' to make it ready for

DOS use.  Press 'I' at the MYDOS main menu and

format the disk (not ENHANCED density).

To verify the disk was formatted Ok press '2' at the

MYDOS main menu for a listing of files on drive 2.  It

should read well over 65,000 free sectors.

Now choose option 'H' and write the MYDOS disk operating

system to your image.    Once completed you are

ready to boot off your virtual hard drive!  Yeah!

FORMATTING WITH SPARTADOS

Boot SpartaDOS and mount your mega disk image in bay 3.

APE will show the image as having 65535 DD sectors.

Spartados is a modular DOS system, its utilities

functions being stored in separate executable files

on the main system disk.  To initialize a disk image

this large, you will require the Spartados

utility 'HDINIT' (Hard Disk Init)

Run HDINIT.  It will ask you which hard disk you want

to initialize, 3 or 4.  The program apparently only

thinks disks this big would be on those drive

numbers, so for now its important your image be

loaded in bay 3. (or 4)

It will ask you if you're sure, and once confirmed

will proceed to format and write the SpartaDOS

FAT to your disk image.   There apparently is

another program needed to make the disk image

bootable, but I don't have it on my SpartaDOS

disk, so I cant advise you as to its use.

Without DOS on your disk, you will have to leave

your 16 meg image mounted as one of drives 2-8, and

boot your SpartaDOS system off a separate disk

image in drive 1.  Spartados can still read and

write the disk image, so you can store your files

on it, but you must use another disk for booting.

[H] Help. Displays the quick keystroke reference chart.

[L] Load Image.  This function is used to select an already

created ATR image file and mount it in the drive 'bay' of

your choice.

You will first be prompted to select the bay # for the image

your going to load.  If an image in already using that bay

you will be prompted to confirm before displacing it.

Once you have selected the drive bay, you will then be

asked for the filename of your disk image.  If you know

the filename and want to type it in you may, or you can

just press RETURN for an easy to use file selector with

which you may browse your ATR images.

After you select the name of your image, APE will take

care of everything else such as reading the image

configuration, etc, and mount it for you in your selected bay.

PROTECTED DISK SOFTWARE

APE can now emulate ALL copy protection, even the most

complex 'Phantom' sector schemes.

Using a special program to read your protected disks, APE

can create .PRO disk images.  This is a new form of

disk image, incorporating timing, error and other

information critical to emulating the copy protection

present on the orignal diskette.

APE can currently emulate defective sectors and the resultant

contents of the FD1771 status register when reading them. APE

can also emulate protection schemes that do not produce

controller errors, such as the Phantom Sector protection

used on Electronic Arts titles.

APE has been tested with the following software originals and

has performed perfectly.   These PRO images are not

'cracked' copies of games, but contain information

representing the real copy protection on the original

diskette:

Alternate Reality, The City (Phantom Sector Protection)

Archon                      (Phantom Sector Protection)

7 Cities Of Gold            (Phantom Sector Protection)

Agent USA                   (Standard Protection)

Ultima ][                   (Standard Protection)

Jumpman                     (Phantom Sector Protection)

Koronis Rift                (Phantom Sector Protection)

Apple Bandits               (Standard Protection)

The Edilon                  (Phantom Sector Protection)

Spy vs Spy: Artic Antics    (Phantom Sector Protection)

Spy vs Spy                  (Standard Protection)

Gumball                     (Standard Protection)

Karateka                    (Phantom Sector Protection)

As other images are reported to work by registered users,

they will be added to this list.  Based on the complexity

of the copy protection used by the above companies, I

am confident that APE can emulate 100% of existing

software.

[Q] Quit program.  Unmounts all images/drives and exits to your

operating system.

[R] Release Image.  This function will unmount your ATR image

file from the bay (drive) you select and make that slot

then available for other disk images if you wish.

[S] Set the PC MIRROR drive.  The PC MIRROR is by far one

of the most powerful features available to you in APE.  When

you mount at drive as a 'PC MIRROR' that drive acts like

a window to the files that actually reside on the MSDOS

portion of your PC hard drive.

Where before it was very difficult to get files from your

PC to your Atari, using null modem cables, modems, terminal

software, etc;  You can now access these files DIRECTLY!!

Lets try an example of this powerful function.  Boot MYDOS

4.5x and once at the main menu choose APE option 'S' and

mount your PC mirror drive in bay 2.

Now, press '2' on the atari keyboard, you will get an

atari directory of the files in the root directory

of your PC hard drive!  APE takes the information from

your PC files and makes your atari THINK they are on

a real atari disk!

Lets try and copy one of your MSDOS files to the atari.  For

this example we will use AUTOEXEC.BAT since most anyone with

MSDOS will have this file.

Choose MYDOS option 'C' (Copy file) and then enter

'2:AUTOEXEC.BAT,1'.  This will tell MYDOS to copy file

'AUTOEXEC.BAT' off drive 2, and put it on drive 1.

Its just that easy!  The entire contents of your PC hard drive

are now at your fingertips!

Note: Files on the pc mirror are sequential access only!  If

you wish random access to a file on the PC MIRROR for

editing, etc, simply copy it to an ATR drive.

NAVIGATING PC DIRECTORIES IN THE PC MIRROR

Getting a directory listing of the root directory of your

PC will show not only the files there, but also the

subdirectories on your hard drive.

One of my main goals when creating APE was to make it

DOS independent.   SpartaDOS, MYDOS, etc all use separate

methods for storing information in subdirectories, but APE

will work with them all.

To change to a subdirectory on your PC MIRROR drive, all you

have to do is to make the atari access that 'file' somehow.

Its just that easy.

If you using MYDOS, use the 'L' command and 'load' the

directory.  You will receive an error, which is *NORMAL*

and expected. APE is telling MYDOS that it cant load that

file and mydos goes back to its main menu.   Now press

'2' once again. WOW!  APE has changed directories and your

now looking at the subdirectory you selected on your

PC hard disk.

If your using SPARTADOS all you need to do is type the

name of the directory (you may need to add a '.' to the

end of the name if there is not extension, i.e. 'DOS.')

and APE will do the rest.  After you receive the

error message, type 'DIR' once again and you will see

APE has changed directories for you! :)

Also note that the subdirectory you are in contains a

directory called 'UP'.  'L'oading or otherwise accessing

this directory will cause the PC MIRROR to go UP the

MSDOS directory tree to the parent (or root in this case)

directory.

WRITING FILES TO THE PC MIRROR

Registered users may now WRITE files directly to the PC

mirror using MyDOS.  To transfer a file to the

current PC MIRROR directory from your Atari all you

need to do is use the MyDOS (C)opy command and copy

your file to the PC MIRROR drive.

NO SPECIAL SOFTWARE OR DRIVER is required for this

function!  APE takes care of EVERYTHING for you, once

the copy operation is complete, a file of the same name

and size will exist on your PC hard drive! :)

This advanced feature can make file transfer problems

between your PC and atari a thing of the past!

[T] Toggle Emulation.  This command switches APE between its

'SMART' mode of emulation, and its 'STUPID' mode.  This

toggle is useful for booting some programs that refuse

to run on 'enhanced' drives.  This switch may be obsolete

since the addition of full support for Happy Warp Speed I/O.

It was really only useful with SUPERDOS which now works

perfectly in Happy Warp Speed Mode.

SMART MODE

APE operates at full tilt, all heuristics are enabled and

APE will accept the 'FORCE ULTRASPEED' command from

SpartaDOS.

STUPID MODE

All Ultraspeed SIO heuristics are disabled and APE will

refuse any request to switch into an enhanced drive

emulation mode.  APE is most like a stock 1050 disk drive

when used in this mode.

[U] Ultraspeed Toggle.  This command will FORCE APE to switch

the method it uses to talk on the SIO bus.  Normally

APE uses advanced heuristic methods to automatically

determine what speed devices on the SIO bus are talking

at, but if you encounter a problem with SpartaDOS

desyncing you can use this command to force the switch.

This command is left over from the early beta test versions

of the program, and since I installed the new syncro code

and switching logic I've never had to use it again.

[V] View Atascii/Ascii/Unix Text File.   You can browse any

text file with this command.  Its is a completely integrated

version of my Multitype file browser.  Once you select

the file to view, APE will attempt to decide what type

of text is it automatically (atascii, etc).  If APE

fails to correctly display your text (rare) you can

toggle the format manually with the [F] command inside

the browser.

You may browse multiple files once the browser has been

started by selecting the [N]ew filespec option.  Press

ESC to exit the browser.

Registered users may start the browser program from the command

line, as a stand alone program within APE.

[W] Write Protect Image.   This function marks your ATR image

as write protected, and affords it the protection offered

by real physical write protection on floppies.  Once

write protected, any writes to the image will be totally

ignored.  The Atari cannot damage or write to the image in

any way.

Write protect is a toggle, and your images may be unprotected

by using 'W' on them again.

NOTE: This does not protect your images from being damaged

by the MSDOS operating system, only by APE.  As always, keep

appropriate backups of your data.

CONFIGURING APE

[Z] Configuration Menu.   This is used to not only setup APE

for proper operation with your system, and to setup

your quick Function Key macros.

When activated, the configuration menu will give you 5

choices as shown below.

���������������������������������Ŀ

�        Configuration Menu       �

���������������������������������Ĵ

�     1) General Configuration    �

�     2) Printer Configuration    �

�   3) Default Path/Quick Macros  �

�    4) Set COM IRQ & BASE Addr   �

�     5) Configure Ultraspeed     �

�                                 �

�       Or Press ESC to EXIT      �

�����������������������������������

1)  General/Serial Configuration Menu

�������������������Ŀ

���������������������������Ĵ APE Configuration ����������������������������Ŀ

� Atari Perpheral Emulator  ���������������������    (c)1995 Steven J Tucker �

������������������������������������������������������������������������������

�        � Serial Core �        �        Keystroke Definitions        �

�        �������������Ľ        �������������������������������������Ĵ

� 1) SIO2PC Device Type: 2 Chip � A)               � P)               �

� 2) SIO2PC Device Port: Comm 2 � B)               � Q)               �

� 3) PC Modem Comm Port: Comm 1 � C)               � R)               �

� 4) PC Modem Baud Rate: 57600  � D)               � S)               �

� 5) PC DTR Line: HIGH          � E)               � T)               �

� 6) PC RTS Line: HIGH          � F)               � U)               �

� 7) HW Flow Window: 1000       � G)               � V)               �

� 8) Framing Threshold: 4       � H)               � W)               �

�������������������������������Ĵ I)               � X)               �

�      � User Interface �       � J)               � Y)               �

�      ����������������Ľ       � K)               � Z)               �

� 9) Command Monitor: On        � L)               � !)               �

� 0) Verbose Drive Monitor: On  � M)               � @)               �

�������������������������������Ĵ N)               � #)               �

� %) Command Filters: On        � O)               � $)               �

� ^) Filter: $52  &) Filter: $00���������������������������������������

� *) Filter: $00  -) Filter: $00�      ^S = Save Configuration

� +) Filter: $00  =) Filter: $00�      ^L = Load Configuration

���������������������������������    ^D = Reload Program Defaults

This screen controls the operation of most serial core functions,

the hardware flow control for concurrent i/o, the sio command

window, the verbose command monitor, sio command monitor filters,

and registered user keystroke definitions.

[1] Sio2PC device type.  This tells APE what kind of serial

conversion cable you are using.  Older versions of the

cable (and better IMHO) use a 2 IC setup which requires

manual control over the serial bus tri-state.  Its very

important you set this correctly. If you have a 2 chip

device and you set it to 1 chip, it will fail to

function correctly or will cause interference with

other, real SIO peripherals.

[2] Sio2PC device port.  This is the Com port to which your

serial conversion device is attached.

[3] PC Modem comm port.  This is the Com port on which your

pc modem is attached/configured.  Both internal and external

modems communicate with the PC via a serial IRQ.  Configuring

this properly will allow APE to use concurrent i/o and

enable your PC modem as an 850 interfaced device.

[4] PC Modem baud rate.  This is the baud rate at which APE will

talk to your PC modem.  Consult your modem manual for

the proper PC -> MODEM serial rate (DTE).  This should

be set to '2400' for most 2400 baud modems, and most

14.4/28.8k modems will accept 57600.

UNREGISTERED USERS are restricted to using 2400 baud

DTE speeds and below.  Please consider upgrading to

the registered version today.  Registered versions

now support up to 19.2k bps communication with the

atari, and future versions will support 38.4k bps

rates!

NOTE: This *DOES NOT* control the rate at which APE

communicates with your Atari via 850 interface emulation.

You should always set your atari terminal software

to 19.2k bps.

Registered users will receive instructions on how to

set up thier terminal software for use at 38.4k and

56kbps.

[5] PC DTR Line.  This controls the state of the DTR line

to your PC modem.  Consult your modem manual for the

proper setting.  Most modems should always be set to HIGH.

[6] PC RTS Line.  This controls the state of the RTS line

to your PC modem during normal operations.  This should

always be set to HIGH unless your modem is very

unusual.  This line is reversed during hardware flow

control to pause the flow of incoming data.

[7] HW Flow window.  This controls the number of bytes the

pc will release to the Atari before allowing the PC

modem to continue sending data, after the FIFO buffer

on the PC side has become full.

A setting of 1000 is recommended, as this keeps the data

flowing very smoothly to the atari at 19.2k bps.  If you

experience problems with timeouts or overruns on the

atari side you can adjust this window accordingly.

[8] Framing Threshold.  This is used by the heuristic

ultraspeed switching routines to decide how quickly

to change modes when new speeds are detected on the

SIO bus.

If APE switches to ultraspeed when it should not, you

may have the framing threshold set too low.  A setting

of below 8 may cause this.

If APE fails to switch into ultraspeed when using mixed

speeds, and you get command timeouts or drive errors, you

may have the framing threshold set too high.

This will depend greatly on the speed of your machine.  I

recommend a setting of 8 for most machines, but if this

causes problems you will need to experiment with this

setting.  Framing thresholds of below 4 or above 15 are

of no use and are not allowed.

[9] Command monitor.  This switch controls the scrolling

SIO Command Monitor at the bottom of the Status screen.

If your machine is slow this will help increase the

speed of the emulator.

[0] Verbose command monitor.  This switch controls the

display of the command names/sector numbers for each

SIO command received.  Turning it off can help speed up

the emulator on very slow machines.

[%] Command filters.  Command filters are used to eliminate

the display of commands you dont wish to see from the

scrolling SIO command monitor.  I.E. you may wish to

filter out the read sector command [$52] to be able

to see the more unusual SIO commands only.

All other switches in this field control the HEX value of

the filters you wish to use.

[A-Z] Defined keystrokes.  Registered users may change the

default key bindings for any APE function.  This is

to both convience to you, and to allow top level

keystroke backward-compatibility with older software

you may have used.

[Crtl-S] Saves the configuration you have set in this menu, the

printer configuration menu, and the path/macros menu

to the file called APE.CFG in the path from which you

launched APE.

[Ctrl-L] Loads the configuration stored in APE.CFG, in the path

which you launched APE from.  If APE.CFG does not exist

you can create one with the [Ctrl-S] save function.

This file is automatically loaded when you start APE also.

[Ctrl-D] Restore system defaults.  This loads the default paramaters

that are hard coded into APE.  This should be used only if

you damage your configuration and connot figure out how

to reset it.

2) Printer Configuration

��������������������������������������Ŀ

�           Printing Options           �

�������������������������������������Ŀ�

��         Eol Conversion [ ]         ��

��   [N] CR -> CR   [Y] CR -> CR/LF   ��

��                                    ��

��       Global Conversions [ ]       ��

��  [N] 8 Bit Data   [Y] 7 Bit Data   ��

��                                    ��

��     Atascii TAB Conversion [ ]     ��

��          [N] No  [Y] Yes           ��

��������������������������������������ٳ

����������������������������������������

This screen controls the operation of the pc printer.  All

commands sent from the atari to the P: printer device will

automatically be intercepted and printer on your PC printer

by APE.

Values set in this section can be saved to APE.CFG.

a) EOL Conversion.  When the atari prints data, it terminates

each line with the ATASCII (Atari Ascii) EOL character

which is 155.  The PC terminates each line diffrently, with

a CR/LF pair.  Even though this is the standard for PC

text, some printers require only the CR part be sent.

If your printer requires line feeds (such as the HP

deskjet series) then select YES.  If your printouts

are all on one line, text overlapping text then your

printer requires line feeds and this should be turned ON.

b) Global coversions.  Enabling this will strip the high bit

from all characters sent to the printer.  Useful with

some printers that may react badly to ATASCII graphic

characters, etc.

c) Atasii TAB conversion.  The Atascii character set uses a

different value for TAB than the PC and PC printers.

Enabling this will automatically convert these characters

to PC compatible ones.

3) Default Path/Filenames & Quick Macros

������������������������������������������������������������������������������͸

�                              � Default Paths �                               �

�                              ���������������;                               �

� [A] Default Image Path [Drive:\PATH\] :                                      �

� [B] Default Disk Image Filename Extension [ATR, DSK, etc] [   ]              �

� [C] Default Block Cfg Path [Drive:\PATH\] :                                  �

� [D] Default PC Mirror Mask [Drive:\PATH\FILEMASK.EXT] :                      �

������������������������������������������������������������������������������͵

�                          � Quick Keyboard Macros �                           �

�                          �����������������������;                           �

�       Quick Keyboard Macros are literal keyboard strings that will be        �

�          executed from the emulator as if you actually typed them!           �

�   (i.e. if F1 is L8C:\TEST.ATR| then C:\TEST.ATR would be loaded in bay 8)   �

�                                                                              �

�  [F1]                                                                        �

�  [F2]                                                                        �

�  [F3]                                                                        �

�  [F4]                                                                        �

�  [F5]                                                                        �

�  [F6]                                                                        �

�  [F7]                                                                        �

�  [F8]                                                                        �

�  [F9]                                                                        �

� [F10]                                                                        �

������������������������������������������������������������������������������;

This screen is used to set the default paths and filename extensions for

your disk images, the PC mirror default path, and all of your

quick keyboard macros.

Values set in this screen may be saved to APE.CFG.

[A] Default Image Path.   This is the default path which will be

called up by the file selector.  For example, if you set this

path to 'C:\ATARI8\APE\DISKS\' then when you call the file

selector to load an image it will start in this directory.

[B] Default Image Extension.  Although I recommend you always use

the extension ATR on your disk images for easy identification,

you may configure any default extension here.  This is the

extension that will be searched by the file selector.  For

example if you set this field to 'DSK' the file selector

would look for files called *.DSK in your default path.

Setting this field to * allows you to browse for any type

of image. (i.e. PRO Images)

[C] Default Block Configuration Path.  Same as function [A], except

it controls where the file selector will look for your saved

block configurations.

[D] Default PC MIRROR Path/Filespec.  By default the first time you

access the PC MIRROR from your Atari DOS, it will provide a

listing of the files in 'C:\*.*'.  If you would like to change

the directory or filespec it starts at, you may do so here.

For example if you set 'D:\STUFF\*.ZIP' it would start in

D:\STUFF\ and see only files matching *.ZIP.

[F1-F10] Define Quick Macro.  The quick macro feature of APE can be

a very powerful time saver.  By allowing you to enter

a series of keystrokes to be executed on the function

key of your choice, most any feature of APE (and even in

the DOS shell!) can be automated!

All characters entered in your macro will be executed

EXACTLY as if you had typed them from the PC keyboard.

The only exception to this is the '|' character which

is used for a embedded carriage return.

Any function that requires a RETURN when done should be

trailed by the '|' character since APE will not press

return for you be default.

For example:  Setting F1 to the string 'Z3' will cause

the emulator to jump to configuration screen #3 when

F1 is pressed at the status screen!

This feature can also execute functions OUTSIDE APE!  You

can use the '!' function to shell to DOS inside your

macro and any remaining keystrokes will be executed by

your dos shell.  (i.e. '!EDIT AUTOEXEC.BAT|' would cause

APE to shell to DOS and execute EDIT AUTOEXEC.BAT)

All quick macros are executed from the main status

screen only. To access deeper functions simply add them

to your macro. (i.e. 'L1C:\TEST.ATR|' to tell APE you

want to load an image, then in Drive 1, then the filename

and RETURN to load it)

** NOTE ** Many XT class machines lack the BIOS hooks

reqired to use this function.

4) Comm IRQ & Base Address

������������������������������Ŀ

�    Configure RS232 Ports     �

������������������������������Ĵ

� Comport � IRQ � Base Address �

������������������������������Ĵ

�    1    �     �              �

�    2    �     �              �

�    3    �     �              �

�    4    �     �              �

������������������������������Ĵ

�  Configure Which Port (1-4)  �

������������������������������Ĵ

�  To disable a COM port, set  �

�  it's base address to $0000  �

������������������������������Ĵ

� WARNING: Selecting an IRQ or �

�base address that is incorrect�

�  can result in your machine  �

�  locking up or acting in an  �

�        erratic manor.        �

��������������������������������

The above menu will allow you to confure APE to function with

the IRQ/BASE address settings used on your computer.  APE

is pre configured with the 'standard' AT values for these

ports, so you should only need to use this function if APE

fails to perform properly for you.

To find the IRQ/BASE address of your com port use the MSD.EXE

utility provided with MSDOS v5 and above, or consult your

users manual.

You may totally disable any Com ports you wont be using by

setting its BASE ADDRESS to $0000.  This is normally NOT

required, but may help if you are running strange TSR

software, etc.

5) Configure Ultraspeed

����������������������������������������������Ŀ

�             Configure Ultraspeed             �

�                                              �

�   1. Force 1x Rate [1x Normal SIO Always]    �

� 2. USDoubler Ultraspeed [2x Normal SIO Rate] �

�   3. APE+ Ultraspeed [3x Normal SIO Rate]    �

�                                              �

�         Current Ultraspeed Type [ ]          �

�                                              �

� WARNING: APE+ Ultraspeed functions at 3x the �

�  normal SIO rate.  This has been tested and  �

�   found to be totally stable under version   �

�   3.2g of Spartados.  Earlier versions of    �

� Spartados cannot maintain the high transfer  �

�  rate, and may result in choppy disk IO or   �

�            corrupted disk images.            �

�                                              �

�  APE+ Ultraspeed WRITES are available only   �

�         to REGISTERED users of APE.          �

������������������������������������������������

The above menu will allow you to tell APE what speed it

should operate at with Spartados and other programs that

use the US Doubler enhanced warp speed mode.

This option effects ONLY the operation of USdoubler type

I/O and has NO EFFECT on the operation of other high speed

systems such as Happy Warp Speed or XF551 mode.

Options in order are:

1) Force 1x SIO operation in Ultraspeed mode.  This will

force APE to *NEVER* switch into 2x or 3x ultraspeed

mode, even when asked to.  This option should NEVER

be used unless you have very serious problems with

Ultraspeed I/O. (Please contact me if you do!)

2) Set 2x SIO Ultraspeed operation.  This is the normal

USDoubler ultraspeed mode, and operates at 2x the

standard SIO rate.  This is the speed you normally

see with enhanced disk peripherals such as the

USdoubler, Happy Enhancment, etc.

3) Set 3x APE+ Ultraspeed.  APE+ 3X SIO operation is a

feature NEVER BEFORE available on the atari computer. It

allows your Atari computer to commuicate with APE at

3x the normal SIO rate.  Thats 1.5x faster than

Happy, XF551, and USDoubler drives!

No special software or patch is required to use APE+

Ultraspeed.  Spartados 3.2g will AUTOMATICALLY switch

to this mode at bootup when APE instructs it to.

APE+ IO pushes Spartados to its serial speed limits.  You

should use FTE's Spartados 3.2g (earlier versions may

not be fast enough) and have KEY turned OFF.  Having

KEY turned on slows the Atari to the point it cannot

keep up w/ APE+ 3X SIO.

USING YOUR PC MODEM WITH APE

Having your inexpensive PC modem do double duty on your

your atari is now possible!  You may use any program

that is compatible with the Atari 850 interface. No

additional software or hardware is required!

For this example we will use BOBTERM.  If you already

have bobterm make sure you use the bare version of the

program that does NOT have an R: handler appended to it.

Load BOBTERM using your favorite, upon loading BOBTERM will

'call' for the 850 interface handler.  APE will download

the handler and bobterm will finish loading.

Notice your APE screen now says 'Concurrent I/O running', it

has been commanded by BOBTERM to start talking to your

PC modem!

Make sure BOBTERM is set for 19.2k bps and then go ahead and

enter terminal mode.  You should be able to type commands

to your PC modem just as if it was attached through an

850 interface!

If you cant seem to talk to your PC modem go back to

the configuration section and review the instructions for

setting up your modem with APE.  Also, make sure bobterm

is set for 19.2k bps.

Unregistered Users Note: You also should set your terminal

rate to 19.2k bps but your actual throughput will be

restricted to 2400 bps until you register.

USING YOUR PC PRINTER WITH APE

First review the above configuration section on setting up

your printer for use with APE.  Once set up properly

all commands sent through the atari P: (printer) device

will be intercepted by APE and sent to your PC printer!

COMMON QUESTIONS AND ANSWERS

Q: Why is APE 'crippled'?

A: APE is provided in its current form for your evaluation.  Even

in its UNREGISTERED form, it provides unmatched power,

speed and flexibility.   You are given free of charge the

ability to 'try before you buy' all of APE's powerful

features, such as the PC MIRROR, Copy Protection Emulation,

R: Modem emulation, 3X SIO operation, unmatched hruristic

Ultraspeed speed switching, Diskcomm3 compatibility,

and much more!

Q: I created a .PRO image of 'Game X' and it will no run under

APE.

A: If you are a registered user, and cannot get your title to

run under APE using all the available emulation modes, it

it possible I can add a new emulation mode that can

deal with this new scheme.  Please send the original

or 100% functional copy to me at the address listed in

REGISTER.DOC and I will do my best to make it part of the

next APE release.

Q: When I create disk images with APE, dos shows them all

as having 000 sectors free!

A: When you create a 'disk image', it is the same as a real

floppy disk you bring home from the store.  Its totally

blank and needs to be FORMATTED with your favorite DOS

before it can be used.  See the above section on creating

disk images for more assistance.

Q: How can I boot my Atari if I dont have a real disk drive?

A: With the serial conversion device attached to the SIO port

of your Atari computer, the atari cannot tell APE from

a real Atari disk drive.  If you mount an image in drive 1

and turn on the Atari it will boot from that 'disk' just

as if it were a real drive.  Ever better because APE is

faster than a real disk drive.

Q: Will APE overwrite or format my PC hard drive?

A: NO.  Ape stores all its data and disk images in regular

PC files like every other MSDOS application and will not

harm, alter or otherwise interfere with the normal

operation of your filesystem.

Q: Can I use real hardware in combination with APE?

A: APE can cooperate fully with your existing SIO hardware such

as disk drives, cassette drives, etc.  Just make sure you

dont assign an APE drive to the drive number of a REAL disk

drive that is turned on.  If both APE *AND* the real disk

drive try to answer on the SIO bus at the same time it will

confuse the atari and cause an error.  Your serial conversion

cable should be the last device on your SIO chain.

Q: All my data is on floppy disks, how can I convert them to

ATR images?

A: If you wish to move your data to faster more reliable ATR

images all you need is a floppy drive capable of reading

your disks.

1) Set up your REAL floppy disk drive as Drive 1.

2) Create a new ATR image that matches the

physical size  and density of your real disk using the

'C' function of APE.  Mount this image in bay #2.

3) Using a SECTOR COPIER program such as MYCOPIER, DISK WIZARD

or FASTCOPY copy the REAL diskette in drive 1 to the

emulated diskette in drive 2.  (I suggest you use MYCOPIER

since it supports ULTRASPEED high speed I/O)

The disk image in bay #2 is now an exact copy of the real

physical disk in drive 1.  You may use it in the same

manner (except you cant play Frisbee with it anymore :)

This can obviously be a time consuming process. See later

in this document, or in ORDER.FRM for information on having

your disk images converted for you.

Q: When I try to access drives higher than #2 I get error 160.

A: Atari DOS 2.0 and other early dos systems were not by default

capable of addressing disk drives higher than 2. (At $1000

each who had more than 2 disk drives back then anyway? :)

You can attempt to patch your DOS.SYS file using the

instructions available on many BBS's, or you can switch

to a superior dos system.  I suggest MYDOS because it is

DOS 2 compatible.

Q: After using the '!' command to exit to DOS, I cant run APE

again.  I get an error message!

A: When you use the '!' command to shell out to DOS, APE does

not actually terminate.  APE places itself into the background

and lauches a new copy of your command shell.  To return

to APE after using the '!' command, type 'EXIT' then return.

Q: Do I need to save my disk images before I leave APE?

A: No.  Any time the Atari writes to a disk 'image' APE

updates that file automatically.

Q: Can I use my copy protected software on APE?

A: YES!  All copy protected software can now be emulated

sucessfully with APE!  Using the Pro System included and

a stock 1050 (or 100% compatible) drive, you can not

create .PRO images of even the most complex diskettes.

See 'PROTECTED DISK EMULATION' in the APE users manual for

complete information on .PRO images.

TECHNICAL SUPPORT

Though technical support is not restricted to registered

users, they will be given top priority.  Please review the

documentation and common questions sections also.

You may contact me in the following ways.

Internet E-mail: classics@nacs.net  [this is the preferred addr]

dh395@cleveland.freenet.edu  [emergency backup]

Telephone: 216-237-5308.  Anytime you can catch me at home

is pretty much ok.  Just try and call before 11pm EST.

Snail Express: Steven J Tucker

9731 Sunrise Blvd #M-33

North Royalton, Ohio  44133 USA

PROGRAM DISTRIBUTION

APE is user supported. This means that you may copy it freely

and give the copies away to anyone you wish. They are in turn

requested to send in the registration if they decide to use it.

You can help me distribute APE to others who might find it

useful by uploading it local BBSs and by giving copies of it

to friends, user groups, and business associates.

Thank you for supporting the Atari Peripheral Emulator and the

Shareware concept.

NEW FEATURES/PROGRAM HISTORY

This version of APE has many new features not available in the

previous beta test releases.

11/20/95 Added or improved since APE 1.06:

* Most of the changes I had noted were lost when I had a

file corruption problem.  So anything new thats not

listed here... :)

* Screen blanker was causing many systems to crash after loading

APE.  It has been removed.   Other more stable freeware

screen blankers are available anyway.

* Old .PRO format totally scrapped.  Old .PRO riders are no

longer compatible with APE and will not be supported in

the future.

* New .PRO v2 image format created.

* Coded or re-coded about 1000 lines of code to install

full dual ATR/PRO compatability.  First public release

in 1.07.

* Bug causing ignored writes to disk images for many users has

hopefully been eliminated.

* Beta .PRO image creator coded and made avilable to registered

users.

* APE now 100% functional in dual ATR/PRO mode.

* Added multiple compatibility modes for PRO images, now

allows all my copy protected software to run!!! :)

* Added compatibility timing to PRO v2 format.  Disks which

require reads at normal 1050 speed (300 rpm) can now be

emulated.

* Added sector overrides for all .PRO functions.  Each sector

can now be modified individually (if desired) to bypass

any of the global .PRO settings.

* Minor improvments/bugfixes for the file selector

* Minor improvments/bugfixed for the text browser

* Misc improvments to the core routines

* Pro System 1.02b now available to the public.

* Plans for the PRO CABLE added to APE.DOC

9/2/95   Added or improved since APE 1.04:

* Titles added to file selector

* TAB can now be used to edit file selector path/filespec

* Misc HELP screen fixes

* Found memory leak in load disk image routine, fixed.

* Added first Diskcomm3 -> ATR conversion code

* DCM -> ATR converter now part of APE program, option [A]

* DCM -> ATR converter can now handle batch processing

using wildcards

* DCM -> ATR converter now asks if you want to mount the

image you just converted (if you convert only 1 image).

* Integrated new COMM routines, speed and reliability improved

* Verified HAPPY HS I/O w/ Diskcomm3, ok.

* Verified Ultraspeed I/O w/ Bitter Reality Demo, ok.

* APE now rejects corrupt/invalid ATR files

* HAPPY SECTOR COPY from the Happy System Disk verified to

work ok.

* Bug in Happy write routine found and fixed.

* Bug in computation of high sector numbers found and

squashed.  Nobody complained about this, anyone out

there? :)

* Added UART/COMM configuration screen and code, now available

in [Z] Configuration, [4] Option 4.

* UART/COM settings now saved in APE.CFG

* Eliminated '.' from file selector, useless entry and its now

easier to navigate using the browser.

* Added '>' and '<' commands to traverse directory tree, as

requested for spartados users.

* PC Mirror updated to work a 2x speed w/ Happy HS I/O

** PC Mirror now compatible with Spartados. Can now copy files

of any size from the PC mirror into spartados using COPY,

XCOPY, etc.   Spartados still has a bug in its i/o routine

but i found a way around it.

* Happy HS I/O writes were still causing jumps to ultraspeed

on occasion.  Found a problem in the timing code and its

now a very solid routine.

* XF551 high speed I/O didnt work most of the time, again nobody

reported this monster bug.  Fixed and verified with XFTOOLS.ZIP.

* Much of APE recoded and optimized. Many sections were in

the orignal beta 'just make it work' form and were real

electronic elephants.  A few sections of non time-critical

repetitive code centralized. Code and data segment sizes reduced

signifigantly and program speed increased.

* Started recoding I/O routines for 3x SIO.

* Recoding of 3x SIO routines finished.

* Patched WARP.COM to allow 3x SIO speed, now renamed to

APE_WARP.COM to avoid confision.  Verified reads and writes

with MyDOS 4.53.

** Added support for 3x SIO speed using Spartados 3.2g with

NO DRIVER REQUIRED.  Spartados now boots in APE+ (3x SIO)

mode automatically.

* 'Force' in the help screed was spelled 'Forse', duh.

* Added Spartados Configuration Menu (Cfg [Z] Option [5]) to

allow selection of default Spartados speed for very slow

PC machines that cannot handle 3x SIO.

* 3x SIO verified NOT to work with Spartados versions 3.2d and

earlier.  Spartados SIO routines in these versions cannot

keep up with APE+ SIO and get choppy.

* Cursor turned off, guess it annoyed some folks just sitting there

and blinking.

* Added 'APE+' display to indicate 3x sio running.

* APE+ SIO (3x) verified to work w/ 3.2g in single and double

density.

* APE+ SIO (3x) verified to work at 3x speed w/ bitter reality

demo.   Any program that follows the Spartados model should

work w/ APE+ IO.

* APE+ SIO (3x) verified to work with MyCOPIER at 3x SIO speed.

* APE+ SIO (3x) verified to work w/ Diskcomm3

** Added support for 'simple' PROTECTED DISK images.  Protection

information is stored in a 1k file of the same name with

the extension '.PRO'.   Sample disk images:

APPLE.ATR/PRO - Apple Bandits protected version

SPY.ATR/PRO - Spy vs Spy protected version

* Fixed a bug in the DCM->ATR conversion code, leftover

junk from the standalone version eliminated.

* Unregistered users restricted to only 2 command line

switches.

* Fixed PC MIRROR to allow APE+ (3x) sio access.

[The following code is complete, but is not included in APE 1.06

because without the ability to use the READ/WRITE TRACK commands

properly, they are not really useful except for show.  When I

get a Happy/Archiver drive to test with, these may be included]

* Happy Controller Ram emulated.

* HAPPY System Disk now recognizes APE as HAPPY w/ Controller

Option installed.

* Happy Tracer Buffer emulated.  HAPPY system disk now recognizes

its presence and displays it contents.

* Happy code buffer emulated and high speed transfers

working.  Able to completely load 'BACKUP ANY DISK' and

accept drive programming.

* Happy move head command recognized

* Archiver READ TRACK command recognized, only response figured

out so far indicates a totally empty track.

* Archiver FORMAT TRACK command accepted.

* Screen blanker added, now blanks screen after 2 minutes of

keyboard and SIO inactivity.

* Ultraspeed timing tightened, better performance.

Added or improved since public beta 8:

* Bugfix for 1050 formatting and status routines

* Happy high speed reads now supported

* New syncro code added, great improvment!

* Better swapping method, you no longer have to press the

existing image first, either is fine.

* Happy high speed write support added

* Some extended Happy commands now supported

* Write protection of images now effective

* Command line options added

* User now asked to mount image after creation

* 850 Interface serial emulation now working at 9600 baud

* Scrapped 850 interface code and starting rewriting

some public domain R: code.

* Automatic switching between command and concurrent i/o mode

now working

* PC MIRROR directories are now built on the fly

* Support for 43/50 line screens installed

* 850 Interface R: emulation now working at 19.2k bps

* Added General/Serial configuation screen

* Added command filters

* Added slow machine display options

* Added user definable framing value

* New streamlined core serial i/o routines.  Much faster now.

* Now recognizes many more extended happy commands

* Added hardware flow control for PC modem, now works

automatically.

* Added user definable flow control threshold

* Added the ability to load and save block configurations

* Added enhanced command line help and registered CL features

* Added Path/Macro configuration screen

* Added support for default paths and file extensions

* Added support for quick macros, and editing code

* New consolidated configuration menu, 'I' function for printer

configuration now in this menu.

* Improved and incorporated my multi-type text browser into

APE.  Can also be called from the command line. (reg)

* Improved file selector, now has title and user can enter

own path/filespec using the TAB key.

REGISTRATION AND ORDER FORM

Send to:  Steven J Tucker                 Orders: (216) 237-5308

9731 Sunrise Blvd #M-33

North Royalton, Ohio  44133       All amounts in U.S.$

USA

Single Computer License Includes:

1. Enhanced version of APE (including FULL R: and .PRO support!)

2. Choose APE Disk Size: 5.25_____ 3.5_____

3. Priority Tech Support (E-Mail, Phone, Mail)

4. Notice of Major Upgrades

5. More!  See REGISTERED BENEFITS section

ITEM DESCRIPTION                             PRICE       QUANTITY

------------------------------------------------------------------

Registered and Enhanced version of APE ..... ONLY $39.99  [      ]

Serial Interface Cable ..................... ONLY $59.99  [      ]

Pro System Interface Cable ................. ONLY $59.99  [      ]

$5 Discount on APE per cable purchased......     -$ 5.00  [      ]

Diskette to ATR Conversion ................. See Below    [      ]

U.S.A. .......... $5

Canada .......... $6         SHIPPING CHARGES  ___________________

All others ...... $9

Floppy disk to ATR conversion service.  You must purchase at

least 10 sides of conversion to use this service. Your ATR images

will be delivered to you on 1.44M PC floppy disks along with your

original diskettes. The fee for converting your 5.25" floppy

disks to ATR images is as follows:

Single Density ....... $1 per SIDE  [Disks ( ) x Sides ( ) x Price]

1050 Density ......... $1 per SIDE

Double Density ....... $2 per SIDE

Please include complete instructions for processing your

images (ie. what sides to convert, etc) or your order may

be delayed.

Note on payment: Canadian and overseas users must pay by

check or money order PAID IN US DOLLARS AND DRAWN DIRECTLY ON A

US BANK.  In Canada, a postal money order in US DOLLARS is

acceptable.

REMEMBER TO ADD SHIPPING CHARGES ........ TOTAL  _____________

================================

Contact  ________________________________________________________

Company  ________________________________________________________

Address  ________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

Phone  _________________________   FAX  _________________________

How did you get your copy of APE?

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

Suggestions or Comments:

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

BUILDING THE APE Pro System INTERFACE YOURSELF

Here is a netlist of connections and a parts listing for

building the APE Pro System cable that allows you to connect

your PC to the Atari SIO bus.

PC->SIO Chain Interface

Materials Required        Source

------------------        ------------------

Any general purpose PCB.  (Radio Shack)

1/2 of Cut SIO Cable      (American Technavision, B&C, Etc)

IBM Serial Connector      (Radio Shack)

(DB9 Pinouts given)

5+ conductor wire.        (Radio shack, but you might want to consider

just re-using some dead cable you have

laying around.  Like a joystick cable w/ both

ends cut off)

~1.2k Resistor            (Radio Shack)

4 x 22uf 25v caps,        (Radio shack, or Digikey part # P6236-ND)

electrolytic.            (Digikey cost ~ 8 cents each)

ICL232 Cmos<-->TTL

RS232 converter          (Digikey part # ICL232CPE-ND ~ $3 each)

Connections

-----------

Pc Ground -> ICL232 Pin 15

Pc DTR    -> ICL232 Pin 8

Pc TxD    -> ICL232 Pin 13

Pc RxD    -> ICL232 Pin 14

+5v Source -> ICL232 Pin 16   (9v battery works OK for me)

Ground -> ICL232 Pin 15

Atari Ready   (LtBlue) -> ICL232 Pin 16

Atari DataOut (Green)  -> ICL232 Pin 12

Atari Command (Purple) -> ICL232 Pin 9

Atari DataIn  (Orange) -> ICL232 Pin 11

Atari Ground  (Black)  -> ICL232 Pin 15

1.2K Resistor - ICL232 Pin 16 -> ICL232 Pin 11

Cap #1 - (+) ICL232 Pin 1

(-) ICL232 Pin 3

Cap #2 - (+) ICL232 Pin 4

(-) ICL232 Pin 5

Cap #3 - (+) ICL232 Pin 15

(-) ICL232 Pin 6

Cap #4 - (+) ICL232 Pin 2

(-) ICL232 Pin 16

BUILDING THE APE INTERFACE YOURSELF

If you have the technical skill, tools, parts and time available,

building the interface yourself can be fun and will also

save you money! :)

Here is an article posted to comp.sys.atari.8bit on the internet

detailing how to build the low cost version of the interface.

Instructions on building the more reliable 2 chip version of

the interface can also be found on the internet group

comp.sys.atari.8bit.

From: Rick_Michael_Cortese@cup.portal.com

Newsgroups: comp.sys.atari.8bit

Subject: SIO2PC, 7400 version

Date: 1 Aug 1995 14:46:04 -0700

Organization: The Portal System (TM)

All IBM pins refer to a 9 pin serial port.

ATARI 5------:-1  7  14-:--ATARI 10

:-2  4  13-:

IBM pin 2------3  L  12-:

ATARI 7------:-4  S  11 open

:-5  0  10-:--see below

IBM 9----------6  0   9-:

ATARI 4--------7      8--:<--ATARI 3

& IBM 5                 1N914 diode

Pin 10---:-----/\/\/\/----IBM 3

_:_/      1K

/ /\   5.1V zener

:

GROUND

The colors I've found are

ATARI PIN

5   green

3   orange

7   violet   =>cmd

6   black    =>ground

10  blue     =>5V

I've also tried it with a 4.7k pull-up resistor going from ATARI pin 3

to the +5V supply (like Nick did) & it didn't seem to change the

results.

1 2 3 4 5             2 4 6 8 10 12

6 7 8 9             1 3 5 7 9 11 13

View of the rear      View of the SIO

of the DB-9F connector  jack on the Atari

Just a few more notes, I've found pin 4 is a little more reliable of a

ground then pin 6. 4 is logic ground while 6 is shield ground,

shouldn't make a difference, but it has on occasion. One more thing;

since the inputs do not have a pullup or down resistor on them, you

should never leave the IBM side unplugged when you power up your

Atari. It could cause unwanted oscillation in the circuit. If you're

feeling really energetic, go ahead & add maybe a 100k resistor to

ground on the IBM inputs.

Richard: The magic in the thing is two fold. First the zener diode

limits the voltage on the 7400 to 5.1V when the IBM throws +9V at it.

Second, when the IBM puts out a -9V the zener becomes a regular diode

& forward conducts. This limits the voltage on the input to maybe a

-.4V which is just about the tolerance of the chip, but with the 1k

resistor in line it's not a problem.

As far as RS232 specs go, one of my job duties was writing specs.  It

doesn't matter what the spec writters put down on paper, the engineers

& manufactures do what ever the hell they want. In RS232 case even

though the specs call for a minimum of +3V to -3V for logic level

transition every device I've tested over the last 7 years responds to

+5V to 0V. The only thing is you have to protect the device

<zener/resistor> & invert the signal.

BTWI: I'm mostly using 74HC00 now so they should work with unmodified

1200XLs & whoever I was making one for in the UK, I finally finished

the MAX232 version for you last night. I think this means I'm only

running about 2 months behind a normal human being.

BTWII:  Marlin, I really owe you a favor for having your BBS up all

those years. I'd like to say I'll make you a cable, but my time is

really iffy right now. You'd be better off going some other route if

you're in a hurry. On the bright side, I just asked for a

promotion/pay raise at work & if I don't get it I will have all the

time in the world<g>.

Rick